3GPP TSG-RAN WG4 Meeting #107									              R4-2307656

Incheon, Korea, May 22 – 26, 2023

Agenda item:	8.26.2.4

Source:	Apple

Title:	On R18 MUSIM - network B requirements

Document for:	Discussion

# Introduction

Network B requirements were widely discussed during the previous RAN4 meetings. The last agreements can be found in [1], in which there are still some open issues.

In this contribution, we continue discussing the open issues.

# Discussion

The remaining issue is about network B requirements scope and how to define the requirements:

Issue 4-1-2: Network B requirements framework

- Proposals

  - P1: Framework of idle/inactive mode RRM requirements for NR-U can be used as starting point to accommodate MUSIM gap cancellation.

  - P2: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline (Qualcomm xiaomi CMCC ZTE Huawei Ericsson vivo Nokia oppo)

    - P2-1: With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps. FFS other adaptation (Qualcomm vivo Xiaomi Huawei)

    - P2-2: DRX cycle is replaced by max(DRX cycle, MGRP); For MUSIM gaps repetition period = 5120ms, requirements need to be added. (CMCC ZTE)

    - P2-3: When RAN4 reuses existing IDLE mode cell reselection requirements for NW-B, the UE shall request MUSIM gaps with MGRP larger than 160ms when NW-B configures DRX cycle larger than 640ms (Ericsson)

  - P3: The existing UE idle mode measurement and accuracy requirements can be re-used for Network B measurement requirements. (Nokia)

Agreements (1st round):

P2

Candidate options:

Recommendations: Continue discussion P2-1; P2-2 and P2-3 and P3

Technically, NR-U framework can also be reused here with similar justification provided in our contribution regarding network A requirement.

One difference here is that for cell reselection in NW B, longer MGRP can be expected and it is not expected to be dropped due to collision with NW A gap, otherwise all the MUSIM gaps would be dropped. Therefore, it makes sense to us that RAN4 can assume no MUSIM gap is dropped in NW B requirements. With this assumption, we are also fine with P2-1 and P2-2.

Proposal 1: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline, With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps. Besides, for MUSIM gaps repetition period = 5120ms, requirements need to be added.

Regarding test case:

Issue 4-1-5: Network B requirements test case

- Proposals

  - Option 1: Do not define test cases to verify any new requirements in network B. (Qualcomm Huawei MTK vivo)

Agreements: No

Although this can be discussed in performance part, we are fine with option 1.

Proposal 2: test case scope can be discussed in performance part. It is not preferred to define test cases to verify any new requirements in network B.

# Conclusion

In this contribution, we provide further discussion on improvement on network B requirements. After discussion, the following conclusions are provided:

Proposal 1: The measurement/cell reselection requirements in IDLE/inactive mode for NW B could reuse the existing idle/inactive requirements as the baseline, With DRX cycle replaced by max(DRX cycle, MGRP_max), where MGRP_max is the maximum MGRP among all configured MUSIM gaps. Besides, for MUSIM gaps repetition period = 5120ms, requirements need to be added.

Proposal 2: test case scope can be discussed in performance part. It is not preferred to define test cases to verify any new requirements in network B.

# References

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2303310, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2303249, LS on priority levels for MUSIM gaps, vivo