3GPP TSG-RAN WG4 Meeting #107 	R4- 2307682

Incheon, South Korea, 22nd – 26th May, 2023

Source:	Ericsson

Title:	Simulation results for FR2-2 PUSCH demodulation requirements

Agenda item:	5.2.9.6.2

Document for:	Information

# Introduction

During the RAN4#106 meeting, there was a discussion on feasibility of 2 layers (1Tx2Rx, 2Tx2Rx) requirements and new configurations were proposed for further simulations in WF [1].

  - For 1x2 Low, use TDLA30-650 for 100MHz CBW, TDLA10-650 for 400MHz CBW, MCS 16

  - For 2x2 Low, use TDLA30-200 for 100MHz CBW, TDLA10-200 for 400MHz CBW

- Consider MCS16 for the requirements

- Update requirements on CRs with

  - TDLA30-650 - 120 kHz and 100 MHz– add requirements in []

  - TDLA10-650 - 120 kHz and 400 MHz – add requirements in []

  - TDLA10-650 - 480 kHz and 400 MHz – add requirements in []

- MCS 16

  - TDLA30-200 - 120 kHz and 100 MHz

  - TDLA10-200 - 120 kHz and 400 MHz

  - TDLA10-200 - 480 kHz and 400 MHz

- MCS 12

  - TDLA30-650 - 120 kHz and 100 MHz

  - TDLA10-650 - 120 kHz and 400 MHz

  - TDLA10-650 - 480 kHz and 400MHz

Decide between MCS12 and MCS16 for 2x2 requirements based on the results provided

In this contribution, simulation results of MCS16 and MCS12 based on WF are delivered.

# Simulation result

Table 2.1 Simulation parameters for FR2-2 normal PUSCH demodulation

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled and enabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS:3D1S1U, S=10D:2G:2U 480kHz SCS: 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | 120kHz SCS: 100M, 400M full bandwidth 480kHz SCS: 400M |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| Test metric | Normalized throughput | 70% |
| Antenna | Tx and Rx configuration | 2Tx2Rx |
| Channel model |  | 100MHz CBW: TDLA30-650, TDLD30-200; 400MHz CBW: TDLA10-650, TDLD10-200 |
| MCS | 64QAM MCS table index | 12, 16 |
| Phase noise | Model sets in TR38.808 | Set#1 |
| Note1: Companies deliver two sets of ideal simulation results for requirement discussion. Result set#1 is without phase noise and set#2 is with phase noise. • No Tx phase noise is modelled • Rx Phase noise is modelled only to find feasible FRC configuration (i.e. achieve maximum throughput and loss in comparison to scenarios without Rx phase noise is less than 1 dB) • 71 GHz carrier frequency is assumed Note2: CPE compensation is considered. |  |  |

Table 2.2 Simulation results for FR2-2 normal PUSCH demodulation, 120kHz SCS, 100MHz

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | MCS | PN Model | Precoding | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | No CPE | CPE |
| 2 | 2 | Normal | TDLA30-650 | 70% | 12 | Set1 | Disabled | 9.8 | 10.2 |
|  |  |  | TDLA30-200 |  | 16 | Set1 | Disabled | 14.5 | 15.2 |

Table 2.3 Simulation results for FR2-2 normal PUSCH demodulation, 120kHz SCS, 400MHz

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | MCS | PN Model | Precoding | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | No CPE | CPE |
| 2 | 2 | Normal | TDLA10-650 | 70% | 12 | Set1 | Disabled | 10.3 | 10.7 |
|  |  |  | TDLA10-200 |  | 16 | Set1 | Disabled | 16.2 | 16.8 |

Table 2.3 Simulation results for FR2-2 normal PUSCH demodulation, 480kHz SCS, 400MHz

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | MCS | PN Model | Precoding | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | No CPE | CPE |
| 2 | 2 | Normal | TDLA10-650 | 70% | 12 | Set1 | Disabled | 9 | 9.4 |
|  |  |  | TDLA10-650 |  | 16 | Set1 | Disabled | 13.7 | 14.4 |

# Conclusions

Simulation results for FR2-2 PUSCH demodulation are delivered.

# References

[1]	R4-2302971, WF for NR extended to 71GHz PUSCH demodulation, Nokia, RAN4#106