3GPP TSG-RAN WG4 Meeting # 107	R4-2307755

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	TP on antenna observation

Agenda Item:	8.2.2

Document for:	Approval

1 Introduction

In the last meeting RAN4#106bis-e an observation for paper R4-2304633 [1] on multiple and single feed antennas was agreed but was not captured in the TR, the observation was as follows:

Observation 1:	From antenna performance perspective, the difference between single input and dual input of stacked patch multi-band antenna is small, and both types could achieve 4 to 5dBi element gain.

This TP captures this in the TR. The text fits sensibly in clause 5.3.3.

2 TP for TR 38.877

<Start of TP>

5.3.3	Interleaved array structures

If the required multi-band antenna bandwidth is too great to be handled by a single antenna array (for the required steering range) it is also possible to use interleaved array structures such as stacked patches. Using stacked patches allows the different bands to feed different elements which are tuned for the specific band. This resolves some of the physical restrictions on the element size. However, if the arrays are stacked then restrictions still exist on the element separation in the array.

Stacked patches can be designed with either a single dual band input or with separate input ports. As the bands are separated (for example 26GHz and 40GHz), it is possible to implement a diplexer to separate the bands for each of the antenna arrays, if necessary.

From antenna performance perspective, the difference between single input and dual input of stacked patch multi-band antenna is small, and both types could achieve 4 to 5dBi element gain.

Figure 5.3.3-1: Multi-band PA with stacked single band arrays (single and dual input ports)

A number of papers have been identified [19], [20], [21], and [22] which have demonstrated the feasibility of operational stacked patch antennas in the range 26 to 40 GHz

Dual band performance can be achieved by using multiple patches stacked on top of each other. For example, in [20] a simple stack with a high band patch and a low band patch with a single input port was shown. Using these patches, a 2x2 array was demonstrated with acceptable levels of input match, boresight gain isolation and scanning angle. Although of course such small arrays are more suitable for a handset than a BS.

In [22] a larger 8x4 array was demonstrated along with separate single band beamformers. While this is still small for BS purposes, it demonstrates the scalability of such structures and that larger BS-size arrays may be technically feasible to implement.

<End of TP>

References

[1] R4-2304633, FR2-1 multi-band antenna analysis, Murata