3GPP TSG-RAN WG4 Meeting # 107	R4-2307761

# Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Reply LS on interference modelling

Agenda Item:	8.20.2.3

Document for:	Approval

- Introduction

Once again RAN1 send a LS to RAN4 on interference modelling for duplex evolution R4-230 (R1-2304183). In the contribution we discuss the question asked by RAN1 and provide our answers.

- Discussion

RAN1 asks RAN4 to provide feedback on the following questions.

  - Question-1: Whether the above values of A, B, C and D can be used for all the BS classes in FR1? If not, what are the values of A, B, C and D for each of BS classes in FR1?

  - Question-2: Whether fixed noise figure can be used for FR2-1 in RAN1 evaluation? If not, what are the values of A, B, C and D for BS classes in FR2-1?

  - Question-3: The feasibility and applicable scenarios of improved noise figure, e.g., by introducing additional interference reduction techniques like subband filtering.

For the first question, RAN4 provided the values for FR1 MR BS and LA BS in the reply LS R4-2306004. So we can reply that the values for other BS classes in FR1 can be found in the LS R4-2306004.

The second question is about the values for FR2-1. The following feedback was provided in last meeting.

- For FR2-1 BS, the following noise figure model can be used as starting point:

  - The values of A, B, C and D:

    - A = [-58] dBm

    - B = [-40] dBm

    - C = 10dB

    - D = [19] dB

We had the agreement that reference point in NF model refer to input to LNA. To be precise, for FR2 it is referring to the input to LNA array for one RX chain. In our evaluation, the values of A and B can be improved. While we are fine to current values for FR2-1 simulation. So we propose to remove [].

For the third question, similar as the other questions, previous LS R4-2306004 already provides the values of A and B applied to the noise figure model if sub-band filter is adopted. Regarding on the feasibility, as discussed in [2], analogue sub-band filter after LNA can provide the needed suppression for the receiver parts after the filter, and the impacts to RX sensitivity due to insertion loss is negligible.

Based on the discussion above, we propose to give the following reply.

  - Question-1: Whether the above values of A, B, C and D can be used for all the BS classes in FR1? If not, what are the values of A, B, C and D for each of BS classes in FR1?

- Answer from RAN4:

For FR1 different set of values are used for each of BS classes. The values for FR1 MR BS and LA BS can be found in the LS R4-2306004.

  - Question-2: Whether fixed noise figure can be used for FR2-1 in RAN1 evaluation? If not, what are the values of A, B, C and D for BS classes in FR2-1?

- Answer from RAN4:

The noise figure model is also applicable to FR2-1 BS. The values of A, B, C and D for FR2-1 can be found in the LS R4-2306004. RAN4 confirms that the [] can be removed.

  - Question-3: The feasibility and applicable scenarios of improved noise figure, e.g., by introducing additional interference reduction techniques like subband filtering.

- Answer from RAN4:

The previous LS R4-2306004 already provides the values of A and B applied to the noise figure model if sub-band filter is adopted

- Conclusion

In this contribution, we discuss the question asked by RAN1 in R1-2304183 and provide our answers.

  - Question-1: Whether the above values of A, B, C and D can be used for all the BS classes in FR1? If not, what are the values of A, B, C and D for each of BS classes in FR1?

- Answer from RAN4:

For FR1 different set of values are used for each of BS classes. The values for FR1 MR BS and LA BS can be found in the LS R4-2306004.

  - Question-2: Whether fixed noise figure can be used for FR2-1 in RAN1 evaluation? If not, what are the values of A, B, C and D for BS classes in FR2-1?

- Answer from RAN4:

The noise figure model is also applicable to FR2-1 BS. The values of A, B, C and D for FR2-1 can be found in the LS R4-2306004. RAN4 confirms that the [] can be removed.

  - Question-3: The feasibility and applicable scenarios of improved noise figure, e.g., by introducing additional interference reduction techniques like subband filtering.

- Answer from RAN4:

The previous LS R4-2306004 already provides the values of A and B applied to the noise figure model if sub-band filter is adopted

- Reference

[1] R4-2306004, “Reply LS on interference modelling for duplex evolution”, Samsung

[2] R4-2305302, “Discussion on the feasibility of FR1 BS aspects”, Huawei, HiSilicon