3GPP TSG-RAN WG4 Meeting # 107	R4-2307808

# Incheon, Korea (Republic of), May 22 – 26, 2023

Source:	LG Electronics

Title:	Discussion on NR sidelink CA operation

Agenda item:	8.31.3.1

Document for:	Discussion

Introduction

In the last meeting, the scope of RRM requirements for NR SL CA was agreed upon [1] as below.

- RRM requirements specified for LTE V2X CA could be used as baseline for NR SL CA. The requirements could be further discussed based on progress in RAN1/2. - FFS: Interruption requirements of SL carrier addition/release for NR SL CA - FFS: Delay requirements of carrier addition/release for NR SL CA - FFS: Requirements of Selection / Reselection of Synchronization Reference Source under NR SL CA operation - RAN4 to further investigate if there is any NR specific impact for NR SL CA.

In this contribution, based on the above agreements, we provide our views on how to define RRM requirements for NR SL CA.

Discussion

Based on the revised WID [2], the assumptions of NR sidelink CA operation are

- Intra-band contiguous CA

- Same subcarrier spacing among CA carriers

- No primary/secondary carrier differentiation

- Only Mode 2 operation

- Reuse the LTE sidelink CA design

  - Sidelink carrier (re)selection, synchronization of aggregated carriers

Even from the RAN1/2 perspective, there would be no enhancement affecting the RRM requirements.

- Observation: There would be no enhancement for NR SL CA in RAN1/2 affecting the RRM requirements based on WID.

Interruption to WAN due to sidelink carrier aggregation

The interruption on the serving cell/PCell could be introduced due to sidelink component carrier addition/release. In NR sidelink CA operation, only mode 2 operation is applicable. It means that there is no serving cell/PCell based sidelink carrier addition/release control. In the RF session, there is no con-current operation with Uu+NR SL CA in Rel-18. So, interruption requirement to WAN due to sidelink carrier addition/release is not needed.

- Proposal 1: Do not introduce interruption to WAN due to sidelink component carrier addition/release in Rel-18.

Component carrier addition and release delay for sidelink carrier aggregation

In LTE sidelink CA operation, when a UE is configured for sidelink transmission mode 4, the carrier addition/release delay is up to UE implementation. Based on this, in NR sidelink CA operation, the carrier addition/release delay requirement does not need since only mode 2 operation is considered. RAN4 RRM specification can be captured as “For UE configured in sidelink transmission mode 2 in NR SL CA, the delay for component carrier addition and release delay is up to UE implementation”.

- Proposal 2: Do not introduce specific requirements for component carrier addition/release delay in NR sidelink CA operation. If needed, RAN4 can add “For UE configured in sidelink transmission mode 2 in NR SL CA, the delay for component carrier addition and release delay is up to UE implementation” to RRM specification.

Selection/reselection of V2X synchronization reference source

As mentioned above, there would be no enhancement affecting the RRM requirements. Since there might be discussions related to the procedure of selection/reselection of a synchronization reference source for NR SL CA in RAN1/2, RAN4 needs to wait for RAN1/2 progress.

- Proposal 3: Wait for RAN1/2 progress for the procedure of selection/reselection of a synchronization reference source for NR SL CA.

Conclusion

In this contribution, we provide views on NR sidelink CA operation, and, we observe and propose

- Observation: There would be no enhancement for NR SL CA in RAN1/2 affecting the RRM requirements based on WID.

- Proposal 1: Do not introduce interruption to WAN due to sidelink component carrier addition/release in Rel-18.

- Proposal 2: Do not introduce specific requirements for component carrier addition/release delay in NR sidelink CA operation. If needed, RAN4 can add “For UE configured in sidelink transmission mode 2 in NR SL CA, the delay for component carrier addition and release delay is up to UE implementation” to RRM specification.

- Proposal 3: Wait for RAN1/2 progress for the procedure of selection/reselection of a synchronization reference source for NR SL CA.

Reference

[1] R4-2306364, “WF on NR_SL_enh2_part2,” OPPO.

[2] RP-222806, “WID revision: NR sidelink evolution,” OPPO.