3GPP TSG-RAN WG4 Meeting #107	R4-2307901

Incheon, KR, May 22 – May 26, 2023

Source:	LG Electronics

Title:	Discussion on Rel-18 RRM requirement for NES

Agenda item:	8.35.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on RRM requirements for NES.

Discussion

Feasibility of SSB-less operation

In the last meeting following WFs were agreed [1] in RRM session.

< Agreement >Issue 2-1-2: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-1? Online session (Monday April 24, 2023) Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-1 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a < Agreement >Issue 2-1-3: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-2? Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-2 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a < Agreement >Issue 2-1-4: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-2a? Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-2a - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

As described in above, the frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC was discussed in RRM session. In the last meeting, some companies thinks that frequency domain separation can be discussed in RF session. We also think this issue can be discussed in RF session. Since center frequency affects the pathloss between gNB and UE, frequency domain separation also affects the difference of reception power. So, we think that the issues on the difference of the reception power with the FR1 inter-band active serving cell can be discussed together frequency domain separation in RF session.

- Proposal 1: frequency domain separation can be discussed together with the difference of the reception power in RF session.

L1/L3 measurement operation on the SSB-less SCell

Issue 2-4-1: Scenario for L1 measurement operation on the SSB-less SCell apart from SCell activation procedure - Candidate Options: - Option 1: No SSB but with CSI-RS resource for L1 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L1 measurement on the inter-band SSB-less SCell Issue 2-4-2: Scenario for L3 measurement operation on the SSB-less SCell apart from SCell activation procedure - Candidate Options: - Option 1: No SSB but with CSI-RS resource for L3 measurement on the inter-band SSB-less SCell - Option 2: No SSB and No CSI-RS resource for L3 measurement on the inter-band SSB-less SCell

In our understating, the CSI-RS L1 measurement is optional feature. So, RAN4 should study two options can be discussed in parallel.

- Proposal 2: For L1/L3 measurement, following two options can be discussed in parallel.

  - Option 1: No SSB but with CSI-RS resource for L1/L3 measurement on the inter-band SSB-less SCell

  - Option 2: No SSB and No CSI-RS resource for L1/L3 measurement on the inter-band SSB-less SCell

Cell DTX/DRX

< Agreement >Issue 3-1: RRM impacts due to Cell DTX/DRX (obj#2) - RRM impacts due to cell DTX can wait for more inputs from RAN1/2. < Agreement >Issue 3-1: RRM impacts due to Cell DTX/DRX (obj#2) - RRM impacts due to cell DTX can wait for more inputs from RAN1/2.

  - Proposal 3: RRM impacts due to cell DTX/DRX can wait for more inputs from RAN1/2.

Conclusion

In this contribution, we provide following proposal for NES.

- Proposal 1: frequency domain separation can be discussed together with the difference of the reception power in RF session.

- Proposal 2: For L1/L3 measurement, following two options can be discussed in parallel in RRM session.

  - Option 1: No SSB but with CSI-RS resource for L1/L3 measurement on the inter-band SSB-less SCell

  - Option 2: No SSB and No CSI-RS resource for L1/L3 measurement on the inter-band SSB-less SCell

- Proposal 3: RRM impacts due to cell DTX/DRX can wait for more inputs from RAN1/2.

Reference

[1] R4-2306369, WF on RRM requirements for NR network energy saving, Huawei