3GPP TSG-RAN WG4 Meeting # 107                                    R4-2307924

Incheon, Korea, 22th - 26th May 2023

Agenda item:	5.2.5.3

Source: 	ZTE Corporation

Title: 	Disscussion on configuring margin for 1 Rx RedCap UEs

Document for:	Approval

Introduction

In RAN4#106 meeting, RAN4 received a reply LS from RAN2 regarding the issue of configuring margin for 1 Rx RedCap UEs in R2-2211115/R4-2214484. RAN2 shared their views as below [1].

For the list of cell-specific RSRP thresholds provided by RAN4, RAN2’s understanding is that the offset shall not apply to delta thresholds, i.e. s-SearchDeltaP-r16 and s-SearchDeltaP-Stationary-r17. For Rel-16/Rel-17 not-at-cell-edge criterion evaluation, RAN2 understands that the offset should not apply to both cell selection thresholds (i.e. Qrxlevmin and Qqualmin) and not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17) and RAN2 will update specifications to avoid the double offset. RAN2 would also like to inform RAN4 that the following cell-specific RSRP thresholds should be included in the list to apply the offset. - rsrp-ThresholdSSB-SUL - rsrp-ThresholdMsg3

In this paper, we will discuss and modify the list of 1 Rx RedCap offset based on RAN2's comments, including Cell-specific RSRP thresholds and their values.

Discussion

RAN4 has discussed the value of cell-specific RSRP offset at the 104bis-e meeting and reached following consensus in the WF [2]. The specific information is copied below:

Cell-specific RSRP thresholds and their values:

| No | Type of threshold | Threshold [dB] |
| --- | --- | --- |
| 1 | rsrp-ThresholdSSB, | + 1 |
| 2 | msgA-RSRP-ThresholdSSB, | + 1 |
| 3 | msgA-RSRP-Threshold | + 1 |
| 4 | absThreshSS-BlocksConsolidation | + 1 |
| 5 | sdt-RSRP-Threshold | + 1 |
| 6 | s-SearchDeltaP-r16 | - 1 |
| 7 | s-SearchDeltaP-Stationary-r17 | - 1 |
| 8 | s-SearchThresholdP-r16 | + 1 |
| 9 | s-SearchThresholdQ-r16 | + 1 |
| 10 | s-SearchThresholdP2-r17 | + 1 |
| 11 | s-SearchThresholdQ2-r17 | + 1 |
| 12 | Qrxlevmin and Qqualmin | - 1 |

According to RAN2's response, the discussion on Cell-specific RSRP thresholds and their values of 1 Rx RedCap can be divided into three parts, including

1. Do delta thresholds need to be deleted?

2. How to avoid double offset (cell selection thresholds and not at cell edge thresholds)?

3. Addition of cell specific RSRP thresholds.

Regarding the first issue, an agreement was reached in the RAN4 #106 meeting, and RAN4 referred to RAN2's suggestion that the offset should not apply to delta thresholds. Consequently, remove the offset of  s-SearchDeltaP-r16 and s-SearchDeltaP-Stationary-r17 from the list.

Offset for low mobility and stationary relaxed measurement criteria thresholds - Agreements Remove the offset from s-SearchDeltaP-r16 and s-SearchDeltaP-Stationary-r17.

For the second issue, according to RAN2's prompt, it may cause a double offset if the offset apply to both cell selection thresholds (i.e. Qrxlevmin and Qqualmin) and not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17), resulting in ineffective consideration of the offset.

In the existing agreements, RAN4 suggests that a “-1 dB” offset should be applied to the cell selection threshold (i.e. Qrxlevmin and Qqualmin), which means that the threshold for 1Rx RedCap UE cell selection should be lowered.  Because under the same conditions, the number of beams that meet the threshold measured by 1Rx UE will be smaller compared to 2RX UE, “-1dB” offset can compensate for the decrease in cell coverage caused by the lower accuracy of 1Rx UE, thereby achieving network coverage. Meanwhile, for non cell edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17, and s-SearchThresholdQ2-r17), RAN4 suggests that an offset of “+1dB” should be applied to them, that is, the cell search threshold of 1Rx RedCap UE should be increased. Because the measurement results of 1Rx UE will have a greater measurement error compared to 2Rx UE, using a positive offset value is beneficial in avoiding unnecessary relaxation of 1Rx UE due to overestimation of RSRP.

However, RAN2 mentions that offset should not be applied to both the cell selection threshold and the cell edge threshold, as this may result in double offset. According to the definition in TS 38.133, if these two types of offsets are applied simultaneously, they may cancel out each other, resulting in the disappearance of the offset advantage we mentioned earlier. The specific expression is reproduced as follows.

5.2.4.9.2 Relaxed measurement criterion for UE not at cell edge The relaxed measurement criterion for UE not at cell edge is fulfilled when: - Srxlev > SSearchThresholdP, and, - Squal > SSearchThresholdQ, if SSearchThresholdQ is configured, Where: - Srxlev = current Srxlev value of the serving cell (dB). - - Squal = current Squal value of the serving cell (dB). 5.2.3.2 Cell selection criterion The cell selection criterion S is fulfilled when: Srxlev > 0 AND Squal > 0 where: Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

We can observe that there is a cancellation phenomenon for the relaxed measurement criterion for UE not at cell edge. For example, when comparing the values of Srxlev and SSearchThreshold, compared to 2Rx UE, the value of SSearchThresholdP is positively offset by one, and the value of Qrxlevmin is negatively offset by one. From a mathematical calculation perspective, both sides are “+1” at the same time and it does not affect the final result. To address the double offset issue, we currently have the following options based on WF [2].

Offset for cell selection and not-at-cell-edge criteria thresholds No consensus on whether to remove the offset from all not-at-cell-edge criteria thresholds. Following options were discussed. - Option 1: Remove the offset from all not-at-cell-edge thresholds. - Option 2: RAN4 to keep the current offset value determined in previous RAN4 meetings. - Option 2a: Keep the offset and change it to +2dB for all not-at-cell-edge thresholds. - Option 3: - Keep current RAN4 agreed offsets for SearchThresholdP-r16 and s-SearchThresholdP2-r17 - Remove the offset from the thresholds SearchThresholdQ-r16 and s-SearchThresholdQ2-r17. - Option 4: Apply “0 dB” 1 Rx. RedCap offset to Qrxlevmin and Qqualmin

To solve this issue, we can consider the modification to keep existing RAN4 decision on 1 Rx offset in cell selection or measurement relaxation. From a mathematical calculation perspective, options 1, 2a, and 4 can all solve the double offset problem mentioned by RAN2. But we prefer option 1 that remove the offset from all not-at-cell-edge thresholds directly. This is consistent with RAN2's suggestion that the offset should not be applied to both cell selection thresholds and not at cell edge thresholds.

Proposal 1: Remove the offset from all not-at-cell-edge thresholds to avoid the double offset.

Regarding the third issue, RAN2 suggests adding rsrp-ThresholdSSB-SUL and rsrp-ThresholdMsg3 in cell-specific RSRP thresholds. In our understanding, rsrp-ThresholdMsg3 is the threshold used for MSG3 duplication, which aims to expand coverage. Therefore, for 1 RX RedCap, we recommend adding and tend to apply “+1 dB” offset to  rsrp-ThresholdMsg3. Meanwhile, we are open to rsrp-ThresholdSSB-SUL.

Proposal 2: The offset of +1 dB should be applied for rsrp-ThresholdMsg3.

Conclusions

In this contribution, we have discussed the list of 1 Rx RedCap offset based on RAN2's comments, including Cell-specific RSRP thresholds and their values. Meanwhile, we have expressed the modification method  based on our understanding. Based on the discussion, we have made following proposals:

Proposal 1: Remove the offset from all not-at-cell-edge thresholds to avoid the double offset.

Proposal 2: The offset of +1 dB should be applied for rsrp-ThresholdMsg3.

References

- R2-2213069, Reply LS on configuring margin for 1 Rx RedCap UEs

- R4-2217214, WF on RedCap RRM requirements