3GPP TSG-RAN WG4 Meeting #107	R4-2307932

# Incheon, KR, May 22 – May 26, 2023

Title:	Further discussion on FR2 Multi-RX DL 2AoA spherical coverage

Source:	Samsung

Agenda item:	8.8.2.2

Document for:	Discussion

1.	Introduction

In RAN4#106 meeting, most simulation assumptions were agreed and captured in the WF [1]; in RAN4#106bis-e meeting, the discussion based on simulation results were becoming more and more convergent. However, the simulation results among companies are still quite different, one dedicated clause was captured in the WF [2] to align the simulation procedure:

2.10 Further alignment of simulation procedure

Motivation: Simulation results from companies seem quite diverged. Companies are encouraged to propose further candidates for alignment, like:

- procedures and assumptions on how SINR is calculated

- how electromagnetic simulation data is used

- simulation steps details and so on.

Agreement:

The following procedure can be used as a guideline for simulation:

1. For one UE implementation

2. For one UE orientation

3. Run EM simulation to obtain per-beam antenna gain patterns

      - Constant step size is suggested <= 5°

      - Performance difference between V/H element can be considered

      - Normalize antenna gain to align with the gain drop between peak EIS and spherical coverage in current spec (option 1 in 2.6)

        - Other calibration method that described in 2.6 also can be used.

4. For one angular separation

5. For one test grid point in 3D scan

- Select beam based on RSRP (or SINR)

5.1 Calculate SINR of AoA+ and AoA- respectively

- SINR = P_signal/(Noise + P_interf)

Where the P_signal is the power of wanted signal and the P_interf is the power of interference, Noise(dBm)= -174 +10*log10(CBW) +NF, CBW is channel bandwidth, NF =10

  - If SINR>=-1, PASS, otherwise, FAIL

5.3a OR combining the results of AoA+ and AoA-

5.3b No logic combination of the results of AoA+ and AoA-, but treat them as two separate points (e.g., arithmetic mean)

- Other methods for +/- offset data are not precluded

- Companies are encouraging to provide analysis on the pros and cons for each “combination” method

5.4 Add weighting (sin θ or Clenshaw-Curtis Quadrature)

6. Repeat for other test grid point

7. Calculate the spherical coverage percentage

8. Repeat for other angular separation

9. Repeat for other UE orientations

10. Repeat for other UE implementations

In above aligned simulation procedure, there are still some open items, including beam selection criteria (RSRP vs SINR), data combining and weighting, etc. And the WF [2] also captured open items including UE orientation and angular separation, DL power requirements, etc. In this contribution, we further discuss the remaining open issues in simulation and requirements.

2. 	Discussion

2.1	beam pairing criteria (RSRP vs SINR)

In last meeting the agreed baseline assumption for beam pairing criteria is based on RSRP: “UE assigns ‘first’ module to track TRP that yields highest RSRP among all combinations of modules and TRPs. The best of the other modules is assigned to track the other TRP”. Meanwhile the SINR criteria is still on the table.

According to the agreement in RRM session [3], Rel-17 group-based reporting was agreed as prerequisite to define requirements for simultaneous reception. Note that Rel-17 group-based reporting is only based on RSRP criteria.

#### Issue 1-2-7: Necessity of group-based beam reporting for simultaneous reception - Rel-17 group-based reporting is used as a prerequisite to define requirement for simultaneous reception - Note: Simultaneous reception term above includes Data/Data, RS/RS and Data/RS simultaneous reception cases.

In our view, the criteria should be aligned between RF and RRM sessions. So it is proposed to adopt RSRP criteria as beam paring assumption for simulation and RF requirements.

Proposal 1:	it is proposed to adopt RSRP criteria as beam paring assumption for simulation and RF requirements.

2.2	data combining and weighting

According to previous agreement, there are two separate tests for each test grid point in 3D scan (UE rotation), i.e., AoA+ pair and AoA- pair respectively. Then there will be issue how to handle the two data for the same test point. Existing options including OR combing, and No logic combination but treating them as two separate points (e.g., arithmetic mean which is actually Average combing).

On top of our previous simulation [4] which was based on OR combining, we further simulated with Average combining shown in Figure 2.2-1.

Figure 2.2-1. Simulation results with Average combining

It can be observed that for typical “side + back” two modules implementation, the simulated 2AoA spherical coverage percentage is only 15% in best case (best UE orientation at best angular separation).

Observation 1:	for typical “side + back” two modules implementation, the simulated 2AoA spherical coverage percentage is only 15% in best case (best UE orientation at best angular separation).

One more issue identified for Average combining is the big performance difference between 150° and 180° angular separation. Due to testability limitation, 180° angular separation may not be testable and potentially the measured performance at 150° angular separation may be used to approaching the performance at 180° angular separation.

Figure 2.2-2 shows the comparison between OR combing and Average combing. With OR combining, the performance is similar between 150° and 180° angular separation, but with Average combing, there is big performance gap obviously between 150° and 180° angular separation.

Figure 2.2-2. Simulation results comparison: OR combining (left) vs Average combining (right)

Observation 2:	OR combing shows similar performance between 150° and 180° angular separation, but Average combing shows big performance gap between 150° and 180° angular separation.

Previous Qualcomm contribution [5] also shows the same trend, reproduced in Figure 2.2-3

Figure 2.3.2.2-1: OR combining method comparison

Figure 2.3.2.2-2: mean combining method comparison

Figure 2.2-3. OR combining (left) vs Average combining (right) in [5]

Given 180° angular separation may not be testable, it is important that the adopted data combing method should show similar performance between 150° and 180° angular separation.

Proposal 2:	the adopted data combing method should show similar performance between 150° and 180° angular separation given 180° angular separation may not be testable.

Moreover, the agreed Pass/Fail criterion is “min[SINR_AoA1, SINR_AoA2] >= -1dB” which is stringent, and OR combining can be considered as a balance under test constraints.

Based on above, we prefer to adopt OR combining.

Proposal 3:	adopt OR combing for the results of AoA+ and AoA-.

About weighting, above simulations are all based on sinθ. In last meeting, options including sinθ1.sinθ2 and Clenshaw Curtis weighting were also discussed. Though our simulation with sinθ1.sinθ2 weighting shows some difference (depending on UE implementation) compared with sinθ, generally speaking the difference is not big. The main problem is that sinθ1.sinθ2 weighting is not applicable for OR combining.

Observation 3:	sinθ1.sinθ2 weighting is not applicable for OR combining.

We also simulated with Clenshaw Curtis weighting. The simulation results are almost the same as that of sinθ weighting. It is aligned with expectation since 2deg constant step sized is used in our simulation. This comparison between Clenshaw Curtis weighting and sinθ weighting on the other hand provides a method to verify if the constant step size is fine enough. In simulation we should make sure the simulation results are almost the same between Clenshaw Curtis weighting and sinθ weighting with fine enough grid. Potential difference between Clenshaw Curtis weighting and sinθ weighting with large measurement grid step size in practical test should be discussed in test SI.

Proposal 4:	Clenshaw Curtis weighting and sinθ weighting should show almost the same simulation results in simulation for requirements derivation.

2.3	UE orientation and angular separation

In last meeting there were extensive discussions on 3-axis positioner and the conclusion is that the enhanced positioner is not adopted in testing and the requirement is agreed to be framed based on the previously agreed legacy positioner. With legacy 2-axis positioner, RAN4 confirmed that different measurement results for different UE orientations occur for 2AoA test. Options regarding UE orientation and angular separation are as following:

- The requirement applies for one of the FFS options below: Option 1: one or more UE-declared orientations in the positioner - FFS if UE orientation is packaged with an AoA separation angle as a pair. Option 2: one or more standard-specified UE orientations in the positioner. Option 3: the requirement applies for two fixed AoA configurations, the UE orientation can be declared (no capability needed) for each of these two directions.

Simulation results [4] show obviously different performance for different UE implementations, Figure 2.2-1 also shows the same trend. Based on such fact, declared UE orientation and declared angular separation is the best choice.

In FR2 MIMO OTA which is also multi-probe test system, standardized UE orientation is specified. Because the multiple probes in FR2 MIMO OTA are close to each other within a small angular spread, and the DL power for each probe are quite different, consequently the MIMO OTA performance for different UE orientation is not very obvious compared with FR2 Multi-Rx DL 2AoA spherical coverage where large angular separation and equal DL power between probes are there.

Observation 4:	standardized UE orientation can be specified for FR2 MIMO OTA because the multiple probes in FR2 MIMO OTA are close to each other within a small angular spread, and the DL power for each probe are quite different

So standard-specified UE orientation can be applied for FR2 MIMO OTA but could not be applied to FR2 Multi-Rx DL 2AoA spherical coverage.

Based on above discussion, declared UE orientation and declared angular separation is preferred to accommodate different UE implementations

Proposal 5:	adopt one UE orientation and one angular separation based on UE declaration.

2.4	DL power condition

The requirement concept (go or no-go) has been officially confirmed in last meeting, i.e., the UE can achieve EIS performance not worse than YdBm on the test point pair (corresponding to 2 AoAs) and the ratio of qualified test points over the whole sphere is M%. Options for the DL power YdBm are as following and the starting point is agreed as “Both DL powers are same as the legacy spherical coverage EIS requirement”:

Option 1: One DL power is higher that legacy spherical coverage requirement: - Use different DL power levels for the two AoAs. One of the DL power levels should be equal to or close to the legacy spherical coverage EIS requirement level. ∆R2TRP needs to be defined for the second DL direction for small AoA separation (<90 degree) and for large AoA separation (≥90 degree) the effect of inter-beam interference is negligible, ∆R2TRP need not be defined. (R4-2304121). Option 2: Both DL powers are higher that legacy spherical coverage requirement: - The relaxation definition for 2AoAs should include both of the relaxation of the fixed DL power level based on the peak EIS of legacy RX and the relaxation of coverage fraction to count the qualified test point. (R4-2305071) - consider an additional relaxation factor due to DL polarization scheme change compared with legacy scheme (R4-2304824). Option 3: Both DL powers are same as the legacy spherical coverage requirement: - Re-use the legacy spherical coverage receiver sensitivity level (single probe) as the DL power to set the core requirement, and then derive the minimum coverage percentile requirement for multi-Rx chain DL reception accordingly. (R4-2305750) Option 4: Other Agreement: Both DL powers are same as the legacy spherical coverage EIS requirement as starting point.

Note that Option 1 configures different DL power for different TRPs. Unequal DL power between TRPs will break the symmetry of 2AoA measurement even both AoA+ pair and AoA- pair are already measured. In our understanding, no matter whether the final DL power is the same as the legacy spherical coverage EIS requirement or not, one principle should be followed, i.e., equal DL power applied to both TRPs.

Observation 5:	Unequal DL power between TRPs will break the symmetry of 2AoA measurement even both AoA+ pair and AoA- pair are already measured

Proposal 6:	equal DL power shall be applied to both TRPs.

2.5	verification aspects

In last meeting channel BW was discussed and the agreement is as following:

Agreement: - Core requirement for multiRx will be defined for all supported channel bandwidths. - Single CBW is selected for core requirement verification, FFS which CBW is chosen

Regarding which CBW is chosen for verification, in our view the most popular BW for FR2 UEs should be selected. 400MHz is optional CBW and can be ruled out. Among 50MHz, 100MHz and 200MHz, 100MHz CBW is supported and measured in most cases. So 100MHz is preferred to be chosen for verification.

Proposal 7:	100MHz CBW shall be chosen for Multi-Rx requirements verification.

Legacy spherical coverage requirement are required to be verified under normal temperature condition. When coming to spherical coverage requirement of multiple AoAs, due to more complicated test system, it is not only difficult but also not necessary to verify the 2AoA spherical coverage requirements with ETC conditions. So it is proposed to resume the NTC test condition as legacy.

Proposal 8:	2AoA spherical coverage requirements shall be verified under normal temperature condition.

2.6	simulation alignment

Simulation alignment is important for achieving consensus on final requirements. It is normal that detailed data may be different for simulation results of different companies. However, attentions should be paid if the simulation results curves show obviously different trend among companies for the same UE implementation. For example, most companies have simulated the back-to-back two modules UE implementation, some simulation results show monotonic curve in terms of 2AoA spherical coverage percentage versus angular separation (i.e., bad percentage for <90° angular separation, and good percentage for >90° angular separation, refer to Figure 2.2-2), while some simulation results show obviously different curved trend. It would be better to align the curve trend before discussing final requirement value.

Proposal 9:	For the same UE implementation, it would be better to align companies’ simulation results curve trend in terms of 2AoA spherical coverage percentage versus angular separation before discussing final requirement value.

2.7	other power classes than PC3

Currently this WI is focusing on deriving requirements for simultaneous multi-RX DL for PC3 handheld UE. The discussion in other WI e.g. PC6 HST UE shows that non-handheld UE maybe could not reuse the requirements for PC3 []. For other power classes like FWA and so on, it may or may not reuse the requirements of PC3. In order to be future proof, it is proposed to specify 2AoA spherical coverage per power classes. In future if it is identified that requirements could be reused, it is still doable to direct the requirements to PC3’s.

Proposal 10:	It is proposed to specify 2AoA spherical coverage requirements per power classes.

2.8	specification impact

Multi-RX DL is a new feature and there would be issue which clause the multi-RX requirements would be written into. As the simultaneous multi-panel operation is still single carrier, so one possibility is to capture the multi-RX requirements into clause 7.3.

On the other hand, UL MIMO is also single carrier but UL MIMO requirements are specified into clause with dedicated suffix D. Similarly, it is also an option to capture the multi-RX requirement into clause 7.3E, i.e., creating a new suffix E for multiple angle of arrival:

There is another possibility of creating a new sub-clause 7.11 for reception of multiple angle of arrival.

Observation 6: there might be 3 options for multi-RX DL requirements to be captured as following

Option 1: existing single carrier clause 7.3

Option 2: new clause with dedicated suffix 7.3E

Option 3: new sub-clause 7.11

Option 1 will make single carrier requirements much more complicated. The issue of option 3 is that the structure of specification would be diversified. Especially, in case there would be other reception requirements than 2AoA spherical coverage in the future, the 7.11 would have repeated sub-clauses for other reception requirements which are supposed to be in 7.4/7.5/7.6/etc respectively.

As we can see, though sensitivity test is not used but instead checking throughput directly for 2AoA spherical coverage, it is still reference sensitivity concept as the required downlink power is still associated with 95% of max throughput. So the best choice would be to capture multi-RX requirements with option 2, i.e., creating a new suffix for multiple angle of arrival.

Proposal 11:	It is proposed to create new suffix E for multiple angle of arrival.

The specification could be as following

3. 	Conclusion

Proposal 1:	it is proposed to adopt RSRP criteria as beam paring assumption for simulation and RF requirements.

Observation 1:	for typical “side + back” two modules implementation, the simulated 2AoA spherical coverage percentage is only 15% in best case (best UE orientation at best angular separation).

Observation 2:	OR combing shows similar performance between 150° and 180° angular separation, but Average combing shows big performance gap between 150° and 180° angular separation.

Proposal 2:	the adopted data combing method should show similar performance between 150° and 180° angular separation given 180° angular separation may not be testable.

Proposal 3:	adopt OR combing for the results of AoA+ and AoA-.

Observation 3:	sinθ1.sinθ2 weighting is not applicable for OR combining.

Proposal 4:	Clenshaw Curtis weighting and sinθ weighting should show almost the same simulation results in simulation for requirements derivation.

Observation 4:	standardized UE orientation can be specified for FR2 MIMO OTA because the multiple probes in FR2 MIMO OTA are close to each other within a small angular spread, and the DL power for each probe are quite different

Proposal 5:	adopt one UE orientation and one angular separation based on UE declaration.

Observation 5:	Unequal DL power between TRPs will break the symmetry of 2AoA measurement even both AoA+ pair and AoA- pair are already measured

Proposal 6:	equal DL power shall be applied to both TRPs.

Proposal 7:	100MHz CBW shall be chosen for Multi-Rx requirements verification.

Proposal 8:	2AoA spherical coverage requirements shall be verified under normal temperature condition.

Proposal 9:	For the same UE implementation, it would be better to align companies’ simulation results curve trend in terms of 2AoA spherical coverage percentage versus angular separation before discussing final requirement value.

Proposal 10:	It is proposed to specify 2AoA spherical coverage requirements per power classes.

Observation 6: there might be 3 options for multi-RX DL requirements to be captured as following

Option 1: existing single carrier clause 7.3

Option 2: new clause with dedicated suffix 7.3E

Option 3: new sub-clause 7.11

Proposal 11:	It is proposed to create new suffix E for multiple angle of arrival.

4. 	References

- R4-2303078    “WF on NR FR2 multi-Rx chain DL reception”, Samsung, Apple

- R4-2306604    “WF on FR2-1 UERF requirements for FR2 multi-Rx”, vivo, Apple

- R4-2303302    “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)”, vivo

- R4-2304824    “FR2 Multi-RX 2AoA spherical coverage simulation and requirements”, Samsung

- R4-2304603    “On UE RF requirements for 2AoA FR2 DL MIMO”, Qualcomm

- R4-2306606     “WF on NR FR2 HST UE RF”, Samsung.