3GPP TSG-RAN WG4 Meeting #107	R4-2307934

# Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on FR2 beam correspondence requirement in initial access

Source:	Samsung

Agenda item:	8.7.3.1

Document for:	Discussion

1.	Introduction

In RAN4#106bis-e meeting, there were still controversial views on beam correspondence requirements for msg1. There are concerns on re-use the same requirements as connected mode without restriction of narrow beam, and companies also identified that the Rel-18 beam correspondence requirements are related with BC tolerance, UE capability and release independence issues.

In this contribution, we further discuss above issues and observe the necessity to specify Rel-18 beam correspondence requirements together with UE capability, release independence and test condition issues as a package.

2. 	Discussion

For the convenience of discussion, the WF [1] of last meeting are reproduced as below:

- Sub-topic 1-1 Minimum peak EIRP and spherical coverage for msg1 Issue 1-1-1: minimum peak EIRP for msg1 + spherical coverage package - Proposals - Option 1: Do not specify the min peak EIRP requirements but Same spherical coverage as RRC_Connected mode - Option 2: Lower than the min peak EIRP of RRC_Connected mode + Same spherical coverage as RRC_Connected mode - Option 3: Same as min peak EIRP of RRC_Connected mode + Same spherical coverage as RRC_Connected mode - Option 4: For beam correspondence in initial access, reuse the same beam lock mode and the same beam type assumption as that of connected mode beam correspondence. And accordingly EIRP requirements and RS side condition can also be reused. Way forward/Agreements: Further discuss in RAN4#107 meeting. Implication between Issue 1-1/1-5/1-6 need to be investigated. - Companies are encouraged to provide justification.

For peak EIRP and EIRP spherical coverage of msg1, Option 1 to 3 are all reusing same EIRP spherical coverage requirements as connected state without mentioning beam type assumption. If the requirements would accommodate both fine beam and rough beam, we’d like to highlight that previous RAN4 assumption is that 7dB gain difference between fine beam and rough beam applies at both beam peak direction and 50%-tile direction, refer to Annex B of TS38.133:

B.2.1.3.1 Minimum SSB_RP values for Rx Beam Peak angle of arrival Minimum SSB_RP values in Tables B.2.2-2 and B.2.3-2 are based on Reference sensitivity for the Operating band and for the UE Power class, and taking a baseline of UE Power class 3 in Band n260 with 50 MHz channel bandwidth. Minimum SSB_RP = Reference sensitivity PC3, n260, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ΣMBP where: Reference sensitivity PC3, n260, 50MHz is the reference sensitivity value in dBm specified for Power Class 3 in Band n260 for 50 MHz Channel bandwidth in Table 7.3.2.3-1 of TS 38.101-2 [19]. Y is the gain difference between fine and rough beams, and is defined in Table B.2.1.3.1-1. Table B.2.1.3.1-1: Gain difference Y between fine and rough beams, Rx beam peak direction Value “Y” in dB, for each UE Power class. 1; 2; 3; 4. FFS; 9.0; 7.0; FFS. …… ##### B.2.1.3.2 Minimum SSB_RP values for angle of arrival within Spherical coverage Minimum SSB_RP values in Tables B.2.2-2 and B.2.3-2 are based on EIS Spherical coverage for the Operating band and for the UE Power class, and taking a baseline of UE Power class 3 in Band n260 with 50 MHz channel bandwidth. Minimum SSB_RP = EIS spherical coverage PC3, n260, 50MHz +Z -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ΣMBS where: EIS spherical coverage PC3, n260, 50MHz is the EIS spherical coverage value in dBm specified for Power Class 3 in Band n260 for 50MHz Channel bandwidth in TS 38.101-2 [19] Table 7.3.4.3-1. Z is the gain difference between fine and rough beams, and is defined in Table B.2.1.3.2-1. Table B.2.1.3.2-1: Gain difference Z between fine and rough beams, Spherical coverage directions Value “Z” in dB, for each UE Power class. 1; 2; 3; 4. FFS; 9.0; 7.0; FFS.

Observation 1:	Previously RAN4 assumes 7dB gain difference between fine and rough beams for both beam peak directions and spherical coverage directions.

There were contributions to provide justification that at 50%-tile direction the gain difference between rough beam and fine beam can be small according to simulation. However, if the rough beam is achieved by switching off other elements (e.g., from 4 active elements to 1 active element), that means EIRP loss up to 12dB at both beam peak direction and 50%-tile direction; If the rough beam is achieved by switching off half elements, that means 6dB EIRP loss at both beam peak direction an 50%-tile direction.

Observation 2:	if rough beam is achieved by switching off elements, same EIRP loss occurs at both beam peak direction and 50%-tile direction, which leading to relaxation compared with requirements of connected state

If rough beam is achieved by all elements active, then the simulation should also be done with such assumption. In our view, if RAN4 would reuse the same spherical coverage requirements for initial access as that of connected state, there should be exact beam assumption how it is achieved.

Proposal 1:	if RAN4 would reuse the same spherical coverage requirements for initial access as that of connected state, there should be exact beam assumption how it is achieved.

If looking into RAN1 assumption on PRACH power control in TS38.213, the UE calculated transmission power based on PRACH target reception power and path loss could be even higher than Pcmax. That implicitly indicates that RAN1 assumes UE could use fine beam for PRACH in initial access.

7.4 Physical random access channel A UE determines a transmission power for a physical random access channel (PRACH), , on active UL BWP of carrier of serving cell based on DL RS for serving cell in transmission occasion as [dBm], where is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3] for carrier of serving cell within transmission occasion , is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers [11, TS 38.321] for the active UL BWP of carrier of serving cell , and is a pathloss for the active UL BWP of carrier based on the DL RS associated with the PRACH transmission on the active DL BWP of serving cell and calculated by the UE in dB as referenceSignalPower – higher layer filtered RSRP in dBm, where RSRP is defined in [7, TS 38.215] and the higher layer filter configuration is defined in [12, TS 38.331]. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in clause 13, the UE determines based on the SS/PBCH block associated with the PRACH transmission.

Observation 3:	RAN1 assumes any UE could use fine beam for PRACH transmission in initial access

Option 1 is considered as a compromise, i.e., reuse same spherical coverage requirements of connected state but not specifying peak EIRP requirements. However, in existing specifications since Rel-15 there is already MPR requirements for PRACH. This issue has been discussed in previous meetings [2] [3], and most companies prefer to come back after Rel-18 beam correspondence requirements for initial access are concluded. If Rel-18 beam correspondence requirements for initial access will go with option 1, i.e., no peak EIRP requirements for PRACH, then all existing MPR requirements for PRACH in existing specifications should be removed for all releases.

Proposal 2:	if RAN4 would not specify peak EIRP requirements for msg1 beam correspondence, all existing MPR requirements for PRACH in existing specifications should be removed for all releases.

Previous RAN4 discussion also indicates that msg1 BC requirements are also related with BC tolerance, UE capability and release independence issues, as the new Rel-18 BC requirements may require UE performance in idle state better than that of legacy UE in connected state for UE not supporting beamCorrespondenceWithoutUL-BeamSweeping

- Sub-topic 1-5 BC tolerance - Proposals - Option 1: BC tolerance is applicable. - Option 1a: The same as Rel-16. - Option 1b: New tolerance is introduced. - Option 1c: it’s suggested to study the tolerance requirements especially for UE supporting BC with beam sweeping in RRC_CONNECTED. - Option 2: BC tolerance is not applicable. - Option 3: If these two types of UE capability will be reused in R18, which means UE supporting BC without beam sweeping has better performance, tolerance requirement is needed for UE with relatively bad BC performance like R16 UE with beam sweeping. If there is only a kind of UE compared with R16, tolerance requirement is no need. Way forward/Agreements: - BC tolerance requirement defined in Rel-15 is not applicable to initial access and RRC_INACTIVE state, according to WID “Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping.” - Encourage companies to provide justification if relaxation is needed in IA for UE not supporting BC without UL beam sweeping. - Sub-topic 1-6 UE capability - Proposals - Option 1: Only the UE support both beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered can support msg1 beam correspondence. - Option 2: For supporting UE beam correspondence requirements for RRC_INACTIVE and initial access UE needs to support both beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 UE capabilities. - Option 3: RAN4 specs to clarify that Rel-18 or newer UEs supporting beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 is considered as a Msg1 beam correspondence capable UE. Way forward/Agreements: - Follow previous RAN4 decision that new UE capability is not introduced for initial access. - FFS for the options above.

Note that beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 are both optional UE capability and they are capability for connected state. Now it is not very clear whether Rel-18 newly defined msg1 beam correspondence requirements are mandatory or optional for all UE.

If it is mandatory for all UE regardless of UE capability, then Rel-18 msg1 beam correspondence requirements should not be release independent to previous releases.

Proposal 3:	RAN4 to further discuss if Rel-18 msg1 beam correspondence requirements are mandatory or optional. If it is mandatory for all UE regardless of UE capability, then it should not be release independent to previous releases.

If msg1 beam correspondence requirements are mandatory only for UE supporting beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16, then the release independence issue depends on what the final requirement is. If final requirements are the same as that of connected state, but the test condition is not equivalent (UBF enabled, transmission period at max power no less than 1ms, etc.), then Rel-18 msg1 beam correspondence requirements should not be release independent to previous releases either.

Observation 4:	If final requirements are the same as that of connected state, but the test condition is not equivalent (UBF enabled, transmission period at max power no less than 1ms, etc.), then Rel-18 msg1 beam correspondence requirements should not be release independent to previous releases.

To enable release independence for UE supporting beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16, it is critical to guarantee the test conditions are equivalent as that of connected state, including enabling beam lock function, holding RAR to enable power ramping and to maintain max power transmission period no less than 1ms, etc.

Proposal 4:	it is proposed to guarantee the test conditions are equivalent as that of connected state, including enabling beam lock function, holding RAR to enable power ramping and to maintain max power transmission period no less than 1ms, etc.

Based on above discussion, it can be seen that msg1 beam correspondence requirements are correlated with UE capability, release independence and test conditions. It is proposed to discuss the msg1 BC requirements together with UE capability, release independence and test conditions as a package.

Proposal 5:	It is proposed to discuss the msg1 BC requirements together with UE capability, release independence and test conditions as a package.

3. 	Conclusion

Observation 1:	Previously RAN4 assumes 7dB gain difference between fine and rough beams for both beam peak directions and spherical coverage directions.

Observation 2:	if rough beam is achieved by switching off elements, same EIRP loss occurs at both beam peak direction and 50%-tile direction, which leading to relaxation compared with requirements of connected state

Proposal 1:	if RAN4 would reuse the same spherical coverage requirements for initial access as that of connected state, there should be exact beam assumption how it is achieved.

Observation 3:	RAN1 assumes any UE could use fine beam for PRACH transmission in initial access

Proposal 2:	if RAN4 would not specify peak EIRP requirements for msg1 beam correspondence, all existing MPR requirements for PRACH in existing specifications should be removed for all releases.

Proposal 3:	RAN4 to further discuss if Rel-18 msg1 beam correspondence requirements are mandatory or optional. If it is mandatory for all UE regardless of UE capability, then it should not be release independent to previous releases.

Observation 4:	If final requirements are the same as that of connected state, but the test condition is not equivalent (UBF enabled, transmission period at max power no less than 1ms, etc.), then Rel-18 msg1 beam correspondence requirements should not be release independent to previous releases.

Proposal 4:	it is proposed to guarantee the test conditions are equivalent as that of connected state, including enabling beam lock function, holding RAR to enable power ramping and to maintain max power transmission period no less than 1ms, etc.

Proposal 5:	It is proposed to discuss the msg1 BC requirements together with UE capability, release independence and test conditions as a package.

4. 	References

- R4-2306600    “WF on beam correspondence in initial access and RRC_INACTIVE”, Nokia

- R4-2218558    “FR2 beam correspondence requirement for initial access”, Samsung

- R4-2300990    “Discussion on revisiting PRACH requirements since Rel-15”, Samsung