3GPP TSG-RAN4 Meeting #107		R4-2307951

Incheon, KR, 22th May – 26th May, 2023

Title:	Discussion on improvement on SCell/SCG setup delay

Source:	Xiaomi

Agenda Item:	8.25.4

Document for:	Discussion

- Introduction

In RAN4#106bis-e meeting, RAN4 had some discussion on improvement on SCell/SCG setup delay and the related WF was approved in [1]. In this contribution, we would like to discuss the feasibility to use the idle/inactive mode measurement results to improve SCell/SCG setup delay.

- Discussion

Solutions based on existing measurement

Issue 2-2-1: overall solution for UE which is configured with EMR measurement. - Agreements: - FFS whether to introduce indication of availability and/or validity status of measurement results from UE to NW into the [existing] EMR reports. Issue 2-2-2: overall solution for UE which is not capable of EMR measurement. - Candidate solutions: - Option 1: - For non-EMR capable UE, UE can report measurement results of carrier/cell configured for cell re-selection for CA/DC setup. - Option 1a: - the measurement results can be reported in following two ways: - in RRCResumeComplete (or RRCSetupComplete) message - after UE in connected mode (i.e. in UEInformationResponse message) - Option 2: FFS

For the solution based on the existing measurement, RAN4 can use the existing measurement including legacy measurement result and EMR result to indicate the validity of the results. The UE is required to report the validity indication for the EMR results or cell reselection measurement results if the results are considered as valid. For EMR capable UE, the UE needs to report the EMR measurement result with validity indication in the existing EMR reports. For non-EMR capable UE, the UE can report the cell selection measurement result with validity indication in RRCResumeComplete message or UEInformationResponse message.

Proposal 1: RAN4 to introduce the validity indication for the measurement results to improve RRC connection setup/resume delay

- For EMR capable UE, the UE reports the indication of validity EMR results in existing EMR report.

- For non-EMR capable UE, the UE reports the indication of valid cell reselection measurement results in RRCResumeComplete message or UEInformationResponse message.

Issue 2-2-3: definition of ‘valid’ in solution based on existing measurement - Agreements: - Candidate criteria for measurements validity definition - A) the measurement are performed within the last [X] seconds before it is reported - B) the reported measurement results satisfy measurement accuracy - C) variation of serving cell RSRP/RSRQ does not exceed [Y] dB - FFS whether a single or several criteria should be used for measurements validity definition.

In our understanding, condition B is the test requirement to guarantee the measurement performance, other than the validity condition. For condition A and C, we think both condition can improve the reliability that the reported result is valid. From this perspective, we support the measurement results are considered as valid if both condition A and condition C are met.

Proposal 2: The measurement results are considered as valid if both condition A and condition C are met.

If the measurement results are invalid, RAN4 needs to study whether and how to report the invalid measurement results. One of the alternative is that the UE does not report any results to NW. Another alternative is that UE can report the invalid indication to NW.

Proposal 3: If the measurement results are invalid, RAN4 to study whether and how to report the invalid measurement results. The following 2 ways are considered for further study:

- The UE does not report the any results to NW;

- The UE reports invalid indication to NW;

Solutions based on enhanced measurement

Issue 2-3-2: feasibility of enhanced measurement which starts from RRC setup/resume procedure. - Candidate solutions: - FFS: Feasibility of additional measurement during RRC connection setup is questionable. For instance, Rx beam sweeping is still needed for additional measurement during RRC connection setup unless target cell has been measured recently. However, if target cell has been measured recently before RRC connection setup, UE could directly send that measurement result, instead of doing another round of measurement. - FFS: It is not clear how much gain would be achieved by allowing UE perform enhanced measurement just dozens of milliseconds in advance, compared with the case network configure MO upon UE enters to connected mode. Moreover whether UE can acquire a complete measurement result during dozens of milliseconds is questionable.

According to the objective description, RAN4 is required to study how to use the measurement results obtained during and/or after IDLE/INACTIVE mode during RRC connection procedure in order to improve SCell/SCG setup delay. During the discussion in previous meetings, the controversial issue is whether UE is allow to perform addition measurement starting from RRC connection setup/resume procedure. During the study phase, RAN4 has discussed the feasibility of introducing additional measurement during RRC connection setup/resume procedure. And the companies had the concern on introducing the additional measurement at least from the following aspects:

- The available time used for additional measurement is typical 20ms during RRC connection setup/resume;

- The UE is required to use additional Rx chain for the additional measurement, otherwise, the RACH procedure will be impacted.

- The accuracy of the additional measurement will not be guaranteed if reduced sample(s) are used.

- How the UE to select the carrier(s) to be measured

In our understanding, if RAN4 continue to discuss the feasibility issue for enhanced measurement, RAN4 still needs to have debate from above aspects. In addition, It is not clear how much gain would be achieved by allowing UE perform enhanced measurement just 20ms - 100ms in advance, compared with the case network configure MO upon UE enters to connected mode. Moreover whether UE can acquire a complete measurement result during 20ms – 100ms is questionable. Thus, we propose not to introduce the enhanced measurement starting from RRC connection setup/resume procedure.

Proposal 4: It is questionable on the feasibility of introducing the enhanced measurement starting from RRC connection setup/resume procedure.

- Conclusion

In this contribution, we discuss the potential feasibility on using the idle/inactive mode measurement results to improve SCell/SCG setup delay and provide our proposals as follows.

Proposal 1: RAN4 to introduce the validity indication for the measurement results to improve RRC connection setup/resume delay

- For EMR capable UE, the UE reports the indication of validity EMR results in existing EMR report.

- For non-EMR capable UE, the UE reports the indication of valid cell reselection measurement results in RRCResumeComplete message or UEInformationResponse message.

Proposal 2: The measurement results are considered as valid if both condition A and condition C are met.

Proposal 3: If the measurement results are invalid, RAN4 to study whether and how to report the invalid measurement results. The following 2 ways are considered for further study:

- The UE does not report the any results to NW;

- The UE reports invalid indication to NW;

Proposal 4: It is questionable on the feasibility of introducing the enhanced measurement starting from RRC connection setup/resume procedure.

- Reference

[1]	R4-2306396	WF on NR Mobility Enhancements RRM requirements (part 2),	Apple