3GPP TSG-RAN4 Meeting #107	R4-2307975

# Incheon, KR, May 22nd - 26th, 2023                                                     Revision of R4-23xxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | DraftCR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-1 to introduce HPUE NR CA configuration CA_n78C_n80A-n84A and CA_n78C_n81A-n84A. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_FR1_TDD_NR_CADC_SUL_R18 |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce HPUE band combination as below: CA_n78C_n80A-n84A CA_n78C_n81A-n84A |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce HPUE band combination as below: CA_n78C_n80A-n84A CA_n78C_n81A-n84A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current spec can’t support HPUE band combination as below: CA_n78C_n80A-n84A CA_n78C_n81A-n84A |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5C, 6.2C.2, 7.3C.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

5.5C	Configurations for SUL

The configuration tables for SUL describe Bandwidth Combination Sets. Bandwidth Combination Set 4 and 5 contains all possible defined channel bandwidths for each band in the combination. The fact that BCS4 and BCS5 contains all channel bandwidths for each band does not alter if a bandwidth is mandatory or optional for a given band. Bandwidths that are identified as optional in Table 5.3.5-1 for a given release are still optional for UEs that support BCS4 or BCS5. , where the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities. Note that the minimum bandwidth is indicated only in BCS5 and BCS5 shall not be indicated together with BCS4 for a SUL configuration. For SUL band combinations including FR1 intra-band CA and with BCS4 or BCS5, the Bandwidth Combination Sets for the FR1 intra-band CA are BCS4 or BCS5.

<Unchanged Contents omitted>

Table 5.5C-5: Supported channel bandwidths per SUL band combination with inter-band CA (two SUL cells)

| SUL band combination with CA | Uplink CA configuration or SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n95A_n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n98A_n79A-n95A | SUL_n41A-n98A SUL_n79A-n95A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n95 | 5, 10, 15 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n83A_n79A-n98A | SUL_n41A-n83A SUL_n79A-n98A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20,30 |  |
|  |  | n98 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n41A-n83A_n79A-n95A | SUL_n41A-n83A SUL_n79A-n95A CA_n41A-n79A | n41 | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | 10, 20, 30,40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n83 | 5, 10, 15, 20,30 |  |
|  |  | n95 | 5, 10, 15 |  |
| CA_n78C_n80A-n84A | SUL_n78A-n80A SUL_n78A-n84A CA_n78C2 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 of TS 38.101-1 | 0 |
|  |  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n78C_n81A-n84A | SUL_n78A-n81A SUL_n78A-n84A CA_n78C2 | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 of TS 38.101-1 | 0 |
|  |  | n81 | 5, 10, 15, 20 |  |
|  |  | n84 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. NOTE 2: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

6.2C	Transmitter power for SUL

<Unchanged clauses omitted>

6.2C.2	ΔTIB,c

For the UE which supports SUL band combination, ΔTIB,c in Tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

<Unchanged Contents omitted>

Table 6.2C.2-2: ΔTIB,c for SUL band combination (Three bands)

| Band combination for SUL | ΔTIB,c for NR bands (dB)3 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration4 |  |  |
| CA_n1_SUL_n78-n80 | 0.6 | 0.8 | 0.6 |
| CA_n1_SUL_n78-n81 | 0.3 | 0.8 | 0.6 |
| CA_n1_SUL_n78-n84 | 0.6 | 0.8 | 0.6 |
| CA_n3_SUL_n41-n80 | 0.5 | 0.31 / 0.82 | 0.5 |
| CA_n3_SUL_n78-n80 | 0.6 | 0.8 | 0.6 |
| CA_n3_SUL_n79-n80 | 0.3 | 0.8 | 0.3 |
| CA_n8_SUL_n78-n81 | 0.6 | 0.8 | 0.6 |
| CA_n28_SUL_n41-n83 | 0.3 | 0.3 | 0.3 |
| CA_n28_SUL_n79-n83 | 0.5 | 0.8 | 0.5 |
| CA_n41_SUL_n79-n80 | 0.31 / 0.82 | 0.8 | 0.3 |
| CA_n41_SUL_n79-n83 | 0.3 | 0.8 | 0.5 |
| CA_n41_SUL_n79-n95 | 0.3 | 0.8 | 0.3 |
| CA_n41_SUL_n79-n97 | 0.5 | 0.5 | 0.5 |
| CA_n41_SUL_n79-n98 | 0.3 | 0.8 | 0.3 |
| CA_n78_SUL_n1-n80 | 0.8 | 0.6 | 0.6 |
| CA_n78_SUL_n1-n81 | 0.8 | 0.3 | 0.6 |
| CA_n78_SUL_n1-n89 | 0.8 | 0.6 | 0.6 |
| CA_n78_SUL_n3-n84 | 0.8 | 0.6 | 0.6 |
| CA_n78_n80-n84 | 0.8 | 0.6 | 0.6 |
| CA_n78_n81-n84 | 0.8 | 0.6 | 0.3 |
| CA_n79_SUL_n41-n80 | 0.31 / 0.82 | 0.8 | 0.3 |
| CA_n79_SUL_n41-n83 | 0.3 | 0.8 | 0.5 |
| CA_n79_SUL_n41-n95 | 0.3 | 0.8 | 0.3 |
| CA_n79_SUL_n41-n97 | 0.5 | 0.5 | 0.5 |
| CA_n79_SUL_n41-n98 | 0.3 | 0.8 | 0.3 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515MHz. NOTE 3: “-” denotes ΔTIB,c = 0. NOTE 4: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

7.3C	Reference sensitivity for SUL

<Unchanged Contents omitted>

7.3C.3.2.2	ΔRIB,c  for three bands

Table 7.3C.3.2.2-1: ΔRIB,c due to SUL (three bands)

| Band combination for SUL | ΔRIB,c for NR bands (dB)2 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration3 |  |  |
| CA_n1_SUL_n78-n80 | 0.2 | 0.5 | - |
| CA_n1_SUL_n78-n81 | - | 0.5 | - |
| CA_n1_SUL_n78-n84 | 0.2 | 0.5 | - |
| CA_n3_SUL_n41-n80 | - | 0.51 | - |
| CA_n3_SUL_n78-n80 | 0.2 | 0.5 | - |
| CA_n3_SUL_n79-n80 | - | 0.5 | - |
| CA_n28_SUL_n41-n83 | 0.2 | - | - |
| CA_n8_SUL_n78-n81 | 0.2 | 0.5 | - |
| CA_n28_SUL_n79-n83 | 0.2 | 0.5 | - |
| CA_n41_SUL_n79-n80 | 0.5 | 0.5 | - |
| CA_n41_SUL_n79-n83 | 0.5 | 0.5 | - |
| CA_n41_SUL_n79-n95 | - | 0.5 | - |
| CA_n41_SUL_n79-n97 | - | 0.8 | - |
| CA_n41_SUL_n79-n98 | - | 0.5 | - |
| CA_n78_SUL_n1-n80 | 0.5 | 0.2 | - |
| CA_n78_SUL_n1-n81 | 0.5 | - | - |
| CA_n78_SUL_n1-n89 | 0.5 | 0.2 | - |
| CA_n78_SUL_n3-n84 | 0.5 | 0.2 | - |
| CA_n79_SUL_n41-n80 | 0.5 | 0.5 | - |
| CA_n78_n80-n84 | 0.5 | - | - |
| CA_n78_n81-n84 | 0.5 | - | - |
| CA_n79_SUL_n41-n83 | 0.5 | 0.5 | - |
| CA_n79_SUL_n41-n95 | - | 0.5 | - |
| CA_n79_SUL_n41-n97 | - | 0.8 | - |
| CA_n79_SUL_n41-n98 | - | 0.5 | - |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 2: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE 3: The component band order in the configuration should be listed by the order of NR bands and SUL band, such as for CA_n1_SUL_n78-n80 the band order from left to right is n1, n78 and n80. |  |  |  |

## <End of Changes>