3GPP TSG-RAN WG4 Meeting #107	R4-2307987

# Incheon, South Korea, May 22 – 26, 2023

Title:	Further discussion on intraBandENDC-Support in RAN4 spec

Source:	ZTE Corporation

Agenda item:	10.2.3

Document for:	Approval

- 1.	Introduction

- For Case 3 with all CCs are contiguous in DL but neither carrier is contiguous to each other in UL, the update solution for Option 2c is suggested to be further checked in RAN4#106bis-e.

    - Option 2c:From Rel-16, UE shall also support non-contiguous operation in the DL (DC_48C_n48A or DC_48D_48A), then the network can configure DL_(n)48CA or DC_(n)48DA with the middle LTE cell DL-only and the UL with a gap (non-contiguous).

- For extending the meaning of ‘both’ in the IE intraBandENDC-Support for B48+n48 EN-DC in both case 3 and case 4, companies are requested to provide further input on the following two options in RAN4#106bis-e.

    - Option 1: Extend the meaning of ‘both’ in the IE intraBandENDC-Support for B48+n48 EN-DC in both case 3 and case 4.

    - Option 2: Keep existing RAN4 understanding of ‘both’ as communicated in the RAN4 LS to RAN2 in R4-2220837.

- For Case 4 in which LTE and NR adjacent carriers are contiguous but carriers in LTE or NR are non-contiguous, the update solution for Option 2b is suggested to be further checked in RAN4#106bis-e.

    - Option 2b: From Rel-16, band combinations of contiguous and non-contiguous intra-band EN-DC should be limited to two sub-blocks one of which consists of a contiguous EN-DC configuration in table Table 5.3B.0-1 in 38.101-3. For these the UE must support both contiguous and non-contiguous EN-DC in the UL, i.e.

      - UE indicate “both” capability for DL DC_48A_(n)48AA with UL DC_(n)48AA and UL DC_48A_n48A

      - The Case 4 is limited to one NR sub-block (band entry) with one or more E-UTRA sub-blocks, the intraBandENDC-Support still indicting the relation between any one of the E-UTRA sub-blocks (band entries) and the single NR sub-block

- For future extensions, companies are requested to provide further input on the following options.

    - Option 1: Consider future extension with intra-band EN-DC configurations having three band entries with two entries from NR band and one entry from E-UTRA band. Decide whether the following configurations in ‘Case x’ and ‘Case y’ will be possible or not in future extensions

      - ‘Case x’:    DL: DC_(n)XCA    UL: DC_XA_nXA

      - ‘Case y’:    DL: DC_YA-(n)YAA    UL: DC_YA_nYA.

    - Option 2: Defer until RAN4 develops common understanding on existing cases.

    - Option 3: Wait for response from RAN2 to RAN4 LS in R4-2220837.

- For handling of Intra-band EN-DC configurations in the specification, companies are requested to provide further input in RAN4#106bis-e.

    - Option 1: RAN4 to specify Config#1 (both UL and DL has contiguous spectrum), Config#2 (both UL and DL has non-contiguous spectrum) and Config#3 (UL and DL has different contiguity) in different sub-clauses.

    - Option 2: Keep existing clauses and handle any combinations with intra-band DL contiguous EN-DC with intra-band UL non-contiguous EN-DC in mixed contiguous and non-contiguous tables.

RAN4 was tasked by RAN plenary to study the inconsistency issues for intra-band EN-DC band combinations and a WF has been approved at last RAN4 meeting in [1] as below.

Principles

- A new UE capability parameter intraBandENDC-Support-UL, with the values {non-contiguous, both}, is introduced.

- The UE capability parameter intraBandENDC-Support-UL is only signalled when UL capability is different from the one indicated by the existing parameter intraBandENDC-Support.

- If the new parameter intraBandENDC-Support-UL is not signalled. the existing UE capability parameter intraBandENDC-Support indicates:

  - either the UE capability only for DL (when the UE supports intra-band EN-DC only in DL)

  - or the common capability for DL and UL (when the UE supports intra-band EN-DC in DL and UL)

- Support for non-contiguous intra-band EN-DC in DL and contiguous in UL is considered an invalid case.

- Early implementation from Release-15 shall be supported.

In RAN2#121bis-e meeting, RAN2 discussed RAN4’s suggestion regarding the interpretation of the UE capability parameter intraBandENDC-Support set to “both”. During the discussion, there were concerns raised on the forward compatibility of RAN4’s solution. RAN2’s understanding for “both” is that the UE supports both contiguous and non-contiguous intra-band EN-DC for the same band entries. Since there is no differentiation between DL and UL in the existing UE capability parameter intraBandENDC-Support, RAN4’s solution was considered non-backward compatible. RAN2 therefore agreed on a new signalling solution outlined as below [2].

In this paper, we analyse the feasibility of intra-band EN-DC band combination support mentioned above. Some considerations on how to implement the solutions in RAN4 are provided.

- 2. 	Discussion

In the previous RAN2/4 meetings, the exact meaning of “both” in IE intraBandENDC-Support has been discussed separately in the two working groups. In RAN4, the previous decision is that for Case 3 the UE supports non-contiguous in UL paired with contiguous in DL; and for Case 4 the UE supports contiguous in DL paired with contiguous / non-contiguous in UL, which is limited to three band entries and two contiguous spectrum sub-blocks. However in RAN2, it is stated that “both” in IE intraBandENDC-Support means both contiguous BC and non-contiguous BC with the same band entries are supported, which has different understanding between RAN2 and RAN4.

One discussion paper on intraBandENDC-Support was proposed in RAN4#106bis-e meeting for information [3]. It mentioned the agreements in RAN2#120 and RAN2#121. The observation is that the introduction of new capability IE for UL intraBandENDC-Support-UL can completely solve the inconsistency issue of intra-band ENDC indication, since the essence of the issue is that in current NR spec the indicator for intra-band ENDC is used for both uplink and downlink without distinguishing them.

Observation 1  The origin of the confusion in indicating the support of intra-band ENDC configuration is that the current IE intraBandENDC-Support is used for both UL and DL without distinguishing them.

Observation 2  A new capability IE intraBandENDC-Support-UL to be introduced in RAN2 to represent the intra-band EN-DC support in UL can completely sovle the inconsistency issue of intra-band ENDC indication. The restriction to the existing capability intraBandENDC-Support to DL is needed.

If RAN4 can agree the above solution in RAN2, it seems there is no problem for future extensions as mentioned in RAN4 WF in [1]. The only leftover issue is that how to implement the changes in RAN4 spec.

Observation 3  There is no need to further discuss the future extension for the support of intra-band ENDC since the introduction of new IE intraBandENDC-Support-UL eliminates the inconsistencies.

With regard to the “Principle 4” in [2], i.e., support for non-contiguous intra-band EN-DC in DL and contiguous in UL being considered an invalid case, we don’t think this restriction is needed. For example, in current configuration table, we have the following configuration:

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_48A-(n)48AA | DC_(n)48AA | Yes |

In this case, we think the downlink should be configured as “non-contiguous” while the uplink is “contiguous”.

Observation 4  There is case in current RAN4 spec which supports non-contiguous intra-band EN-DC in DL and contiguous in UL.

Proposal 1  It is supposed to agree the new signalling solution outlined by RAN2 except for “Principle 4” since such case already exists in RAN4. The corresponding changes in RAN4 spec for intra-band ENDC configuration should be done accordingly.

In last RAN4 meeting, we submitted a draft CR for information to implement the changes in RAN4 spec [4], which we think it is still reasonable under the condition that a new IE for intraBandENDC-Support-UL is introduced.

2.1 Handling of Intra-band EN-DC in RAN4 Spec

As for the EN-DC configurations for B48+n48 in Case 3/4, to our understanding, it is reasonable to set a separate sub-clause for mixed intra-band contiguous and non-contiguous EN-DC configurations with different contiguous and non-contiguous aspects in DL and UL so as to reflect the new changes in RAN2 for introducing the IE intraBandENDC-Support-UL.

Furthermore, if we consider the “both” value for both IE intraBandENDC-Support and intraBandENDC-Support-UL from the perspective of signalling, we think the main purpose of “both” is to reduce the load of signalling. It is within the work scope of RAN2 and not too much related to RAN4. Therefore, we propose not to reflect “both” in RAN4 intra-band EN-DC configuration table.

Proposal 2  For mixed intra-band EN-DC configurations with different contiguous and non-contiguous aspects in DL and UL, it is suggested to set a new sub-clause in TS 38.101-3.

Proposal 3  There is no need to emphasize “both” value for intraBandENDC-Support and intraBandENDC-Support-UL in RAN4 intra-band ENDC configuration tables.

A draft reply LS to RAN2 is proposed in [5].

A CR is proposed in [6].

The proposed changes in RAN4 spec is as below.

5.3B.1.3	BCS for Intra-band non-contiguous EN-DC

Table 5.3B.1.3-1: EN-DC configurations and bandwidth combination sets defined for intra-band non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_48A-(n)48AA5 | DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |  |  |  |

      - 5.3B.1.4	BCS for mixed Intra-band contiguous and non-contiguous EN-DC

Table 5.3B.1.4-1: EN-DC configurations and bandwidth combination sets defined for mixed intra-band contiguous and non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration |  | Uplink EN-DC configurations |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) |  |  | Bandwidth combination set |  |
|  |  |  |  |  | Channel bandwidths for E-UTRA carrier (MHz) |  |  | Channel bandwidths for NR carrier (MHz) |  |  | Channel bandwidths for E-UTRA carrier (MHz) |  |  |  |  |  |  |  |
| DC_(n)48CA5 | DC_48A_n48A2 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)48DA5 | DC_48A_n48A2 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_48A-(n)48AA |  | DC_(n)48AA |  |  | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  | 5, 10, 15, 20, 40 |  |  |  |  |  | 80 |  |  | 0 |  |
|  |  |  |  |  |  |  |  |  | 5, 10, 15, 20, 40 |  |  | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 5: The UE supporting these configurations indicates ‘both’ by IE intraBandENDC-Support. |  |  |  |  |  |  |

5.5B.4	Mixed Intra-band contiguous and non-contiguous EN-DC

Table 5.5B.4-1: Mixed intra-band contiguous and non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)48CA6 | DC_48A_n48A5 | Yes5 |
| DC_(n)48DA6 | DC_48A_n48A5 | Yes5 |
| DC_48A-(n)48AA6 | DC_(n)48AA5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void NOTE 3: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: The UE supporting these configurations indicates ‘both’ by IE intraBandENDC-Support. |  |  |

- 3. 	Conclusion

In this paper, we analyse the feasibility of the solution for intra-band EN-DC band combination support mentioned by RAN2. Some considerations on how to implement the solutions in RAN4 are provided. The following observations and proposals are proposed.

Observation 1  The origin of the confusion in indicating the support of intra-band ENDC configuration is that the current IE intraBandENDC-Support is used for both UL and DL without distinguishing them.

Observation 2  A new capability IE intraBandENDC-Support-UL to be introduced in RAN2 to represent the intra-band EN-DC support in UL can completely sovle the inconsistency issue of intra-band ENDC indication. The restriction to the existing capability intraBandENDC-Support to DL is needed.

Observation 3  There is no need to further discuss the future extension for the support of intra-band ENDC since the introduction of new IE intraBandENDC-Support-UL eliminates the inconsistencies.

Observation 4  There is case in current RAN4 spec which supports non-contiguous intra-band EN-DC in DL and contiguous in UL.

Proposal 1  It is supposed to agree the new signalling solution outlined by RAN2 except for “Principle 4” since such case already exists in RAN4. The corresponding changes in RAN4 spec for intra-band ENDC configuration should be done accordingly.

Proposal 2  For mixed intra-band EN-DC configurations with different contiguous and non-contiguous aspects in DL and UL, it is suggested to set a new sub-clause in TS 38.101-3.

Proposal 3  There is no need to emphasize “both” value for intraBandENDC-Support and intraBandENDC-Support-UL in RAN4 intra-band ENDC configuration tables.

A draft reply LS to RAN2 is proposed in R4-2307988 [5].

A CR is proposed in R4-2307989 [6].

The proposed changes in RAN4 spec is as below.

5.3B.1.3	BCS for Intra-band non-contiguous EN-DC

Table 5.3B.1.3-1: EN-DC configurations and bandwidth combination sets defined for intra-band non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_48A-(n)48AA5 | DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |  |  |  |

      - 5.3B.1.4	BCS for mixed Intra-band contiguous and non-contiguous EN-DC

Table 5.3B.1.4-1: EN-DC configurations and bandwidth combination sets defined for mixed intra-band contiguous and non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration |  | Uplink EN-DC configurations |  |  | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth (MHz) |  |  | Bandwidth combination set |  |
|  |  |  |  |  | Channel bandwidths for E-UTRA carrier (MHz) |  |  | Channel bandwidths for NR carrier (MHz) |  |  | Channel bandwidths for E-UTRA carrier (MHz) |  |  |  |  |  |  |  |
| DC_(n)48CA5 | DC_48A_n48A2 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)48DA5 | DC_48A_n48A2 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_48A-(n)48AA |  | DC_(n)48AA |  |  | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  | 5, 10, 15, 20, 40 |  |  |  |  |  | 80 |  |  | 0 |  |
|  |  |  |  |  |  |  |  |  | 5, 10, 15, 20, 40 |  |  | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 5: The UE supporting these configurations indicates ‘both’ by IE intraBandENDC-Support. |  |  |  |  |  |  |

5.5B.4	Mixed Intra-band contiguous and non-contiguous EN-DC

Table 5.5B.4-1: Mixed intra-band contiguous and non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)48CA6 | DC_48A_n48A5 | Yes5 |
| DC_(n)48DA6 | DC_48A_n48A5 | Yes5 |
| DC_48A-(n)48AA6 | DC_(n)48AA5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void NOTE 3: For TDD bands, the minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: The UE supporting these configurations indicates ‘both’ by IE intraBandENDC-Support. |  |  |

# References

- R4-2303635, WF on inconsistency issue for intra-band EN-DC band combinations, AT&T

- R2-2304431, Reply LS on intraBandENDC-Support, Qualcomm

- R4-2304727, Further discussion on intra-band EN-DC support, ZTE

- R4-2304728, CR for TS 38.101-3 on corrections to intra-band EN-DC configurations, ZTE

- R4-2307988, Draft Reply LS on intraBandENDC-Support-UL, ZTE

- R4-2307989, CR for TS 38.101-3 on corrections to the mixed intra-band contiguous and non-contiguous EN-DC configurations, ZTE