3GPP TSG-RAN WG4 Meeting #107 	R4-2308025

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.8.3.3

Source: 	Ericsson

Title:	On RLM and beam management

Document for:	Discussion

# Introduction

The agreements from RAN4#106bis-e are summarized in [12] and [13] for the 1st and 2nd threads, respectively. And the discussion summaries are available in [14] and [15] for the 1st and 2nd threads, respectively.

The following was agreed for RLM and BM in RAN4#106bis-e:

- Cell specific RLM and BFD/CBD

  - No further discussion on TRP specific RLM as there is no such procedure in existing specification..

- Issue 2-1-2: Number of samples

  - For RLM with multi-Rx chain operation, the same number of samples Mout and Min are used as in Rel-17.

  - For link recovery with multi-Rx chain operation, the same number of samples MBFD and MCBD are used as in Rel-17.

- Issue 2-1-3: PDCCH configuration

  - For RLM with multi-Rx chain operation, the same PDCCH transmission configuration is used as in Rel-17.

  - For link recovery with multi-Rx chain operation, the same PDCCH transmission configuration is used as in Rel-17.

# Discussion

## RLM

### General

Currently TRP-specific RLM is not supported in RAN1, so, according to the current RLM procedure, the UE has only one RLM procedure per serving cell. However, the UE can still keep track of multiple RLM-RSs:

From TS 38.133:

…When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout, layer 1 of the UE shall send an out-of-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the out-of-sync indications as specified in TS 38.331 [2].

When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin, layer 1 of the UE shall send an in-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the in-sync indications as specified in TS 38.331 [2]…

The multiple RLM-RSs can be transmitted from multiple TRPs. When multiple RLM-RS are configured, if they are overlapped in time domain, and received from different directions, they can be received using two panels at the same time.

Considering the limited amount of remaining time for this WI, we propose to focus on simultaneous reception of two CSI-RSs (with different QCL type D from different directions).

- Proposal 1: For RLM based at least on CSI-RS, the multi-rx operation includes simultaneous reception of two CSI-RSs (with different QCL type D from different directions).

NOTE: for BFD/CBD, the simultaneous reception of two CSI-RSs has been agreed already in RAN4#104bis-e [R4-2217587].

The in-sync and out-of-sync evaluation periods are defined per RLM-RS and can be enhanced for multi-rx operation.

### Evaluation period reduction for multi-rx operation

Three approaches for enhancing RLM evaluation period have been proposed so far:

- Approach 1: Beam sweeping factor N reduction

Disadvantages of Approach 1:

- WI scope is the simultaneous reception of different RSs, while with reduced beam sweeping factor the UE attempts to receive the same RS from different directions,

- The multi-rx gain is not always from beam sweeping only,

- In many cases N=1 already,

- Re-interpreting the meaning of the legacy definition of a parameter is not always desirable and may impact the implementation.

- Approach 2: Sharing factor reduction

Disadvantages of Approach 2:

- The multi-rx gain is not always from sharing only

- Re-interpreting the meaning of the legacy definition of a parameter is not always desirable and may impact the implementation

- There is no agreement to introduce sharing factor reduction.

- Approach 3: Introducing a separate scaling factor for multi-rx

This approach is cleaner, since it does not re-interpret the legacy parameters.

It also addresses the disadvantages of Approach 1 and Approach 2, since there is no need to explicitly write in the specification when the reduction is due to beam sweeping or due to sharing factor reduction, leaving flexibility for UE implementation.

Since Approach 1 does not fulfil the WI goal with respect to simultaneous reception of different RSs, Approach 3 can be used. Approach 3 can be implemented in the specification, e.g., as shown in the table below. The new parameter (e.g., “L”) could take values 1 for non-simultaneous reception and L<1 (e.g., ½) for simultaneous reception with multi-rx when the necessary conditions for multi-rx operation are met.

Table 1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2 (based on Table 8.1.3.2-2 in TS 38.133)

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P×N×L)×TCSI-RS) | Max(100, Ceil(Min×P×N×L) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P×N×L)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P×N×L)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P×N×L) × TDRX | Ceil(Min×P×N×L) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

- Proposal 2: Since beam seeping factor reduction is not fulfilling the WI goal with respect to different RSs, the RLM evaluation periods (in-sync and out-of-sync) can be enhanced with a new scaling parameter L. L=1 for non-simultaneous reception, and L<1 for simultaneous reception with multi-rx (e.g., L=1/2 but can be TBD for now) when the necessary conditions for multi-rx are met:

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P×N×L)×TCSI-RS) | Max(100, Ceil(Min×P×N×L) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P×N×L)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P×N×L)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P×N×L) × TDRX | Ceil(Min×P×N×L) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

## Beam management

### General

The following was agreed in [4] for beam management procedures:

- BFD/BFR:

  - CSI-RS based BFD/BFR is in the scope of the current WI

BFD/CBD can be based only on SSB, can be based only on CSI-RS, or can be based on both SSB and CSI-RS. The above proposal in [4] missed to cover BFD/BFR based on both SSB and CSI-RS, so we propose to further clarify this.

- Proposal 3: Simultaneous CSI-RS reception applies for BFD/BFR which is based at least on CSI-RS.

NOTE: for BFD/CBD, the simultaneous reception of two CSI-RSs has been agreed already in RAN4#104bis-e [R4-2217587].

There can be different approaches for evaluation period reduction with multi-rx, e.g., beam sweeping factor (N) reduction or applying directly another scaling factor, provided the conditions for multi-rx operation are met. The conditions are discussed in section 2.2.1.

### Evaluation period enhancement

Three approaches for enhancing BFD/CBD evaluation period have been proposed so far:

- Approach 1: Beam sweeping factor N reduction

Disadvantages of Approach 1:

- WI scope is the simultaneous reception of different RSs, while with reduced beam sweeping factor the UE attempts to receive the same RS from different directions,

- The multi-rx gain is not always from beam sweeping only,

- In many cases N=1 already,

- Re-interpreting the meaning of the legacy definition of a parameter is not always desirable and may impact the implementation.

- Approach 2: Sharing factor reduction

Disadvantages of Approach 2:

- The multi-rx gain is not always from sharing only

- Re-interpreting the meaning of the legacy definition of a parameter is not always desirable and may impact the implementation

- There is no agreement to introduce sharing factor reduction.

- Approach 3: Introducing a separate scaling factor for multi-rx

This approach is cleaner, since it does not re-interpret the legacy parameters.

It also addresses the disadvantages of Approach 1 and Approach 2, since there is no need to explicitly write in the specification when the reduction is due to beam sweeping or due to sharing factor reduction, leaving flexibility for UE implementation.

Since Approach 1 does not fulfil the WI goal with respect to simultaneous reception of different RSs, Approach 3 can be used. Approach 3 can be implemented in the specification, e.g., as shown in the table below. The new parameter (e.g., “L”) could take values 1 for non-simultaneous reception and L<1 (e.g., ½) for simultaneous reception with multi-rx when the necessary conditions for multi-rx operation are met.

Table 2: Evaluation period TEvaluate_BFD_CSI-RS for FR2 (based on Table 8.5.3.2-2 in TS 38.133)

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(MBFD P N PBFD u L) TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 × MBFD P N PBFD L) Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | Ceil(MBFD P N PBFD L) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Table 3: Evaluation period TEvaluate_CBD_CSI-RS for FR2 (based on Table 8.5.6.2-2 in TS 38.133)

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(MCBD P N PCBD L) TCSI-RS) |
| DRX cycle > 320ms | Ceil(MCBD P N PCBD L) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

- Proposal 4: Since beam seeping factor reduction is not fulfilling the WI goal with respect to different RSs, the BFD evaluation period can be enhanced with a new scaling parameter L. L=1 for non-simultaneous reception, and L<1 for simultaneous reception with multi-rx (e.g., L=1/2 but can be TBD for now) when the necessary conditions for multi-rx are met:

Evaluation period TEvaluate_BFD_CSI-RS for FR2 (based on Table 8.5.3.2-2 in TS 38.133)

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(MBFD P N PBFD L) TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 × MBFD P N PBFD L) Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | Ceil(MBFD P N PBFD L) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

- Proposal 5: Since beam seeping factor reduction is not fulfilling the WI goal with respect to different RSs, the CBD evaluation period can be enhanced with a new scaling parameter L. L=1 for non-simultaneous reception, and L<1 for simultaneous reception with multi-rx (e.g., L=1/2 but can be TBD for now) when the necessary conditions for multi-rx are met:

Evaluation period TEvaluate_CBD_CSI-RS for FR2 (based on Table 8.5.6.2-2 in TS 38.133)

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(MCBD P N PCBD L) TCSI-RS) |
| DRX cycle > 320ms | Ceil(MCBD P N PCBD L) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

# Summary

The following have been observed and proposed in this contribution.

- Proposal 1: For RLM based at least on CSI-RS, the multi-rx operation includes simultaneous reception of two CSI-RSs (with different QCL type D from different directions).

- Proposal 2: Since beam seeping factor reduction is not fulfilling the WI goal with respect to different RSs, the RLM evaluation periods (in-sync and out-of-sync) can be enhanced with a new scaling parameter L. L=1 for non-simultaneous reception, and L<1 for simultaneous reception with multi-rx (e.g., L=1/2 but can be TBD for now) when the necessary conditions for multi-rx are met:

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil(Mout×P×N×L)×TCSI-RS) | Max(100, Ceil(Min×P×N×L) × TCSI-RS) |
| DRX ≤ 320ms | Max(200, Ceil(1.5×Mout×P×N×L)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×Min×P×N×L)× Max(TDRX, TCSI-RS)) |
| DRX > 320ms | Ceil(Mout×P×N×L) × TDRX | Ceil(Min×P×N×L) × TDRX |
| NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. |  |  |

- Proposal 3: Simultaneous CSI-RS reception applies for BFD/BFR which is based at least on CSI-RS.

NOTE: for BFD/CBD, the simultaneous reception of two CSI-RSs has been agreed already in RAN4#104bis-e [R4-2217587].

- Proposal 4: Since beam seeping factor reduction is not fulfilling the WI goal with respect to different RSs, the BFD evaluation period can be enhanced with a new scaling parameter L. L=1 for non-simultaneous reception, and L<1 for simultaneous reception with multi-rx (e.g., L=1/2 but can be TBD for now) when the necessary conditions for multi-rx are met:

Evaluation period TEvaluate_BFD_CSI-RS for FR2 (based on Table 8.5.3.2-2 in TS 38.133)

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(MBFD P N PBFD L) TCSI-RS) |
| DRX cycle ≤ 320ms | Max(50, Ceil(1.5 × MBFD P N PBFD L) Max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | Ceil(MBFD P N PBFD L) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

- Proposal 5: Since beam seeping factor reduction is not fulfilling the WI goal with respect to different RSs, the CBD evaluation period can be enhanced with a new scaling parameter L. L=1 for non-simultaneous reception, and L<1 for simultaneous reception with multi-rx (e.g., L=1/2 but can be TBD for now) when the necessary conditions for multi-rx are met:

Evaluation period TEvaluate_CBD_CSI-RS for FR2 (based on Table 8.5.6.2-2 in TS 38.133)

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320ms | Max(25, Ceil(MCBD P N PCBD L) TCSI-RS) |
| DRX cycle > 320ms | Ceil(MCBD P N PCBD L) TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2214344, WF on FR2 multi-Rx RRM, vivo, Aug. 2022.

[3] R4-2217246, WF on RRM impacts and general aspects for multi-Rx, vivo, Oct. 2022.

[4] R4-2217587, WF on L1 measurements, beam sweeping factors and simultaneous reception, Qualcomm, Oct. 2022.

[5] R4-2217248, WF on TCI state switching for multi-RX chain DL reception, Ericsson, Oct. 2022.

[6] R4-2220742, WF on NR FR2 multi-Rx chain DL reception – Part 1, Nov. 2022.

[7] R4-2220743, WF on NR FR2 multi-Rx chain DL reception – Part 2, Nov. 2022.

[8] R4-2220402, WF on NR FR2 multi-Rx chain DL reception – Part 3, Nov. 2022.

[9] R4-2303193, Ad-hoc minutes for NR FR2 multi-Rx chain DL reception RRM, vivo, March 2023.

[10] R4-2303302, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, March 2023.

[11] R4-2303317, WF on NR FR2 multi-Rx chain DL reception RRM requirements(part2), Qualcomm, March 2023.

[12] R4-2306318, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, Apr. 2023.

[13] R4- 2306319, WF on FR2_multiRx_part2, Ericsson, Apr. 2023.

[14] R4-2306392, Topic summary for [106bis-e][202] FR2_multiRx_part1, vivo, Apr. 2023.

[15] R4-2306234, Topic summary for [106bis-e][203] FR2_multiRx_part2, Ericsson, Apr. 2023.