3GPP TSG-RAN WG4 Meeting #107 	R4-2308038

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On NCR RRM Requirements

Agenda item:	8.29.5

Document for:	Discussion

Introduction

At RAN4#106bis-e meeting, consensus was reached on many of the NCR-MT RRM requirements [1]. However, there are a few issues which remained open as shown below. For several RRM procedures, agreements were reached to define the requirements; however, requirements for those procedures remains to be specified (shown in green). For a few other issues, further discussions are required (shown in yellow).

Issue 1-1: RRC re-establishment - Agreements - Define the RRC re-establishment requirement for LA NCR-MT only in Rel-18 Issue 2-1: Initial transmit timing requirement Te - Agreements - Define Te requirement for WA and LA NCR-MT Issue 4-2: adaptive beamforming for NCR access link - Agreements - FFS whether to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency. Issue 5-1: BFD/BFR/RLM for NCR-MT - Agreements - For LA NCR-MT - Define RLM/BFD/BFR requirements - For WA NCR-MT - Do not define BFD/BFR requirements - FFS for RLM requirements.

In this paper, we address these open issues.

Discussion

Requirements for RRC Re-establishment

At RAN4#106bis-e, it has been agreed to define the RRC Re-establishment requirements for LA NCR only for Rel-18. Considering the similarity in deployments of NCR with that of IAB, we recommend using the requirements for RRC Re-establishment procedure as described in Clause 12.1.1.1 of 3GPP TS 38.174 for IAB-MT.

NCR and IAB nodes have similar deployment characteristics in terms of their location, and Release-18 NCR is intended for static deployments.

For the RRC Re-establishment procedure in LA NCR-MT, use the requirements for the RRC Re-establishment for IAB-MT as described in Clause 12.1.1.1 of 3GPP TS 38.174.

Initial Transmit Timing requirement

For the same reason described in Section 2.1 for RRC Re-establishment requirement, RAN4 should perform an analysis if the initial transmit timing requirement for IAB-MT can be used for LA NCR.

Determine if the initial transmit timing requirements for IAB-MT as described in Clause 12.2.1 can be applied to the LA NCR-MT initial transmit timing requirements.

Adaptive beamforming for NCR access link

RAN1 agreed on NCR capability to configure the NCR-Fwd Access link beam based on the indication in DCI or MAC-CE. At RAN1#112, the following agreements were reached regarding the time reference for NCR-Fwd access link beam configuration/switching:

Agreements: For the aperiodic beam indication, the reference of slot offset for each time resource is defined as the slot n+k where n refers to the slot that NCR-MT receive the DCI carrying the indication and: - Option-2: k refers to the offset value [defined by NCR-MT capability and/or declared by vendor]. - Note: This k is different from the parameter used to define the Slot offset for the time resource. For the semi-persistent beam indication, the indicated information should be applied starting from the first slot that is after slot n+3Nslotsubframe,μ, where n refers to the slot that NCR-MT would transmit HARQ-ACK information corresponding to the PDSCH carrying the activation command and μ is the SCS configuration for the channel carrying the HARQ-ACK information.

Access link beam switching is one of the distinguishing and essential functionalities of NCR node. Depending on the approach, the access link beam switching can be configured in a static manner (with RRC signalling) or change dynamically. When access link beam is indicated dynamically, it is important to ensure that the access UEs can be scheduled by the gNB over NCR-Fwd links only when the access beam is ready. Otherwise, the transmitted packets might be forwarded though the wrong beam or lost completely.

To avoid any impacts on the quality of data transmission to/from the access UEs, it is important that NCR node is strictly following the access link beam configuration/switching procedure.

We do not think that any new procedures on top of already existing aperiodic and semi-persistent beam indication defined in RAN1 are needed. However, RAN4 needs to reflect the RAN1 agreements in the core requirements:

RAN4 to define core requirements on the application latency of aperiodic and semi-persistent access link beam indication.

Then, it is still up to RAN4 discussion how these requirements to be defined. This will also depend how the related procedures are described in RAN1 and/or RAN2 TSs.

During the last meeting, some of the companies were also wondering whether it could be possible to measure the latency of access beam switching. In our view, it should be already possible based on the existing NR repeater conformance testing setup defined in Rel-17. In Figure 1, we show an example of EIRP measuring system set-up for conformance testing Over The Air (OTA) setup is described in Annex E.1 of TS 38.115-2. The changes in the access link beam can be identified based on the changes of the signal measured by the testing antenna.

Figure 1: Measuring system set-up for maximum EIRP testing.

The latency in the access beam application can be detected by the EIPR measurements of the NCR-Fwd access link.

BFD/BFR/RLM for NCR-MT

The following agreement was reached in RAN1#111:

As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported

It was also agreed in RAN2#120 that, NCR-Fwd will be OFF in the case of RLF happens and NCR-MT enters RRC_IDLE state due to not finding a suitable cell.

It was agreed in RAN4#106bis-e to define RLM/BFD/BFR requirements for LA NCR-MT. Considering the similarity of deployment scenario between IAB and NCR, RAN4 should determine whether the IAB requirements for these procedures can be used for NCR-MT.

In the TS 38.174, Clause 12.3.2 describes requirements for BFD and BFR for IAB-MT, and Clause 12.3.1 describes requirements for RLM for IAB-MT.  RAN4 should analyze applicability of these requirements for NCR-

For LA NCR_MT, RAN4 should analyze applicability of the requirements for BFD and BFR described in Clause 12.3.2 in TS 38.174 and the applicability of the requirements for RLM described in Clause 12.3.1 in TS 38.174.

RLM is in not necessarily critical functionality for the WA nodes, where the link is stable. Therefore, it is not required to define RRM requirements for RLM procedure.

The C-link of a WA NCR is expected to be very stable due to unobstructed LOS deployment with its serving base station. Hence, it is very unlikely to suffer radio link failure.

Do not define RLM requirements for WA NCR-MT C-link.

Conclusion

In this document, RRM requirements for a few procedures of NCR-MT, remaining from the RAN4#106bis meeting, were discussed.

The following Observations and Proposals were made:

Observation 1: NCR and IAB nodes have similar deployment characteristics in terms of their location, and Release-18 NCR is intended for static deployments.

Proposal 1: For the RRC Re-establishment procedure in LA NCR-MT, use the requirements for the RRC Re-establishment for IAB-MT as described in Clause 12.1.1.1 of 3GPP TS 38.174.

Proposal 2: Determine if the initial transmit timing requirements for IAB-MT as described in Clause 12.2.1 can be applied to the LA NCR-MT initial transmit timing requirements.

Observation 2: To avoid any impacts on the quality of data transmission to/from the access UEs, it is important that NCR node is strictly following the access link beam configuration/switching procedure.

Proposal 3: RAN4 to define core requirements on the application latency of aperiodic and semi-persistent access link beam indication.

Observation 3: The latency in the access beam application can be detected by the EIPR measurements of the NCR-Fwd access link.

Proposal 4: For LA NCR_MT, RAN4 should analyze applicability of the requirements for BFD and BFR described in Clause 12.3.2 in TS 38.174 and the applicability of the requirements for RLM described in Clause 12.3.1 in TS 38.174.

Observation 4: The C-link of a WA NCR is expected to be very stable due to unobstructed LOS deployment with its serving base station. Hence, it is very unlikely to suffer radio link failure.

Proposal 5: Do not define RLM requirements for WA NCR-MT C-link.

- References

- R4-2306360, “WF on NCR-MT RRM requirements”, ZTE, RAN4#106bis, E-meeting, April 17-26, 2023.