3GPP TSG-RAN WG4 Meeting # 107	R4-2308075

Incheon Meeting, May 22nd – 27th, 2023

Title:	TP to TR 38.899 Addition of CA_n5-n48-n77

Source:	Nokia, Verizon

Agenda item:	7.20.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.899 to add CA_n5-n48-n77. At RAN4#106bis this combination was submitted as a draftCR [1], however it was questioned if proper MSD analysis were performed, hence it is at this meeting resubmitted as a TP.

************************************* Start of TP*****************************************

## 6.x	CA_n5-n48-n77

### 6.x.1	Configurations

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n48A-n77A | n777, 9 CA_n5A-n48A CA_n5A-n77A7 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Power Class 1.5 is allowed for this single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.x.2	Maximum output power

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n5A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.x.3	REFSENS requirements

Analysis performed for the PC3 CA_n5A-n48A-n77A as captured in TR 38.717-03-02, the following co-existence issues due to dual uplink are addressed:

-	Simultaneous Tx/Rx operation between NR n48 and n77 carriers are not supported hence.

-	CA_n48-n77 is limited for downlink, hence no UL configuration exists.

#### 6.x.3.1	Power class 2 case a, b

No additional MSD is needed

### 6.x.4	Void

************************************* End of TP******************************************

# Reference

[1] R4-2306543, Draft CR 38.101-1 to add HPUE for 3 CA of n2 n5 n48 n66 n77, Nokia, Verizon

[2] R4-2201057, TP for TR 38.717-03-02 CA_n5-n48-n77, Samsung, Verizon