3GPP TSG-RAN WG4 Meeting #107 	R4-2308104

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on power back off for aerial UEs

Agenda item:	8.36.1

Document for:	Discussion and Approval

Introduction

The UAV WI, as presented in [1], includes the following objectives with RAN4 impact highlighted:

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. 2. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2] Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. 3. Specify the support for UAV identification broadcast (BRID) in NR PC5. Support of DAA using the same framework as BRID without DAA specific enhancements can be considered [RAN2]. Note:. UAV use of NR PC5 is to be used only in designated bands as defined in regulation for BRID/DAA use. 4. Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling [RAN1, RAN2]: - FR1 with directional antenna at UE side 5. Based on the technical conditions defined for aerial UE usage in ECC Decision (22)07, study and specify the necessary UE types and additional OOBE requirements for aerial UEs in 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz. [RAN4]. Note: In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE Note2: Applicability of power classes for aerial UE may need to be addressed in RAN4 Note3: RAN4 to identify the supported bands for aerial UE impacted by above information.

In this contribution we present further considerations for support of UAVs by the RAN4 specification.

Discussion

Background

During RAN99, an objective with RAN4 impact was added to the UAV WID [1]. RAN4 is requested to specify necessary UE types for additional Out Of Band Emissions (OOBE) requirements for aerial UE operating the frequency ranges; 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz according to ECC Decision (22)07.

The LS sent by ECC includes 4 requests to ETSI[2]:

Based on the ECC Decision (22)07, ECC invites ETSI to include the following requirements in future ETSI harmonised standard on aerial UE to ensure a coherence with this ECC framework: - Additional OOBE requirements applicable to aerial UEs in the following frequency bands 1710-1785 MHz, 2500-2570 MHz, 2570-2620 MHz, as defined by ECC Decision 22(07). In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE. - a mechanism/feature coherent with the above aerial UE definition in order to differentiate aerial UE, as defined by ECC Decision 22(07) from terrestrial UE operating under LTE/NR 5G networks - differentiation of aerial UE from other terrestrial UE shall not be changed by the end-user - d) the aerial UE shall not be capable to connect to LTE/NR 5G networks without aerial subscription

The objective added to the WID for addressing the LS from ECC clearly states that it is necessary “study and specify the necessary UE types and additional OOBE requirements for aerial UEs…”

It is part of the WID to specify additional OOBE requirements for aerial UEs for further restrictions in emission by the ECC decision.

The LS sent by ECC presents a list of requests to 3GPP, as previously mentioned in the discussion. The requests identified by b) and c) concern the discussion of the aerial UE types and the applicability of the respective further restrictions on emissions:

- Additional OOBE requirements applicable to aerial UEs in the following frequency bands 1710-1785 MHz, 2500-2570 MHz, 2570-2620 MHz, as defined by ECC Decision 22(07). In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE.

In this document we will address the OOBE requirements and the resulting A-MPR. The remaining aspects of the ECC LS is addressed in [3].

Affected NR bands

When considering the frequency ranges requested protected by ECC it can be identified that they correspond or overlap the NR bands as defined below from 38.101-1.

TS 38.101-1 - Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n3810 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD5 |
| NOTE 5: Unless otherwise stated, the applicability of requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the requirements for Band n41. A UE supporting Band n90 shall also support band n41. NOTE 10: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. |  |  |  |

In similar approach the affected LTE bands from 36.101 is listed below.

36.101 - Table 5.5-1 E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 9 | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz | FDD |
| 10 | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| 41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD4 |
| 69 | N/A |  |  | 2570 MHz | – | 2620 MHz | FDD2 |
| NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 4: The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 5: A UE that supports E-UTRA Band 66 shall receive in the entire DL operating band NOTE 6: A UE that supports E-UTRA Band 66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_66B, CA_66C and CA_66A-66A. NOTE 7: A UE that complies with the E-UTRA Band 66 minimum requirements in this specification shall also comply with the E-UTRA Band 4 minimum requirements. |  |  |  |  |  |  |  |

According to the ECC decision:

There is a need for OOBE limits for aerial UE operating in the following frequency bands: 1710-1785 MHz: Protection of MetSat operating in the 1675-1710 MHz frequency band Out-of-band limit: -40 dBm/MHz in the frequency range 1675-1710 MHz for aerial UE operating in 1710-1785 MHz. 2500-2570 MHz / 2570-2620 MHz: Protection of RAS operating in 2690 – 2700 MHz and radars operating in 2700-2900 MHz Out-of-band limit: -50 dBm/MHz in the frequency range 2690-2900 MHz for aerial UE operating in 2500-2570 MHz or 2570-2620 MHz. In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE.

From the list of potential affected bands only UL of band 3, 7 and 38 may be affected since these to our understanding are the only ones operated in Europe and this additional restriction comes from ECC with jurisdiction only here.

Affected bands of the additional ECC OOBE limits are UL of both LTE and NR bands 3, 7 and 38.

As a result of this the study for needed MPR/A-MPR is restricted to these three bands.

Simulation alignment

During RAN4#10bis a WF [4] were agreed which outlines the initial assumptions for the back off study (MPR/A-MPR). The agreed points have been included below:

<Way forward>: - P1 - For n3, RAN4 will perform back off analysis against MetSat protection on 1675-1710 MHz focusing on edge channels and specify the frequency distance when no back off is needed - P2 - For n7 and n38 protection of RAS 2690 – 2700 MHz and radars operating in 2700-2900 MHz analysis is done separately for n7 and n38 - Next meeting focuses on n38 - Concerned companies are encouraged to bring studies for n7 if seen necessary - P3 - For calibration, the current NR MPR will be used for submitted results for NR, meaning 0 MPR is as defined currently for n3, n7 and n38.

A-MPR Study and Proposals for n3

Additional spurious emission limits for UE co-existence are considered according to 38.101-1, as shown in Table 1. The additional requirement from ECC is added in the high-lighted row.

Table 1: Additional spurious emissions for UE co-existence (n3)

| Frequency range (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 1884.5 MHz – 1915.7 MHz | -41 | 0.3 MHz | NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |
| 1675 MHz – 1710 MHz | -40 | 1 MHz | NOTE xx: Applicable only for aerial UEs. |

Simulation Assumptions

IQ-Image and LO leakage = -28 dBc

CIM3 = -60 dBc

PA calibration point was 20 MHz, 15 kHz, QPSK, DFT-S-OFMA, 100 RB at lower channel edge with 0.5 dB MPR

No IBE or EVM was simulated

Simulation Results

We simulated the A-MPR due to the ECC mask in the 1675-1710 MHz range. Complete sets of results for 5 and 50 MHz channel BWs at the lower edge of the band can be found in Annex A; below we present examples.

Figure 1: Examples of the A-MPR when the channel is placed at the lower edge of the band n3. The A-MPR search is performed only up to 20 dB and some points marked with 20 dB require more A-MPR.

Very large values of A-MPR, even above 20 dB, are required if the channel is placed at the lower edge of the band n3.

Complete sets of results for channel BWs from 30 to 50 MHz at the upper edge of the band can be found in Annex A; below we present examples for 50 MHz.

Figure 2: Examples of the A-MPR with the 50 MHz channel BW when the channel is at the upper edge of the band n3.

Smaller A-MPR is required for channel BWs from 30 to 50 MHz if the channel is at the upper edge of the band n3.

A-MPR proposal

By restricting the channel placement, it may be possible to avoid A-MPR, at least for these placements, as the impact of the ECC restriction have the greatest impact at the lower edge of band n3. The minimum possible frequency offset for different channel bandwidths if A-MPR is to be avoided is shown below in Table 2.

Table 2: Frequency offset from the lower edge of band n3 to avoid A-MPR. An empty cell means the A-MPR can’t be avoided even if the channel is at the upper edge of the band.

| Channel BW, MHz | Freq. offset, MHz |
| --- | --- |
| 5 | 8.27 |
| 10 | 16.53 |
| 15 | 24.53 |
| 20 | 32.42 |
| 25 | 40.96 |
| 30 |  |
| 35 |  |
| 40 |  |
| 45 |  |
| 50 |  |

A-MPR can be avoided for some channel placements of specific channel bandwidths.

In specific it can be proposed to consider the following:

Table 3: Proposed minimum frequency offset from the lower edge of band n3 to avoid A-MPR.

| Channel BW, MHz | Freq. offset, MHz |
| --- | --- |
| 5 | 10 |
| 10 | 20 |
| 15 | 25 |
| 20 | 35 |
| 25 | 45 |
| 30 | N/A |
| 35 | N/A |
| 40 | N/A |
| 45 | N/A |
| 50 | N/A |

RAN4 shall consider no A-MPR for CBWs 5, 10, 15, 20 and 25 MHz if placed with a minimum frequency offset from the lower edge of band n3 as shown in Table 3.

Since it is first meeting presenting the A-MPR study results as captured in full in Annex A, it may be premature to agree A-MPR before also considering input from others. Hence, it is proposed to at least consider the presented results for n3 for now.

RAN4 shall consider the presented A-MPR results when defining A-MPR for n3 due to the ECC requirements.

A-MPR Study and Proposals for n38

Additional spurious emission limits for UE co-existence are considered according to 38.101-1, as shown in Table 3. The additional requirement from ECC is added in the high-lighted row.

Table 3: Additional spurious emissions for UE co-existence (n38)

| Frequency range (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 2620 MHz – 2645 MHz | -15.5 | 5 MHz | NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths up to 20 MHz. For channel bandwidth within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. . For carriers overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |
| 2645 MHz – 2690 MHz | -40 | 1 MHz | NOTE 15: See above NOTE 22: See above |
| 2690 MHz – 2900 MHz | -50 | 1 MHz | NOTE xx: Applicable only for aerial UEs. |

Simulation Assumptions:

IQ-Image and LO leakage = -28 dBc

CIM3 = -60 dBc

PA calibration point was 20 MHz, 15 kHz, QPSK, DFT-S-OFMA, 100 RB at lower channel edge with 0.5 dB MPR

No IBE or EVM was simulated

Simulation Results

We simulated the A-MPR due to the ECC mask in the 2690-2900 MHz range. A-MPR is needed for 40 MHz channel BW, but not for 30 MHz. As the emissions generally reach further with larger channel BWs, we conclude that the channel BWs smaller than 30 MHz will not require A-MPR.

Only the 40 MHz channel BW requires A-MPR in the case of n38.

A complete set of results for the 40 MHz channel BW at the upper edge of the band can be found in the Annex B; below we present examples.

Figure 3: Examples of the A-MPR when the channel is placed at the upper edge of the band n38.

A-MPR proposal

Give the presented simulation results it is proposed to only consider A-MPR for channel bandwidths above 30MHz, i.e. 40 MHz for band n38.

RAN4 shall only consider A-MPR for 40MHz channel bandwidth for n38 due to the ECC requirements.

Since it is first meeting presenting the A-MPR study results as captured in full in Annex B, it may be premature to agree A-MPR before also considering input from others. Hence, it is proposed to at least consider the presented results for n3 for now.

RAN4 shall consider the presented A-MPR results when defining A-MPR for n38 due to the ECC requirements.

A-MPR Study and Proposals for n7

Additional spurious emission limits for UE co-existence are considered according to 38.101-1, as shown in Table 4. The additional requirement from ECC is added in the high-lighted row.

Table 4: Additional spurious emissions for UE co-existence (n7)

| Frequency range (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 2570 MHz – 2575 MHz | +1.6 | 5 MHz | NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 21: This requirement is applicable for any channel bandwidths up to 20MHz within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |
| 2575 MHz – 2595 MHz | -15.5 | 5 MHz | NOTE 15: See above NOTE 21: See above NOTE 26: See above |
| 2595 MHz – 2620 MHz | -40 | 1 MHz | NOTE 15: See above NOTE 21: See above |
| 2690 MHz – 2900 MHz | -50 | 1 MHz | NOTE xx: Applicable only for aerial UEs. |

Simulation Results

Given the presented results for n38 which have a relatively small need for A-MPR and the fact that n7 is even less impacted by the new requirements our assessment is that no A-MPR is needed for band n7.

No A-MPR is needed for band n7.

A-MPR proposal

RAN4 shall define no A-MPR for n7 due to the ECC requirements

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: It is part of the WID to specify additional OOBE requirements for aerial UEs for further restrictions in emission by the ECC decision.

Observation 2: Affected bands of the additional ECC OOBE limits are UL of both LTE and NR bands 3, 7 and 38.

Observation 3: Very large values of A-MPR, even above 20 dB, are required if the channel is placed at the lower edge of the band n3.

Observation 4: Smaller A-MPR is required for channel BWs from 30 to 50 MHz if the channel is at the upper edge of the band n3.

Observation 5: A-MPR can be avoided for some channel placements of specific channel bandwidths.

Proposal 1: RAN4 shall consider no A-MPR for CBWs 5, 10, 15, 20 and 25 MHz if placed with a minimum frequency offset from the lower edge of band n3 as shown in Table 3.

Proposal 2: RAN4 shall consider the presented A-MPR results when defining A-MPR for n3 due to the ECC requirements.

Observation 6: Only the 40 MHz channel BW requires A-MPR in the case of n38.

Proposal 3: RAN4 shall only consider A-MPR for 40MHz channel bandwidth for n38 due to the ECC requirements.

Proposal 4: RAN4 shall consider the presented A-MPR results when defining A-MPR for n38 due to the ECC requirements.

Observation 7: No A-MPR is needed for band n7.

Proposal 5: RAN4 shall define no A-MPR for n7 due to the ECC requirements

- References

- RP-230782, Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)

- RP-230025/R4-2304033, Reply to LS to 3GPP on ECC request for standardisation support related to ECC Decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands”, ETSI TC MSG/TFES

- R4-2308349, Discussion on identification of aerial UEs, Nokia, Nokia Shanghai Bell

- R4-2306642, WF on back off study (MPR/A-MPR) for UAV, Qualcomm Incorporated

- Annex A

Note that the A-MPR search is performed only up to 20 dB and some points marked with 20 dB require more A-MPR.

- Annex B