3GPP TSG-RAN WG4 Meeting #107 	R4-2308112

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On system parameters for sidelink in unlicensed spectrum

Agenda item:	8.31.2.1.1

Document for:	Approval

Introduction

The updated WID [1] of NR sidelink evolution from 3GPP TSG RAN Meeting #99-e includes an objective related to the support of operation of sidelink in unlicensed spectrum and also has an objective related to support of new introduced features (e.g. SL unlicensed) and frequency bands. Relevant objectives are copied below.

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel. - UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation [RAN4] - The exact frequency bands for both licensed and ITS-dedicated spectrum in FR1 and FR2 are to be determined based on company input during the WI. - The frequency band combinations is limited to the ITS-dedicated spectrum only (Band n47) for contiguous carrier aggregation. - Frequency bands for the unlicensed spectrum in FR1 are [n46 and n96/n102] (i.e., 5GHz and 6GHz) in accordance with corresponding national regulatory requirements. - Support of new sidelink frequency bands and band combinations should ensure coexistence between sidelink and Uu interface in the same and adjacent channels in licensed spectrum.

In this paper we discuss potential RAN4 impact on RF requirements for SL operating in unlicensed spectrum.

Discussion

Currently, sidelink requirements in RAN4 RF specifications are defined at least on the following specific sections for V2X communications:

- 5.2E	Operating bands for V2X

- 5.3E	Channel bandwidth for V2X

- 5.4E	Channel arrangement for V2X

- 6.2E	Transmit power for V2X

- 6.3E	Output power dynamics for V2X

- 6.4E	Transmit signal quality for V2X

- 6.5E	Output RF spectrum emissions for V2X

- 7.3E	Reference sensitivity for V2X

- 7.4E	Maximum input level for V2X

- 7.5E	Adjacent channel selectivity for V2X

- 7.6E	Blocking characteristics for V2X

- 7.7E	Spurious response for V2X

- 7.8E	Intermodulation characteristics for V2X

In NR-U, a number of RF requirements were introduced in RAN4 specifications in Rel-16. As an example, the following list shows impacted or introduced sections (refer to first large RAN4 CR from NR-U Rel-16, RP-202117):

- 5.2	Operating bands

- 5.2D	Operating bands for UL MIMO

- 6.2F	Transmit power for shared spectrum channel access

- 6.3F	Output power dynamics for shared spectrum channel access

- 6.4F	Transmit signal quality for shared spectrum channel access

- 6.5F	Output RF spectrum emissions for shared spectrum channel access

- 7.3F	Reference sensitivity for shared spectrum channel access

- 7.5F	Adjacent channel selectivity for shared spectrum channel access

- 7.6F	Blocking characteristics for shared spectrum channel access

- 7.7F	Spurious response for shared spectrum channel access

- 7.8F	Intermodulation characteristics for shared spectrum channel access

Since SL-U is being specified in Rel-18 to operate in same bands as supported by NR-U in FR-1, i.e., including Band n46 (5 GHz) and Band n96/n102 (6 GHz), it is expected that SL-U should also follow similar RF requirements introduced by NR-U in RAN4 specs or utilize NR-U requirements as baseline.

SL-U is being specified to operate in same bands as NR-U in FR1, thus SL-U may reuse NR-U RF requirements in large extent.

During RAN4#106-bis-e it was discussed if sidelink in unlicensed spectrum should be include to the specification as a new suffix or reuse either “E” as used for V2X or a new suffix “J” for sidelink on shared spectrum access.

Suffix E is already defined for V2X and thereby sidelink operation/feature.

Since sidelink in unlicensed spectrum is a sub-set of the feature resulting in the suffix “E” for V2X there is no obvious reason for introducing yet a new suffix only for this type of operation. Hence, it is beneficial to introduce any requirements specific for sidelink in unlicensed spectrum under existing suffix E.

Any requirements specific for sidelink in unlicensed spectrum shall be defined under existing suffix E in the RAN4 specification.

It was discussed how to handle V2X abbreviation. In RAN4 RRM specifications “V2X Sidelink” is used and in RAN1 specifications it is adopted “Sidelink” mainly. Either of these options can be used in RAN4 RF specifications.

RAN4 to capture the requirements for new operating bands in existing V2X sections, possibly renamed to “V2X Sidelink” or only “Sidelink”.

Regulation restrictions on application of unlicensed spectrum where also discussed and resulted in the following WF.

Way forward: Companies are encouraged to bring TP or CR for regulation restriction note for SL-U operating bands in TR 38.786 and/or TS 38.101-1 in the next meeting, and further refinement or discussion on below suggested wordings is allowed. - Client devices are prohibited from connecting directly to another client device in the frequency range 5850 – 5895 MHz and 5925 – 7125 MHz in USA, and in the frequency range 5925 – 7125 MHz in Canada per regulatory rules. Direct connection between client devices in the shared spectrum band is subject to country-specific conditions. - For bands n96 and/or n102, SL-U operation is excluded for vehicular UEs. - UAV BRID/DAA operation is not applicable in this band.” Companies can check and provide information if there are other region-wide regulations related to SL-U.

In our view, for scalability of the specification, TS does not need to mention the specific frequency range values, since the values should be available in the country/region specific standards.

RAN4 TS 38.101-1 may mention “Direct connection between client devices in the shared spectrum band is subject to country-specific conditions”, without need to specifically display the frequency range value for generality.

It should be noted that, as agreed in RAN#99, Sidelink carrier aggregation should be specified in Rel-18 only for ITS band in FR1 (Band n47), i.e., combination with SL-U new operating bands not overlapping with band n47 is not expected. Therefore, RAN4 should focus mainly on defining the requirements for single carrier SL-U operation.

RAN4 to focus mainly on single carrier SL-U operation.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: Suffix E is already defined for V2X and thereby sidelink operation/feature.

- Observation 2: Suffix E is already defined for V2X and thereby sidelink operation/feature.

Proposal 1: Any requirements specific for sidelink in unlicensed spectrum shall be defined under existing suffix E in the RAN4 specification.

- Proposal 2: RAN4 to capture the requirements for new operating bands in existing V2X sections, possibly renamed to “V2X Sidelink” or only “Sidelink”.

- Proposal 3: RAN4 TS 38.101-1 may mention “Direct connection between client devices in the shared spectrum band is subject to country-specific conditions”, without need to specifically display the frequency range value for generality.

- Proposal 4: RAN4 to focus mainly on single carrier SL-U operation.

- References

- RP-230077, WID revision: NR sidelink evolution, OPPO.