3GPP TSG-RAN WG4 Meeting # 107	R4-2308125

# Incheon, Korea, 22th - 26th, May, 2023

Source: 	Samsung, KT corporation

Title: 	Views on Rx requirements for band combination with 3Tx

Agenda Item:	7.29.3.2

Document for:	Discussion

- Introduction

A new WID was approved in RAN#98 while further revised in RAN#99 as [1], in which enhancement of 3Tx for inter-band CA/EN-DC with two bands is included as one objective. In last meeting, the group had some initial discussion on the applicable Tx/Rx requirement and spec impact, the agreements are captured in [2].

In this paper, we share our views on the remaining issues for Rx requirements.

- Discussion

2.1 General

In terms of PC2 IMD, 8 combos included in current WID require 3Tx for PC2, however 2Tx for PC2 has not been introduced yet for some of them according to Rel-18 v1.0.0, the status is summarized in below table.

| 2Tx for PC2 has been introduced | 2Tx for PC2 has NOT been introduced |
| --- | --- |
| CA_n28A-n41A | CA_n26A-n78A |
| CA_n28A-n78A | CA_n8A-n78A |
| CA_n41A-n71A | DC_40A_n78A |
| CA_n41A-n77A |  |
| DC_3A_n78A |  |

Observation 1: For PC2 CA_n26-n78, CA_n8-n78, DC_40-n78, currently there is no corresponding PC2 2Tx (23+23) MSD to be referred to according to Rel-18.1.0 spec.

2.2 Harmonic/Harmonic mixing

Proposal 1: In terms of harmonic mixing, same MSD requirements can be applied for a band combination with either 1Tx or 2Tx in aggressor band with same power class.

Proposal 2: In terms of cross band isolation, same MSD requirements can be applied for a band combination with either 1Tx or 2Tx in aggressor band with same power class.

2.3 PC2 IMD

Proposal 3: Same IMD MSD requirements can be applied for 1Tx-1Tx and 1Tx-2Tx PA configuration for the same PC2 2-band combination.

2.4 PC1.5 IMD

Proposal 4: For PC1.5 3Tx (PC3@FDD, PC1.5@TDD) IMD, adopt below test configuration.

| MSD test configuration | PC3 FDD band | PC1.5 TDD band |
| --- | --- | --- |
| Option 2 | 23 dBm | 23 dBm |

- 3. Conclusion

Observation 1: For PC2 CA_n26-n78, CA_n8-n78, DC_40-n78, currently there is no corresponding PC2 2Tx (23+23) MSD to be referred to.

Proposal 1: In terms of harmonic mixing, same MSD requirements can be applied for a band combination with either 1Tx or 2Tx in aggressor band with same power class.

Proposal 2: In terms of cross band isolation, same MSD requirements can be applied for a band combination with either 1Tx or 2Tx in aggressor band with same power class.

Proposal 3: Same IMD MSD requirements can be applied for 1Tx-1Tx and 1Tx-2Tx PA configuration for the same PC2 2-band combination.

Proposal 4: For PC1.5 3Tx (PC3@FDD, PC1.5@TDD) IMD, adopt below test configuration.

| MSD test configuration | PC3 FDD band | PC1.5 TDD band |
| --- | --- | --- |
| Option 2 | 23 dBm | 23 dBm |

- 4. Reference

[1] RP-230161, 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, OPPO, RAN#99

[2] R4-2306555, WF on Rx requirements for 3Tx UE, Apple, RAN4#106bis-e