# 3GPP TSG-RAN WG4 Meeting # 107                                R4-2308182

# Athens, Greece, February 27 – March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1558 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into TS 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R18_2BDL_xBUL-Core |  |  |  |  |  | Date: |  |  | 2023-05-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This big CR is to reflect the completed inter-band CA combinations are introduced into TS 38.101-1 from RAN4 #106bis meeting and #107 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The inter-band CA band combinations for 2 bands DL with up to 2 bands UL within FR1 completed in the following contributions are added from RAN4 #106bis and RAN4 #107 meetings. The endorsed draft CRs and approved TPs in RAN4 #106 meeting are listed: - R4-2306484 Addition of FR1 inter-band BCS 4 and 5 - R4-2306503 TP to TR 38.718-02-01 Addition of CA_n3A-n7A - R4-2306504 TP to TR 38.718-02-01 Addition of CA_n3A-n75A - R4-2306505 TP to TR 38.718-02-01 for CA_n46-n77 and DC_n46-n77 - R4-2306506 TP for TR 38.718-02-01 to include CA_n2A-n41A, CA_n2(2A)-n41A - R4-2306507 TP for TR 38.718-02-01 to include CA_n5A-n41A). - R4-2306508 TP for TR 38.718-02-01 to include CA_n2A-n71A, CA_n2(2A)-n71A - R4-2304752 TP for TR 38.718-02-01 to include CA_n7A-n12A - R4-2306509 TP for TR 38.718-02-01 to include CA_n7A-n71A - R4-2304754 TP for TR 38.718-02-01 to include CA_n12A-n41A - R4-2306510 TP for TR 38.718-02-01 to include CA_n12A-n78A, CA_n12A-n78(2A) - R4-2306511 TP to TR 38.718-02-01 Addition of CA_n1-n105 - R4-2304976 TP to TR 38.718-02-01 Addition of CA_n3-n105 - R4-2306512 TP to TR 38.718-02-01 Addition of CA_n40-n105 - R4-2306513 TP to TR 38.718-02-01 Addition of CA_n78-n105 - R4-2306514 TP to TR 38.718-02-01 Addition of CA_n7-n105 - R4-2304980 TP to TR 38.718-02-01 Addition of CA_n3-n102 and DC_n3-n102 - R4-2306515 TP to TR 38.718-02-01 Addition of CA_n77-n102 and DC_n77-n102 - R4-2306516 TP for TR38.718-02-01_CA_n3A-n41C with UL CA_n3A-n41C - R4-2304220 Draft CR for 38.101-1 to correct CA_n28-n75 - R4-2306517 draft CR to TS38.101-1_CA_n40A-n41C BCS4 and 5 The inter-band CA band combinations for 2 bands DL with up to 2 bands UL within FR1 completed in the following contributions are added from RAN4 #107 meetings. - R4-2310349 TP to TR 38.718-02-01 Addition of UE co-existence studies for 2 Uplink in one Intra-Band CA - R4-2310347 Acronym fix for CA_n48B-n66(2A) and CA_n40B-n77A - R4-2310350 Draft CR for TS 38.101-1 Support of CA_n2-n77 CA_n5-n77 CA_n12-n77 CA_n66-n77 CA_n71-n77 - R4-2310355 TP to TR 38.718-02-01 Addition of CA_n78(2A) UL to CA_n78-n102 - R4-2308164 Draft CR to TS38.101-1: CA_n40A-n79C and CA_n8A-n40A - R4-2308175 TP for TR38.718-02-01_CA_n8A-n79C with n79C - R4-2310411 Updated TP for TR 38.718-02-01 to intoduce BCS4 and BCS5 for CA_n5-n77 - R4-2310364 TP for TR 38.718-02-01 to intoduce BCS4 and BCS5 for CA_n78-n79 - R4-2310365 Draft CR for 38.101-1 to add CA_n3A-n40A_BCS4 and 5 - R4-2309392 draft CR 38.101-1 to add missing MSD CA_n7-n78 - R4-2307168 Draft CR to 38.101-1 CA_n41-n79 - R4-2310320 Addition of FR1 inter-band BCS 4 and 5 - R4-2309290 DraftCR for TS 38.101-1 to introduce CA_n12-n71 Requirements - R4-2310317 TP for 38.718-02-01 on MSD value due to cross band isolation for CA_n5A-n71A combination |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for above band combinations are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.2, 5.5A.3, 5.5B, 6.2A.1, 6.2A.4.2.3, 6.2B, 6.5A.3.2.3, 7.3A.2.4, 7.3A.3.2.1, 7.3A.4, 7.3A.5, 7.3A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, 5.2A.2.2-1 and Table 5.2A.2.3-1, where all operating bands are within FR1.

Table 5.2A.2-1: Void

Table 5.2A.2-2: Void

Table 5.2A.2-3: Void

5.2A.2.1	Inter-band CA (two bands)

Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n1-n3 | n1, n3 |  |
| CA_n1-n5 | n1, n5 |  |
| CA_n1-n7 | n1, n7 |  |
| CA_n1-n8 | n1, n8 |  |
| CA_n1-n18 | n1, n18 |  |
| CA_n1-n20 | n1, n20 |  |
| CA_n1-n26 | n1, n26 |  |
| CA_n1-n28 | n1, n28 |  |
| CA_n1-n38 | n1, n38 |  |
| CA_n1-n40 | n1, n40 |  |
| CA_n1-n411 | n1, n41 |  |
| CA_n1-n46 | n1, n46 |  |
| CA_n1-n67 | n1, n67 |  |
| CA_n1-n74 | n1, n74 |  |
| CA_n1-n75 | n1, n75 |  |
| CA_n1-n771 | n1, n77 | No |
| CA_n1-n781 | n1, n78 | No |
| CA_n1-n791 | n1, n79 | No |
| CA_n1-n102 | n1, n102 |  |
| CA_n1-n105 | n1, n105 |  |
| CA_n2-n5 | n2, n5 |  |
| CA_n2-n7 | n2, n7 |  |
| CA_n2-n12 | n2, n12 |  |
| CA_n2-n14 | n2, n14 |  |
| CA_n2-n29 | n2, n29 |  |
| CA_n2-n30 | n2, n30 |  |
| CA_n2-n41 | n2, n41 |  |
| CA_n2-n48 | n2, n48 |  |
| CA_n2-n66 | n2, n66 |  |
| CA_n2-n71 | n2, n71 |  |
| CA_n2-n77 | n2, n77 |  |
| CA_n2-n78 | n2, n78 |  |
| CA_n3-n5 | n3, n5 |  |
| CA_n3-n7 | n3, n7 |  |
| CA_n3-n8 | n3, n8 |  |
| CA_n3-n18 | n3, n18 |  |
| CA_n3-n20 | n3, n20 |  |
| CA_n3-n26 | n3, n26 |  |
| CA_n3-n28 | n3, n28 |  |
| CA_n3-n341 | n3, n34 |  |
| CA_n3-n38 | n3, n38 |  |
| CA_n3-n401 | n3, n40 | No |
| CA_n3-n411 | n3, n41 | No |
| CA_n3-n67 | n3, n67 |  |
| CA_n3-n74 | n3, n74 |  |
| CA_n3-n771 | n3, n77 | No |
| CA_n3-n781 | n3, n78 | No |
| CA_n3-n791 | n3, n79 | No |
| CA_n3-n102 | n3, n102 |  |
| CA_n3-n105 | n3, n105 |  |
| CA_n5-n7 | n5, n7 |  |
| CA_n5-n12 | n5, n12 |  |
| CA_n5-n14 | n5, n14 |  |
| CA_n5-n25 | n5, n25 |  |
| CA_n5-n28 | n5, n28 |  |
| CA_n5-n29 | n5, n29 |  |
| CA_n5-n30 | n5, n30 |  |
| CA_n5-n40 | n5, n40 |  |
| CA_n5-n41 | n5, n41 |  |
| CA_n5-n48 | n5, n48 |  |
| CA_n5-n66 | n5, n66 |  |
| CA_n5-n71 | n5, n71 |  |
| CA_n5-n771 | n5, n77 |  |
| CA_n5-n781 | n5, n78 | No |
| CA_n5-n791 | n5, n79 | No |
| CA_n7-n8 | n7, n8 |  |
| CA_n7-n12 | n7, n12 |  |
| CA_n7-n25 | n7, n25 |  |
| CA_n7-n26 | n7, n26 |  |
| CA_n7-n28 | n7, n28 |  |
| CA_n7-n40 | n7, n40 |  |
| CA_n7-n466 | n7, n46 |  |
| CA_n7-n66 | n7, n66 |  |
| CA_n7-n67 | n7, n67 |  |
| CA_n7-n71 | n7, n71 |  |
| CA_n7-n75 | n7, n75 |  |
| CA_n7-n77 | n7, n77 |  |
| CA_n7-n781 | n7, n78 |  |
| CA_n7-n79 | n7, n79 |  |
| CA_n7-n102 | n7, n102 |  |
| CA_n7-n105 | n7, n105 |  |
| CA_n8-n20 | n8, n20 |  |
| CA_n8-n28 | n8, n28 |  |
| CA_n8-n341 | n8, n34 |  |
| CA_n8-n38 | n8, n38 |  |
| CA_n8-n391 | n8, n39 |  |
| CA_n8-n401 | n8, n40 |  |
| CA_n8-n411 | n8, n41 | No |
| CA_n8-n751 | n8, n75 |  |
| CA_n8-n771 | n8, n77 |  |
| CA_n8-n781 | n8, n78 | No |
| CA_n8-n791 | n8, n79 | No |
| CA_n12-n25 | n12, n25 |  |
| CA_n12-n30 | n12, n30 |  |
| CA_n12-n41 | n12, n41 |  |
| CA_n12-n48 | n12, n48 |  |
| CA_n12-n66 | n12, n66 |  |
| CA_n12-n71 | n12, n71 |  |
| CA_n12-n77 | n12, n77 |  |
| CA_n12-n78 | n12, n78 |  |
| CA_n13-n25 | n13, n25 |  |
| CA_n13-n66 | n13, n66 |  |
| CA_n13-n77 | n13, n77 |  |
| CA_n14-n30 | n14, n30 |  |
| CA_n14-n66 | n14, n66 |  |
| CA_n14-n77 | n14, n77 |  |
| CA_n18-n28 | n18, n28 |  |
| CA_n18-n40 | n18, n40 |  |
| CA_n18-n411 | n18, n41 |  |
| CA_n18-n74 | n18, n74 |  |
| CA_n18-n7710 | n18, n77 |  |
| CA_n18-n7811 | n18, n78 |  |
| CA_n20-n282 | n20, n28 |  |
| CA_n20-n40 | n20, n40 |  |
| CA_n20-n67 | n20, n67 |  |
| CA_n20-n75 | n20, n75 |  |
| CA_n20-n78 | n20, n78 |  |
| CA_n24-n41 | n24, n41 |  |
| CA_n24-n48 | n24, n48 |  |
| CA_n24-n77 | n24, n77 |  |
| CA_n25-n29 | n25, n29 |  |
| CA_n25-n38 | n25, n38 |  |
| CA_n25-n41 | n25, n41 |  |
| CA_n25-n466 | n25, n46 |  |
| CA_n25-n48 | n25, n48 |  |
| CA_n25-n66 | n25, n66 |  |
| CA_n25-n71 | n25, n71 |  |
| CA_n25-n77 | n25, n77 |  |
| CA_n25-n78 | n25, n78 |  |
| CA_n25-n85 | n25, n85 |  |
| CA_n26-n2819 | n26, n28 |  |
| CA_n26-n66 | n26, n66 |  |
| CA_n26-n70 | n26, n70 |  |
| CA_n26-n77 | n26, n77 |  |
| CA_n26-n78 | n26, n78 |  |
| CA_n28-n34 | n28, n34 |  |
| CA_n28-n38 | n28, n38 |  |
| CA_n28-n39 | n28, n39 |  |
| CA_n28-n401 | n28, n40 |  |
| CA_n28-n411 | n28, n41 |  |
| CA_n28-n466 | n28, n46 |  |
| CA_n28-n50 | n28, n50 |  |
| CA_n28-n7112 | n28, n71 |  |
| CA_n28-n74 | n28, n74 |  |
| CA_n28-n752 | n28, n75 |  |
| CA_n28-n771 | n28, n77 | No |
| CA_n28-n781 | n28, n78 | No |
| CA_n28-n791 | n28, n79 |  |
| CA_n28-n94 | n28, n94 |  |
| CA_n28-n102 | n28, n102 |  |
| CA_n29-n30 | n29, n30 |  |
| CA_n29-n66 | n29, n66 |  |
| CA_n29-n70 | n29, n70 |  |
| CA_n29-n71 | n29, n71 |  |
| CA_n29-n77 | n29, n77 |  |
| CA_n30-n66 | n30, n66 |  |
| CA_n30-n77 | n30, n77 |  |
| CA_n34-n40 | n34, n40 |  |
| CA_n34-n419 | n34, n41 |  |
| CA_n34-n791 | n34, n79 |  |
| CA_n38-n409 | n38, n40 |  |
| CA_n38-n66 | n38, n66 |  |
| CA_n38-n781 | n38, n78 |  |
| CA_n38-n791 | n38, n79 |  |
| CA_n39-n40 | n39, n40 |  |
| CA_n39-n41 | n39, n41 | No |
| CA_n39-n791 | n39, n79 | No |
| CA_n40-n41 | n40, n41 |  |
| CA_n40-n771 | n40, n77 |  |
| CA_n40-n781 | n40, n78 |  |
| CA_n40-n791,4 | n40, n79 | No |
| CA_n40-n105 | n40, n105 |  |
| CA_n41-n481 | n41, n48 |  |
| CA_n41-n501 | n41, n50 |  |
| CA_n41-n66 | n41, n66 |  |
| CA_n41-n70 | n41, n70 |  |
| CA_n41-n711 | n41, n71 |  |
| CA_n41-n74 | n41, n74 |  |
| CA_n41-n771 | n41, n77 |  |
| CA_n41-n781 | n41, n78 |  |
| CA_n41-n791,3 | n41, n79 | No |
| CA_n41-n85 | n41, n85 |  |
| CA_n46-n481,6 | n46, n48 |  |
| CA_n46-n666 | n46, n66 |  |
| CA_n46-n771,6 | n46, n77 |  |
| CA_n46-n781,6 | n46, n78 |  |
| CA_n46-n9615,16,17,18 | n46, n96 |  |
| CA_n48-n539 | n48, n53 |  |
| CA_n48-n66 | n48, n66 |  |
| CA_n48-n70 | n48, n70 |  |
| CA_n48-n71 | n48, n71 |  |
| CA_n48-n7713,14,18 | n48, n77 |  |
| CA_n48-n961, 6 | n48, n96 |  |
| CA_n50-n78 | n50, n78 |  |
| CA_n66-n70 | n66, n70 |  |
| CA_n66-n71 | n66, n71 |  |
| CA_n66-n77 | n66, n77 |  |
| CA_n66-n78 | n66, n78 |  |
| CA_n66-n85 | n66, n85 |  |
| CA_n67-n78 | n67, n78 |  |
| CA_n70-n71 | n70, n71 |  |
| CA_n70-n77 | n70, n77 |  |
| CA_n70-n78 | n70, n78 |  |
| CA_n71-n77 | n71, n77 |  |
| CA_n71-n78 | n71, n78 |  |
| CA_n74-n771 | n74, n77 |  |
| CA_n74-n781 | n74, n78 |  |
| CA_n75-n781 | n75, n78 |  |
| CA_n76-n781 | n76, n78 |  |
| CA_n77-n787 | n77, n78 |  |
| CA_n77-n795,7 | n77, n79 |  |
| CA_n77-n85 | n77, n85 |  |
| CA_n77-n102 | n77, n102 |  |
| CA_n78-n795 | n78, n79 |  |
| CA_n78-n92 | n78, n92 |  |
| CA_n78-n94 | n78, n94 |  |
| CA_n78-n102 | n78, n102 |  |
| CA_n78-n105 | n78, n105 |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 3: The frequency range below 2506 MHz for Band n41 is not used in this combination. NOTE 4: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 5: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 6: The PCell is allocated in the licensed band in this combination. NOTE 7: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n78 or n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 10 The frequency range in band n77 is restricted for this band combination to 3520-3560 MHz, 3700-3800 MHz, 4000-4100 MHz. NOTE 11: The frequency range in band n78 is restricted for this band combination to 3520 -3560 MHz and 3700– 3800 MHz. NOTE 12: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 13: Simultaneous Rx/Tx capability for TDD combinations does not apply for UEs supporting band n48 with an n77 implementation. NOTE 14: The band n48 and n77 will synchronize their uplink and downlink configurations and in commonly TDD network coordination NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting CA_n46-n96. Same restrictions are applied when applicable NR CA configuration is part of a higher order configurations NOTE 16: The minimum requirements for intra-band non-contiguous CA/DC apply for CA_n46-n96 and related higher order CA/DC configurations. NOTE 17: The combination is not used alone as fall back mode of other band combinations in which UL in Band 48 is not used. NOTE 18: The minimum requirements for inter-band CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable CA configuration is a subset of a higher order CA configuration. NOTE 19: Uplink is only in n28 for CA_n26-n28 |  |  |

<< Next change >>

5.5A.3		Configurations for inter-band CA

Table 5.5A.3-1: Void

Table 5.5A.3-2: Void

Table 5.5A.3-3: Void

5.5A.3.1	Configurations for inter-band CA (two bands)

Table 5.5A.3.1-1a: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A | CA_n1A-n3A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n3B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
| CA_n1B-n3A | CA_n1A-n3A | n1 | CA_n1B_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n1 | CA_n1B_BCS0 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n3(2A) | CA_n1A-n3A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | CA_n3(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
| CA_n1(2A)-n3A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n3(2A) | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3(2A)_BCS1 |  |
| CA_n1(2A)-n3B | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n3 | CA_n3B_BCS0 |  |
| CA_n1A-n5A | CA_n1A-n5A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1(2A)-n5A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n1A-n7A | CA_n1A-n7A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n7B | CA_n1A-n7A CA_n7B | n1 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n1(2A)-n7A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n1A-n8A | CA_n1A-n8A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1(2A)-n8A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n1A-n18A | CA_n1A-n18A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
| CA_n1A-n20A | CA_n1A-n20A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
| CA_n1A-n26A | CA_n1A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n1A-n26(2A) | CA_n1A-n26A | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n1A-n28A | CA_n1A-n28A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n1(2A)-n28A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n1A-n38A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1(2A)-n38A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n1A-n40A | CA_n1A-n40A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n1 | 5, 10, 15, 20 | 1 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n40B | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n40 | CA_n40B_BCS0 |  |
| CA_n1A-n41A | CA_n1A-n41A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n1A-n46A | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 10, 20, 40, 60, 80 |  |
| CA_n1A-n46C | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n1A-n46D | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n1A-n67A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n1A-n74A | CA_n1A-n74A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n1A-n75A | - | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n77A | CA_n1A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n77(2A) | CA_n1A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n1A-n77(3A) | CA_n1A-n77A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n1A-n78A | n18 n788 CA_n1A-n78A8 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n1 | 5, 10, 15, 20 | 3 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  | CA_n78(2A) CA_n1A-n78A | n1 | 5, 10, 15, 20 | 2 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n1A-n78C | CA_n78C CA_n1A-n78A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 3 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n1(2A)-n78A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n79A | CA_n1A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1A-n79C | CA_n1A-n79A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n1(2A)-n79A | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n79 | 40, 60, 80, 100 |  |
|  |  | n1 | CA_n1(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n1(2A)-n79C | - | n1 | CA_n1(2A)_BCS0 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n1 | CA_n1(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n1A-n102A | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n102(2A) | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n102B | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n102C | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n102D | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n102E | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n1A-n105A | CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

Table 5.5A.3.1-1b: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n5A | CA_n2A-n5A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n2A-n5B | CA_n2A-n5A CA_n5B | n2 | 5, 10, 15, 20 | 0 |
|  |  | n5 | CA_n5B_BCS0 |  |
| CA_n2(2A)-n5A | CA_n2A-n5A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n2A-n7A | CA_n2A-n7A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n2A-n7(2A) | CA_n2A-n7A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7(2A)_BCS0 |  |
| CA_n2A-n12A | CA_n2A-n12A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n2(2A)-n12A | CA_n2A-n12A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n2A-n14A | CA_n2A-n14A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n2(2A)-n14A | CA_n2A-n14A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n2A-n29A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
| CA_n2(2A)-n29A | - | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n29 | 5, 10 |  |
| CA_n2A-n30A | CA_n2A-n30A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n2(2A)-n30A | CA_n2A-n30A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n2A-n38A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n38 | 5, 10, 15, 20, 40 |  |
| CA_n2A-n41A | CA_n2A-n41A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n2(2A)-n41A | CA_n2A-n41A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n48A | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701, 801, 901, 1001 |  |
| CA_n2A-n48B | CA_n48B CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n2A-n48C | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n2A-n48(2A) | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n2A-n48(A-B) | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n2A-n48(A-C) | CA_n2A-n48A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(A-C)_BCS0 |  |
| CA_n2A-n66A | - | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2(2A)-n66A | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n2A-n66(2A) | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n66(2A) | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n2(2A)-n66(3A) | CA_n2A-n66A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n66(3A) | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n2A-n66B | CA_n2A-n66A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
| CA_n2A-n71A | CA_n2A-n71A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2(2A)-n71A | CA_n2A-n71A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2A-n77A | n778,9 CA_n2A-n77A8 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2A-n77(2A) | n778,9 CA_n2A-n77A8 CA_n77(2A)7 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n77B | - | n2 | n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n2A-n77C | n778, 9 CA_n77C CA_n2A-n77A8 | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
| CA_n2(2A)-n77A | n778, 9 CA_n2A-n77A8 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2(2A)-n77B | - | n2 | CA_n2(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n2(2A)-n77(2A) | n778 CA_n2A-n77A8 CA_n77(2A)7 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n2A-n77(3A) | CA_n2A-n77A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n2(2A)-n77C | n778, 9 CA_n2A-n77A8 | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n2A-n78A | n78 CA_n2A-n78A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n2 | See n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n2A-n78(2A) | CA_n2A-n78A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n2 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n2 | See n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |

Table 5.5A.3.1-1c: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n5A | CA_n3A-n5A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n3(2A)-n5A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n5 | 5, 10, 15, 20 |  |
| CA_n3A-n7A | CA_n3A-n7A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3A-n7B | CA_n3A-n7A CA_n7B | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n3(2A)-n7A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n3 | CA_n3(2A)_BCS1 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3B-n7A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n3B-n7B | CA_n3A-n7A CA_n7B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n8A | CA_n3A-n8A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n3(2A)-n8A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n3A-n18A | CA_n3A-n18A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
| CA_n3A-n20A | CA_n3A-n20A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
| CA_n3A-n26A | CA_n3A-n26A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n3A-n26(2A) | CA_n3A-n26A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3B-n26A | CA_n3A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
| CA_n3B-n26(2A) | CA_n3A-n26A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n3A-n28A | CA_n3A-n28A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 | 3 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n3(2A)-n28A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n3A-n34A | CA_n3A-n34A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n34 | 5, 10, 15 |  |
| CA_n3A-n38A | CA_n3A-n38A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n38 | 5, 10, 15, 20, 40 |  |
| CA_n3B-n38A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3(2A)-n38A | - | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n3A-n40A | CA_n3A-n40A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 2 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41A | n418,9 CA_n3A-n41A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 3 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n41B | CA_n3A-n41A | n3 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41B_BCS0 |  |
| CA_n3A-n41C | CA_n41C CA_n3A-n41A CA_n3A-n41C | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n3A-n41(2A) | CA_n3A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | CA_n41(2A)_BCS0 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 |  |
| CA_n3A-n67A | - | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n3A-n74A | CA_n3A-n74A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n3A-n75A | - | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  | CA_n3A-n75A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n77A | n778,9 CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n77(2A) | CA_n77(2A) CA_n3A-n77A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35,40 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n3A-n77(3A) | CA_n3A-n77A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
| CA_n3A-n78A | n38 n788 CA_n3A-n78A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n78C | CA_n78C CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  | CA_n3A-n78A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n3A-n78(2A) | CA_n3A-n78A CA_n78(2A) | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n3(2A)-n78A | - | n3 | CA_n3(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n3 | CA_n3(2A)_BCS1 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n78A | CA_n3B CA_n3A-n78A | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n78(2A) | CA_n3A-n78A CA_n3B | n3 | CA_n3B_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n79A | CA_n3A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3(2A)-n79A | CA_n3A-n79A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | CA_n3(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3A-n79C | CA_n79C CA_n3A-n79A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3(2A)-n79C | CA_n3A-n79A | n3 | CA_n3(2A)_BCS1 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n3 | CA_n3(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3B-n79A | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n3 | CA_n3B_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n3B-n79C | - | n3 | CA_n3B_BCS0 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n3 | CA_n3B_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n3A-n102A | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n3A-n102(2A) | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n3A-n102B | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n3A-n102C | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n3A-n102D | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n3A-n102E | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n3A-n105A | CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

Table 5.5A.3.1-1d: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n7A | CA_n5A-n7A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n5A-n7B | CA_n5A-n7A CA_n7B | n5 | 5, 10, 15, 20 | 0 |
|  |  | n7 | CA_n7B_BCS0 |  |
| CA_n5A-n12A | CA_n5A-n12A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n5A-n14A | CA_n5A-n14A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n14 | 5, 10 |  |
| CA_n5A-n25A | CA_n5A-n25A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n25(2A) | CA_n5A-n25A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
| CA_n5A-n28A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n5A-n29A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n29 | 5, 10 |  |
| CA_n5A-n30A | CA_n5A-n30A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n5A-n40A | CA_n5A-n40A | n5 | 5, 10, 15, 20, 251 | 0 |
|  |  | n40 | 55, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80,90,100 |  |
| CA_n5A-n41A | CA_n5A-n41A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48A | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n48(2A) | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(2A)_BCS1 |  |
| CA_n5A-n48B | CA_n48B CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n5A-n48C | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n5A-n48(A-B) | CA_n5A-n48A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(A-B)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n48 | CA_n48(A-B)_BCS1 |  |
| CA_n5A-n66A | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5B-n66A | CA_n5A-n66A CA_n5B | n5 | CA_n5B_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n5A-n66(2A) | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n66(3A) | CA_n5A-n66A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n5B-n66(2A) | CA_n5A-n66A CA_n5B | n5 | CA_n5B_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n5A-n71A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n5A-n77A | n778, 9 CA_n5A-n77A8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n77B | - | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n5A-n77(2A) | n778 CA_n5A-n77A8 CA_n77(2A) | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n5A-n77(3A) | CA_n77(2A) CA_n5A-n77A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n5(2A)-n77A | n778, 9 CA_n5A-n77A8 | n5 | CA_n5(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5A-n77C | 778, 9 CA_n5A-n77A8 CA_n77C | n5 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5(2A)-n77C | n778,9 CA_n5A-n77A8 | n5 | CA_n5(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | CA_n5(2A)_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5B-n77A | n778 CA_n5A-n77A8 CA_n5B | n5 | CA_n5B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n5B-n77C | n778 CA_n5A-n77A8 CA_n5B CA_n77C | n5 | CA_n5B_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n5 | CA_n5B_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n5A-n78A | n788 CA_n5A-n78A8 | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n78(2A) | CA_n5A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n5A-n78C | CA_n5A-n78A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n5 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78C_BCS1 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n5A-n79A | CA_n5A-n79A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n79C | CA_n5A-n79A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

Table 5.5A.3.1-1e: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n8A | CA_n7A-n8A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
| CA_n7A-n12A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |
| CA_n7A-n25A | CA_n7A-n25A | n7 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n25(2A) | CA_n7A-n25A | n7 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
| CA_n7(2A)-n25A | CA_n7A-n25A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7(2A)-n25(2A) | CA_n7A-n25A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n25 | CA_n25(2A)_BCS0 |  |
| CA_n7A-n26A | CA_n7A-n26A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n7A-n26(2A) | CA_n7A-n26A | n7 | 5, 10, 15, 20, 25, 30, 35, 40, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n7B-n26A | CA_n7A-n26A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n7B-n26(2A) | CA_n7A-n26A | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
| CA_n7A-n28A | CA_n7A-n28A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n7B-n28A | CA_n7A-n28A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
| CA_n7A-n40A | CA_n7A-n40A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n46A | CA_n7A-n46A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
| CA_n7A-n46C | CA_n7A-n46A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n7A-n46D | CA_n7A-n46A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n7A-n46(2A) | CA_n7A-n46A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
| CA_n7A-n66A | CA_n7A-n66A | n7 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 10, 15, 20, 40 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7A-n66(2A) | CA_n7A-n66A | n7 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7(2A)-n66A | CA_n7A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n7(2A)-n66(2A) | CA_n7A-n66A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n7A-n67A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n7A-n71A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n7A-n75A | - | n7 | 10, 15, 20 | 0 |
|  |  | n75 | 5, 10, 15, 20 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n77A | CA_n7A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7(2A)-n77A | CA_n7A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7A-n77(2A) | CA_n7A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7(2A)-n77(2A) | CA_n7A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n7A-n77(3A) | CA_n77(2A) CA_n7A-n77A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7(2A)-n77(3A) | CA_n77(2A)CA_n7A-n77A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n7A-n78A | n788 CA_n7A-n78A8 | n7 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n78C | CA_n7A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n7B-n78A | n788 CA_n7A-n78A8 CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n7B-n78(2A) | CA_n7A-n78A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n7A-n78(2A) | CA_n7A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A) CA_n7A-n78A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n7(2A)-n78A | CA_n7A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n7 | CA_n7(2A)_BCS0 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n7 | CA_n7(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7(2A)-n78(2A) | CA_n7A-n78A | n7 | CA_n7(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n7 | CA_n7(2A)_BCS0 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n7 | CA_n7(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n7A-n79A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n7A-n79C | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n7A-n102A | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n7A-n102(2A) | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n7A-n102B | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n7A-n102C | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n7A-n102D | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n7A-n102E | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n7A-n105A | CA_n7A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n8A-n20A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n20 | 5, 10, 15, 20 |  |
| CA_n8A-n28A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n8A-n34A | CA_n8A-n34A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n34 | 5, 10, 15 |  |
| CA_n8A-n38A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n8A-n39A | CA_n8A-n39A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n8A-n40A | CA_n8A-n40A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n41A | CA_n8A-n41A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n41C | CA_n8A-n41A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n8A-n75A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n75 | 5, 10, 15, 20 |  |
|  |  | n8 | 5, 10,15, 20 | 1 |
|  |  | n75 | 5, 10,15, 20, 25, 30, 40, 50 |  |
| CA_n8A-n77A | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n8A-n77(2A) | - | n8 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n8A-n78A | CA_n8A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | 5, 10, 15, 20 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n8A-n78C | CA_n8A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n8A-n78(2A) | CA_n8A-n78A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n8A-n79A | CA_n8A-n79A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n79 | 10, 20, 40, 50, 60, 80, 100 |  |
| CA_n8A-n79C | CA_n8A-n79A CA_n79C | n8 | 5, 10, 15, 20 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |

Table 5.5A.3.1-1f: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n12A-n25A | CA_n12A-n25A | n12 | 5, 10, 15 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n12A-n30A | CA_n12A-n30A | n12 | 5, 10, 15 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n12A-n41A | - | n12 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n48A | - | n12 | 5, 10, 15 | 0 |
|  |  | n48 | 10, 15, 20, 30, 40 |  |
| CA_n12A-n66A | CA_n12A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
| CA_n12A-n66(2A) | CA_n12A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n12A-n66(3A) | CA_n12A-n66A | n12 | 5, 10, 15 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n12A-n71A | - | n12 | 5, 10, 15 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n12A-n77A | n778, 9 CA_n12A-n77A8 | n12 | 5, 10, 15 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n12 | n12 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n12A-n77B | - | n12 | n12 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n12A-n77C | - | n12 | n12 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n12A-n77(2A) | n778, 9 CA_n12A-n77A8 | n12 | 5, 10, 15 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n12A-n78A | CA_n12A-n78A | n12 | 5, 10, 15 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n78(2A) | CA_n12A-n78A | n12 | 5, 10, 15 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n13A-n25A | CA_n13A-n25A | n13 | 5, 10 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n13A-n66A | CA_n13A-n66A | n13 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n13 | 5, 10, | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n13A-n77A | n778, 9 CA_n13A-n77A8 | n13 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n13A-n77(2A) | CA_n77(2A) CA_n13A-n77A | n13 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n14A-n30A | CA_n14A-n30A | n14 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n14A-n66A | CA_n14A-n66A | n14 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n14A-n66(2A) | CA_n14A-n66A | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n14A-n66(3A) | CA_n14A-n66A | n14 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n14A-n77A | n778, 9 CA_n14A-n77A8 | n14 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n14A-n77(2A) | n778, 9 CA_n14A-n77A8 | n14 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n18A-n28A | CA_n18A-n28A | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
| CA_n18A-n40A | CA_n18A-n40A | n18 | 5, 10, 15 | 0 |
|  |  | n40 | 10, 15, 20, 30, 40 |  |
| CA_n18A-n41A | CA_n18A-n41A | n18 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| CA_n18A-n74A | CA_n18A-n74A | n18 | 5, 10, 15 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n18A-n77A | CA_n18A-n77A | n18 | 5, 10, 15 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n18A-n77(2A) | CA_n18A-n77A | n18 | 5, 10, 15 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n18A-n77(3A) | CA_n18A-n77A | n18 | 5, 10, 15 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n18A-n78A | CA_n18A-n78A | n18 | 5, 10, 15 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n18A-n78(2A) | CA_n18A-n78A | n18 | 5, 10, 15 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n18 | See n18 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |

Table 5.5A.3.1-1g: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n20A-n28A | CA_n20A-n28A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n20 | 5, 10, 15, 20 | 1 |
|  |  | n28 | 5, 10, 15, 20, 30 |  |
| CA_n20A-n40A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n20A-n67A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |
| CA_n20A-n75A | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n75 | 5, 10, 15, 20 |  |
|  |  | n20 | 5, 10,15, 20 | 1 |
|  |  | n75 | 5, 10,15, 20, 25, 30, 40, 50 |  |
| CA_n20A-n78A | CA_n20A-n78A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n20A-n78C | - | n20 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n24A-n41A | CA_n24A-n41A | n24 | 5, 10 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n24 | See n24 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n24A-n41(2A) | CA_n24A-n41A | n24 | 5, 10 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n24 | See n24 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 |  |
| CA_n24A-n48A | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n24A-n48B | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48B_BCS1 |  |
| CA_n24A-n48(2A) | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n24A-n48(3A) | CA_n24A-n48A | n24 | 5, 10 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n24A-n77A | CA_n24A-n77A | n24 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n24 | See n24 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n24A-n77C | CA_n24A-n77A | n24 | 5, 10 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n24A-n77(2A) | CA_n24A-n77A | n24 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n24 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n25A-n29A | - | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n29 | 5, 10 |  |
| CA_n25A-n38A | CA_n25A-n38A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25(2A)-n38A | CA_n25A-n38A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n25A-n41A | n418, 9 CA_n25A-n41A8 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41A | n418, 9 CA_n25A-n41A8 | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS1 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n41C | n418, 9 CA_n25A-n41A8 CA_n41C8 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | CA_n41C_BCS2 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |
| CA_n25(2A)-n41(2A) | n418, 9 CA_n25A-n41A 8 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n41 | CA_n41(2A)_BCS3 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
| CA_n25A-n41C | n418, 9 CA_n25A-n41A8 CA_n41C8 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C BCS 4 and 5 |  |
| CA_n25A-n41(2A) | n418, 9 CA_n25A-n41A8 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | CA_n41(2A)_BCS1 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | CA_n41(2A)_BCS3 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 |  |
| CA_n25A-n41(3A) | n418, 9 CA_n25A-n41A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(3A)_BCS0 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 |  |
| CA_n25A-n41(A-C) | n418, 9 CA_n25A-n41A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  | CA_n41C8 | n41 | CA_n41(A-C)_BCS0 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(A-C)BCS 4 and 5 |  |
| CA_n25(2A)-n41(3A) | CA_n25A-n41A | n25 | CA_n25(2A)_ BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 |  |
| CA_n25(2A)-n41(A-C) | CA_n41C CA_n25A-n41A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 |  |
| CA_n25A-n46A | - | n25 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
| CA_n25A-n48A | CA_n25A-n48A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n25A-n48(2A) | CA_n25A-n48A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n25A-n48C | CA_n25A-n48A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n25A-n66A | CA_n25A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n66(2A) | CA_n25A-n66A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n66A | CA_n25A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | 10, 15, 20, 30, 40 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | CA_n25(2A)_BCS1 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n66(2A) | CA_n25A-n66A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS1 | 2 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n25A-n71A | CA_n25A-n71A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n71B | CA_n25A-n71A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25A-n71(2A) | CA_n25A-n71A | n25 | 5, 10, 15, 20 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n71A | CA_n25A-n71A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n71(2A) | CA_n25A-n71A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n71B | CA_n25A-n71A | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n25A-n77A | n778,9 CA_n25A-n77A8 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n77(2A) | n778,9 CA_n77(2A) CA_n25A-n77A8 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25A-n77(3A) | CA_n77(2A) CA_n25A-n77A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n25(2A)-n77A | n778,9 CA_n25A-n77A8 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n77(2A) | n778,9 CA_n25A-n77A8 | n25 | CA_n25(2A)_BCS1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n25(2A)-n77(3A) | CA_n25(2A) CA_n77(2A) CA_n25A-n77A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n25A-n78A | CA_n25A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25A-n78(2A) | CA_n25A-n78A | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n25(2A)-n78A | CA_n25A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | CA_n25(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n78(2A) | CA_n25A-n78A | n25 | CA_n25(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n25 | CA_n25(2A)_BCS0 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n25 | CA_n25(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n25A-n85A | CA_n25A-n85A | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n85A | CA_n25A-n85A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

Table 5.5A.3.1-1h: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n28A | - | n26 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n26 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n26A-n66A | CA_n26A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n26A-n66(2A) | CA_n26A-n66A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n26A-n70A | CA_n26A-n70A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n26A-n77A | CA_n26A-n77A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n26A-n78A | CA_n26A-n78A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n26(2A)-n78A | CA_n26A-n78A | n26 | CA_n26(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n26A-n78(2A) | CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n26(2A)-n78(2A) | CA_n26A-n78A | n26 | CA_n26(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n28A-n34A | CA_n28A-n34A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n34 | 5, 10, 15 |  |
| CA_n28A-n38A | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n28A-n39A | CA_n28A-n39A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n28A-n40A | CA_n28A-n40A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n28A-n40B | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n40 | CA_n40B_BCS0 |  |
| CA_n28A-n41A | n418 CA_n28A-n41A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 30 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n41B | CA_n28A-n41A | n28 | 5, 10 | 0 |
|  |  | n41 | CA_n41B_BCS0 |  |
| CA_n28A-n41C | CA_n28A-n41A CA_n41C | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n28A-n46A | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 20, 40, 60, 80 |  |
| CA_n28A-n46C | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n28A-n46D | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n28A-n46(2A) | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |
| CA_n28A-n50A | CA_n28A-n50A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n50 | 5, 10, 15, 20, 40, 50, 60, 801 |  |
| CA_n28A-n71A | - | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n28A-n74A | CA_n28A-n74A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n28A-n75A | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n75 | 5, 10, 15, 20 |  |
|  |  | n28 | 5, 10, 15, 20 | 1 |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 2 |
|  |  | n75 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
| CA_n28A-n77A | CA_n28A-n77A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n77(2A) | CA_n77(2A) CA_n28A-n77A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n28A-n77(3A) | CA_n28A-n77A | n28 | 5, 10 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(3A)_BCS4 and 5 |  |
| CA_n28A-n78A | n788 CA_n28A-n78A8 | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n28 | 5, 10, 15, 20, 30 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n78C | CA_n28A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n28A-n78(2A) | CA_n78(2A) CA_n28A-n78A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n28 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n28A-n79A | n798 CA_n28A-n79A8 | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n28A-n79C | CA_n79C | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n28A-n94A | - | n28 | 5, 10, 15, 20 | 0 |
|  |  | n94 | 5, 10, 15, 20 |  |
| CA_n28A-n102A | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n28A-n102(2A) | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n28A-n102B | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n28A-n102C | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n28A-n102D | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n28A-n102E | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n29A-n30A | - | n29 | 5, 10 | 0 |
|  |  | n30 | 5, 10 |  |
| CA_n29A-n66A | - | n29 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n29 | 5, 10 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n29A-n66B | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
| CA_n29A-n66(2A) | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n29 | 5, 10 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n29A-n66(3A) | - | n29 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n29A-n70A | - | n29 | 5, 10 | 0 |
|  |  | n70 | 5, 10, 15, 201,, 251 |  |
| CA_n29A-n71A | - | n29 | 5, 10 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n29A-n77A | n778, 9 | n29 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n29A-n77(2A) | n778, 9 | n29 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |

Table 5.5A.3.1-1i: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n30A-n66A | CA_n30A-n66A | n30 | 5, 10 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n30A-n66(2A) | CA_n30A-n66A | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n30A-n66(3A) | CA_n30A-n66A | n30 | 5, 10 | 0 |
|  |  | n66 | CA_n66(3A)_BCS0 |  |
| CA_n30A-n77A | n778, 9 CA_n30A-n77A8 | n30 | 5, 10 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n30A-n77(2A) | n778, 9 CA_n77(2A) CA_n30A-n77A8 | n30 | 5, 10 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n34A-n40A | CA_n34A-n40A | n34 | 5, 10, 15 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
| CA_n34A-n41A | CA_n34A-n41A | n34 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30 ,40 ,50, 60, 70, 80, 90, 100 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n41C | CA_n41C CA_n34A-n41A CA_n34A-n41C | n34 | 5, 10, 15 | 0 |
|  |  | n41 | CA_n41C_BCS1 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n34A-n79A | CA_n34A-n79A | n34 | 5, 10, 15 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n34A-n79C | CA_n34A-n79A | n34 | 5, 10, 15 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n38A-n40A | - | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n38A-n66A | CA_n38A-n66A | n38 | 5, 10, 15, 20 | 0 |
|  |  | n66 | 5, 10, 15, 20, 30, 40 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n38A-n66(2A) | CA_n38A-n66A | n38 | 5, 10, 15, 20 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
| CA_n38A-n71A | - | n38 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n38A-n78A | CA_n38A-n78A | n38 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n38 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n38 | See n38 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n38A-n78(2A) | CA_n38A-n78A | n38 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  |  | n38 | 5, 10, 15, 20 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n38 | See n38 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n38A-n79A | - | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n38 | See n38 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n38A-n79C | - | n38 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n38 | See n38 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n39A-n40A | CA_n39A-n40A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |
| CA_n39A-n41A | CA_n39A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n39A-n41C | CA_n41C CA_n39A-n41A CA_n39A-n41C | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n39A-n41(2A) | CA_n39A-n41A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41(2A)_BCS0 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 |  |
| CA_n39A-n79A | CA_n39A-n79A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n39A-n79C | CA_n79C CA_n39A-n79A | n39 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n40A-n41A | n418 CA_n40A-n41A8 | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
| CA_n40A-n41C | CA_n41C12 CA_n40A-n41A CA_n40A-n41C12 | n40 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |
| CA_n40A-n77A | CA_n40A-n77A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n40A-n77(2A) | CA_n40A-n77A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n40A-n77C | - | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n40B-n77A | n778 CA_n40A-n77A | n40 | CA_n40B_BCS1 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 704, 80, 904, 100 |  |
| CA_n40B-n77(2A) | CA_n40A-n77A | n40 | CA_n40B_BCS1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n40B-n77C | CA_n40A-n77A | n40 | CA_n40B_BCS1 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n40A-n78A | CA_n40A-n78A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n40A-n78(2A) | CA_n40A-n78A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n40A-n78C | CA_n40A-n78A | n40 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n40B-n78A | - | n40 | CA_n40B_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  | CA_n40A-n78A | n40 | CA_n40B_BCS1 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n40B-n78(2A) | CA_n40A-n78A | n40 | CA_n40B_BCS1 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n40B-n78C | CA_n40A-n78A | n40 | CA_n40B_BCS1 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n40A-n79A | CA_n40A-n79A | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n40 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n40A-n79C | CA_n40A-n79A CA_n79C | n40 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n40A-n105A | CA_n40A-n105A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

Table 5.5A.3.1-1j: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n48A | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | See n48 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n48B | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n41A-n48C | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n48 | CA_n48C_BCS1 |  |
| CA_n41A-n48(2A) | CA_n41A-n48A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
| CA_n41C-n48A | CA_n41A-n48A | n41 | CA_n41C_BCS2 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n41C-n48B | CA_n41A-n48A | n41 | CA_n41C_BCS2 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n41C-n48C | CA_n41A-n48A | n41 | CA_n41C_BCS2 | 0 |
|  |  | n48 | CA_n48C_BCS1 |  |
| CA_n41(2A)-n48A | CA_n41A-n48A | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n48 | See n48 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n48B | CA_n41A-n48A | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n48 | CA_n48B_BCS2 |  |
| CA_n41(2A)-n48C | CA_n41A-n48A | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n48 | CA_n48C_BCS1 |  |
| CA_n41(2A)-n48(2A) | CA_n41A-n48A | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
|  |  | n41 | CA_n41(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 |  |
| CA_n41A-n50A | CA_n41A-n50A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n50 | 5, 10, 15, 20, 40, 50, 60, 801 |  |
| CA_n41A-n66A | n418,9 CA_n41A-n66A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n66A | n418, 9 CA_n41A-n66A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n41 | CA_n41(2A)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n66(2A) | n418, 9 CA_n41A-n66A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41C-n66A | n418, 9 CA_n41A-n66A8 CA_n41C | n41 | CA_n41C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n66(2A) | n418, 9 CA_n41A-n66A8 CA_n41C | n41 | CA_n41C_BCS2 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n66(2A) | n418, 9 CA_n41A-n66A8 | n41 | CA_n41(2A)_BCS3 | 0 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41(3A)-n66A | n418, 9 CA_n41A-n66A8 | n41 | CA_n41(3A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40, |  |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n66(2A) | CA_n41A-n66A | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41(A-C)-n66A | n418, 9 CA_n41C8 CA_n41A-n66A8 | n41 | CA_n41(A-C)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n66(2A) | CA_n41C CA_n41A-n66A | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 |  |
| CA_n41A-n70A | CA_n41A-n70A | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n41A-n71A | n418,9 CA_n41A-n71A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n71B | n418,9 CA_n41A-n71A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41A-n71(2A) | n418,9 CA_n41A-n71A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41C-n71A | n418, 9 CA_n41A-n71A8 CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71(2A) | n418,9 CA_n41A-n71A8 CA_n41C8 | n41 | CA_n41C_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n71A | n418, 9 CA_n41A-n71A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(2A)_BCS3 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n71(2A) | n418,9 CA_n41A-n71A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41(2A)-n71B | n418,9 CA_n41A-n71A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n41 | CA_n41(2A)_BCS1 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41(3A)-n71A | n418,9 CA_n41A-n71A8 | n41 | CA_n41(3A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n71B | CA_n41A-n71A | n41 | CA_n41(3A)_BCS4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS4 and 5 |  |
| CA_n41(3A)-n71(2A) | CA_n41A-n71A | n41 | CA_n41(3A)_BCS4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS4 and 5 |  |
| CA_n41(A-C)-n71A | n418,9 CA_n41A-n71A8 CA_n41C8 | n41 | CA_n41(A-C)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  | n418,9 CA_n41A-n71A CA_n41C | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n71B | CA_n41A-n71A CA_n41C | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41(A-C)-n71(2A) | CA_n41A-n71A CA_n41C | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41C-n71B | n418,9 CA_n41A-n71A8 CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n41 | CA_n41C_BCS1 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n41A-n74A | CA_n41A-n74A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n74 | 5, 10, 15, 20 |  |
| CA_n41A-n77A | n418,9 n778,9 CA_n41A-n77A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41B-n77A | CA_n41A-n77A | n41 | CA_n41B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n41(2A)-n77A | n418,9 n778,9 CA_n41A-n77A8 | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(3A)-n77A | n418,9 n778,9 CA_n41A-n77A8 | n41 | CA_n41(3A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(3A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(A-C)-n77A | n418,9 n778,9 CA_n41C CA_n41A-n77A8 | n41 | CA_n41(A-C)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41(A-C)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n77A | n418,9 n778,9 CA_n41A-n77A8 CA_n41C8 | n41 | CA_n41C_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n77(2A) | n418,9 n778,9 CA_n41A-n77A8 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41C-n77(2A) | n418,9 n778,9 CA_n41A-n77A8 CA_n41C | n41 | CA_n41C_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n41A-n77(3A) | CA_n41A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(3A)_BCS 4 and 5 |  |
| CA_n41(2A)-n77(2A) | - | n41 | CA_n41(2A)_BCS1 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  | CA_n41A-n77A | n41 | CA_n41(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A) BCS 4 and 5 |  |
| CA_n41A-n77C | CA_n41A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n41A-n78A | CA_n41A-n78A | n41 | 10, 15, 20, 40, 50, 60, 80, 100 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n78(2A) | CA_n41A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n41A-n78C | CA_n41A-n78A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |
| CA_n41A-n79A | n418 n798 CA_n41A-n79A8 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 2 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n79C | CA_n41A-n79A CA_n79C | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n41C-n79A | CA_n41A-n79A CA_n41C | n41 | CA_n41C_BCS0 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n79C | CA_n41C CA_n79C CA_n41A-n79A | n41 | CA_n41C_BCS0 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  |  | n41 | CA_n41C_BCS4 and 5 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |
| CA_n41A-n85A | CA_n41A-n85A | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n85A | CA_n41A-n85A CA_n41C | n25 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n85A | CA_n41A-n85A | n25 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

Table 5.5A.3.1-1k: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n46A-n48A | CA_n46A-n48A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | 20 |  |
|  |  | n46 | 20, 40, 60, 80 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 |  |
| CA_n46A-n48(2A) | CA_n46A-n48A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46A-n48(3A) | CA_n46A-n48A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46A-n48(4A) | CA_n46A-n48A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46A-n48B | CA_n46A-n48A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46A-n48C | CA_n46A-n48A CA_n46A-n48B | n46 | 20, 40, 60, 80 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46B-n48A | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | 20 |  |
|  |  | n46 | CA_n46B_BCS0 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 |  |
| CA_n46B-n48(2A) | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46B-n48(3A) | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46B-n48(4A) | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46B-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46B-n48C | CA_n46A-n48A | n46 | CA_n46B_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46C-n48A | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | 20 |  |
|  |  | n46 | CA_n46C_BCS0 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 |  |
| CA_n46C-n48(2A) | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46C-n48(3A) | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46C-n48(4A) | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46C-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46C-n48C | CA_n46A-n48A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46D-n48A | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | 20 |  |
|  |  | n46 | CA_n46D_BCS0 | 1 |
|  |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 |  |
| CA_n46D-n48(2A) | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46D-n48(3A) | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46D-n48(4A) | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46D-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46D-n48C | CA_n46A-n48A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46M-n48A | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | 20 |  |
| CA_n46M-n48(2A) | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46M-n48(3A) | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46M-n48(4A) | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46M-n48B | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46M-n48C | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46N-n48A | CA_n46A-n48A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 |  |
| CA_n46N-n48(2A) | CA_n46A-n48A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |
| CA_n46N-n48(3A) | CA_n46A-n48A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(3A)_BCS0 |  |
| CA_n46N-n48(4A) | CA_n46A-n48A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48(4A)_BCS0 |  |
| CA_n46N-n48B | CA_n46A-n48A CA_n46A-n48B | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48B_BCS0 |  |
| CA_n46N-n48C | CA_n46A-n48A | n46 | CA_n46N_BCS0 | 0 |
|  |  | n48 | CA_n48C_BCS0 |  |
| CA_n46A-n66A | - | n46 | 20, 40, 60, 80 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n46A-n77A | CA_n46A-n77A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46C-n77A | CA_n46A-n77A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46D-n77A | CA_n46A-n77A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46A-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46C-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46D-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46(2A)-n77A | CA_n46A-n77A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46(2A)-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46A-n78A | CA_n46A-n78A | n46 | 20, 40, 60, 80 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46C-n78A | CA_n46A-n78A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46D-n78A | CA_n46A-n78A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46A-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46C-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46D-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46(2A)-n78A | CA_n46A-n78A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46(2A)-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46A-n96A | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46B-n96A | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46C-n96A | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46D-n96A | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46M-n96A | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46N-n96A | - | n46 | CA_n46N_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n46A-n96B | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46B-n96B | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46C-n96B | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46D-n96B | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46M-n96B | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46N-n96B | - | n46 | CA_n46N_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n46A-n96C | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46B-n96C | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46C-n96C | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46D-n96C | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46M-n96C | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46N-n96C | - | n46 | CA_n46N_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n46A-n96D | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46B-n96D | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46C-n96D | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46D-n96D | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46M-n96D | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46N-n96D | - | n46 | CA_n46N_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n46A-n96E | - | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46B-n96E | - | n46 | CA_n46B_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46C-n96E | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46D-n96E | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46M-n96E | - | n46 | CA_n46M_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n46N-n96E | - | n46 | CA_n46N_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |

Table 5.5A.3.1-1l: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n48A-n53A | - | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n53 | 5, 10 |  |
| CA_n48(2A)-n53A | - | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n53 | 5, 10 |  |
| CA_n48A-n66A | CA_n48A-n66A | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | 5, 10, 15, 20, 40, 501, 601, 801, 901, 1001 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701 , 801, 901, 1001 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40, |  |
| CA_n48A-n66B | CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701 , 801, 901, 1001 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
| CA_n48A-n66(2A) | CA_n48A-n66A | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701 , 801, 901, 1001 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n48B-n66A | CA_n48B CA_n48A-n66A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48B-n66(2A) | CA_n48B CA_n48A-n66A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
|  |  | n48 | CA_n48B_BCS2 | 1 |
|  |  | n66 | CA_n66(2A)_BCS1 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n66 | CA_n66(2A)_BCS2 |  |
| CA_n48C-n66A | CA_n48B CA_n48A-n66A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | CA_n48C_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48(2A)-n66A | CA_n48A-n66A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48(2A)-n66B | CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | CA_n66B_BCS0 |  |
| CA_n48(2A)-n66(2A) | CA_n48A-n66A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n66 | CA_n66(2A)_BCS0 |  |
| CA_n48(A-B)-n66A | CA_n48A-n66A | n48 | CA_n48(A-B)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48(A-C)-n66A | CA_n48A-n66A | n48 | CA_n48(A-C)_BCS0 | 0 |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n48 | CA_n48(A-C)_BCS0 | 1 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
| CA_n48A-n70A | CA_n48A-n70A | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701 , 801, 901, 1001 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48(2A)-n70A | CA_n48A-n70A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n70 | 5, 10, 15, 20, 25 |  |
| CA_n48B-n70A | CA_n48A-n70A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n48A-n71A | CA_n48A-n71A | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701, 801, 901, 1001 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n71(2A) | CA_n48A-n71A | n48 | 5, 10, 15, 20, 30, 40, 501, 601, 701 , 801, 901, 1001 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48(2A)-n71A | CA_n48A-n71A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(2A)-n71(2A) | CA_n48A-n71A | n48 | CA_n48(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48(3A)-n71A | CA_n48A-n71A | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48(4A)-n71A | CA_n48A-n71A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n71A | CA_n48A-n71A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48B-n71(2A) | CA_n48A-n71A | n48 | CA_n48B_BCS2 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n48C-n71A | CA_n48A-n71A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n48A-n77A | n778,9 | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | See n48 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48A-n77C | n778,9 | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48A-n77(2A) | - | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n48 | See n48 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n48(2A)-n77A | n778,9 | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n48(2A)-n77C | n778,9 | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 2 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS1 | 3 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48(2A)-n77(2A) | - | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n48 | CA_n48(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n48B-n77A | CA_n48B n778,9 | n48 | CA_n48B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48B-n77C | CA_n48B n778,9 CA_n77C | n48 | CA_n48B_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48B_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n48 | CA_n48B_BCS2 | 2 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n48 | CA_n48B_BCS2 | 3 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n48(A-B)-n77A | n778,9 | n48 | CA_n48(A-B)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n48 | CA_n48(A-B)_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n48A-n96A | CA_n48A-n96A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48(2A)-n96A | CA_n48A-n96A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48(2A)-n96B | CA_n48A-n96B | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48(2A)-n96C | CA_n48A-n96A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48(2A)-n96D | CA_n48A-n96A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48(2A)-n96E | CA_n48A-n96A | n48 | CA_n48(2A)_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48(3A)-n96A | CA_n48A-n96A | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48(3A)-n96B | CA_n48A-n96B | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48(3A)-n96C | CA_n48A-n96A | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48(3A)-n96D | CA_n48A-n96A | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48(3A)-n96E | CA_n48A-n96A | n48 | CA_n48(3A)_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48(4A)-n96A | CA_n48A-n96A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48(4A)-n96B | CA_n48A-n96A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48(4A)-n96C | CA_n48A-n96A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48(4A)-n96D | CA_n48A-n96A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48(4A)-n96E | CA_n48A-n96A | n48 | CA_n48(4A)_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48A-n96B | CA_n48A-n96A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48A-n96C | CA_n48A-n96A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48A-n96D | CA_n48A-n96A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48A-n96E | CA_n48A-n96A | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48B-n96A | CA_n48A-n96A CA_n48B-n96A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48B-n96B | CA_n48A-n96A CA_n48B-n96A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48B-n96C | CA_n48A-n96A CA_n48B-n96A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48B-n96D | CA_n48A-n96A CA_n48B-n96A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48B-n96E | CA_n48A-n96A CA_n48B-n96A | n48 | CA_n48B_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n48C-n96A | CA_n48A-n96A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n96 | 20, 40, 60, 80 |  |
| CA_n48C-n96B | CA_n48A-n96A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n96 | CA_n96B_BCS0 |  |
| CA_n48C-n96C | CA_n48A-n96A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n96 | CA_n96C_BCS0 |  |
| CA_n48C-n96D | CA_n48A-n96A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n96 | CA_n96D_BCS0 |  |
| CA_n48C-n96E | CA_n48A-n96A | n48 | CA_n48C_BCS0 | 0 |
|  |  | n96 | CA_n96E_BCS0 |  |
| CA_n50A-n78A | CA_n50A-n78A | n50 | 5, 10, 15, 20, 30, 40, 50, 60, 801 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |

Table 5.5A.3.1-1m: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n70A | - | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66B-n70A | - | n66 | CA_n66B_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66(2A)-n70A | - | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n70 | 5, 10, 15, 201, 251 |  |
| CA_n66A-n71A | CA_n66A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n71B | CA_n66A-n71A | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | CA_n71B_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n66A-n71(2A) | CA_n66A-n71A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n66(2A)-n71A | CA_n66A-n71A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n71B | CA_n66A-n71A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71B_BCS2 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |
| CA_n66(2A)-n71(2A) | CA_n66A-n71A | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n66B-n71A | CA_n66A-n71A | n66 | CA_n66B_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n66A-n77A | n778,9 CA_n66A-n77A8 | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n77B | - | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n66(2A)-n77A | n778,9 CA_n66A-n77A8 | n66 | CA_n66(2A)_BCS1 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n77B | - | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n66A-n77(2A) | n778,9 CA_n66A-n77A8 CA_n77(2A) | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n66(3A)-n77A | n778 CA_n66A-n77A8 | n66 | CA_n66(3A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66(2A)-n77(2A) | n778,9 CA_n66A-n77A8 CA_n77(2A) | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n66(3A)-n77(2A) | n778 CA_n66A-n77A8 | n66 | CA_n66(3A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| CA_n66A-n77C | n778,9 CA_n77C CA_n66A-n77A8 | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n66A-n77(3A) | CA_n77(2A) CA_n66A-n77A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n77 | CA_n77(3A)_BCS0 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n66(2A)-n77(3A) | CA_n66(2A) CA_n77(2A) CA_n66A-n77A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n66(2A)-n77C | n778,9 CA_n66A-n77A8 | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS1 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n66B-n77A | n778,9 CA_n66A-n77A8 | n66 | CA_n66B_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n66B-n77C | n778,9 CA_n66A-n77A8 | n66 | CA_n66B_BCS0 | 0 |
|  |  | n77 | CA_n77C_BCS0 |  |
|  |  | n66 | CA_n66B_BCS0 | 1 |
|  |  | n77 | CA_n77C_BCS1 |  |
| CA_n66A-n78A | CA_n66A-n78A | n66 | 5, 10, 15, 20, 40 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66A-n78(2A) | CA_n66A-n78A | n66 | 5, 10, 15, 20, 30, 40 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n66(2A)-n78A | CA_n66A-n78A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | CA_n66(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n78(2A) | CA_n66A-n78A | n66 | CA_n66(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n66 | CA_n66(2A)_BCS1 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n66 | CA_n66(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n66A-n85A | CA_n66A-n85A | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n85A | CA_n66A-n85A | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n67A-n78A | - | n67 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n67A-n78(2A) | CA_n78(2A) | n67 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |

Table 5.5A.3.1-1n: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n70A-n71A | CA_n70A-n71A | n70 | 5, 10, 15, 201, 251 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n70A-n71(2A) | CA_n70A-n71A | n70 | 5, 10, 15, 201, 251 | 0 |
|  |  | n71 | CA_n71(2A)_BCS0 |  |
| CA_n70A-n77A | CA_n70A-n77A | n70 | 5, 10, 15, 201, 251 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n70A-n78A | CA_n70A-n78A | n70 | 5, 10, 15, 201, 251 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n71A-n77A | n778, 9 CA_n71A-n77A8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71A-n77(2A) | n778, 9 CA_n71A-n77A8 | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n71A-n77(3A) | CA_n77(2A) CA_n71A-n77A | n71 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(3A)_BCS1 |  |
| CA_n71A-n77B | - | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n71A-n77C | - | n71 | n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n71B-n77A | n778, 9 CA_n71A-n77A8 | n71 | CA_n71B_BCS2 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n71B-n77(2A) | n778, 9 CA_n71A-n77A8 | n71 | CA_n71B_BCS2 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n71 | CA_n71B_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n71(2A)-n77A | n778, 9 CA_n71A-n77A8 | n71 | CA_n71(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71(2A)-n77(2A) | n778, 9 CA_n71A-n77A8 | n71 | CA_n71(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77(2A)_BCS 4 and 5 |  |
| CA_n71(2A)-n77B | - | n71 | CA_n71(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77B_BCS 4 and 5 |  |
| CA_n71(2A)-n77C | - | n71 | CA_n71(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | CA_n77C_BCS 4 and 5 |  |
| CA_n71A-n78A | CA_n71A-n78A | n71 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71A-n78(2A) | CA_n71A-n78A | n71 | 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n74A-n77A | CA_n74A-n77A | n74 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n74A-n78A | CA_n74A-n78A | n74 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n75A-n78A | - | n75 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n75A-n78(2A) | - | n75 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS1 |  |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n76A-n78A | - | n76 | 5 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| CA_n77A-n78A7 | - | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77A-n78C2 | - | n77 | 10,15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78C_BCS1 |  |
| CA_n77A-n78(2A)2 | - | n77 | 10,15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n77A-n79A | n77A8,9 n79A8,9 CA_n77A-n79A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n77(2A)-n79A | CA_n77A-n79A | n77 | CA_n77(2A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n77(3A)-n79A | CA_n77A-n79A | n77 | CA_n77(3A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n77A-n85A | CA_n77A-n85A | n77 | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77(2A)-n85A | CA_n77A-n85A | n77 | CA_n77(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77A-n102A | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n77A-n102(2A) | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n77A-n102B | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n77A-n102C | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n77A-n102D | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n77A-n102E | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n77(2A)-n102A | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n77(2A)-n102(2A) | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n77(2A)-n102B | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n77(2A)-n102C | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n77(2A)-n102D | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n77(2A)-n102E | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4 and 5 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78A-n79A | n78A8,9 n79A8,9 CA_n78A-n79A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78A-n79C | - | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
| CA_n78(2A)-n79A | CA_n78A-n79A | n78 | CA_n78(2A)_BCS1 | 0 |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78A-n92A | CA_n78A-n92A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n92 | 5, 10, 15, 20 |  |
|  |  | n78 | See n78 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n92 | See n92 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78(2A)-n92A | CA_n78A-n92A | n78 | CA_n78(2A)_BCS0 | 0 |
|  |  | n92 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n92 | See n92 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78A-n94A | - | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n94 | 5, 10, 15, 20 |  |
| CA_n78A-n102A | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n78A-n102(2A) | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n78A-n102B | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n78A-n102C | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n78A-n102D | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n78A-n102E | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78(2A)-n102A | CA_n78A-n102A CA_n78(2A) | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n78(2A)-n102B | CA_n78A-n102A CA_n78(2A) | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n78(2A)-n102C | CA_n78A-n102A CA_n78(2A) | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n78(2A)-n102D | CA_n78A-n102A CA_n78(2A) | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n78(2A)-n102E | CA_n78A-n102A CA_n78(2A) | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78(2A)-n102(2A) | CA_n78A-n102A CA_n78(2A) | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n78A-n105A | CA_n78A-n105A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

The following notes are applied to the above tables:

NOTE 1:	This UE channel bandwidth is applicable only to downlink.

NOTE 2:	The minimum requirements for intra-band contiguous or non-contiguous CA apply.

NOTE 3:	The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1.

NOTE 4:	This UE channel bandwidth is optional in this release of the specification.

NOTE 5:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration.

NOTE 6:	For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration

NOTE 7:	Limited to operation at 3450-3550 MHz and 3700–3980 MHz.

NOTE 8:	Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination

NOTE 9:	Power Class 1.5 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination

NOTE 10: 	Only single uplink carriers with power class other than PC3 are listed.

NOTE 11: The CA configurations are given in Table 5.5A.1-1 or Table 5.5A.2-1 in this specification

NOTE 12: UL configurations are for non-simultaneous Rx/Tx operation.

<< Next change >>

5.5B	Configurations for DC

For an NR DC configuration specified in 5.5B.1-1, the bandwidth combination sets for the corresponding NR CA configuration in 5.5A.3,i.e.,dual uplink inter-band carrier aggregation with uplink assigned to two NR bands, are applicable to Dual Connectivity.

Table 5.5B.1-1: Inter-band NR DC configurations  (two bands)

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n1A-n3A | DC_n1A-n3A |
| DC_n1A-n7A | DC_n1A-n7A |
| DC_n1A-n28A | DC_n1A-n28A |
| DC_n1A-n41A | DC_n1A-n41A |
| DC_n1A-n46A DC_n1A-n46C DC_n1A-n46D | DC_n1A-n46A |
| DC_n1A-n46(2A) | DC_n1A-n46A |
| DC_n1A-n77A2 | DC_n1A-n77A |
| DC_n1A-n78A | DC_n1A-n78A |
| DC_n1A-n78(2A) | DC_n1A-n78A |
| DC_n1A-n79A2 | DC_n1A-n79A |
| DC_n1A-n102A DC_n1A-n102B DC_n1A-n102C DC_n1A-n102D DC_n1A-n102E | DC_n1A-n102A |
| DC_n1A-n102(2A) | DC_n1A-n102A |
| DC_n2A-n5A DC_n2A-n5B | DC_n2A-n5A |
| DC_n2A-n48A DC_n2A-n48B DC_n2A-n48C | DC_n2A-n48A |
| DC_n2A-n48(2A) DC_n2A-n48(A-C) | DC_n2A-n48A |
| DC_n2A-n66A DC_n2A-n66B | DC_n2A-n66A |
| DC_n2A-n77A DC_n2A-n77C | DC_n2A-n77A |
| DC_n2A-n77(2A) DC_n2A-n77(3A) DC_n2(2A)-n77A DC_n2(2A)-n77C | DC_n2A-n77A |
| DC_n3A-n7A | DC_n3A-n7A |
| DC_n3A-n28A | DC_n3A-n28A |
| DC_n3A-n41A | DC_n3A-n41A |
| DC_n3A-n77A2 | DC_n3A-n77A |
| DC_n3A-n77(2A) 2 | DC_n3A-n77A |
| DC_n3A-n78A2 | DC_n3A-n78A |
| DC_n3A-n78(2A)2 | DC_n3A-n78A |
| DC_n3A-n79A | DC_n3A-n79A |
| DC_n3A-n102A DC_n3A-n102B DC_n3A-n102C DC_n3A-n102D DC_n3A-n102E | DC_n3A-n102A |
| DC_n3A-n102(2A) | DC_n3A-n102A |
| DC_n5A-n48A DC_n5A-n48B DC_n5A-n48C | DC_n5A-n48A |
| DC_n5A-n48(2A) | DC_n5A-n48A |
| DC_n5A-n66A DC_n5B-n66A | DC_n5A-n66A |
| DC_n5A-n66(2A) DC_n5B-n66(2A) | DC_n5A-n66A |
| DC_n5A-n77A DC_n5A-n77C | DC_n5A-n77A |
| DC_n5A-n77(2A) DC_n5A-n77(3A) DC_n5(2A)-n77A DC_n5(2A)-n77C | DC_n5A-n77A |
| DC_n7A-n28A | DC_n7A-n28A |
| DC_n7A-n46A DC_n7A-n46C DC_n7A-n46D | DC_n7A-n46A |
| DC_n7A-n46(2A) | DC_n7A-n46A |
| DC_n7A-n78A | DC_n7A-n78A |
| DC_n7A-n78(2A) | DC_n7A-n78A |
| DC_n7A-n102A DC_n7A-n102B DC_n7A-n102C DC_n7A-n102D DC_n7A-n102E | DC_n7A-n102A |
| DC_n7A-n102(2A) | DC_n7A-n102A |
| DC_n12A-n77A | DC_n12A-n77A |
| DC_n12A-n77(2A) | DC_n12A-n77A |
| DC_n28A-n41A | DC_n28A-n41A |
| DC_n28A-n46A DC_n28A-n46C DC_n28A-n46D | DC_n28A-n46A |
| DC_n28A-n46(2A) | DC_n28A-n46A |
| DC_n28A-n77A2 | DC_n28A-n77A |
| DC_n28A-n77(2A) | DC_n28A-n77A |
| DC_n28A-n78A2 | DC_n28A-n78A |
| DC_n28A-n78(2A)2 | DC_n28A-n78A |
| DC_n28A-n79A | DC_n28A-n79A |
| DC_n28A-n102A DC_n28A-n102B DC_n28A-n102C DC_n28A-n102D DC_n28A-n102E | DC_n28A-n102A |
| DC_n28A-n102(2A) | DC_n28A-n102A |
| DC_n41A-n77A | DC_n41A-n77A |
| DC_n41A-n78A | DC_n41A-n78A |
| DC_n41A-n79A2, 3 | DC_n41A-n79A |
| DC_n46A-n48A DC_n46A-n48B DC_n46A-n48C DC_n46B-n48A DC_n46B-n48B DC_n46B-n48C DC_n46C-n48A DC_n46C-n48B DC_n46C-n48C DC_n46D-n48A DC_n46D-n48B DC_n46D-n48C DC_n46N-n48A DC_n46N-n48B DC_n46N-n48C | DC_n46A-n48A DC_n46A-n48B |
| DC_n46A-n77A DC_n46C-n77A DC_n46D-n77A | DC_n46A-n77A |
| DC_n46A-n77(2A) DC_n46C-n77(2A) DC_n46D-n77(2A) DC_n46(2A)-n77A DC_n46(2A)-n77(2A) | DC_n46A-n77A |
| DC_n46A-n78A DC_n46C-n78A DC_n46D-n78A | DC_n46A-n78A |
| DC_n46A-n78(2A) DC_n46C-n78(2A) DC_n46D-n78(2A) DC_n46(2A)-n78A DC_n46(2A)-n78(2A) | DC_n46A-n78A |
| DC_n48A-n66A DC_n48B-n66A DC_n48C-n66A | DC_n48A-n66A |
| DC_n48A-n66(2A) DC_n48B-n66(2A) DC_n48(2A)-n66A DC_n48(2A)-n66(2A) DC_n48(A-C)-n66A | DC_n48A-n66A |
| DC_n48A-n70A DC_n48B-n70A | DC_n48A-n70A |
| DC_n48(2A)-n70A | DC_n48A-n70A |
| DC_n48A-n71A DC_n48B-n71A DC_n48C-n71A | DC_n48A-n71A |
| DC_n48A-n71(2A) DC_n48(2A)-n71A DC_n48(2A)-n71(2A) DC_n48(3A)-n71A DC_n48(4A)-n71A DC_n48B-n71(2A) | DC_n48A-n71A |
| DC_n48A-n96A DC_n48B-n96A DC_n48C-n96A DC_n48A-n96B DC_n48B-n96B DC_n48C-n96B DC_n48A-n96C DC_n48B-n96C DC_n48C-n96C DC_n48A-n96D DC_n48B-n96D DC_n48C-n96D DC_n48A-n96E DC_n48B-n96E DC_n48C-n96E | DC_n48A-n96A DC_n48B-n96A |
| DC_n66A-n77A DC_n66A-n77C DC_n66B-n77A DC_n66B-n77C | DC_n66A-n77A |
| DC_n66A-n77(2A) DC_n66A-n77(3A) DC_n66(2A)-n77(2A) DC_n66(2A)-n77C | DC_n66A-n77A |
| DC_n71A-n77A | DC_n71A-n77A |
| DC_n71A-n77(2A) | DC_n71A-n77A |
| DC_n77A-n79A1 | DC_n77A-n79A |
| DC_n77(2A)-n79A1 | DC_n77A-n79A |
| DC_n78A-n79A DC_n78(2A)-n79A | DC_n78A-n79A |
| DC_n77A-n102A DC_n77A-n102B DC_n77A-n102C DC_n77A-n102D DC_n77A-n102E | DC_n77A-n102A |
| DC_n77A-n102(2A) DC_n77(2A)-n102A DC_n77(2A)-n102B DC_n77(2A)-n102C DC_n77(2A)-n102D DC_n77(2A)-n102E DC_n77(2A)-n102(2A) | DC_n77A-n102A |
| DC_n78A-n102A DC_n78A-n102B DC_n78A-n102C DC_n78A-n102D DC_n78A-n102E | DC_n78A-n102A |
| DC_n78A-n102(2A) DC_n78(2A)-n102A DC_n78(2A)-n102B DC_n78(2A)-n102C DC_n78(2A)-n102D DC_n78(2A)-n102E DC_n78(2A)-n102(2A) | DC_n78A-n102A |
| NOTE 1: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR DC configuration is part of a higher order configuration. NOTE 2: Applicable for UE supporting inter-band NR DC with mandatory simultaneous Rx/Tx capability. NOTE 3: The frequency range below 2506 MHz for Band n41 is not used in this combination. |  |

<< Next change >>

6.2A	Transmitter power for CA

6.2A.1	UE maximum output power for CA

6.2A.1.1	Void

6.2A.1.2	Void

6.2A.1.3	UE maximum output power for Inter-band CA

For power class 3 inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements power class 3 in clause 6.2 apply.

For other power class except class 3 inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the maximum output power is specified in Table 6.2A.1.3-2. The period of measurement shall be at least one sub frame (1 ms).

For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.3-1.

Table 6.2A.1.3-1 UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n18A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n20A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n26A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n46A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n74A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n78A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n12A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n14A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n77A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n2A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n18A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n20A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n26A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n34A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n38A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n41A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n41C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n74A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n75A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n78A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n12A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n14A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n25A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n77A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n5A-n78A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n25A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n26A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n46A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n78A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n7A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n34A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n39A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n12A-n25A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n12A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n12A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n12A-n77A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n12A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n13A-n25A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n13A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n13A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n14A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n14A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n14A-n77A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n18A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n18A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n18A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n18A-n74A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n18A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n18A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n20A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n20A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n38A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n41A |  |  | 266,7 | +2/-32 | 23 | +2/-3 |  |  |
| CA_25A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n77A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n25A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n85A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n34A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n39A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n41A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n46A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n50A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n74A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n78A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n79A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n34A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n30A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n30A-n77A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n34A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n34A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n34A-n41C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n38A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n38A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n39A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n39A-n41C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n39A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n40A-n41A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n40A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n40A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n40A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n40A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n50A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n66A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n71A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41A-n74A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n77A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n79A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41A-n85A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n46A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n46A-n48B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n46A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n46A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n96A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48B-n96A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n96B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n50A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n66A-n77A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n66A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n66A-n85A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n70A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n70A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n71A-n77A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n71A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n74A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n74A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n77A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n77A-n85A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n77A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78A-n92A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: Void NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports PC3 within an NR TDD or FDD band and supports PC2 within a second NR TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. |  |  |  |  |  |  |  |  |

<< Next change >>

# 6.2A.4.2	ΔTIB,c for CA

For the UE which supports inter-band NR CA configuration, ΔTIB,c in tables below applies. Unless otherwise stated, ΔTIB,c is set to zero.

6.2A.4.2.1	Void

6.2A.4.2.2	Void

# 6.2A.4.2.3	ΔTIB,c for Inter-band CA (two bands)

Table 6.2A.4.2.3-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n1-n3 | 0.3 | 0.3 |
| CA_n1-n5 | 0.3 | 0.3 |
| CA_n1-n7 | 0.5 | 0.6 |
| CA_n1-n8 | 0.3 | 0.3 |
| CA_n1-n18 | 0.3 | 0.3 |
| CA_n1-n20 | 0.3 | 0.3 |
| CA_n1-n26 | 0.3 | 0.3 |
| CA_n1-n28 | 0.3 | 0.6 |
| CA_n1-n38 | 0.5 | 0.5 |
| CA_n1-n40 | 0.5 | 0.5 |
| CA_n1-n41 | 0.5 | 0.5 |
| CA_n1-n67 | 0.3 | - |
| CA_n1-n74 | 0.3 | 0.3 |
| CA_n1-n75 | 0.3 | - |
| CA_n1-n77 | 0.6 | 0.8 |
| CA_n1-n78 | 0.3 | 0.8 |
| CA_n1-n102 | 0.6 | 0.8 |
| CA_n1-n105 | 0.3 | 0.6 |
| CA_n2-n5 | 0.3 | 0.3 |
| CA_n2-n7 | 0.5 | 0.5 |
| CA_n2-n12 | 0.3 | 0.3 |
| CA_n2-n14 | 0.3 | 0.3 |
| CA_n2-n29 | 0.3 | - |
| CA_n2-n30 | 0.5 | 0.3 |
| CA_n2-n41 | 0.5 | 0.45/0.96 |
| CA_n2-n48 | 0.6 | 0.8 |
| CA_n2-n66 | 0.5 | 0.5 |
| CA_n2-n71 | 0.3 | 0.6 |
| CA_n2-n77 | 0.6 | 0.8 |
| CA_n2-n78 | 0.6 | 0.8 |
| CA_n3-n7 | 0.5 | 0.5 |
| CA_n3-n8 | 0.3 | 0.3 |
| CA_n3-n18 | 0.3 | 0.3 |
| CA_n3-n20 | 0.3 | 0.3 |
| CA_n3-n26 | 0.3 | 0.3 |
| CA_n3-n28 | 0.3 | 0.3 |
| CA_n3-n34 | 0.5 | 0.5 |
| CA_n3-n38 | 0.5 | 0.5 |
| CA_n3-n40 | 0.5 | 0.5 |
| CA_n3-n41 | 0.5 | 0.34 / 0.85 |
| CA_n3-n74 | 0.8 | 0.9 |
| CA_n3-n77 | 0.6 | 0.8 |
| CA_n3-n78 | 0.6 | 0.8 |
| CA_n3-n79 | 0.3 | 0.8 |
| CA_n3-n102 | 0.3 | 0.8 |
| CA_n3-n105 | 0.3 | 0.6 |
| CA_n5-n7 | 0.3 | 0.3 |
| CA_n5-n12 | 0.8 | 0.4 |
| CA_n5-n14 | 0.5 | 0.5 |
| CA_n5-n25 | 0.3 | 0.3 |
| CA_n5-n28 | 0.5 | 0.5 |
| CA_n5-n29 | 0.5 | - |
| CA_n5-n30 | 0.3 | 0.3 |
| CA_n5-n40 | 0.3 | 0.3 |
| CA_n5-n41 | 0.6 | 0.3 |
| CA_n5-n48 | 0.3 | 0.3 |
| CA_n5-n66 | 0.3 | 0.3 |
| CA_n5-n71 | 0.5 | 0.5 |
| CA_n5-n77 | 0.6 | 0.8 |
| CA_n5-n78 | 0.6 | 0.8 |
| CA_n7-n8 | 0.3 | 0.6 |
| CA_n7-n12 | 0.3 | 0.3 |
| CA_n7-n25 | 0.5 | 0.5 |
| CA_n7-n26 | 0.3 | 0.3 |
| CA_n7-n28 | 0.3 | 0.3 |
| CA_n7-n40 | 0.5 | 0.6 |
| CA_n7-n46 | 0.3 | - |
| CA_n7-n66 | 0.5 | 0.5 |
| CA_n7-n67 | 0.3 | N/A |
| CA_n7-n71 | 0.6 | 0.3 |
| CA_n7-n75 | 0.7 | - |
| CA_n7-n77 | 0.5 | 0.8 |
| CA_n7-n78 | 0.5 | 0.8 |
| CA_n7-n79 | 0.5 | 0.8 |
| CA_n7-n102 | 0.5 | 0.8 |
| CA_n7-n105 | 0.3 | 0.6 |
| CA_n8-n20 | 0.4 | 0.4 |
| CA_n8-n28 | 0.6 | 0.5 |
| CA_n8-n34 | 0.3 | 0.3 |
| CA_n8-n38 | 0.6 | 0.3 |
| CA_n8-n39 | 0.3 | 0.3 |
| CA_n8-n40 | 0.3 | 0.3 |
| CA_n8-n41 | 0.6 | 0.3 |
| CA_n8-n75 | 0.3 | - |
| CA_n8-n77 | 0.6 | 0.8 |
| CA_n8-n78 | 0.6 | 0.8 |
| CA_n8-n79 | 0.3 | 0.8 |
| CA_n12-n25 | 0.3 | 0.3 |
| CA_n12-n30 | 0.3 | 0.3 |
| CA_n12-n41 | 0.3 | 0.3 |
| CA_n12-n48 | 0.3 | 0.3 |
| CA_n12-n66 | 0.8 | 0.3 |
| CA_n12-n71 | 1.0 | 1.0 |
| CA_n12-n77 | 0.5 | 0.8 |
| CA_n12-n78 | 0.5 | 0.8 |
| CA_n13-n25 | 0.3 | 0.3 |
| CA_n13-n66 | 0.3 | 0.3 |
| CA_n13-n77 | 0.5 | 0.8 |
| CA_n14-n30 | 0.3 | 0.3 |
| CA_n14-n66 | 0.3 | 0.3 |
| CA_n14-n77 | 0.5 | 0.8 |
| CA_n18-n28 | 0.5 | 0.5 |
| CA_n18-n40 | 0.3 | 0.3 |
| CA_n18-n41 | 0.3 | 0.3 |
| CA_n18-n74 | 0.3 | 0.3 |
| CA_n18-n77 | 0.3 | 0.8 |
| CA_n18-n78 | 0.3 | 0.8 |
| CA_n20-n28 | 0.5 | 0.5 |
| CA_n20-n40 | 0.3 | 0.3 |
| CA_n20-n75 | 0.3 | - |
| CA_n20-n78 | 0.6 | 0.8 |
| CA_n20-n67 | 0.5 | - |
| CA_n24-n41 | 0.3 | 0.46 / 0.97 |
| CA_n24-n48 | 0.6 | 0.8 |
| CA_n24-n77 | 0.6 | 0.8 |
| CA_n25-n29 | 0.3 | - |
| CA_n25-n38 | 0.5 | 0.5 |
| CA_n25-n41 | 0.5 | 0.46 / 0.97 |
| CA_n25-n48 | 0.6 | 0.8 |
| CA_n25-n66 | 0.5 | 0.5 |
| CA_n25-n71 | 0.3 | 0.6 |
| CA_n25-n77 | 0.6 | 0.8 |
| CA_n25-n85 | 0.3 | 0.6 |
| CA_n26-n28 | 0.5 | 0.5 |
| CA_n26-n66 | 0.3 | 0.3 |
| CA_n26-n70 | 0.3 | 0.3 |
| CA_n26-n77 | 0.3 | 0.8 |
| CA_n26-n78 | 0.3 | 0.8 |
| CA_n28-n34 | 0.3 | 0.3 |
| CA_n28-n38 | 0.3 | 0.3 |
| CA_n28-n39 | 0.3 | 0.3 |
| CA_n28-n40 | 0.3 | 0.3 |
| CA_n28-n41 | 0.3 | 0.3 |
| CA_n28-n50 | 0.3 | 0.4 |
| CA_n28-n71 | 1.1 | 1.1 |
| CA_n28-n74 | 0.6 | 0.4 |
| CA_n28-n75 | 0.3 | - |
| CA_n28-n77 | 0.5 | 0.8 |
| CA_n28-n78 | 0.5 | 0.8 |
| CA_n28-n79 | 0.5 | 0.8 |
| CA_n28-n94 | 0.5 | 0.6 |
| CA_n28-n102 | 0.5 | 0.8 |
| CA_n29-n30 | - | 0.3 |
| CA_n29-n66 | - | 0.3 |
| CA_n29-n70 | - | 0.3 |
| CA_n29-n71 | - | 0.5 |
| CA_n29-n77 | - | 0.8 |
| CA_n34-n79 | 0.3 | 0.8 |
| CA_n30-n66 | 0.5 | 0.8 |
| CA_n30-n77 | 0.3 | 0.8 |
| CA_n34-n41 | 0.3 | 0.3 |
| CA_n34-n79 | 0.3 | 0.8 |
| CA_n38-n40 | 0.53 | 0.53 |
| CA_n38-n66 | 0.5 | 0.5 |
| CA_n38-n78 | 0.3 | 0.8 |
| CA_n38-n79 | 0.3 | 0.8 |
| CA_n39-n41 | 02 / 0.5 | 02 / 0.5 |
| CA_n39-n79 | 0.3 | 0.8 |
| CA_n40-n41 | 0.53 | 0.53 |
| CA_n40-n77 | - | 0.5 |
| CA_n40-n78 | - | 0.5 |
| CA_n40-n79 | 0.3 | 0.8 |
| CA_n40-n105 | 0.3 | 0.6 |
| CA_n41-n48 | 0.3 | 0.8 |
| CA_n41-n50 | 0.3 | 0.4 |
| CA_n41-n66 | 0.86 / 1.37 | 0.5 |
| CA_n41-n70 | 0.5 | 0.5 |
| CA_n41-n71 | 0.3 | 0.6 |
| CA_n41-n74 | 0.3 | 0.3 |
| CA_n41-n771 | 0.3 | 0.8 |
| CA_n41-n781 | 0.3 | 0.8 |
| CA_n41-n79 | 0.3 | 0.8 |
| CA_n41-n85 | 0.3 | 0.6 |
| CA_n46-n48 | - | 0.5 |
| CA_n46-n77 | - | 0.8 |
| CA_n46-n78 | - | 0.8 |
| CA_n46-n96 | - | 0.5 |
| CA_n48-n53 | 0.53 | 0.33 |
| CA_n48-n66 | 0.8 | 0.6 |
| CA_n48-n70 | 0.8 | 0.6 |
| CA_n48-n71 | 0.3 | 0.3 |
| CA_n48-n96 | 0.5 | 0.5 |
| CA_n50-n78 | 02 / 0.53 | 02 / 0.53 |
| CA_n66-n70 | 0.5 | 0.5 |
| CA_n66-n71 | 0.3 | 0.3 |
| CA_n66-n77 | 0.6 | 0.8 |
| CA_n66-n78 | 0.6 | 0.8 |
| CA_n66-n85 | 0.3 | 0.3 |
| CA_n67-n78 | N/A | 0.8 |
| CA_n70-n71 | 0.3 | 0.6 |
| CA_n70-n77 | 0.6 | 0.8 |
| CA_n70-n78 | 0.6 | 0.8 |
| CA_n71-n77 | 0.5 | 0.8 |
| CA_n71-n78 | 0.5 | 0.8 |
| CA_n74-n77 | 0.4 | 0.8 |
| CA_n74-n78 | 0.4 | 0.8 |
| CA_n75-n78 | - | 0.8 |
| CA_n76-n78 | - | 0.8 |
| CA_n77-n79 | 0.5 | 0.5 |
| CA_n77-n85 | 0.7 | 0.5 |
| CA_n77-n102 | 1.5 | 1.5 |
| CA_n78-n79 | 0.5 / 1.58 | 0.5 / 1.58 |
| CA_n78-n92 | 0.8 | 0.6 |
| CA_n78-n102 | 1.5 | 1.5 |
| CA_n78-n105 | 0.8 | 0.5 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

<< Next change >>

6.2.B	Transmitter power for NR-DC

6.2B.0	General

The requirements apply for inter-band NR-DC with one uplink serving cell configured per CG.

6.2B.1	UE maximum output power for NR-DC

For inter-band NR-DC with one uplink carrier assigned per NR band, the transmitter power requirements in clause 6.2 apply per band.

For inter-band NR-DC with one uplink assigned per band, the UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, the maximum output power is measured as the sum of maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2B.1.3-1.

Table 6.2B.1.3-1 UE Power Class for inter-band NR-DC

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_n1A-n3A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n1A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n1A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n1A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n1A-n46A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n1A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n1A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n1A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n1A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n2A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n2A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n2A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n2A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n3A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n3A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n3A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n3A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n3A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n3A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n3A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n5A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n5A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n5A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n7A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n7A-n46A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n7A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n7A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n12A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n28A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n28A-n46A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n28A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n28A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n28A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n28A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n41A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n41A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n41A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n46A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n46A-n48B |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n46A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n46A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n48A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n48A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n48A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n48A-n96A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n48B-n96A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n66A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n77A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n77A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n78A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| DC_n78A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass is the maximum UE power specified without account of the tolerance NOTE 3: The maximum power requirement applies to the total transmitted power over both the MCG and SCG. NOTE 4: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

<< Next change >>

6.5A.3.2	Spurious emissions for UE co-existence

6.5A.3.2.1	Void

6.5A.3.2.2	Void

6.5A.3.2.3	Spurious emissions for UE co-existence for Inter-band CA

For inter-band carrier aggregation with the uplink assigned to two NR bands, the requirements in Table 6.5A.3.2.3-1 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band carrier aggregation with uplink assigned to two NR bands the requirements in Table 6.5A.3.2.3-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.5A.3.2.3-1 would be considered to be verified by the measurements verifying the one uplink inter-band CA UE to UE co-existence requirements.

Table 6.5A.3.2.3-1: Requirements for uplink inter-band carrier aggregation (two bands)

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4,6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n5 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n1-n7 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 68, 72, 74, 75, 76 NR Band n78, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 4. 7, 6 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 4. 7, 6 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n1-n8 | E-UTRA Band 20, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n18 | E-UTRA Band 1, 11, 21, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_n1-n20 | E-UTRA Band 3, 7, 8, 22, 31, 32, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 NR band n1, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 38, 42, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 NR band n20 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
|  | Frequency range | 1900 | - | 1910 | -15.5 | 5 | 4, 6, 7 |
| CA_n1-n26 | E-UTRA Band 1, 3, 5, 7, 11, 18, 19, 21, 26, 31, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
| CA_n1-n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR band n78, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | NR band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 4, 14 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 4, 7, 14 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 4, 7, 14 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n1-n41 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 27, 28, 42, 44, 45, 50, 51, 52, 65, 73, 74 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4,6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n46 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n78, n79, n100, n104 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| CA_n1-n74 | E-UTRA Band 1, 5, 7, 8, 18, 19, 20, 26, 28, 31, 38, 40, 41, 42, 43, 52, 65, 67, 68 NR Band n78, n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -28 | 1 | 4, 21 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 4, 22 |
|  | Frequency range | 1488 | - | 1510.9 | -35 | 1 | 4, 23 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
| CA_n1-n77 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 40, 41, 65, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n1-n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| CA_n2-n7 | E-UTRA Band 2, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n2-n12 | -UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 53, 54, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 12, 25, 85 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 4, 10, 50, 51, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n14 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 2, 25, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| CA_n2-n30 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 41, 42, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 NR band n30 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | NR Band n2 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 43, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n48 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n2-n66 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n77 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 54, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n2-n78 | E-UTRA Band 5, 7, 12, 13，26, 28, 41, 66, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n3-n5 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 26, 28, 31, 38, 40, 43, 50, 51, 65, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 22, 41, 42, 52 Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n7 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA band 22, 42, 52 NR-band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n3-n8 | E-UTRA Band 1, 11, 20, 21, 28, 31, 32, 33, 34, 38, 39, 40, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 8 | FDL_low | - | FDL_high | -50 | 1 | 2, 4 |
|  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n18 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_n3-n20 | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n3, n20 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 22, 38, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| CA_n3-n26 | E-UTRA Band 1, 5, 7, 11, 18, 19, 21, 26, 34, 39, 40, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA band 22, 41, 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| CA_n3-n34 | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 69,72, 73, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 52 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n38 | E-UTRA Band 1, 5, 8, 20, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA band 22, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| CA_n3-n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31 38, 40, 41, 72 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 32, 42, 43, 50, 51, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -30 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n3-n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n41 | E-UTRA Band 1, 5, 8, 11, 18, 19, 20, 21, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n74 | E-UTRA Band 1, 5, 7, 8, 18, 19, 20, 26, 28, 31, 34, 38, 39, 40, 41, 43, 65, 67, 68 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -28 | 1 | 4, 21 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 4, 22 |
|  | Frequency range | 1488 | - | 1510.9 | -35 | 1 | 4, 23 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
| CA_n3-n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n3-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n7 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 28, 29, 30, 31, 34, 40, 42, 43, 65, 66, 71, 85, 103 NR Band n7 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 13 |
| CA_n5-n12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 43, 54, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 41, 42, 48, 50, 51, 66, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n5-n14 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 41, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n5-n25 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 50, 51, 54, 66, 70, 71,74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43, 48, 53, n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n5-n30 | E-UTRA Band 2, 4, 7, 12, 13, 14, 17, 24, 25, 26, 29, 38, 48, 54, 66, 70, 71, 85, 103 NR band n5, 30 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n5-n40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 28, 31, 34, 38, 42, 43, 45, 50, 51, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA band 41, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2, 22 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 54, 65, 66, 70, 71, 73, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n5-n66 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n77 | E-UTRA Band 1, 2, 3, 4, 5, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 40, 54, 65, 66, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n5-n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 7, 2 |
| CA_n5-n79 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 21, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85, 103 | FDL_low | - | FDL_high |  |  |  |
|  | E-UTRA Band 41, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n7-n8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 42, 43, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n7-n25 | E-UTRA Band 4, 5, 7, 12, 13, 14 17, 26, 27, 28, 29, 30, 42, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n7-n26 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 22, 26, 29, 30, 31, 40, 42, 43, 65, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 8 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 8 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 14 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n7-n28 | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40 72 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band n1 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n7-n40 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 42, 43, 50, 51, 52, 65, 67, 68, 72, 74, 75, 76, 77, 78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 18, 23 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 18, 23 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n7-n46 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n7-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 66, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n7-n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 6 |
| CA_n7-n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| CA_n7-n79 | E-UTRA Band 1, 3, 5, 7, 8, 28, 34, 40, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| CA_n8-n34 | E-UTRA Band 1, 20, 28, 31, 32, 33, 38, 39, 40, 45, 50, 51, 65, 67, 69,72, 73, 74, 75, 76 NR Band n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n39 | E-UTRA Band 1, 34, 40, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 41, 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n8-n40 | E-UTRA Bands 1, 11, 20, 21, 28, 31, 32, 33, 34, 38, 39, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76NR Band n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 3, 7, 22, 41, 42, 43, 52 NR Bands n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n41 | E-UTRA Band 1, 11, 12, 28, 34, 39, 45, 50, 51, 65, 73,74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA band 3, 42, 52, 54 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n8-n78 | E-UTRA Band 1, 8, 11, 20, 21, 28, 34, 39, 40, 65, 74 NR Band n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n8-n79 | E-UTRA Band 1, 8, 11, 21, 28, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 41, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n12-n25 | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 53, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 12, 25, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n12-n30 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n12-n66 | E-UTRA Band 2, 5, 13, 14, 17, 25, 26, 27, 30, 41, 53, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n12-n77 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 54, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 66, 70, | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n12-n78 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n13-n25 | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 54, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2,14, 25, 103 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 30, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| CA_n13-n66 | Bands 2, 4, 5, 7, 12, 13, 17, 25, 26, 27, 29, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14, 103 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 24, 30, 46, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| CA_n13-n77 | E-UTRA Band 2, 5, 7, 12, 13, 25, 26, 41, 54, 66, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| CA_n14-n30 | E-UTRA Band 2, 4, 5,12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 48, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| CA_n14-n66 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4 |
| CA_n14-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 4, 20 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 4, 20 |
| CA_n18-n28 | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2,11, 15 |
|  | E-UTRA Band 42, 65 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 34, 40 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_n18-n40 | E-UTRA Band 1, 3, 11, 21, 34, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 22 |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_n18-n41 | E-UTRA Band 1, 3, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n18-n74 | E-UTRA Band 1, 3, 34, 40, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -28 | 1 | 4, 21 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 4, 22 |
|  | Frequency range | 1488 | - | 1510.9 | -35 | 1 | 4, 23 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_n18-n77 | E-UTRA Band 1, 3, 11, 21, 34, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_n18-n78 | E-UTRA Band 1, 3, 11, 21, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| CA_n20-n28 | E-UTRA Band 3, 7, 31, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 20, 22, 32, 38, 42, 43, 65, 75, 76 NR Band n78, 100, n101 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| CA_n20-n78 | E-UTRA Band 1, 3, 7, 8, 34, 40, 65 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n24-n41 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n24-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n24-n77 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 29, 30, 41, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n25-n41 | E-UTRA Band 4, 5, 12, 13 , 14, 17, 24, 26, 27, 28, 29, 30, 42, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n25-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, NR band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n25-n66 | E-UTRA Band 4, 5, 7, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 38, 41, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 43, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n25-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 53, 54, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 48, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| CA_n25-n77 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 54, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n25-n78 | E-UTRA Band 5, 7, 12, 13, 26, 28, 41, 66, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n25-n85 | E-UTRA Band 5, 12, 13, 14, 17, 24, 26, 30, 53, 54, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| CA_n26-n66 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 43, 47, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 53 NR band 77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n26-n70 | E-UTRA Band 2, 5, 10, 12, 13, 14, 17, 24, 25, 29, 30, 48, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n26-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n26-n78 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n34 | - UTRA Band 3, 7, 8, 18, 19 ,20, 26, 31, 38, 40, 41, 72 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n39 | - UTRA Band 8, 26, 34, 40, 41 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 50, 51, 52, 73, 74 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| CA_n28-n40 | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 41, 72 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21, 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n28-n41 | E-UTRA Band 2, 3, 5, 8, 25, 26, 27, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 42, 50, 51, 52, 65, 66, 73, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -50 | 1 | 11 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n46 | E-UTRA Band 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| CA_n28-n50 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 25, 26, 27, 31, 34, 38, 39, 40, 41, 72 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 22, 42, 43, 48, 52, 65, 66, 73 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 10, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n77 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n74 | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 26, 31, 34, 38, 39, 40, 41 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 11, 15 |
|  | E-UTRA Band 4, 42, 43, 52, 65, 66 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20, 2 |
|  | Frequency range | 1475 | - | 1488 | -28 | 1 | 4, 21, 2 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 4, 22, 2 |
|  | Frequency range | 1488 | - | 1510.9 | -35 | 1 | 4, 23, 2 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4, 2 |
| CA_n28-n78 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 11, 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 11, 12 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 11 |
| CA_n28-n79 | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 10, 11 |
| CA_n30-n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 38, 41, 70, 71, 103 NR band n30, n66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n30-n77 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n34-n40 | E-UTRA Band 1, 3, 7, 8, 11, 20, 21, 22, 26, 28, 31, 32, 33, 38, 39, 41, 42, 43, 44, 45, 50, 51, 65, 67, 69, 72, 73, 74, 75, 76 NR band n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n34-n41 | E-UTRA Band 1, 3, 8, 26, 28, 39, 42, 44, 45, 50, 51, 52, 65, 73, 74 NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n34-n79 | E-UTRA Band 1, 3, 8, 11, 18, 19, 21, 28, 39, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n38-n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 19 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 5, 19, |
| CA_n38-n78 | E-UTRA Band 1, 3, 5, 8, 20, 28, 34, 40, 65, NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| CA_n39-n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 8 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n39-n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 4, 8 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 4, 7, 8 |
| CA_n40-n41 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n40-n77 | UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 | 22 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n40-n78 | UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n40-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74, NR band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 53, 54, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n41-n50 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 20, 25, 26, 27, 28, 29, 30, 31, 34, 39, 42, 43, 44, 48, 52, 65, 66, 67, 68, 70, 71, 73, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n41-n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n41-n70 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n41-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 54, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| CA_n41-n74 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 17, 18, 19, 26, 28, 29, 34, 39, 42, 48, 52, 65, 66, 85, 103 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -28 | 1 | 4, 21 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 4, 22 |
|  | Frequency range | 1488 | - | 1510.9 | -35 | 1 | 4, 23 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
| CA_n41-n77 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 44, 45, 50, 51, 53, 54, 65, 66, 70, 71, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n78 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 42, 44, 45, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 40 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n41-n85 | E-UTRA Band 5, 12, 13, 14, 17, 24, 26, 30, 54, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 4, 25, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| CA_n46-n48 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, NR band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n46-n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n46-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| CA_n48-n66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n48-n70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n48_n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 54, 66, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| CA_n48-n96 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 53, 54, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n50-n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 25, 26, 27, 28, 29, 31, 33, 34, 38, 39, 40, 41, 44, 65, 66, 67, 68, 69, 72, 73, 85, 103 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n66-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 43, 50, 51, 53, 66, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n66-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n66-n78 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 29, 26, 28, 41, 66, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_n66-n85 | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 30, 43, 53, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 4, 25, 41, 42, 48, 50, 51, 66, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n70-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 30, 48, 66, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| CA_n70-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n70-n78 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n71-n77 | E-UTRA Band 1, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 30, 39, 40, 44, 45, 50, 51, 53, 54, 65, 66, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | E-UTRA Band 2, 7, 25, 41, 34, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n71-n78 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_n74-n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 34, 39, 40, 41, 65, 66, 85, 103 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 21 |
|  | Frequency range | 1475 | - | 1488 | -28 | 1 | 4, 21 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 4, 22 |
|  | Frequency range | 1488 | - | 1510.9 | -35 | 1 | 4, 23 |
| CA_n74-n78 | E-UTRA Band 1, 3, 5, 7, 8, 18, 19, 20, 26, 28, 34, 39, 40, 41, 65, NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 4, 20 |
|  | Frequency range | 1475 | - | 1488 | -28 | 1 | 4, 21 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 4, 22 |
|  | Frequency range | 1488 | - | 1510.9 | -35 | 1 | 4, 23 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 4 |
| CA_n77-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n77-n85 | E-UTRA Band 3, 5, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 30, 39, 40, 45, 53, 54, 73, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | E-UTRA Band 1, 2, 4, 7, 10, 25, 41, 34, 50, 51, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| CA_n78-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| CA_n78-n92 | E-UTRA Band 1, 3, 7, 8, 34, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 9: Void. NOTE 10: Void. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 12: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned NR carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.3.1-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 15: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 17: Void. NOTE 18: This requirement is applicable for any channel bandwidths within the range 2500 – 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 – 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 19: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 20: Applicable for cases and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 5 and 10 MHz bandwidth, and when the lower edge of the assigned NR UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. NOTE 21: Applicable for 5 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for 10 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned NR UL channel bandwidth. NOTE 22: As exceptions, for 90 and 100 MHz channel bandwidth, -40 dBm/MHz is applicable in the frequency range of 2496 – 2505 MHz. NOTE 23: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band.if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5A.3.2.3-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

< Next change >>

# 7.3A.3.2.1	ΔRIB,c for two bands

Table 7.3A.3.2.1-1: ΔRIB,c due to CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n1-n28 | - | 0.2 |
| CA_n1-n67 | - | 0.2 |
| CA_n1-n77 | 0.2 | 0.5 |
| CA_n1-n78 | - | 0.5 |
| CA_n1-n102 | 0.2 | 0.5 |
| CA_n1-n105 | - | 0.3 |
| CA_n2-n48 | 0.2 | 0.5 |
| CA_n2-n66 | 0.3 | 0.3 |
| CA_n2-n71 | - | 0.3 |
| CA_n2-n77 | 0.2 | 0.5 |
| CA_n2-n78 | 0.2 | 0.5 |
| CA_n3-n41 | - | 04 / 0.55 |
| CA_n3-n67 | 0.3 | - |
| CA_n3-n74 | 0.3 | 0.5 |
| CA_n3-n77 | 0.2 | 0.5 |
| CA_n3-n78 | 0.2 | 0.5 |
| CA_n3-n79 | - | 0.5 |
| CA_n3-n102 | - | 0.5 |
| CA_n3-n105 | - | 0.3 |
| CA_n5-n12 | 0.5 | 0.3 |
| CA_n5-n41 | 0.2 | - |
| CA_n5-n77 | 0.2 | 0.5 |
| CA_n5-n78 | 0.2 | 0.5 |
| CA_n7-n8 | - | 0.2 |
| CA_n7-n40 | - | 0.5 |
| CA_n7-n46 | 0.3 | - |
| CA_n7-n66 | 0.5 | 0.5 |
| CA_n7-n71 | 0.2 | - |
| CA_n7-n77 | - | 0.5 |
| CA_n7-n78 | 0.5 | 0.5 |
| CA_n7-n79 | - | 0.5 |
| CA_n7-n102 | - | 0.5 |
| CA_n7-n105 | - | 0.2 |
| CA_n8-n28 | 0.2 | 0.2 |
| CA_n8-n77 | 0.2 | 0.5 |
| CA_n8-n78 | 0.2 | 0.5 |
| CA_n8-n79 | - | 0.5 |
| CA_n12-n66 | 0.5 | - |
| CA_n12-n71 | 0.8 | 0.8 |
| CA_n12-n77 | 0.2 | 0.5 |
| CA_n12-n78 | 0.2 | 0.5 |
| CA_n13-n77 | 0.2 | 0.5 |
| CA_n14-n77 | 0.2 | 0.5 |
| CA_n18-n77 | - | 0.5 |
| CA_n18-n78 | - | 0.5 |
| CA_n20-n78 | - | 0.5 |
| CA_n20-n40 | - | 0.5 |
| CA_n24-n48 | 0.2 | 0.5 |
| CA_n24-n77 | 0.2 | 0.5 |
| CA_n25-n48 | 0.2 | 0.5 |
| CA_n25-n66 | 0.3 | 0.3 |
| CA_n25-n71 | - | 0.3 |
| CA_n25-n77 | 0.2 | 0.5 |
| CA_n25-n78 | 0.2 | 0.5 |
| CA_n25-n85 | - | 0.3 |
| CA_n26-n28 | 0.2 | 0.2 |
| CA_n26-n77 | - | 0.5 |
| CA_n26-n78 | - | 0.5 |
| CA_n28-n71 | 0.7 | 0.7 |
| CA_n28-n74 | 0.2 | - |
| CA_n28-n75 | 0.2 | - |
| CA_n28-n77 | 0.2 | 0.5 |
| CA_n28-n78 | 0.2 | 0.5 |
| CA_n28-n79 | 0.2 | 0.5 |
| CA_n28-n102 | 0.2 | 0.5 |
| CA_n29-n77 | 0.2 | 0.5 |
| CA_n28-n94 | 0.1 | 0.2 |
| CA_n30-n66 | 0.5 | 0.4 |
| CA_n30-n77 | - | 0.5 |
| CA_n34-n40 | 0.3 | 0.3 |
| CA_n34-n79 | - | 0.5 |
| CA_n38-n66 | 0.5 | 0.5 |
| CA_n38-n78 | 0.4 | 0.5 |
| CA_n38-n79 | 0.5 | 0.5 |
| CA_n39-n40 | 0.3 | 0.3 |
| CA_n39-n41 | 0.22 / 0.23 | 0.22 / 0.23 |
| CA_n39-n79 | - | 0.5 |
| CA_n40-n77 | 0.4 | 0.5 |
| CA_n40-n78 | 0.4 | 0.5 |
| CA_n40-n79 | - | 0.5 |
| CA_n40-n105 | - | 0.2 |
| CA_n41-n48 | 0.5 | 0.5 |
| CA_n41-n66 | 0.56 / 17 | 0.5 |
| CA_n41-n71 | - | 0.2 |
| CA_n41-n771 | - | 0.5 |
| CA_n41-n781 | - | 0.5 |
| CA_n41-n79 | 0.5 | 0.5 |
| CA_n41-n85 | - | 0.2 |
| CA_ n46-n48 | - | 0.5 |
| CA_n46-n77 | - | 0.5 |
| CA_n46-n78 | - | 0.5 |
| CA_n48-n53 | 0.53 | - |
| CA_n48-n66 | 0.5 | 0.2 |
| CA_n48-n70 | 0.5 | 0.2 |
| CA_n48-n96 | 0.5 | - |
| CA_n50-n78 | 0.22 / 0.23 | 0.22 / 0.23 |
| CA_n66-n77 | 0.2 | 0.5 |
| CA_n66-n78 | 0.2 | 0.5 |
| CA_n67-n78 | 0.2 | 0.5 |
| CA_n70-n77 | 0.2 | 0.5 |
| CA_n70-n78 | 0.2 | 0.5 |
| CA_n71-n77 | 0.2 | 0.5 |
| CA_n71-n78 | 0.2 | 0.5 |
| CA_n74-n77 | - | 0.5 |
| CA_n74-n78 | - | 0.5 |
| CA_n75-n78 | - | 0.5 |
| CA_n76-n78 | - | 0.5 |
| CA_n77-n85 | 0.5 | 0.2 |
| CA_n77-n102 | 0.5 | - |
| CA_n78-n92 | 0.5 | - |
| CA_n78-n102 | 0.5 | - |
| CA_n78-n105 | 0.5 | 0.2 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. NOTE 2: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

<< Next change >>

7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band in frequency range 1 is impacted by UL harmonic interference from another band which belongs to NR band in frequency range 1 of the same downlink CA configuration. Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for either single band uplink or PC3 or PC2 CA are specified in Table 7.3A.4-1. For these exceptions, only the listed test points in Table 7.3A.4-1 are needed to be tested.

Table 7.3A.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n1 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n2 | n48 | 5 | 15 | 25 (RBstart=0) | 5 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n48 | 10 | 15 | 50 (RBstart=0) | 1007 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n48 | 5 | 15 | 25 (RBstart=0) | 10 | 1.9 | NOTE 6 | UL2/DL1 near-miss |
| n2 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n2 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n2 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n3 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n5 | n411 | 5 | 15 | 16 (RBstart=0) | 10 | 10.3 | NOTE 3 | UL3/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n5 | n778 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n5 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.5 | NOTE 4 | UL4/DL1 direct-hit |
| n5 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n7 | n79 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n8 | n313 | N/A | N/A | N/A | N/A | N/A | NOTE 2 | UL2/DL1 direct-hit |
| n8 | n7 | 5 | 15 | 8 (RBstart=0) | 5 | 10 | NOTE 3 | UL3/DL1 direct-hit |
| n8 | n7 | 5 | 15 | 25 (RBstart=0) | 50 | 1.1 | NOTE 3 | UL3/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 13 | NOTE 3 | UL3/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 3.5 | NOTE 3 | UL3/DL1 direct-hit |
| n8 | n77 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 16 (RBstart=0) | 10 | 12.0 | NOTE 5 | UL5/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 25 (RBstart=0) | 100 | 4.4 | NOTE 5 | UL5/DL1 direct-hit |
| n12 | n48 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n12 | n48 | 5 | 15 | 25 (RBstart=0) | 40 | 4.7 | NOTE 5 | UL5/DL1 direct-hit |
| n12 | n66 | 5 | 15 | 8 (RBstart=0) | 5 | 10 | NOTE 3 | UL3/DL1 direct-hit |
| n12 | n66 | 5 | 15 | 20 (RBstart=0) | 40 | 2.4 | NOTE 3 | UL3/DL1 direct-hit |
| n12 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n12 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n12 | n781 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n13 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n13 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n14 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n18 | n77 | 5 | 15 | 16 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n18 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n20 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n20 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n24 | n7712 | N/A | N/A | N/A | N/A | N/A | NOTE 2 | UL2/DL1 direct-hit |
| n24 | n7712 | N/A | N/A | N/A | N/A | N/A | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n48 | 5 | 15 | 25 (RBstart=0) | 10 | 1.9 | NOTE 6 | UL2/DL1 near-miss |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n25 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n26 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | N/A | NOTE 4 | UL4/DL1 direct-hit |
| n26 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | N/A | NOTE 5 | UL5/DL1 direct-hit |
| n26 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n26 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n28 | n1 | 5 | 15 | 8 (RBstart=0) | 5 | 10.2 | NOTE 3 | UL3/DL1 direct-hit |
| n28 | n1 | 5 | 15 | 25 (RBstart=0) | 50 | 1.1 | NOTE 3 | UL3/DL1 direct-hit |
| n28 | n50 | 5 | 15 | 12 (RBstart=0) | 5 | 23.0 | NOTE 2 | UL2/DL1 direct-hit |
| n28 | n50 | 5 | 15 | 25 (RBstart=0) | 80 | 10.8 | NOTE 2 | UL2/DL1 direct-hit |
| n28 | n74 | 5 | 15 | 12 (RBstart=0) | 5 | 23.1 | NOTE 2 | UL2/DL1 direct-hit |
| n28 | n74 | 5 | 15 | 25 (RBstart=0) | 20 | 16.8 | NOTE 2 | UL2/DL1 direct-hit |
| n28 | n75 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 | NOTE 2 | UL2/DL1 direct-hit |
| n28 | n75 | 10 | 15 | 50 (RBstart=0) | 50 | 18.7 | NOTE 2 | UL2/DL1 direct-hit |
| n28 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n28 | n77 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n28 | n78 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n28 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n28 | n94 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 | NOTE 2 | UL2/DL1 direct-hit |
| n28 | n94 | 10 | 15 | 50 (RBstart=0) | 50 | 18.7 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n48 | 5 | 15 | 12 (RBstart=0) | 5 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n48 | 40 | 15 | 200 (RBstart=0) | 1007 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n48 | 5 | 15 | 12 (RBstart=0) | 10 | 1.9 | NOTE 6 | UL2/DL1 near-miss |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n66 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n78 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n70 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n70 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n70 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n70 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n70 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n70 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n71 | n210 | 5 | 15 | 8 (RBstart=0) | 5 | 10 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n71 | 5 | 15 | 8 (RBstart=0) | 5 | 14.6 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 5 | 10 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n2510,11 | 5 | 15 | 8 (RBstart=0) | 40 | 2.1 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n70 | 5 | 15 | 8 (RBstart=0) | 5 | 9.9 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n70 | 5 | 15 | 20 (RBstart=0) | 25 | 4.1 | NOTE 3 | UL3/DL1 direct-hit |
| n85 | n66 | 5 | 15 | 8 (RBstart=0) | 10 | 10 | NOTE 3 | UL3/DL1 direct-hit |
| n85 | n66 | 5 | 15 | 20 (RBstart=0) | 40 | 2.4 | NOTE 3 | UL3/DL1 direct-hit |
| n85 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n85 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n92 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n92 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.0 | NOTE 4 | UL4/DL1 direct-hit |
| n94 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n94 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n105 | n7 | 5 | 15 | 25 (RBstart=0) | 5 | 14.6 | NOTE 3 | UL3/DL1 direct-hit |
| n105 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: Void. NOTE 10: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 11: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1990 MHz. NOTE 12: In the USA, n77 band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. There is no UL harmonic due to n24 UL in downlink for n77 operating in 3450 – 3550 MHz and 3700 – 3980 MHz. NOTE 13: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.2 apply unless otherwise specified). |  |  |  |  |  |  |  |  |

The reference sensitivity for the shared access band does not apply when there is at least one individual RE within the shared access downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3A.4-1a.

Table 7.3A.4-1a: NR-U reference sensitivity measurement exclusion region in MHz.

| NR Band / Harmonic order / Channel BW in UL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL Band | Harmonic order | DL Band | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40MHz | 45MHz | 50 MHz |
| n1 | 3 | n46 | +/- 15 | +/- 23 | +/- 35 | +/- 45 | +/- 60 | +/- 70 | +/- 90 | +/- 100 | +/- 115 |
| n1 | 3 | n102 | +/- 10 | +/- 20 | +/- 30 | +/- 40 | +/- 50 | +/- 60 | +/- 80 |  | +/- 100 |
| n7 | 2 | n46 | +/- 10 | +/- 20 | +/- 30 | +/- 40 | +/- 50 | +/- 60 | +/- 80 |  | +/- 100 |
| n25 | 3 | n46 | +/- 15 | +/- 23 | +/- 35 | +/- 45 | +/- 60 | +/- 70 | +/- 90 |  |  |
| n66 | 3 | n46 | +/- 15 | +/- 23 | +/- 35 | +/- 45 | +/- 60 | +/- 70 | +/- 90 |  |  |
| n48 | 2 | n96 | +/- 10 | +/- 20 | +/- 30 | +/- 40 |  | +/- 60 | +/- 80 |  |  |
| NOTE 1: Even though UL harmonic does not fall directly into NR-U band the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-2: Void

For a PC2 aggressor NR UL band for NR DL CA FR1, the maximum amount of degradation is specified in Table 7.3A.4-2a for a UE not indicating txDiversity-r16 [15].

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1 for UE not supporting Tx Diversity

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 27.1 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 16.6 | NOTE 2 | UL2/DL1 direct-hit |
| NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

Table 7.3A.4-3: Void

Table 7.3A.4-3a: Void

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band is impacted by receiver harmonic mixing due to another band part which belongs to PC3 NR band or PC2 NR band of the same CA configuration. Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for either PC3 or PC2 CA are specified in Table 7.3A.4-4 and from a PC2 aggressor NR UL band for PC2 CA are specified in Table 7.3A.4-4a.For these exceptions, only the listed test points in Table 7.3A.4-4, Table 7.3A.4-4a and Table 7.3A.4-4b are needed to be tested.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n1053 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| n2 | n713 | 5 | 15 | 25 (RBstart=0) | 5 | 26.5 | NOTE 4 | UL1/DL3 |
| n3 | n26 | 5 | 15 | 25 (RBstart=0) | 5 | 3.7 | NOTE 1 | UL1/DL2 |
| n7 | n263 | 25 | 15 | 25 (RBstart=104) | 5 | 2.0 | NOTE 10 | UL1/DL3 Near miss |
| n7 | n71 | 5 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 8 | UL1/ DL4 |
| n25 | n713 | 5 | 15 | 25 (RBstart=0) | 5 | 26.5 | NOTE 4 | UL1/DL3 |
| n25 | n713 | 20 | 15 | 100 (RBstart=0) | 20 | 15.3 | NOTE 4 | UL1/DL3 |
| n40 | n20 | 5 | 15 | 25 (RBstart=0) | 5 | 27.8 | NOTE 4 | UL1/DL3 |
| n40 | n20 | 20 | 15 | 100 (RBstart=0) | 20 | 20.3 | NOTE 4 | UL1/DL3 |
| n40 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 37.8 | NOTE 4 | UL1/DL3 |
| n40 | n28 | 20 | 15 | 100 (RBstart=0) | 20 | 30.3 | NOTE 4 | UL1/DL3 |
| n40 | n77 | 10 | 30 | 24 (RBstart=0) | 10 | 8.3 | NOTE 1 | UL3/DL2 |
| n40 | n77 | 10 | 30 | 24 (RBstart=0) | 100 | 0.4 | NOTE 1 | UL3/DL2 |
| n40 | n78 | 10 | 30 | 24 (RBstart=0) | 10 | 8.3 | NOTE 1 | UL3/DL2 |
| n40 | n78 | 10 | 30 | 24 (RBstart=0) | 100 | 0.4 | NOTE 1 | UL3/DL2 |
| n41 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | n183 | 5 | 15 | 25 (RBstart=0) | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| n41 | n183 | 15 | 15 | 75 (RBstart=0) | 15 | 22.5 | NOTE 4 | UL1/DL3 |
| n41 | n48 | 10 | 30 | 24 (RBstart=0) | 10 | 8.3 | NOTE 9 | UL4/DL3 |
| n41 | n48 | 10 | 30 | 24 (RBstart=0) | 100 | 0.4 | NOTE 9 | UL4/DL3 |
| n41 | n78 | 10 | 30 | 24 (RBstart=0) | 10 | 8.3 | NOTE 1 | UL3/DL2 |
| n41 | n78 | 10 | 30 | 24 (RBstart=0) | 100 | 0.4 | NOTE 1 | UL3/DL2 |
| n46 | n7 | 5 | 15 | 12 (RBstart=0) | 5 | 8.3 | NOTE 7 | UL1/DL2 |
| n46 | n7 | 5 | 15 | 24 (RBstart=0) | 50 | 0.6 | NOTE 7 | UL1/DL2 |
| n46 | n48 | 5 | 15 | 12 (RBstart=0) | 5 | 22.6 | NOTE 2 | UL2/DL3 |
| n46 | n48 | 20 | 15 | 100 (RBstart=0) | 100 | 12 | NOTE 2 | UL2/DL3 |
| n46 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 19.5 | NOTE 2 | UL2/DL3 |
| n46 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 12 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 19.5 | NOTE 2 | UL2/DL3 |
| n46 | n78 | 20 | 15 | 100 (RBstart=0) | 100 | 12 | NOTE 2 | UL2/DL3 |
| n77 | n2 | 10 | 15 | 25 (RBstart=0) | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n77 | n2 | 20 | 15 | 100 (RBstart=0) | 20 | 3,7 | NOTE 7 | UL1/DL2 |
| n77 | n5 | 5 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n77 | n5 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 | NOTE 8 | UL1/DL4 |
| n77 | n12 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n12 | 15 | 15 | 75 (RBstart=0) | 15 | 26.2 | NOTE 5 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n13 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n14 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |
| n77 | n25 | 10 | 15 | 25 (RBstart=0) | 5 | 6.7 | NOTE 7 | UL1/DL2 |
| n77 | n25 | 20 | 15 | 100 (RBstart=0) | 40 | 1.1 | NOTE 7 | UL1/DL2 |
| n776 | n29 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n776 | n29 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |
| n77 | n30 | 10 | 15 | 12 (RBstart=0) | 5 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n30 | 10 | 15 | 24 (RBstart=0) | 10 | 8.0 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 10 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n40 | 20 | 30 | 50 (RBstart=0) | 100 | 0.9 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 10.4 | NOTE 2 | UL2/DL3 |
| n77 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 6.3 | NOTE 2 | UL2/DL3 |
| n776 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 7 | UL1/DL2 |
| n77 | n85 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n85 | 15 | 15 | 75 (RBstart=0) | 15 | 26.2 | NOTE 5 | UL1/DL5 |
| n78 | n12 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n78 | n40 | 20 | 30 | 50 (RBstart=0) | 5 | 10.4 | NOTE 2 | UL2/DL3 |
| n78 | n40 | 20 | 30 | 50 (RBstart=0) | 80 | 4.5 | NOTE 2 | UL2/DL3 |
| n78 | n41 | 20 | 30 | 50 (RBstart=0) | 10 | 10.4 | NOTE 2 | UL2/DL3 |
| n78 | n41 | 20 | 30 | 50 (RBstart=0) | 100 | 6.3 | NOTE 2 | UL2/DL3 |
| n78 | n67 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n78 | n67 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |
| n79 | n8 | 10 | 15 | 25 (RBstart=0) | 5 | 25 | NOTE 5 | UL1/DL5 |
| n96 | n48 | 5 | 15 | 25 (RBstart=0) | 5 | 5.8 | NOTE 2 | UL2/DL3 |
| n96 | n48 | 20 | 15 | 100 (RBstart=0) | 100 | 0.5 | NOTE 2 | UL2/DL3 |
| n102 | n1 | 20 | 15 | 100 (RBstart=0) | 5 | 13.5 | NOTE 4 | UL1/DL3 |
| NOTE 1: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 5: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 6: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 9: The requirements should be verified for DL NR-ARFCN of the victim (higher) band (superscript HB) such that fDLHB=fULLB/0.75 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 10: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band |  |  |  |  |  |  |  |  |

<Unchanged text are omitted>

The reference sensitivity for the shared access band does not apply when there is at least one individual RE within the shared access downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3A.4-1c.

Table 7.3A.4-4c: NR-U reference sensitivity measurement exclusion region in MHz.

| NR Band / Harmonic order / Channel BW (MHz) in UL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL Band | UL/DL Harmonic Order | DL Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| n78 | UL3/DL2 | n46 |  | +/-23 | +/-35 | +/-45 | +/-60 | +/-70 | +/-90 | +/-115 | +/-140 | +/-160 | +/-180 | +/-210 | +/-230 |
| n77 | UL3/DL2 | n46 |  | +/-23 | +/-35 | +/-45 | +/-60 | +/-70 | +/-90 | +/-115 | +/-140 | +/-160 | +/-180 | +/-210 | +/-230 |
| NOTE 1: Even though UL harmonic does not fall directly into NR-U band the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the ratio of the UL harmonic order over the DL harmonic order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-5: Void

<Unchanged text are omitted>

7.3A.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands given in Table 7.3A.5-1, Table 7.3A.5-1a, Table 7.3A.5-2 and Table 7.3A.5-2a the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.5-1, Table 7.3A.5-1a, Table 7.3A.5-2 and Table 7.3A.5-2a. For these test points the reference sensitivity requirement specified in Table 7.3.2-1 and Table 7.3.2-2 are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.5-1, Table 7.3A.5-1a, Table 7.3A.5-2 and Table 7.3A.5-2a.

Table 7.3A.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | TDD | N/A |
| CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
| CA_n1-n46 | n1 | 1950 | 5 | 25 | 2140 | 5 | FDD | IMD5 |
|  | n46 | 5660 | 10 | 50 | 5660 | N/A | TDD | N/A |
| CA_n1-n77 | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD44 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n1-n102 | n1 | 1977.5 | 5 | 25 | 2167.5 | 13 | FDD | IMD3 |
|  | n102 | 5935 | 20 | 100 | 5935 | N/A | TDD | N/A |
| CA_n1-n105 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n105 | 673 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |
| CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_n2-n66 | n2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | 8.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987.5 | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 RBSTART=10 | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 RBSTART=0 | 3945 |  |  |  |
| CA_n2-n78 | n2 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n3-n5 | n3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | n3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | FDD | IMD23 |
| CA_n3-n7 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | FDD | IMD4 |
| CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 8 | FDD | IMD44 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | FDD | N/A |
| CA_n3-n18 | n18 | 818 | 5 | 25 | 863 | N/A | FDD | N/A |
|  | n3 | 1731 | 5 | 25 | 1826 | 4 | FDD | IMD4 |
| CA_n3-n26 | n3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | n26 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | n3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | n26 | 838 | 5 | 25 | 883 | 26 | FDD | IMD211 |
| CA_n3-n20 | 3 | 1775 | 5 | 25 | 1870 | 4 | FDD | IMD4 |
|  | 20 | 840 | 5 | 25 | 799 | N/A | FDD | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | 20 | 847 | 5 | 25 | 806 | 9 | FDD | IMD4 |
| CA_n3-n38 | n3 | 1713 | 5 | 25 | 1808 | 8.2 | FDD | IMD4 |
|  | n38 | 2617 | 5 | 25 | 2617 | N/A | TDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | N/A14 | FDD | IMD9 |
|  | n41 | 2545 | 60 | 1 RBSTART= 0 | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 RBSTART= 272 | 2625 |  |  |  |
|  | n3 | 1747.5 | 5 | 25 RBSTART= 0 | 1842.5 | 15.3 | FDD | IMD3 |
|  | n41 | 2560 | 60 | 1 RBSTART= 30 | 2560 | N/A | TDD | N/A |
|  |  | 2620 | 60 | 1 RBSTART= 127 | 2620 |  |  |  |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | N/A | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | [2.2] | FDD | IMD7 |
|  | n77 | 3455 | 10 | 1 (RBstart=10) | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBstart=0) | 3945 |  |  |  |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 2.2 | FDD | IMD7 |
|  | n78 | 3305 3780 | 10 10 | 1 (RBstart=3) 1 (RBstart=0) | 3305 3780 | N/A | TDD | N/A |
| CA_n5-n7 | n5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | n7 | 2547 | 10 | 50 | 2667 | N/A | FDD | N/A |
| CA_n5-n14 | n5 | 836 | 5 | 25 | 881 | 25 | FDD | IMD34 |
|  | n14 | 791 | 5 | 25 | 761 | N/A | FDD | N/A |
|  | n5 | 826.5 | 5 | 25 | 871.5 | N/A | FDD | N/A |
|  | n14 | 795.5 | 5 | 25 | 765.5 | 25 | FDD | IMD3 |
| CA_n5_n41 | n5 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | TDD | N/A |
| CA_n5-n66 | n5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n5-n7713 | n5 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
|  | n5 | 829 | 5 | 25 | 874 | 5.5 | FDD | IMD5 |
|  | n5 | N/A | 5 | N/A | 880 | 8.6 | FDD | IMD4 |
|  | n7712 | 3410 | 10 | 1 RBSTART=25 | 3410 | N/A | TDD | N/A |
|  |  | 3850 | 10 | 1 RBSTART=25 | 3850 |  |  |  |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | TDD | N/A |
| CA_n5-n78 | n5 | 844 | 5 | 25 | 889 | 8.3 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| CA_n7-n26 | n7 | 2556 | 5 | 25 | 2676 | N/A | FDD | N/A |
|  | n26 | 837 | 5 | 25 | 882 | 16.0 | FDD | IMD311 |
|  | n7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | FDD | IMD5 |
|  | n26 | 816.5 | 5 | 25 | 861.5 | N/A | FDD | N/A |
| CA_n7-n40 | n7 | 2510 | 5 | 25 | 2630 | 23 | FDD | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | TDD | N/A |
| CA_n7-n46 | n7 | 2550 | 10 | 50 | 2670 | 26.8 | FDD | IMD24 |
|  | n46 | 5220 | 20 | 50 | 5220 | N/A | TDD | N/A |
| CA_n7-n66 | n7 | 2535 | 10 | 50 | 2655 | 15 | FDD | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
| CA_n7-n77 | n7 | 2540 | 5 | 25 | 2660 | 7.1 | FDD | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | TDD | N/A |
|  | n7 | N/A | 5 | N/A | 2687.5 | 15 | FDD | IMD5 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=0) | 3455 | N/A | TDD | N/A |
|  |  | 3835 | 10 | 1 (RBSTART=7) | 3835 |  |  |  |
| CA_n7-n78 | n7 | N/A | 5 | N/A | 2650 | 15 | FDD | IMD5 |
|  | n7812 | 3350 | 10 | 1 RBSTART=25 | 3350 | N/A | TDD | N/A |
|  |  | 3700 | 10 | 1 RBSTART=25 | 3700 |  |  |  |
| CA_n8-n41 | n8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |
| CA_n8-n79 | n8 | 897.5 | 5 | 25 | 942.5 | 4.8 | FDD | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | TDD | N/A |
| CA_n12-n66 | n12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n66 | 1765 | 5 | 25 | 2115 | 5.0 | FDD | IMD4 |
| CA_n12-n77 | n12 | 702 | 5 | 20 | 732 | 5.5 | FDD | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
| CA_n12-n78 | 12 | 710 | 5 | 25 | 740 | 5.5 | FDD | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | TDD | N/A |
| CA_n13-n77 | n13 | 782 | 5 | 20 | 751 | 5.5 | FDD | IMD5 |
|  | n77 | 3880 | 10 | 50 | 3880 | N/A | TDD | N/A |
| CA_n14-n77 | n14 | 793 | 5 | 20 | 763 | 5.5 | FDD | IMD5 |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | TDD | N/A |
| CA_n18-n778 | n18 | N/A | N/A | N/A | N/A | N/A | FDD | IMD4/5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n18-n789 | n18 | N/A | N/A | N/A | N/A | N/A | FDD | IMD4 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n20-n78 | n20 | 850 | 5 | 25 | 809 | 11 | FDD | IMD4 |
|  | n78 | 3359 | 10 | 50 | 3359 | N/A | TDD | N/A |
| CA_n24-n7710 | n24 | N/A | N/A | N/A | N/A | N/A | FDD | IMD4 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n25-n41 | n25 | N/A | 5 | N/A | 1992.5 | 8.5 | FDD | IMD7 |
|  | n41 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=221) | 2640 |  |  |  |
| CA_n25-n48 | n25 | 1852.5 | 5 | 25 | 1932.5 | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_n25-n66 | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | 23 | FDD | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | FDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 | 8.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | N/A | 5 | N/A | 1987.5 | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 RBSTART=10 | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 RBSTART=0 | 3945 |  |  |  |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
| CA_n26-n66 | n26 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
| CA_n26-n70 | n26 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | n70 | 1710 | 5 | 25 | 2020 | N/A | FDD | N/A |
| CA_n26-n7713 | n26 | N/A | N/A | N/A | N/A | N/A | FDD | IMD4 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n26 | N/A | N/A | N/A | N/A | N/A | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |
| CA_n28-n50 | n28 | 730 | 10 | 50 | 775 | 15.3 | FDD | IMD2 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | TDD | N/A |
|  | n28 | 740 | 10 | 50 | 785 | 6.0 | FDD | IMD44 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | TDD | N/A |
| CA_n28-n74 | n28 | 705.5 | 5 | 25 | 760.5 | 24.6 | FDD | IMD2 |
|  | n74 | 1466 | 5 | 25 | 1514 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 11.3 | FDD | IMD411 |
|  | n74 | 1431 | 5 | 25 | 1479 | N/A | FDD | N/A |
|  | n28 | 709 | 5 | 25 | 764 | N/A | FDD | N/A |
|  | n74 | 1466 | 5 | 25 | 1514 | 14.6 | FDD | IMD4 |
|  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | FDD | N/A |
|  | n74 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | FDD | IMD5 |
| CA_n28-n77 | n28 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_n28-n77 | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | FDD | IMD5 |
|  | n77/n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 8.0 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |
|  | n30 | N/A | 5 | N/A | 2352.5 | 3.2 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBstart=17) | 3455 | N/A | TDD | N/A |
|  |  | 3825 | 10 | 1 (RBstart=0) | 3825 |  |  |  |
| CA_n41-n66 | n4112 | 2545 | 90 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2640 | 100 | 1 (RBstart=171) | 2640 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | 32.5 | FDD | IMD5 |
| CA_n41-n71 | n41 | 2614 | 5 | 25 | 2614 | N/A | TDD | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 11 | FDD | IMD4 |
| CA_n41-n77 | n4112 | 2545 | 60 | 1 (RBstart=0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBstart=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | FDD | IMD9 |
| CA_n46-n77 | n46 | N/A | 10 | N/A | 5155 | N/A | TDD | IMD5 |
|  | n7712 | 3385 | 10 | 1 (RBSTART= 25) | 3385 | N/A | TDD | N/A |
|  |  | 3975 | 10 | 1 (RBSTART= 25) | 3975 |  |  |  |
|  | n46 | N/A | 10 | N/A | 5660 | N/A | TDD | IMD6 |
|  | n7712 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
|  | n46 | N/A | 10 | N/A | 5230 | N/A | TDD | IMD7 |
|  | n7712 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
| CA_n46-n78 | n46 | N/A | 10 | N/A | 5660 | N/A | TDD | IMD6 |
|  | n7812 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
|  | n46 | N/A | 10 | N/A | 5230 | N/A | TDD | IMD7 |
|  | n7812 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
| CA_n48-n66 | n48 | 3660 | 5 | 25 | 3660 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
| CA_n48-n70 | n70 | 1697.5 | 25/15 | 25 | 1997.5 | 26 | FDD | IMD24 |
|  | n48 | 3695 | 10 | 50 | 3695 | N/A | TDD | N/A |
| CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | FDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 5.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2197.5 | 15 | FDD | IMD513 |
|  | n7712 | 3305 | 10 | 1 (RBstart=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBstart=8) | 3855 |  |  |  |
|  | n66 | 1730 | 5 | 25 | 2130 | 1.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBstart=10) | 3455 | N/A | TDD | N/A |
|  |  | 3875 | 10 | 1 (RBstart=0) | 3875 |  |  |  |
| CA_n66-n78 | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |
| CA_n66-n85 | n66 | 1770 | 5 | 25 | 2138 | 5 | FDD | IMD4 |
|  | n85 | 701 | 5 | 25 | 731 | N/A | FDD | N/A |
| CA_n67-n78 | n67 | N/A | 5 | N/A | 748 | 8.6 | SDL | IMD415 |
|  | n7812 | 3376 | 10 | 1 RBSTART=25 | 3376 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 RBSTART=25 | 3750 |  |  |  |
| CA_n70-n71 | n70 | 1697.5 | 5 | 25 | 1997.5 | 5 | FDD | IMD4 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 31 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | 5.0 | FDD | IMD5 |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |
| CA_n70-n78 | n70 | 1705 | 5 | 25 | 2005 | 31 | FDD | IMD2 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
|  | n70 | 1705 | 5 | 25 | 2005 | 5.0 | FDD | IMD5 |
|  | n78 | 3560 | 10 | 50 | 3560 | N/A | TDD | N/A |
| CA_n71-n7713 | n71 | 671 | 5 | 25 | 625 | 5.5 | FDD | IMD5 |
|  | n77 | 3309 | 10 | 50 | 3309 | N/A | TDD | N/A |
|  | n71 | N/A | 5 | N/A | 640 | 8.6 | FDD | IMD4 |
|  | n7712 | 3480 | 10 | 1 RBSTART=25 | 3480 | N/A | TDD | N/A |
|  |  | 3800 | 10 | 1 RBSTART=25 | 3800 |  |  |  |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
| CA_n77-n856 | n77 | 3590 | 10 | 25 | 3590 | N/A | TDD | N/A |
|  | n85 | 712 | 10 | 25 | 742 | 5.5 | FDD | IMD5 |
| CA_n78-n102 | n78 | 3320 | 10 | 1 (RBstart=25) | 3320 | N/A | TDD | N/A |
|  |  | 3680 | 10 | 1 (RBstart=25) | 3680 |  |  |  |
|  | n102 | 6280 | 20 | 100 | 6280 | N/A11 | TDD | IMD4 |
| CA_n78-n105 | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
|  | n105 | 682.5 | 5 | 25 | 631.5 | 5.5 | FDD | IMD5 |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 NOTE 2: RBSTART = 0, 15 kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD2 MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE8: There is no IMD4/5 products in band n18 downlink for n77 operating in 3520 – 3560 MHz, 3700 – 3800MHz and 4000 - 4100MHz frequency range. NOTE 9: There is no IMD4 product in band n18 downlink for n78 operating in 3520 – 3560MHz and 3700-3800MHz frequency range. NOTE 10: There is no IMD4 product in band n24 downlink for n77 operating in 3450 – 3980 MHz and n24 uplink restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz. NOTE 11: This band is subject to IMD5 also which MSD is not specified.. NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

<Unchanged texts are omitted>

7.3A.6	Reference sensitivity exceptions due to cross band isolation for CA

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part which belongs to NR band of the same NR CA configuration due to cross band isolation issues. Thereference sensitivity degradation for the victim band due to cross band isolation is specified only for the specific uplink and downlink test points specified in Table 7.3A.6-1 for either PC3 and PC2 NR CA from a PC3 aggressor NR UL band, and for PC2 NR CA, in Table 7.3A.6-1afrom a PC2 aggressor NR UL band, and in Table 7.3A.6-1b from a PC1.5 aggressor NR single band uplink

In Tables 7.3A.6-1, 7.3A.6-1a and 7.3A.6-1b the following terminology is used to define the source of cross-band isolation interference:

- “ACLR1” indicates that the first adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

- “ACLR2” indicates that the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

“>ACLR2” indicates that neither the first, nor the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

Table 7.3A.6-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n1 | n3 | 1922.5 | 5 | 15 | 25 (RBstart=0) | 1877.5 | 5 | 3 | >ACLR2 |
| n1 | n3 | 1945 | 50 | 15 | 128 (RBstart=0) | 1877.5 | 5 | 19.7 | ACLR1 |
| n1 | n38 | 1955 | 50 | 15 | 128 (RBstart=142) | 2572.5 | 5 | 2.9 | >ACLR2 |
| n1 | n38 | 1955 | 50 | 15 | 128 (RBstart=142) | 2590 | 40 | 2.9 | >ACLR2 |
| n1 | n40 | 1970 | 20 | 15 | 100 (RBstart=6) | 2302.5 | 5 | 6.6 | >ACLR2 |
| n1 | n41 | 1955 | 50 | 15 | 128 (RBstart=142) | 2501 | 10 | 6.1 | >ACLR2 |
| n1 | n41 | 1970 | 20 | 15 | 100 (RBstart=6) | 2546 | 100 | 0.7 | >ACLR2 |
| n3 | n41 | 1760 | 50 | 15 | 50 (RBstart=220) | 2501 | 10 | 0.7 | >ACLR2 |
| n3 | n41 | 1760 | 50 | 15 | 50 (RBstart=220) | 2546 | 100 | 0.7 | >ACLR2 |
| n3 | n74 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 2.6 | >ACLR2 |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 4.3 | >ACLR2 |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |
| n5 | n71 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 3.9 | >ACLR2 |
| n7 | n3 | 2525 | 50 | 15 | 45 (RBstart=0) | 1877.5 | 5 | 0.6 | >ACLR2 |
| n7 | n40 | 2525 | 50 | 15 | 45 (RBstart=0) | 2397.5 | 5 | 3.7 | >ACLR2 |
| n12 | n71 | 706.5 | 15 | 15 | 20 (RBstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |
| n18 | n285 | 822.5 | 15 | 15 | 25 (RBstart=0) | 800.5 | 5 | 31.3 | ACLR1 |
| n18 | n28 | 822.5 | 15 | 15 | 25 (RBstart=0) | 785.5 | 5 | 12.7 | ACLR2 |
| n34 | n3 | 2017.5 | 15 | 15 | 75 (RBstart=0) | 1877.5 | 5 | 3 | >ACLR2 |
| n38 | n1 | 2590 | 40 | 15 | 216 (RBstart=0) | 2167.5 | 5 | 1.9 | >ACLR2 |
| n38 | n25 | 2590 | 40 | 15 | 216 (RBstart=0) | 1992.5 | 5 | 0.6 | >ACLR2 |
| n38 | n78 | 2600 | 40 | 15 | 216 (RBstart=0) | 3305 | 10 | 8.3 | >ACLR2 |
| n40 | n1 | 2340 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | [21.9] | ACLR2 |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2622.5 | 5 | 22.3 | >ACLR2 |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2645 | 50 | 15.6 | >ACLR2 |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 18.1 | >ACLR2 |
| n41 | n2 | 2546 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 0.6 | >ACLR2 |
| n41 | n3 | 2546 | 100 | 30 | 270 (RBstart=0) | 1877.5 | 5 | 0.6 | >ACLR2 |
| n41 | n25 | 2546 | 100 | 30 | 270 (RBstart=0) | 1992.5 | 5 | 0.6 | >ACLR2 |
| n41 | n48 | 2680 | 100 | 30 | 270 (RBstart=3) | 3552.5 | 5 | 8.3 | >ACLR2 |
| n411 | n66 | 2546 | 100 | 30 | 270 (RBstart=0) | 2197.5 | 5 | 10.5 | >ACLR2 |
| n41 | n70 | 2546 | 100 | 30 | 270 (RBstart=0) | 2017.5 | 5 | 0.6 | >ACLR2 |
| n41 | n77 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 8.3 | >ACLR2 |
| n41 | n78 | 2640 | 100 | 30 | 270 (RBstart=3) | 3305 | 10 | 8.3 | >ACLR2 |
| n46 | n48 | 5190 | 80 | 30 | 216 (RBstart=0) | 3697.5 | 5 | 13.3 | >ACLR2 |
| n46 | n48 | 5190 | 80 | 30 | 216 (RBstart=0) | 3650 | 100 | 6.2 | >ACLR2 |
| n46 | n77 | 5190 | 80 | 30 | 216 (RBstart=0) | 3975 | 10 | 10.5 | >ACLR2 |
| n46 | n77 | 5190 | 80 | 30 | 216 (RBstart=0) | 3930 | 100 | 5.5 | >ACLR2 |
| n46 | n78 | 5190 | 80 | 30 | 216 (RBstart=0) | 3795 | 10 | 10.4 | >ACLR2 |
| n46 | n78 | 5190 | 80 | 30 | 216 (RBstart=0) | 3750 | 100 | 5.1 | >ACLR2 |
| n48 | n411 | 3570 | 40 | 15 | 216 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n48 | n411 | 3570 | 40 | 15 | 216 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| n48 | n46 | 3680 | 40 | 15 | 216 (RBstart=0) | 5160 | 20 | 15.7 | >ACLR2 |
| n48 | n96 | 3680 | 40 | 15 | 216 (RBstart=0) | 5935 | 20 | 15.7 | >ACLR2 |
| n71 | n5 | 688 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.0 | >ACLR2 |
| n71 | n12 | 683 | 20 | 15 | 20 (RBstart=86) | 731.5 | 5 | 8.2 | ACLR2 |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 17.5 | ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 10 | 4.5 | >ACLR2 |
| n77 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n77 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| n78 | n71 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 4.5 | >ACLR2 |
| n78 | n38 | 3350 | 100 | 30 | 270 (RBstart=0) | 2617.5 | 5 | 3.3 | >ACLR2 |
| n78 | n38 | 3350 | 100 | 30 | 270 (RBstart=0) | 2600 | 40 | 3.3 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 4.5 | >ACLR2 |
| n78 | n401 | 3350 | 100 | 30 | 270 (RBstart=0) | 2350 | 100 | 4.5 | >ACLR2 |
| n78 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2685 | 10 | 4.5 | >ACLR2 |
| n78 | n411 | 3350 | 100 | 30 | 270 (RBstart=0) | 2640 | 100 | 4.5 | >ACLR2 |
| n78 | n46 | 3750 | 100 | 30 | 270 (RBstart=3) | 5160 | 20 | 13.5 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4420 | 40 | 2 | >ACLR2 |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 2 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | 2.6 | >ACLR2 |
| n79 | n783 | 4450 | 100 | 30 | 270 (RBstart=0) | 3750 | 100 | 2.6 | >ACLR2 |
| n96 | n48 | 5965 | 80 | 30 | 216 (RBstart=0) | 3697.5 | 5 | 13.3 | >ACLR2 |
| n96 | n48 | 5965 | 80 | 30 | 216 (RBstart=0) | 3650 | 100 | 6.2 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: Void NOTE 5: The MSD exceptions are applicable to the case that interference of UL band 3rd order IMD product falls into the affected DL channels. |  |  |  |  |  |  |  |  |  |

Table 7.3A.6.2: Void

<< Next change >>

7.6A.3.3	Out-of-band blocking for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the out-of-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in clause 7.6.3 for each component carrier while all downlink carriers are active.

For inter-band carrier aggregation with component carriers in operating bands < 2.7GHz including n48, and for FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz, the appropriate adjacent channel selectivity and in-band blocking requirements in the respective clauses 7.5 and 7.6.2 shall be applied for carrier j. For inter-band carrier aggregation with component carriers in operating bands > 2.7GHz excluding n48, and for FDL_Low(j) – 3* BWchannel ≤ f ≤ FDL_High(j) + 3* BWchannel, the appropriate adjacent channel selectivity and in-band blocking requirements in the respective clauses 7.5 and 7.6.2 shall be applied for carrier j. FDL_Low(j) and FDL_High(j) denote the respective lower and upper frequency limits of the operating band containing carrier j, j = 1,…,X, with carriers numbered in increasing order of carrier frequency and X the number of component carriers in the band combination. BWchannel denotes the channel bandwidth of the wanted signal component carrier j. If CW interferer falls in a gap between FDL_High(j) and FDL_Low(j+1) where the corresponding OOB ranges 1 and 2 overlap, then the lower level interferer limit of the overlapping OOB ranges applies.

For inter-band carrier aggregation with uplink assigned to two NR bands, the out-of-band blocking requirements specified in clause 7.6.3 shall be met with the transmitter power for the uplink set to 7 dB below PCMAX_L,f,c  for each serving cell c.

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.1-1, Pinterferer power defined in Table 7.6.3-2 and 7.6.3-4 is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.1-1.

For inter-band CA combination listed in Table 7.6A.3.3-1, exceptions to the requirement specified in Table 7.6A.3.3-2 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier. Unless otherwise stated, the exceptions apply to any power classes for the listed inter-band CA combinations.

Table 7.6A.3.3-1: CA band combination with exceptions allowed

CA band combination

CA_n1-n102

CA_n5-n77

CA_n5-n78

CA_n5-n79

CA_n7-n8

CA_n7-n46

CA_n7-n105

CA_n8-n77

CA_n8-n78

CA_n8-n79

CA_n12-n48

CA_n12-n77

CA_n12-n78

CA_n13-n77

CA_n14-n77

CA_n18-n77

CA_n18-n78

CA_n20-n78

CA_n26-n77

CA_n26-n78

CA_n28-n46

CA_n28-n77

CA_n28-n78

CA_n28-n79

CA_n28-n102

CA_n48-n71

CA_n48-n77

CA_n71-n77

CA_n71-n78

CA_n77-n85

CA_n78-n92

CA_n78-n105

Table 7.6A.3.3-1a: Void

<< End of change >>