3GPP TSG-RAN WG4 Meeting #107 	R4-2308217

# Incheon, Korea, May 22 – May 26, 2023

Source: 	vivo

Title: 	Discussion on cell switch delay requirements in R18 LTM

Agenda Item:	8.25.3.3

Document for:	Discussion

Introduction

In RAN4 #106 and RAN4 #106-bis-e, R18 L1L2-triggered mobility was discussed in RRM session and the WF was agreed in [1] [2].

Based on discussion RAN4 has common understanding on the following baseline delay requirements for cell switch.

Issue 3-3-1: RACH-based Cell switch delay for Pcell/PSCell

< Agreement>:

  - The baseline of RACH-based cell switch delay requirements is Tdelay = Tcmd + Tprocessing / TLTM-processing + Tsearch + T∆ + Tmargin + Tuncertainity /TIU, where Tuncertainity /TIU is the uncertainty in acquiring the first available PRACH occasion in the new cell.

    - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed.

    - FFS: add/remove/modify other component(s).

Note: ‘/’ means ‘or’ here.

Issue 3-3-2: RACH-less Cell switch delay for Pcell/PSCell

< Agreement>:

  - The baseline of RACH-less cell switch delay requirements is Tdelay = Tcmd + Tprocessing,2 / TLTM-processing + Tsearch + T∆ + Tmargin + Tuncertainity/TIU,

    - FFS: the ending point

    - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed.

    - FFS: add/remove/modify other component(s).

Note: ‘/’ means ‘or’ here.

The above baseline is generally aligned with RAN2’s assumption listed in [3].

Figure 2-1: Components of Mobility Latency (before enhancements) in [3]

Based on all above information, we provide our views on the cell switch delay requirements for R18 L1L2-triggered mobility.

Discussion

<On generals and principles>

In last meeting, the following issues are discussed.

Issue 3-1-3: Whether to define requirements for Direct SCell activation at SpCell change (target SCell is not current PCell)

< Agreement>:

  - No more discussion until RAN2 concludes to support direct SCell activation at SpCell change

Issue 3-1-5: SpCell cell switch delay requirements for SpCell change with SCell change

< Way forward >: FFS the following options

  - Option 1: For the scenario “PCell change with SCell change”, PCell switch delay is not extended by SCell changes, i.e., UE is supposed to perform SCell change after cell switch of PCell is finished.

  - Option 2: FFS

In our understanding, these two issues are technically the same. RAN4 only specify RRM requirements for SCell activation, and normally SCell change is also considered as SCell activation. Since RAN2 has not yet agree on the direct activation part, RAN4 can also hold on the requirements for SpCell change with SCell change case, even though the scenario is already agreed to be support by RAN2.

Proposal 1  RAN4 hold on the discussion on RRM requirements for SpCell change with SCell change until RAN2 concludes to support direct SCell activation at SpCell change.

In last meeting, the following issue is discussed.

Issue 3-2-2: Procedure of cell switch

< Way forward >: FFS the following options

  - Option 1: Further discuss whether UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

  - Option 2:  Under the condition that target cell is known, UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

  - Option 3: if T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2).

  - Option 4: UE cannot perform T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell. If needed, some other L1 configuration, L2/L3 reconfiguration and L2 reset can be performed during cell switch, i.e., in some cases it can be performed after the T/F tracking.

Firstly, in our understanding, for the inter-frequency cell switch, the L1/L2/L3 reconfiguration includes RF warm-up and software processing. T/F tracking should be performed after UE correctly configures at least L1 according to the RRC configuration from gNB. UE should obtain the RRC configuration details, e.g. BWP configuration, QCL configuration, SSB configuration etc., before UE performs T/F tracking. So far, RAN2 has made some of the configuration outside serving cell configuration, which is accessible to the UE right at the point when candidate cell configuration is received.

On the other hand, certainly the L2/L3 reconfiguration is not related to T/F tracking in physical layer. We note that the MAC/RLC/PDCP reset is up-to RAN2 decision. UE processing of these procedures can be postponed after T/F tracking.

Proposal 2  UE can not perform T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell.

- If needed, some other L1 configuration, L2/L3 reconfiguration and L2 reset can be performed during cell switch, i.e. in some cases it can be performed after the T/F tracking.

<On Detail of cell swith delay requirements for Pcell/PSCell >

In last meeting, general timeline for both RACH-based cell switch and RACH-less cell switch are agreed. However, the endpoint for RACH-less Cell switch are still FFS.

For RACH-less cell switch, the UL sync is not performed during cell switch procedure. As the end point, UE may perform UL/DL transmission in the target cell, which is based on the TCI and/or uplink timing indication it has maintained before the cell switch.

For FR1, beam indications are not always necessary, but UL/DL timing indication and UL pathloss indication can be possible. Therefore, we prefer to revise the wording a little.

Proposal 3  For RACH-less cell switch, the end point is defined when UE performs the first DL/UL reception/transmission on the target cell based on the indicated transmission configuration.

In last meeting, the following issue is discussed.

Issue 3-3-3: Tprocessing,2

- What UE has done during Tprocessing, 1 before cell switch command? What UE needs to do during Tprocessing, 2 after cell switch command?

- What is the impact of reference +delta configuration on the processing time?

- What UE has prepared for target cell after performing pre DL synchronization?

- What UE has get prepared for target cell after performing pre UL synchronization?

- The factors to consider when target cell is a current scell ?

- How many different processing time values to define?

< Way forward >: FFS the following options

  - Option 1 (CTC, CMCC, Nokia, Huawei): The time for UE processing could be reduced if some procedures have been done before UE receive the cell switch command or for some scenarios.

    - Option 1a (CMCC): Tprocessing = 0 for the case that DL synchronization for candidate cell(s) is performed before cell switch command

    - Option 1b (Nokia):

      - Some of the RRC processing can be done prior to the cell switch to reduce the delay. FFS what parts are done when the configuration arrives.

      - At least partial RRC processing is done before the cell switch command. Exact conditions can be FFS.

      - LTM-processing Should consider a scenario where the user plane does not need reset, and there is no need for further RRC processing.

  - Other options omitted…

Full configuration, reference configuration and delta configuration are being discussed in RAN2. In RAN2 #112 meeting the following agreements is achieved.

- Agreements in RAN2 #112:

Usage of reference configuration:

- 	Candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration (FFS if done at cell switch or before the cell switch)

- 	The complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset RLC or PDCP.

-	To support reconfigurations that requires reset of RLC PDCP, this should be possible (in principle same a full config)

-	FFS if more than RLC PDCP should be kept and how much of “replacing” need to be specified.

-	FFS if the reference configuration can be derived from the current UE configuration at some point of time.

Agreement in RAN2 #112-bis-e:

Whether the Reference configuration is a complete configuration or not is up to the network implementation.

Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration.

The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current).

Confirm that only the replacement procedure (the “full config without L2 reset”) is supported for Execution of LTM cell switch.

The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed.

Based on reference configuration, the UE internal memory needed for saving RRC configurations can be saved. Forming the full configuration before cell switch does not have significant gain in reducing interruption, but may have quite large cost in UE’s memory if multiple candidate cells are configured.

Proposal 4  For the baseline requirement, RAN4 assume UE forms the complete configuration and applies it during the execution of cell switch command.

In last meeting, the following issues are discussed.

Issue 3-3-8: Whether to define RACH-based cell switch delay requirements for unknown cell

< Way forward >:

  - Not define RACH-based cell switch delay requirements for unknown cell case.

  - FFS wait for RAN1 progress

Issue 3-3-10: Whether to define RACH-less cell switch delay requirements for unknown cell

< Way forward >:

  - Not define RACH-less cell switch delay requirements for unknown cell case.

  - FFS wait for RAN1 progress

As discussed in our companion paper, fine-Rx-beam-based L1 measurement, if configured, should be performed on known cell only. RAN4 not to consider fine-Rx-beam-based L1 measurement requirements if the measurement is indicated to be performed on unknown cell.

For LTM cell switch based on either rough-Rx-beam-based L1 measurement or fine-Rx-beam-based L1 measurement, the cell should also be known when cell switch command is received. Otherwise, UE may still need to perform cell search, which cause quite long delay/interruption during the cell switch, and the potential benefit introduced by LTM is wasted. Considering L3 measurement based HO/PSCel_change already support the unknown case, we think as the optimization to legacy L3 HO/PSCel_change, it is reasonable not to consider the known case at least in RAN4. In other word, RAN4 NOT to define requirements if the cell switch command indicates switching to a cell that is unknown.

Proposal 5  RAN4 not to define cell switch delay requirements for the case that the target cell is unknown.

In last meeting, the following issue is discussed.

Issue 3-3-5: T/F fine tracking: TΔ and Tmargin

< Way forward >: FFS the following options

  - Option 1: The baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms. FFS: whether TΔ and Tmargin can be 0 under certain conditions.

    - Option 1a: TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms.

    - Option 1b: TΔ and Tmargin can be 0 under the condition that DL synchronization for candidate cell(s) has been performed before cell switch command.

    - Option 1c: When TCI state to use for the target cell is already in the active TCI state list, it is also necessary for the UE to meet the Te requirement for an initial transmission, that is, at least one SSB is available at the UE during the last 160ms, then TΔ and Tmargin can be 0.

    - Option 1d: TΔ = 0 for the case that DL synchronization for candidate cell(s) is performed before cell switch command

    - Option 1e: TΔ = 0 for the case that SSB based fine synchronization for candidate cell(s) is performed before cell switch command

  - Option 2: FFS

Issue 3-3-9: Whether to define PCell/PSCell switch delay requirements for unknown TCI state case when TCI state is indicated together with cell switch command

< Way forward >: FFS the following options

  - Option 1 (MTK, vivo, Huawei, CATT, OPPO): When TCI state is indicated together with cell switch command, only define cell switch delay requirements for known TCI state case and not define requirements for unknown TCI state case.

  - Option 2 (Nokia): When TCI state is indicated together with cell switch command, define cell switch delay requirements for both known and unknown TCI state case

  - Option 3 (Ericsson): FFS

Issue 3-3-11: TCI state switching time

< Way forward >: FFS the following options

  - Option 1: no need to add TCI state switching time in cell switch delay.

  - Option 2:

    - If DL TCI state switch is included in cell switch command, there is no extra delay if fine time tracking is already included in cell switch delay requirement or obtained by pre DL-sync.

    - If UL TCI state switch is included in cell switch command, possible extra delay is expected due to non-maintained PL-RS. Further discuss whether to consider non maintained PL-RS case.

  - Option 3: TCI state switching delay shall be considered depending on the scenario. The design requires LTM mobility measurements to be discussed first.

  - Other options omitted…

Firstly, RAN1 has made the following agreements.

Agreement in RAN1 #112

- The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following:

  - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2

Agreement in RAN1 #112-bis-e

For the Rel-17 unified TCI based beam indication in Rel-18 LTM, at least Alt 1 is supported:

- Alt 1: TCI state activation of a candidate cell is received before the reception of beam indication of the candidate cell,

- Alt 2: TCI state activation of a candidate cell is received together with the reception of beam indication of the candidate cell

  - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command

- Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability

FFS: signalling details for TCI state activation

FFS: For Alt 1, whether/how TCI state activation for candidate cell(s) is allowed

Note: If scenarios 1 and 3 are to be supported other beam indication/TCI activation timing relationships are not precluded.

In latest RAN1 agreements, RAN1 has clarified that their previous agreement was for beam indication, which is DCI based TCI switch. Actually, the TCI state activation is assumed to be performed before cell switch. This is what RAN1 has confirmed so far. Therefore, RAN4 should consider to support TCI state activation before cell switch command.

Based on above, the TCI state switch delay would be the beam application time. In R17, RAN1 has the basic design for the beam application time. In R18, the MAC CE based beam application time should also be discussed and clarified in RAN1 first.

The beam application and cell switch can be performed simultaneously. Since TCI state activation is performed before

Proposal 6  At least support TΔ = 0, Tsearch = 0, Tmargin = 0, for the case that DL synchronization for candidate cell(s) is performed before cell switch command.

Proposal 7  Add a new term Tswitch, while the detailed value would equals to beam application time, which will be further specified by RAN1.

Proposal 8  For the scenario 2 that RAN1 has already agreed to support, RAN4 only consider known and activated TCIs, i.e. the time-frequency tracking and PL-RS maintenance are performed before cell switch command. No cell switch requirement is defined for the scenario 2, if the indicated TCI is unknown or not activated.

In last meeting, the following issue is discussed.

Issue 3-3-14: Tinterruption

< Way forward >: FFS the following options

  - Option 1: The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd

  - Option 2: There is almost no interruption during cell switch procedure when target Pcell/SCell is current SCell/PCell.

  - Option 3:

    - For RACH-based cell switch, T_interruption at least include the time of Tprocessing,2 and T_IU.

    - For RACH-less cell switch, T_interruption at least include T_processing,2

The LTM interruption mainly comprise the needed RF warm-up and reconfiguration of L1/L2/L3, which should be T_processing,2 in Figure 1. For the case when RACH is needed, RACH collision needs to be considered in the interruption time. Before RACH is sent to the target cell, there is no possibility that network can schedule this UE in the corresponding CG.

On the other hand, if TCI state is indicated in cell switch command, no interruption is needed according to R17 feMIMO conclusions.

Proposal 9  For RACH-based cell switch, T_interruption at least include the time of Tprocessing,2 and T_IU.

For RACH-less cell switch, T_interruption at least include T_processing,2

Conclusions

Based on above analysis, we have following proposals.

Proposal 1  RAN4 hold on the discussion on RRM requirements for SpCell change with SCell change until RAN2 concludes to support direct SCell activation at SpCell change.

Proposal 2  UE can not perform T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell.

- If needed, some other L1 configuration, L2/L3 reconfiguration and L2 reset can be performed during cell switch, i.e. in some cases it can be performed after the T/F tracking.

Proposal 3  For RACH-less cell switch, the end point is defined when UE performs the first DL/UL reception/transmission on the target cell based on the indicated transmission configuration.

Proposal 4  For the baseline requirement, RAN4 assume UE forms the complete configuration and applies it during the execution of cell switch command.

Proposal 5  RAN4 not to define cell switch delay requirements for the case that the target cell is unknown.

Proposal 6  At least support TΔ = 0, Tsearch = 0, Tmargin = 0, for the case that DL synchronization for candidate cell(s) is performed before cell switch command.

Proposal 7  Add a new term Tswitch, while the detailed value would equals to beam application time, which will be further specified by RAN1.

Proposal 8  For the scenario 2 that RAN1 has already agreed to support, RAN4 only consider known and activated TCIs, i.e. the time-frequency tracking and PL-RS maintenance are performed before cell switch command. No cell switch requirement is defined for the scenario 2, if the indicated TCI is unknown or not activated.

Proposal 9  For RACH-based cell switch, T_interruption at least include the time of Tprocessing,2 and T_IU.

For RACH-less cell switch, T_interruption at least include T_processing,2

- References

[1]  R4-2303175	WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek.  RAN4 #106

[2]  R4-2306395  WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek.  RAN4 #106-bis-e

[3]  R2-2209257	LS on L1/L2-based inter-cell mobility, MediaTek.  RAN2 #119-e