3GPP TSG-RAN WG4 Meeting # 107	R4-2308328

Incheon, KR, May 22 – May 26, 2023

Title: 	Discussion on L1-RSRP measurement requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.25.2.2

Document for:	Discussion

Introduction

L1/L2 based inter-cell mobility was discussed in last meeting [1]. This contribution provides further analysis on L1-RSRP measurement requirements on L1/L2 based inter-cell mobility from RRM requirement perspective.

Discussion

General principles

The LTM procedure is descripted in the latest RAN2 TS38.300 running CR (R2-2302039), see below.

1. The UE sends a MeasurementReport message to the gNB. The gNB decides to use LTM and initiates candidate cell(s) preparation. 2. The gNB transmits an RRCReconfiguration message to the UE including the LTM candidate cell configurations of one or multiple candidate cells. 3. The UE stores the LTM candidate cell configurations and transmits a RRCReconfigurationComplete message to the gNB. 4a/4b. The UE may perform DL synchronization and TA acquisition with candidate cell(s) before receiving the cell switch command. Editor’s note: DL synchronization for candidate cell(s) before cell switch command is supported, at least based on SSB. FFS necessary mechanism. Editor’s note: TA acquisition of candidate cell(s) before LTM cell switch command is supported, at least based on PDCCH ordered RACH, where the PDCCH order is only triggered by source cell. FFS detailed mechanism. 5. The UE performs L1 measurements on the configured candidate cell(s), and transmits lower-layer measurement reports to the gNB. Editor’s note: FFS whether the lower-layer measurement reports are carried on L1 or MAC. Editor’s note: The order of DL/UL sync (step 4a/4b) and L1 measurement (step 5) is not defined and subject to change. 6. The gNB decides to execute cell switch to a target cell, and transmits a MAC CE triggering cell switch by including the candidate configuration index of the target cell. The UE switches to the configuration of the target cell. Editor’s note: FFS how beam indication is done.

As per step 1, 2 and 5, the limited number of candidate target frequency/cells is selected through L3 measurement report and then network configures L1/L2 mobility related measurements on the candidate cells and then UE reports corresponding L1/L2 measurement results. Although it is not explicitly precluded that L1 measurement is configured while L3 measurement on the same frequency is not configured, it is not a typical configuration. From UE perspective, to correct receive L1 measurement resource, UE needs L3 measurement to identify coarse timing of neighbour cell. In other words, before L1-RSRP measurement, UE should achieve frame/slot/symbol level synchronization including acquiring SSB index information (on which symbols the RS to-be-measured) of the to-be-measured neighbour cell, otherwise cell detection delay and SSB index acquisition delay are needed. Based on the analysis, it is suggest to define L1-RSRP measurement requirements for known neighbour cell. That’s to say, UE performs L1 measurement on a neighbour cell after UE has performed L3 measurement on that cell (it is not required UE to report L3 measurement results before L1 measurement).

Proposal 1: UE performs L1 measurement on a neighbour cell after UE has performed L3 measurement on that cell.

With the above understanding, the target cell is supposed to be selected from candidate cell which has been ever measured.

Proposal 2: RAN4 to define L1 measurement requirements for known cell case only.

In last meeting, it is agreed that not to use intermediate L3 measurement lower layer samples for L1 measurement. One left open issue is whether final L3 measurement is used or not.

Issue 2-3-2: Whether to use L3 measurement results or intermediate L3 measurement results for FR1 intra-frequency L1 measurement report Issue 2-3-3: Whether to consider using L3 measurement results or intermediate L3 measurement results for FR1 inter-frequency L1 measurement report < GTW Agreement>: - Do not use L3 intermediate results for FR1 intra-frequency L1 report and FR1 inter-frequency L1 report - FFS on the assumptions on the number of cells used for L1 measurements - Note: Intermediate L3 measurement results are any measurements samples or average of the samples obtained for the purpose of L3 reporting - FFS whether to recommend RAN1/2 to consider a new reporting mechanism - FFS whether to use final L3 measurement results for FR1 intra-frequency L1 measurement report or for FR1 inter-frequency L1 measurement report.

Regarding FR2 L1-RSRP measurement for intra-frequency and inter-frequency, different RX beams are used for L1-RSRP and L3-RSRP measurement. In general, rough beams are used for L3 measurement and fine beams are used for L1 measurement. If final L3 measurement is used for the L1 measurement, the L1 measurement result is derived with rough beam. The selected beam may be not good enough. To achieve better performance, UE has to re-perform fine beam based L1 measurement after cell switch. This mechanism is the same as legacy handover procedure, any performance enhancement and delay reduction is not observed.

<Agreement>: - In FR2: - Fine beam can be assumed for L1 measurement on intra-frequency neighbor cell. FFS on inter-frequency neighbor cell. - FFS whether to consider rough beam also for L1 measurement on neighbor cell (including intra and inter-frequency).

Proposal 3: From specify requirements perspective, NOT to use final L3 measurement results for FR1 intra-frequency L1 measurement report or for FR1 inter-frequency L1 measurement report.

Intra-frequency L1-RSRP measurement

- Scenario

In R17 ICBM, the intra-frequency L1-RSRP measurement requirements are specified for the scenario that SSB from the cell with different PCI is completely contained in the active BWP. In R18 L1/L2 mobility, whether the scenario that SSB from neighbor cell is not within active BWP needs some discussion.

The requirements in clause 9.13 apply, provided the SSB from cell with PCI different from serving cell configured for L1-RSRP if the following conditions are met: - the number of cells with PCI different from seving cells Nmax = 1 for FR2 and Nmax = maxNrofAdditionalPCI for FR1. Where, maxNrofAdditionalPCI is defined in TS 38.331 [2]. - The SSB from the cell with different PCI completely contained in the active BWP or associated with initial downlink BWP of the UE

When the SSB from neighbor cell for intra-frequency L1-RSRP measurement is not within active BWP, in essence, this is alike the scenario which is discussed in FG 6-1a. The conclusion on FG 6-1a can be reused for intra-frequency L1-RSRP measurement where SSB from neighbor cell is not within active BWP. Table 1 provides the summary on L1-RSRP measurement standard status. As the legacy bwp-WithoutRestriction only focus on SSB in serving cell, a new UE capability or reuse bwp-WithoutRestriction with some clarification may be needed.

Table 1. L1-RSRP measurement standard status

| Measurement type | SSB location | Supported or not? |
| --- | --- | --- |
| Measurement on serving cell | Within active BWP | Supported in R15 |
|  | Outside active BWP | On-going discussion on bwp-WithoutRestriction (i.e., FG 6-1a) |
| Intra-frequency neighbor cell | Within active BWP | Supported in Rel-17 FeMIMO |
|  | Outside active BWP | To discuss in Rel-18 LTM |

Proposal 4: The conclusion on FG 6-1a can be reused for intra-frequency L1-RSRP measurement where SSB from neighbor cell is not within active BWP. Open whether to define related requirements in this release.

- Scheduling restriction

For UE incapable of RTD>CP, if the actual RTD of serving cell and neighbour cell is no larger than CP, the legacy measurement restriction and scheduling restriction defined for non-serving cell in R17 apply for intra-frequency L1-RSRP measurement on neighbor cell. If the actual RTD of serving cell and neighbor cell is larger than CP, UE who is incapable of RTD>CP would not correctly perform L1-RSRP measurement on neighbour cell as UE follows serving cell timing. Therefore there is no requirement when actual RTD of serving cell and neighbor cell is larger than CP for UE incapable of RTD>CP.

Proposal 5: For UE incapable of RTD>CP, the legacy measurement restriction and scheduling restriction defined for non-serving cell in R17 apply for intra-frequency L1-RSRP measurement on neighbor cell, when the actual RTD of serving cell and neighbor cell is no larger than CP.

•	when actual RTD>CP, there are no requirements

For UE capable of RTD>CP, UE is able to track different timing of candidate neighbor cells.

- In FR1 legacy scheduling restriction due to L3 measurement, if UE don’t support simultaneousRxDataSSB-DiffNumerology  and deriveSSB_IndexFromCell is not enabled, the UE is not expected to transmit or receive on all symbols within SMTC window duration. In L3 measurement, all SSB are confined to SMTC window. However for SSB based L1-RSRP measurement on neighbor cell, indicating SMTC or SSB in neigbour cell is still under discussion in RAN1.

  - If UE performs neigbour cell L1-RSRP measurement based on SSB occasion, the measurement resources have no such SMTC restriction. In FR1 FDD, the time difference between serving cell and neighbor cell is up to 500us. Considering large time difference between cells, the scheduling restriction would be extended, e.g., UE would not receive and transmit on [Multiple OFDM symbols before and after those OFDM symbols corresponding to the configured LTM L1-RSRP measurement SSBs]. As the SSB periodicity is less than SMTC periodicity, such frequent puncture would severely degrade UE throughput. Therefore to avoid excessive scheduling restriction, if UE doesn’t support simultaneousRxDataSSB-DiffNumerology, RAN4 is to specify scheduling restriction requirements for L1-RSRP measurement on neighbor cell when deriveSSB_IndexFromCell is enabled, and is not to specify requirements when deriveSSB_IndexFromCell is not enabled.

  - If UE performs neigbour cell L1-RSRP measurement based on SMTC, legacy scheduling restriction defined in clause 9.2.5.3.2 can be reused.

- In FR2-1, deriveSSB-IndexFromCell is always enabled. Therefore on top of the legacy scheduling restriction defined for non-serving cell in R17 for intra-frequency L1-RSRP measurement on neighbor cell, additional 1 symbol is added before and after those OFDM symbols corresponding to the configured LTM L1-RSRP measurement SSBs, when RTD of serving cell and neighbor cell larger than CP.

Proposal 6: For UE capable of RTD>CP,

- In FR1, if UE doesn’t support simultaneousRxDataSSB-DiffNumerology,

  - If UE performs neigbour cell L1-RSRP measurement based on SSB occasion, RAN4 is to specify scheduling restriction requirements for L1-RSRP measurement on neighbor cell when deriveSSB_IndexFromCell is enabled, and is not to specify requirements when deriveSSB_IndexFromCell is not enabled.

  - If UE performs neigbour cell L1-RSRP measurement based on SMTC, legacy scheduling restriction defined in clause 9.2.5.3.2 can be reused.

- In FR2-1, for scheduling restriction for L1-RSRP measurement on neighbor cell, on top of the legacy scheduling restriction defined for non-serving cell in R17, additional 1 symbol is added before and after those OFDM symbols corresponding to the configured LTM L1-RSRP measurement SSBs.

Inter-frequency L1-RSRP measurement

Inter-frequency L1-RSRP measurement is supported in R18 LTM.

Agreement

- For Rel-18 LTM, L1 inter-frequency measurement is supported from RAN1 point of view.

In our understanding, the common case is that the target inter-frequency L1-RSRP measurement frequency is not within the active BWP. If RAN4 only define requirements for inter-frequency L1 measurement within active BWP, it would strongly limit the inter-frequency LMT applicable scenarios.

Proposal 7: Specify requirements for

-Inter-frequency SSB-Based L1measurement with gap: The target cell’s SSB is not contained in the DL active BWP.

-Inter-frequency SSB-Based L1measurement without gap: The target cell’s SSB is completely contained in the DL active BWP.

If measurement gap is to be used, which type of gap needs further discussed. Till now, there are four types of gaps.

- Inter-frequency with type 1 gap

- Inter-frequency with type 2 gap

- Inter-frequency with NCSG

- Inter-frequency with needforgap

Inter-frequency with Type-1 gap

In FR1, SSB based L1-RSRP can be performed simultaneously with L3-RSRP measurement during gap occasions. As the configured L1-RSRP measurement frequencies are the subset of L3 measurement, no additional L1 measurement effort is needed. From L1 measurement delay perspective, the CSSFwithingap for L3 measurement can be reused for L1 RSRP.

In FR2, as rough beams are used for L3 measurement and fine beams are used for L1 measurement, either L1 measurement or L3 measurement is required to be measured within one gap occasion. L1 measurement and L3 measurement share the type-1 measurement gap. As type-1 is a basic gap, the inter-frequency L1-RSRP gap based on type-1 gap can be regarded as a baseline.

Proposal 8: Type-1 gap can be a baseline for inter-frequency L1-RSRP measurement if gap is needed.

Inter-frequency with type-2 gap

In FR2, as rough beams are used for L3 measurement and fine beams are used for L1 measurement, either L1 measurement or L3 measurement is required to be measured within one gap occasion. A dedicated measurement gap can be allocated for L1-RSRP measurement. Or when there are concurrent gaps, L1 measurement and L3 measurement can share one measurement gap. As the latter alternative is similar as type-1 gap based scheme, dedicated L1-RSRP gap is preferred to be considered.

Proposal 9: For SSB based L1-RSRP inter-frequency measurement with type 2 gap:

- In FR1, SSB based L1-RSRP can be performed simultaneously with L3-RSRP measurement;

- In FR2, a dedicated measurement gap for L1-RSRP can be considered.

Inter-frequency with NCSG

For UE who supports NCSG, during ML, the UE is expected to transmit and receive data on the corresponding serving carrier(s). For inter-frequency measurement, UE measurement behaviour during ML resembles with that during legacy measurement gap. Therefore SSB-based inter-frequency L1-RSRP measurement with NCSG has the same measurement principle as with legacy gap. From delay perspective, the measurement delay with NCSG is the same as that with legacy gap.

It shall be noted that either concurrent NCSG with legacy gap or concurrent two NCSG(s) is not supported in R17. Therefore one straight forward way is SSB based L1-RSRP measurement shared with L3 measurement with NCSG. In theory, the measurement delay with NCSG is the same as that with shared legacy gap. However considering the limited time for specifying R18 LTM requirement, the NCSG based L1-RSRP measurement can be considered in future release.

Proposal 10: SSB based L1-RSRP inter-frequency measurement with NCSG can be considered as low priority in R18 LTM.

Inter-frequency with Needforgap

Technically the mechanism of needforgap is very similar to that of NCSG. The potential benefit is that there are no interruption and no measurement gap if UE report it is able to support. However Needforgap is still under discussion in R18. Considering the limited time for specifying R18 LTM requirement, the Needforgap based L1-RSRP measurement can be considered in future release.

Proposal 11: SSB based L1-RSRP inter-frequency measurement with Needforgap is not to be considered in R18 LTM.

Accuracy

In last meeting, some companies proposed an issue whether lower SNR would be assumed compared to the case when L1 measurements are only used for beam managements.

Issue 2-7-1: side condition of intra-frequency L1-RSRP measurement accuracy requirements < Way forward >: FFS the following options - Option 1: Reuse legacy value SNR= -3dB - Option 2: SNR = -6dB (same as L3 measurement) - Option 3: For R17 L1 measurements, the side condition for measurement accuracy requirements is -3dB. For new type of L1 measurements if agreed by other WGs, the side condition for measurement accuracy requirements is -6dB.

From below aspects analysis, we don’t find strong view to use a lower SNR for R18 inter-cell mobility.

1. In R15, the side condition of L1-RSRP measurement is -3dB. One reason is that the sample number of L1-RSRP measurement is smaller than L3 measurement, i.e., 1 or 3. To guarantee measurement accuracy, higher SINR is set.

2. -3dB side condition is set for R17 ICBM L1-RSRP as well. The reason is that the non-serving cell is selected by network and it is supposed to be in a good condition. Therefore the L1-RSRP on NSC would be performed at higher SINR.

Considering the above aspects into account, we don’t find strong view to use a lower SNR for R18 inter-cell mobility.

Proposal 12: Reuse legacy value SNR=-3dB in intra-frequency L1-RSRP measurement accuracy.

Measurement Capability

In R17 FeMIMO, only one intra-frequency non-serving cell for FR2 L1-RSRP is supported (see below). In R18 L1/L2 inter-cell mobility, the L1/L2 inter-cell mobility supported UE would perform L3 measurement on multiple neighbor cells firstly. The possible procedure is that the limited number of candidate cells selected through L3 measurement and then network configure L1/L2 related measurements related with the candidate cells and then UE report corresponding L1/L2 measurement report.  There may be more than one (but the number is limited) non-serving cells per frequency layer for L1/L2 mobility.

- 9.13.2 Requirements Applicability The requirements in clause 9.13 apply, provided the SSB from cell with PCI different from serving cell configured for L1-RSRP if the following conditions are met: - the number of cells with PCI different from seving cells Nmax = 1 for FR2 and FFS for FR1 - The cell with different PCI from serving cell is known - The SSB resources configured for L1-RSRP measurements are measurable

In R17 ICBM, sharing factor PSC and PCDP are introduced for FR2 serving cell and cell with different PCI, as UE can not simultaneously receive beams from different directions. If there are more than 1 neighbour cell, the sharing factor will be according updated.

Proposal 13: For intra-frequency inter-cell L1-RSRP measurement, supported to-be measured candidate cells needs to be discussed. If there are more than 1 neighbour cell, the sharing factor (PSC and PCDP) in R17 ICBM shall be modified.

Conclusions

This contribution provides further analysis on L1/L2 based inter-cell mobility for mobility latency reduction. The following proposals are provided:

Proposal 1: UE performs L1 measurement on a neighbour cell after UE has performed L3 measurement on that cell.

Proposal 2: RAN4 to define L1 measurement requirements for known cell case only.

Proposal 3: From specify requirements perspective, NOT to use final L3 measurement results for FR1 intra-frequency L1 measurement report or for FR1 inter-frequency L1 measurement report.

Proposal 4: The conclusion on FG 6-1a can be reused for intra-frequency L1-RSRP measurement where SSB from neighbor cell is not within active BWP. Open whether to define related requirements in this release.

Proposal 5: For UE incapable of RTD>CP, the legacy measurement restriction and scheduling restriction defined for non-serving cell in R17 apply for intra-frequency L1-RSRP measurement on neighbor cell, when the actual RTD of serving cell and neighbor cell is no larger than CP.

•	when actual RTD>CP, there are no requirements

Proposal 6: For UE capable of RTD>CP,

- In FR1, if UE doesn’t support simultaneousRxDataSSB-DiffNumerology,

  - If UE performs neigbour cell L1-RSRP measurement based on SSB occasion, RAN4 is to specify scheduling restriction requirements for L1-RSRP measurement on neighbor cell when deriveSSB_IndexFromCell is enabled, and is not to specify requirements when deriveSSB_IndexFromCell is not enabled.

  - If UE performs neigbour cell L1-RSRP measurement based on SMTC, legacy scheduling restriction defined in clause 9.2.5.3.2 can be reused.

- In FR2-1, for scheduling restriction for L1-RSRP measurement on neighbor cell, on top of the legacy scheduling restriction defined for non-serving cell in R17, additional 1 symbol is added before and after those OFDM symbols corresponding to the configured LTM L1-RSRP measurement SSBs.

Proposal 7: Specify requirements for

- -Inter-frequency SSB-Based L1measurement with gap: The target cell’s SSB is not contained in the DL active BWP.

- -Inter-frequency SSB-Based L1measurement without gap: The target cell’s SSB is completely contained in the DL active BWP.

Proposal 8: Type-1 gap can be a baseline for inter-frequency L1-RSRP measurement if gap is needed.

Proposal 9: For SSB based L1-RSRP inter-frequency measurement with type 2 gap:

- In FR1, SSB based L1-RSRP can be performed simultaneously with L3-RSRP measurement;

- In FR2, a dedicated measurement gap for L1-RSRP can be considered.

Proposal 10: SSB based L1-RSRP inter-frequency measurement with NCSG can be considered as low priority in R18 LTM.

Proposal 11: SSB based L1-RSRP inter-frequency measurement with Needforgap is not to be considered in R18 LTM.

Proposal 12: Reuse legacy value SNR=-3dB in intra-frequency L1-RSRP measurement accuracy.

Proposal 13: For intra-frequency inter-cell L1-RSRP measurement, supported to-be measured candidate cells needs to be discussed. If there are more than 1 neighbour cell, the sharing factor (PSC and PCDP) in R17 ICBM shall be modified.

Reference

- R4-2306395, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek