3GPP TSG-RAN WG4 Meeting # 107										                   R4-2308337

Incheon, KR, May 22 – May 26, 2023

Title: 					Discussion on RRM impact for Enhanced support of reduced capability NR devices

Source: 	Huawei, HiSilicon

Agenda item:	8.32.3

Document for:	Discussion

Introduction

The revised WID on enhanced support of reduced capability NR devices has been approved in [1]. Techniques for further UE complexity reduction have been studied in the study item documented in TR 38.865. The WID has the following objectives:

The objective is to specify support for the following enhancements: Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

This contribution provides the preliminary analysis on the possible RRM impacts due to eRedCap.

Discussion

- Enhanced eDRX in RRC_INACTIVE

In RAN2#121bis-e meeting, the following progress is made for T calculation. In general, T determination is settled down. However some open issues are still under discussion, e.g., Whether RAN PTW is larger than CN PTW, or whether RAN PTW length is always the same as CN PTW length.

T calculation In an overlapped or non-overlapped PH: Within RAN PTW and outside CN PTW, T = RAN configured DRX cycle If this is even a valid case (we will decide later): In an overlapped PH: Within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}. In an overlapped PH: Within both CN PTW and RAN PTW, T = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}.

If RAN PTW length and CN PTW length is different, then there are three kinds of T (shown in Figure 1):

- In non-overlapped PTW: T1. As per latest RAN2 agreement, T1= RAN configured DRX cycle;

- In overlapping PH, T2-1 for overlapping part of CN and RAN PTW, and T2-2 for the non-overlapping part with PTW.

- If CN PTW length is larger than RAN PTW (shown in Figure 1 case 1), as per latest RAN2 agreement,

T2-1 = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information};

T2-2 = min {CN configured DRX cycle, default paging cycle broadcast in system information};

- If RAN PTW is larger than CN PTW as shown in Figure 1 case 2 (whether this configuration is valid depends on RAN2 decision), as per latest RAN2 agreement:

T2-1 = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information};

T2-2 = RAN configured DRX cycle;

Note: The case RAN PTW is larger than CN PTW will be decided in RAN2

| Case 1. CN PTW length is larger than RAN PTW |  |
| --- | --- |
| Case 2. RAN PTW is larger than CN PTW (if valid) |  |

Figure 1 ‘T’ in RAN and CN paging coincide PH and non-coincide PH if RAN PTW length and CN PTW length is different

If RAN PTW length and CN PTW length is always the same, then there are two kinds of T (shown in Figure 2):

- In non-overlapped PTW: T1. As per latest RAN2 agreement, T1= RAN configured DRX cycle;

- In overlapping PH: T2. As per latest RAN2 agreement, T2 = min {CN configured DRX cycle, RAN configured DRX cycle, default paging cycle broadcast in system information}.

Figure 2. ‘T’ in RAN and CN paging coincide PH and non-coincide PH if RAN PTW length and CN PTW length is always the same

As in R17 RedCap, the RRM requirements of serving cell are specified based on T (paging DRX) which are applied for both inside PTW and outside PTW. When two PTWs exist, there are multiple kinds of T. To follow the legacy R17 serving cell measurement principle, the straight way is to measure within PTWs for both overlapping PH and non-overlapping PH. The RRM requirements of serving cell are specified according to associated T within separate PTWs (e.g., T1, T2-1, T2-2).

Proposal 1: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, UE measures serving cell within PTWs for both overlapping PH and non-overlapping PH. The measurement requirements of serving cell are specified according to associated T within separate PTWs (e.g., T1, T2-1, T2-2).

As in R17 RedCap, the measurement period of intra-frequency/inter-frequency measurement depends on inactive eDRX cycles. The reason why T is not used for neighbour cell measurement is that paging DRX T is different within idle_PTW and outside idle_PTW. Measurements as per changed T would complicate UE implementation. We think the same principle can be reused for two PTWs case for R18 RedCap: defining intra-frequency/inter-frequency measurement requirements according to a fixed period when both IDLE eDRX and Inactive eDRX larger than 10.24s. The fixed period can be one of “T”s or other fixed value. As RAN2 is still working on the details of “T” in different PTWs, we are open to further discuss how to determine the fixed period.

For both serving cell and intra-frequency/inter-frequency measurements, one thing is noted that UE shall not perform measurements outside PTW(s) where PTW can be idle_PTW or and inactive_PTW, otherwise the power saving gain of introducing PTW would be vanished.

Proposal 2: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, intra-frequency/inter-frequency measurement requirements are according to a fixed period, where the fixed period can be one of “T”s or other fixed value.

- Complexity/cost reduction

There are some progress in RAN1 on UE BB bandwidth reduction. These complexity reduction focus on reduce the UE BB bandwidth for PDSCH and PUSCH to 5MHz. We assume there is no impact on SSB reception and processing.

#110b - For UE BB bandwidth reduction - the scheduling of SIB1/ broadcast OSI (PDSCH) are allowed to be larger than 5 MHz (as in legacy operation). The size of UE post-FFT buffering is FFS. - a UE is not expected to receive an UL grant, either dynamic grant or configured grant, with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop. - For UE Peak data rate reduction - The value X in “The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X” got no consensus yet. - For standalone PR1, the constraint value can be different from the X above. #111 - For UE BB bandwidth reduction - PR3 is agreed in RAN1 to replace BW3, i.e. “a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.” - For broadcast and unicast PDSCH, RAN1 does not assume that the UE post-FFT buffer size per slot is smaller than 20 MHz. - Broadcast of separate SIB1/OSI (PDSCH) to Rel-18 RedCap UEs is not supported. - For paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). - A UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop. - For RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. Only for this case, a timeline relaxation for Msg3 transmission can be further discussed. #112 Agreement Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

In RAN1 it is agreed that the scheduling of SIB1 is allowed to be larger than 5MHz, this is from network scheduling perspective. Intuitively the CGI reading related requirements may be impacted. However we think as the CGI reading delay based on TSMTC and TRMSI-scheduling, UE has efficient time to decode SIB1 as long as UE received the complete SIB1.

Tidentify_CGI = (TMIB + T SIB1) ms Where: TMIB is the time period used to acquire MIB message. TMIB = 6 * TSMTC ms for target cell carrier frequency on FR1 and TMIB = 25 * TSMTC ms for target cell carrier frequency on FR2. TSIB1 is the time period used to acquire SIB1 message. TSIB1 = 6 * TRMSI-scheduling ms.

For interruption requirements due to CGI reading, if UE BB bandwidth for PDSCH is reduced, the processing time of decoding SIB1 may be enlarged. Then the interrupted duration on serving cell may be larger accordingly. Therefore the legacy interruption due to CGI reading is not applicable to R18 RedCap. If RAN4 would like to define CGI reading related requirements when UE BB bandwidth is limited, simulation evaluation work would be carried out. However as we know CGI reading is for SON/ANR purpose. If network would like to know the CGI of one pico cell, it can request a normal UE with normal BB bandwidth to perform CGI reading. It is not a typical case that network indicate a R18 Redcap UE with reduced BB bandwidth to do such procedure. Moreover the work load of evaluation work is considerable and the current phase of the WI is a bit late, we suggest that no CGI reading related requirements are specified for R18 RedCap.

Proposal 3: It is not typical that network request a R18 RedCap UE with baseband bandwidth reduction to perform CGI reading for the purpose of SON/ANR.

Proposal 4: No CGI reading related requirements are specified for R18 RedCap.

Conclusions

This contribution provides analysis on the measurement requirements for RedCap UE when eDRX is configured. According to the analysis, we propose the following proposals.

Proposal 1: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, UE measures serving cell within PTWs for both overlapping PH and non-overlapping PH. The measurement requirements of serving cell are specified according to associated T within separate PTWs (e.g., T1, T2-1, T2-2).

Proposal 2: For R18 RedCap UE in inactive mode with both RRC_IDLE eDRX and RRC_INACTIVE eDRX larger than 10.24s, intra-frequency/inter-frequency measurement requirements are according to a fixed period, where the fixed period can be one of “T”s or other fixed value.

Proposal 3: It is not typical that network request a R18 RedCap UE with baseband bandwidth reduction to perform CGI reading for the purpose of SON/ANR.

Proposal 4: No CGI reading related requirements are specified for R18 RedCap.

References

- RP-223544, Revised WID on Enhanced support of reduced capability NR devices, Ericsson