3GPP TSG-RAN4 Meeting #107	R4-2308349

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on identification of aerial UEs

Agenda item:	8.36.2

Document for:	Discussion

Introduction

The UAV WI, as presented in [1], includes the following objectives with RAN4 impact highlighted:

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. 2. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2] Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible. 3. Specify the support for UAV identification broadcast (BRID) in NR PC5. Support of DAA using the same framework as BRID without DAA specific enhancements can be considered [RAN2]. Note:. UAV use of NR PC5 is to be used only in designated bands as defined in regulation for BRID/DAA use. 4. Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling [RAN1, RAN2]: - FR1 with directional antenna at UE side 5. Based on the technical conditions defined for aerial UE usage in ECC Decision (22)07, study and specify the necessary UE types and additional OOBE requirements for aerial UEs in 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz. [RAN4]. Note: In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE Note2: Applicability of power classes for aerial UE may need to be addressed in RAN4 Note3: RAN4 to identify the supported bands for aerial UE impacted by above information.

In this contribution we present further considerations for support of UAVs by the RAN4 specification.

Discussion

During RAN99, an objective with RAN4 impact was added to the UAV WID [1]. RAN4 is requested to specify necessary UE types for additional OOBE requirements for aerial UE operating the frequency ranges; 1710-1785 MHz, 2500-2570 MHz and 2570-2620 MHz according to ECC Decision (22)07.

The LS sent by ECC includes 4 requests to ETSI[2]:

Based on the ECC Decision (22)07, ECC invites ETSI to include the following requirements in future ETSI harmonised standard on aerial UE to ensure a coherence with this ECC framework: - Additional OOBE requirements applicable to aerial UEs in the following frequency bands 1710-1785 MHz, 2500-2570 MHz, 2570-2620 MHz, as defined by ECC Decision 22(07). In other frequency bands, OOBE limits applicable to terrestrial UE remain unchanged for aerial UE. - a mechanism/feature coherent with the above aerial UE definition in order to differentiate aerial UE, as defined by ECC Decision 22(07) from terrestrial UE operating under LTE/NR 5G networks - differentiation of aerial UE from other terrestrial UE shall not be changed by the end-user - d) the aerial UE shall not be capable to connect to LTE/NR 5G networks without aerial subscription

Defining aerial UE

The objective added to the WID for addressing the LS from ECC clearly states that it is necessary “study and specify the necessary UE types and additional OOBE requirements for aerial UEs…”

It is part of the WID to specify the necessary UE types that are further restricted in emission by the ECC decision.

The LS sent by ECC presents a list of requests to 3GPP, as previously mentioned in the discussion. The requests identified by b) and c) concern the discussion of the aerial UE types and the applicability of the respective further restrictions on emissions:

- a mechanism/feature coherent with the above aerial UE definition in order to differentiate aerial UE, as defined by ECC Decision 22(07) from terrestrial UE operating under LTE/NR 5G networks

- differentiation of aerial UE from other terrestrial UE shall not be changed by the end-user

In this document we will first address the request in b). ECC has requested that 3GPP introduces:

- A mechanism coherent with the above aerial UE definition.

- That differentiate an aerial UE type from the terrestrial UEs.

The mention to the “above aerial UE definition” corresponds to the description provided by ECC in their LS [2] (highlights are our own):

“According to this ECC Decision, an aerial UE refers to a UE supporting UAS features and services and requiring an aerial subscription. An aerial UE is installed either on-board an Unmanned Aircraft (e.g. drones) or on-board manned aircraft (e.g. helicopter). It identifies itself to the mobile network as being in this class.”

Therefore, ECC operational conditions states that an aerial UE refers to a UE that:

- UE supports UAS features

- Requires an aerial subscription type

- It identifies itself to the network as being in this class

In RAN4 #106-bis, companies have reached an agreement to address the aerial requirements by adding additional requirements under clauses marked by an identifier suffix (e.g. “J”) for aerial UEs [6]:

<Agreement >: - Adding related requirements under a suffix (e.g. J), similar to what have been done for NR-U and V2X, can be done for both NR and LTE, i.e. in both TS 38.101-1 ad TS 36.101

However,  it is important that the applicability rule for the clauses marked by the new suffix is clear and reaches the ECC intent to separate terrestrial and aerial UEs.

RAN4 has to clearly indicate to what kinds of UEs the aerial requirements must apply.

RAN4 has discussed different options to identify the aerial UE, but from 3GPP and specially from RAN point of view, the definition of an aerial UE still needs further clarification.

- From SA point of view: In TS 24.501, SA has indicated that a UE that supports UAS (uncrewed aerial services) is defined as “a UE which supports an aerial vehicle, such as a drone, with an onboard or built-in USIM and is able to perform UE NAS functionalities specified in this specification. Upper layers of the UE supporting UAS services are responsible for UAS related procedures, such as UUAA and C2 authorization, for which the NAS layer of the UE supporting UAS services performs the necessary NAS procedures”. Another condition for this UE to access UAS services, is that before initiating UAS access, the UE is assigned a CAA(civial aviation authority)-level ID.

- From RAN point of view, in TS 36.306 [5] (LTE specifications, which are the baseline for the Rel-18 NR specifications which are not ready for UAVs yet), a UE that has aerial subscription must support two UE capabilities (clauses 4.3.6.34 and 4.3.6.35 in TS 36.306 [5]).

From the NW point of view, the aerial UE type is identified therefore after the authentication and authorization procedure is performed, and NW has validated the UE capabilities; this addresses the ECC concern. On the other hand, the UE knows its own capabilities, and depends on the validation of the authentication and authorization towards the network to confirm whether the subscription is from aerial type. It is worth noting that this UE may still utilize other services before performing this procedure.

Define an aerial UE as a UE supporting aerial capabilities AND that has performed authentication and authorization of the aerial subscription as described in TS 25.401.

Defining regional requirements for aerial UEs

Another discussion taking place on RAN4 #106-bis was how to make the requirements apply only in the regions affected by the ECC decision. Among the options agreed in the previous meeting was the adoption of NS signaling. This is the preferred option, as it makes easier for gNBs within the authority region of ECC to broadcast the signal and it is future proof for additional requirements that might come for UAVs from other regions, as they are also likely to impact other ground systems by their flying nature.

The NS framework must be such that:

- It is only applied for aerial UEs and does not impact any other types of UEs; even though the UL configuration for any given band is conveyed in SIB and can be read by all UEs.

- It is in harmony with other NS signaling already in use for the same bands.

The second point is very important, as there already other NS values defined for the same bands. Creating a new mapping table to apply only to aerial UEs would create replicas of the existing signalling in two parts of the specification, which would make the maintenance much more difficult. In our draftCR R4-2308350, we propose a WF to fulfill both premises above.

Use NS signalling to support the restrictions on UAV transmissions according to the ECC decision.

Endorse the DraftCR R4-2308350

Impact on V2X.

One of the bands impacted by the ECC definition is the band n38, which may be also utilized for sidelink (V2X) deployments. The utilization of sidelink transmission is not a corner case of UAV deployments, as due to local regulations and policies, UAVs will be required to broadcast their remote identification (BRID) continuously, and this operation is performed in 3GPP-compliant  access via the PC5 interface (sidelink).

The goal of ECC decision is to protect other systems from transmissions coming from UAVs, and this is justified by the fact UEs are flying above the receiving antennas of these systems. This situation does not change regardless whether the transmission is performed over Uu or PC5.

The OOBE restrictions on the ECC decision also apply for sidelink transmissions

In TS 38.101-1, the V2X (sidelink) additional requirements are covered under the clauses provided with the suffix E. One important aspect is that the mapping of the NS signalling to be used by V2X (Table 6.2E.3.1-2) is different from the “common” mapping Table 6.2.3.1-1A. Therefore, it is important that the SL requirements are captured in Table 6.2E.3.1-2 for n38 as well.

Capture the NS mapping also for n38 under V2X operation for UAVs in Table 6.2E.3.1-2.

Conclusion

In this paper we discussed the definition of aerial UE types in regards to the applicability rules for OOBE additional restrictions within ECC region of authority:

Observation 1: It is part of the WID to specify the necessary UE types that are further restricted in emission by the ECC decision.

Observation 2: RAN4 has to clearly indicate to what kinds of UEs the aerial requirements must apply.

Proposal 1: Define an aerial UE as a UE supporting aerial capabilities AND that has performed authentication and authorization of the aerial subscription as described in TS 25.401.

Proposal 2: Use NS signalling to support the restrictions on UAV transmissions according to the ECC decision.

Proposal 3: Endorse the DraftCR R4-2308350

Proposal 4: The OOBE restrictions on the ECC decision also apply for sidelink transmissions

Proposal 5: Capture the NS mapping also for n38 under V2X operation for UAVs in Table 6.2E.3.1-2.

- References

- RP-230782, Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)

- RP-230025/R4-2304033, Reply to LS to 3GPP on ECC request for standardisation support related to ECC Decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands”, ETSI TC MSG/TFES

- S2-2303302, “LS Reply on ECC request for standardisation support related to ECC Decision (22)07 on “harmonised framework on aerial UE usage in MFCN harmonised bands”, SA2 Meeting #155, SA2 Meeting #155

- TS 23.356, “Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2”, 3GPP, Release 18.0.0

- TS 36.306, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities”, 3GPP, Release 17.3.0.

- R4-2306641, “WF on NR Support for UAV”, Nokia, Electronic Meeting, 17 April –26 April, 2023