3GPP TSG-RAN WG4 Meeting #107	R4-2308395

Incheon, South Korea, May 22nd – 26th, 2023

Agenda Item:	X.XX

Source:	MediaTek Inc.

Title: 	TP to TR 36.xxxx on introducing L+S FDD band for IoT NTN operation

Document for:	Discussion

# Introduction

In this contribution, we propose TP to the common TR for all IoT NTN bands by capturing the latest discussions and agreements.

# TP

====== Start of TP ==========

# 5 Common band agnostic aspects

## 5.1	System parameters

### 5.1.1 Operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [254] | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

### 5.1.2 Channel arrangements

The following agreements are reached on channel arrangements:

- The addition of Band [254] does not have any impact on the channel bandwidth sub-clauses for category M1 or category NB1/NB2.

- Channel spacing sub-clauses keep untouched with the addition of Band [254] for category M1 and category NB1/NB2.

- Channel raster for category M1 for Band [254] are defined as the following table

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [254] | 100 | 2483.5 | 228571 | 228571 –<1>- 228735 | 1610 | 261339 | 261339 –<1>- 261503 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

### 5.1.3 Tx-Rx frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| [254] | 873.5 MHz |

# 6	FDD band [254] (L+S band)

## 6.1	Regulation

## 6.2	UE requirements

### 6.2.1	UE transmitter characteristics

#### 6.2.1.1	Maximum output power for category M1 and NB1/NB2

For the new FDD frequency bands for NB-IoT/eMTC NTN operation, the UE maximum output power for category M1 and category NB1 and NB2 can be specified in the table below:

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| [254] |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

#### 6.2.1.2	Emission requirements and NS values

### 6.2.2	UE receiver characteristics

#### 6.2.2.1	Reference sensitivity for category M1 and NB1/NB2

#### 6.2.2.2	Blocking requirements

The following agreements are reached for blocking requirements:

  - Reuse the 36.102 in-band blocking requirements

  - Reuse the 36.102 narrow-band blocking requirements

#### 6.2.2.3 Rx spurious emission

The following agreements are reached for Rx spurious emission:

  - Reuse the 36.102 spurious emission requirements

====== End of TP ===========