3GPP TSG-RAN WG4 Meeting # 107 	R4-2308398

Incheon, Korea, May 22 – 26, 2023

Source: 	IIT Delhi, IIT Jodhpur

Title: 	TP to TR 38.877 for clause 5.3 Wideband Antenna Architecture

Agenda Item:	8.2.2

Document for:	Approval

# 1 Introduction

This contribution aims at providing a text proposal on phase adjustment to steering the beam in the same direction for a multiband phase antenna array into TR 38.877.

# 2 TP for TR 38.877

<start of TP>

# 2	References

[1]	Paradigm Phase Shift: RF MEMS Phase Shifters: An Overview, A. Chakraborty et al., IEEE Microwave Magazine, Vol. 18, No. 1, 2017.

<Next Proposal>

5.4  Single direction beam steering in different time frames

As the beamformer ICs provide single-phase shifted value over the entire operating range and hence the beam at the upper band steers less than the beam at the lower band, creating an error. The antenna element requires additional phase shifts at the upper band to steer the beam in the same direction (for both lower and upper bands). For a generic solution, FR2 bands with higher frequency separation 26+38 GHz: n258+n260 bands are targeted. Five beam steering angles are considered, which are 0º, 10º, 20º, 30º, and 40º. The additional phase shift required at antenna element ports is calculated by considering the separation of 0.5λ (at the upper band) between antenna elements. To steer the beam at 0º, 10º, 20º, 30º, and 40º the antenna elements require additional phase shift of 0º, 10º, 20º, 30º, and 40º, respectively, at upper band (d = 0.34λ) compared to lower band (d = 0.34λ).

Figure 5.4.1. shows an example of designing a phase shifter [1] that can be used along with wideband beamforming ICs to steer the beam in a single direction in different time frames. A switched line phase shifter is designed by employing delay arms between two direct paths, as the phase of any microstrip line is proportional to its length. Here, the PIN diodes are employed as electronic switches. Switching the bias current, the state of the diode can be switched to ON (under forward bias) and OFF (under forward bias). To generate phase shifts of 10º, 20º, 30º, and 40º delay arms with length l+nlλ+l10, l+nlλ+l20, l+nlλ+l30, and l+nlλ+l40, respectively are designed.

Note: nlλ corresponds to the addition of n times 360º phase shift

Figure 2.1.1. Switched line phase shifter

.

Case 1: For the lower band (to steer the beam at any angle) and upper band to steer the beam at 0º, where no phase shifts are required, single will follow the straight path (i.e., D1, D2, D3, D4, D5 are switched ON).

Case 2: Upper band to steer the beam at 10º requires an additional 10º phase shift, which can be obtained by switching single to path l+nlλ+l10 (i.e., D6, D7, D3, D4, D5: ON).

Case 3: Upper band to steer beam at 20º requires an additional 20º phase shift, which can be obtained by switching single to path l+nlλ+l20 (i.e., D1, D8, D9, D4, D5: ON)

Case 4: Upper band to steer beam at 30º requires an additional 30º phase shift, which can be obtained by switching single to path l+nlλ+l30 (i.e., D1, D2, D10, D11, D5: ON)

Case 5: Upper band to steer beam at 40º requires an additional 40º phase shift, which can be obtained by switching single to path l+nlλ+l40 (i.e., D1, D2, D3, D12, D13: ON)

In addition, phase shifters can be designed with a variety of switches such as single pole double throw, Field Effect Transistor, and micro-electro-mechanical systems. Also, many wideband variable phase shifters are available that can be used directly along with beamforming ICs required phase shifts can be added at the upper band.

With this method, frequency selective phase shifted signals can be obtained, and the beam can be steered in the same direction for both bands at different time intervals. However, achieving the same steering angle at MB in a single time frame is not viable.

<End of proposal>