3GPP TSG-RAN WG4 Meeting # 107                                                       R4-2308399

Incheon, KR, May 22 – May 26, 2023

Title: 	Draft reply LS on PSFCH and S-SSB transmissions over non-contiguous RB sets

Source: 	LG Electronics

Agenda item:	8.31.2 UE RF requirements

Document for:	Approval

# Background

RAN1 has discussed the scenario where a RX UE needs to transmit PSFCHs (using existing R16/17 PSFCH format 0) across multiple unlicensed channels within a SL BWP in a same slot. Similarly, RAN1 is also discussing UE transmitting S-SSBs across multiple unlicensed channels within a SL BWP in a same slot. RAN1 would like to seek RAN4’s opinion on the following questions [1].

Question 1: whether multiple PSFCHs (using existing R16/17 PSFCH format 0) can be transmitted over non-contiguous RB sets?

Question 2: If multiple PSFCHs (using existing R16/17 PSFCH format 0) can be transmitted over non-contiguous RB sets, is there a limitation(s) on e.g., number of RB sets, max. frequency separation between the RB sets, etc?

The above same two questions also apply to UE transmitting S-SSBs over non-contiguous RB sets. Note that, according to existing RAN1 agreements, it is not possible to transmit PSFCH and S-SSB within a SL BWP in a same slot.

RAN1 respectfully requests RAN4 to provide feedback to the questions above for both multiple PSFCHs and multiple S-SSBs cases.

# Discussion

During the development of the RF requirements for Shared spectrum channel access for operation in un-licensed bands n46, n96 and n102, RAN4 agreed that the UE transmit requirements for wide-band operations are only applicable for single RB set or multiple contiguous RB sets within the UE channel. This is also captured into the section 6.1F of TS 38.101-1 [2].

## 6.1F	General

For wideband operations, the minimum requirements for the transmitter characteristics are specified for transmissions on one scheduled RB set or ≥ 1 scheduled contiguous RB set(s) within the UE channel. The requirements apply with configured UL intra-cell guard bands of non-zero size according to Table 5.3.3-2, with the union of the scheduled RB sets and the intra-cell guard bands.

Unless stated otherwise, when a clause is not present for shared spectrum channel access, the general requirements and the additional clause requirements (suffices A,B,D) in clause 6 apply.

Following this agreement non-contiguous UL transmissions of multiple RB sets is not supported in NR-U (in bands n46, n96 and n102), which is used as basis for defining the UE requirements for NR SL-U and therefore the answer to both questions from RAN1 is No.

# Summary

Answers to RAN1 questions are proposed below. Draft LS is attached to Annex of this document.

Question 1: whether multiple PSFCHs (using existing R16/17 PSFCH format 0) can be transmitted over non-contiguous RB sets?

RAN4 answer 1: UL transmissions with non-contiguous RB sets is not supported for wide-band operations in bands n46, n96 and n102.

Question 2: If multiple PSFCHs (using existing R16/17 PSFCH format 0) can be transmitted over non-contiguous RB sets, is there a limitation(s) on e.g., number of RB sets, max. frequency separation between the RB sets, etc?

RAN4 answer 2: UL transmissions with non-contiguous RB sets is not supported for wide-band operations in bands n46, n96 and n102.

# References

[1] R1-2304218, LS on PSFCH and S-SSB transmissions over non-contiguous RB sets, Source: RAN1, To: RAN4

[2] TS 38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V18.0.0

# Annex: draft reply LS

3GPP TSG-RAN WG4 Meeting #107                                                                     R4-230xxxx

Incheon, KR, May 22 – May 26, 2023

Title:	Draft reply LS on PSFCH and S-SSB transmissions over non-contiguous

Response to:	R1-2304218, LS on PSFCH and S-SSB transmissions over non-contiguous RB sets

Release:	Release 18

Work Item:	NR_SL_enh2-Core

Source:	RAN4

To:	RAN1

Cc:	-

Contact person:	Markus Pettersson

Markus.pettersson@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN4 has discussed the questions in RAN1 LS R1-2304218 and provides following feedback to RAN1:

Question 1: whether multiple PSFCHs (using existing R16/17 PSFCH format 0) can be transmitted over non-contiguous RB sets?

RAN4 answer 1: UL transmissions with non-contiguous RB sets is not supported for wide-band operations in bands n46, n96 and n102.

Question 2: If multiple PSFCHs (using existing R16/17 PSFCH format 0) can be transmitted over non-contiguous RB sets, is there a limitation(s) on e.g., number of RB sets, max. frequency separation between the RB sets, etc?

RAN4 answer 2: UL transmissions with non-contiguous RB sets is not supported for wide-band operations in bands n46, n96 and n102.

2. Actions:

To RAN1: RAN4 respectfully requests RAN1 to take the provided feedback into account for both multiple PSFCHs and multiple S-SSBs cases.

3. Date of Next RAN1 Meetings:

TSG RAN WG4 Meeting #108		21th Aug – 25th Aug 2023 	Toulouse, France

TSG RAN WG4 Meeting #108bis		9th Oct – 13th Oct 2023 	Xiamen, China