# 3GPP TSG-RAN WG4 Meeting # 107                                    R4-2308405

# Incheon, KR, May 22 – May 26, 2023

# Title: 	Discussion on 8Rx PDSCH requirements

# Source: 	ZTE Corporation

# Agenda item:	8.5.3.1.2

# Document for:	Approval

# 1 Introduction

- In RAN4#106bis-e meeting, 8Rx UE demodulation and CSI requirements were discussed and a WF was agreed[1]. In this paper, we will provide our views on some issues and some simulation results on 8 Rx PDSCH for FR1.

# 2 Discussion

## 2.1 PDSCH requirements

Issue 2-3: Propagation conditions for Rank 2 test - Option 1: TDLC300-100 ULA Medium B and TDLA30-10 ULA Medium B - Option 2: Only TDLC300-100 ULA Medium B

- In previous meeting, we agreed that Rank 2 for vehicle case which propagation condition is based on TDLC300-100 ULA Medium B.

Proposal 1: To consider TDLC300-100 ULA Medium B.

Issue 2-4: MCS and antenna correlation for Rank 2 test - Proposals for TDLC300-100 ULA Medium B - Option 1: MCS 2, 7 (Table 2) - Option 2: MCS 19 (Table 1) - Proposals for TDLA30-10 ULA Medium B (if agreed) - Option 1: MCS 13, 26 (Table 2) - Option 2: MCS 19 (Table 1)

- In general, we prefer considering only one MCS for every Rank and define only one requirement.

Proposal 2: Only consider MCS 19 for TDLC300-100 ULA Medium B.

Issue 2-5: MCS for Rank 4 test - Option 1: MCS 13 and MCS26 (Table 1) - Option 2: Only MCS17 (Table 1)

Proposal 3: Only consider MCS 17 for Rank 4.

Issue 2-7: Single MCS configuration for Rank 8 test - Option 1: MCS13 - Option 2: MCS17

Proposal 4: Only consider MCS 13 for Rank 8.

## 2.2 Simulation results

In this section, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). We depict the SNR value corresponding to 70% of the peak throughput. All figures have been provided in the Appendix.

Table 2-1. Summary of PDSCH 8Rx simulation results

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | TDD UL/DL Pattern | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 (106) | 64QAM, 0.50 | FR1.30-1 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.5 |
| 1-2 | 40 / 30 (106) | 64QAM, 0.43 | FR1.30-1 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 10.6 |
| 1-3 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 14.4 |

# 3 Conclusion

In this contribution, we give some discussions on NR 8Rx demodulation. The conclusions are:

Proposal 1: To consider TDLC300-100 ULA Medium B.

Proposal 2: Only consider MCS 19 for TDLC300-100 ULA Medium B.

Proposal 3: Only consider MCS 17 for Rank 4.

Proposal 4: Only consider MCS 13 for Rank 8.

# 4 Reference

- R4-2305888, WF for 8Rx UE performance requirements. RAN4#106bis-e meeting. Huawei, HiSilicon.

- RP-221496, WID revision: RF requirements enhancement for NR frequency range, Huawei, HiSilicon.

# 5 Appendix

Figure 1. Rank 2, MCS 19, TDLC-300-100 Medium B

Figure 2. Rank 4, MCS 17, TDLA-30-10 Low

Figure 3. Rank 8, MCS 13, TDLA-30-10 Low