3GPP TSG-RAN WG4 Meeting #107	R4-2308424

Incheon, Korea (Republic of), 22 – 26 May 2023

Agenda Item:	9.7.7.1

Source:	Ericsson

Title:	Discussion on eMTC/NB-IoT UE demodulation requirements for NTN

Document for:	Discussion

# 1	Introduction

RAN4#106 agreed with the way forward on NB-IoT/eMTC UE demodulation requirements for NTN [1]. This contribution discusses the remaining open issues.

# 2	NPDCCH repetition number of Cat-NB1/NB2 for NTN

- For NB-IoT NPDSCH demodulation requirements, set NPDCCH repetition factor to be 128 in test 2. - Interested companies can verify this repetition number ensures the NPDCCH BLER becomes low enough.

The only one remaining issue on UE demodulation requirements for IoT-NTN is the NPDCCH repetition number during NPDSCH test. The NPDSCH test number 2 in [2] is set by reusing TS36.101 Table 8.12.1.1.2-2 Test 2, where NPDSCH and NPDCCH repetition numbers are both set to 256. According to the discussion in RAN4#106 and RAN4#106bis-e, RAN4 agreed to set the NPDSCH repetition level for IoT-NTN to 128 to adjust the SNR level, as shown in Table 1. Since both NPDSCH and NPDCCH use the convolutional coding and the payload size of DCI N1 (=23bits) is much smaller than NPDSCH TBS (=392bits), we think it is feasible to set NPDCCH repletion level to 128, as shown in Table 2.

[To be added the NPDCCH simulation results]

Proposal: Confirm the NPDCCH repetition level for NPDSCH test 2 is set to 128.

Table 1	Test parameters for NPDSCH demodulation requirements

| Test number | MCS | PDSCH repetition | Propagation condition | Carrier type | SNR (dB) to achieve 70% of maximum throughput |
| --- | --- | --- | --- | --- | --- |
| 1 | QPSK 1/2 | 32 | NTN-TDLC5-200 | Anchor | [-4.8] |
| 2 | QPSK 1/3 | 128 | NTN-TDLA100-10 | Non-anchor | [-10.8] |

Table 2	NPDCCH transmission parameters [2]

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| NOC at antenna port | Noc1 | dBm/15kHz | -93 (Note 1) |
|  | Noc2 | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1 128 for Test 2 |
| Rmax (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1 256 for Test 2 |
| G (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission. Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

# 3	Summary

Proposal: Confirm the NPDCCH repetition level for NPDSCH test 2 is set to 128.

# References

- R4-2305960, “WF on IoT-NTN UE demodulation requirements”, MediaTek.

- R4-2305961, “Draft Big CR to 36.102: Demodulation requirements for IoT-NTN”, MediaTek.