3GPP TSG-RAN WG4 Meeting #107	R4-2308426

Incheon, Korea (Republic of), 22 – 26 May 2023

Agenda Item:	9.7.7.2

Source:	Ericsson

Title:	Discussion on SAN demodulation requirements for IoT-NTN

Document for:	Discussion

# 1	Introduction

RAN4#106bis-e agreed with the way forward on SAN demodulation requirements for IoT over NTN [1], and updated the simulation assumption. This contribution discusses the remaining open issues according to our updated simulation results [2].

# 2	SAN demodulation requirements for eMTC

## 2.1	Propagation channel model for PUSCH

The remaining open issue of the SAN demodulation requirements for eMTC is the propagation channel model.

Table 1	Simulation parameters for PUSCH.

| Parameters | CE Mode A | CE Mode B |
| --- | --- | --- |
| Max number of HARQ transmissions | 4 | 2 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 | 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Number of PUSCH repetitions | 8 | 256 |
| Frequency hopping | OFF | OFF |
| Duplex mode | FDD only | FDD only |
| Number of Tx antennas | 1 | 1 |
| Number of Rx antennas | 1 and 2 | 1 and 2 |
| Propagation channel | Option 1: NTN-TDLC5-5 Option 2: NTN-TDLA100-5 | Option 1: NTN-TDLA100-5 Option 2: NTN-TDLC5-5 |
| FRC | TS36.104 A3-2 | TS36.104 A3-1 |
| System BW | 1.4MHz only | 1.4MHz only |
| Fraction of maximum throughput | 70% | 70% |

Table 2	Summary of simulation results for eMTC PUSCH [2].

| Rx antenna | Propagation condition | CE Mode A | CE Mode B |
| --- | --- | --- | --- |
| 1 | NTN-TDLA100-5 | -6.1 | -15.1 |
|  | NTN-TDLC5-5 | -6.1 | -16.3 |
| 2 | NTN-TDLA100-5 | -10.5 | -17.4 |
|  | NTN-TDLC5-5 | -9.2 | -18.1 |

Table 1 summarizes our simulation results for PUSCH [2]. From the summary, it is not observed significant performance difference between NTN-TDLA100-5 and NTN-TDLC5-5, but NTN-TDLA100-5 requires slightly higher SNR for CE Mode B. Since some companies preferred to consider both LOS and NLOS channel models and also avoiding setting too low SNR test points, we propose to keep the original assumption, that is, CE Mode A uses the LOS-based channel model and CE Mode B uses the NLOS channels.

Proposal 1: Set NTN-TDLC5-5 for PUSCH CE Mode A tests, and NTN-TDLA100-5 for PUSCH CE Mode B tests.

# 3	SAN demodulation requirements for NB-IoT

## 3.1	Propagation channel model for NPUSCH format

The remaining open issue of the SAN demodulation requirements for NB-IoT is also the propagation channel model.

Table 3	Simulation parameters for NPUSCH format 1.

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions | FRC | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 3.75KHz | 1 | Option 1: NTN-TDLA100-1 Option 2: NTN-TDLC5-1 | TS36.104 A16-1 | 4 | 70% |
| 1 | 1 and 2 | 15KHz | 12 | Option 1: NTN-TDLC5-1 Option 2: NTN-TDLA100-1 | TS36.104 A16-5 | 16 | 70% |

Table 4	Summary of simulation results for NB-IoT NPUSCH format 1 [2].

| Subcarrier spacing | Number of tones | Repetitions | Rx antenna | NTN-TDLA100-1 | NTN-TDLC5-1 |
| --- | --- | --- | --- | --- | --- |
| 3.75kHz | 1 | 4 | 1 | -3.5 | -6.1 |
|  |  |  | 2 | -7.1 | -8.7 |
| 15kHz | 12 | 16 | 1 | -7.5 | -10.0 |
|  |  |  | 2 | -11.7 | -13.1 |

Table 4 summarizes our simulation results for NPUSCH format 1 [2]. It is observed that the NTN-TDLA100-1 requires 1.5 - 2.5dB higher SNR to achieve 70% of the maximum throughput compared with NTN-TDLC5-1. In RAN4#106bis-e, since several companies preferred to consider both LOS and NLOS channel models, and to differentiate the SNR levels between 3.75kHz and 15kHz cases, we propose to set NTN-TDLA100-1 for 3.75kHz single tone tests and NTN-TDLC5-1 for 15kHz 12 tones tests.

Proposal 2: Set NTN-TDLA100-1 for 3.75kHz single tone tests and NTN-TDLC5-1 for 15kHz 12 tones tests.

# 4	Summary

Proposal 1: Set NTN-TDLC5-5 for PUSCH CE Mode A tests, and NTN-TDLA100-5 for PUSCH CE Mode B tests.

Proposal 2: Set NTN-TDLA100-1 for 3.75kHz single tone tests and NTN-TDLC5-1 for 15kHz 12 tones tests.

# References

- R4-2305965, “WF for SAN demodulation requirements for IoT over NTN”, Ericsson.

- R4-2308425, “Simulation results of SAN demodulation requirements for IoT-NTN”, Ericsson.