3GPP TSG-RAN WG4 Meeting # 107	R4-2308434

# Incheon, KR, May 22 – May 26, 2023

Source: 	Samsung

Title: 	                      Discussion on simultaneous multi-panel operation for FR2 PC6 UE

Agenda Item:	8.13.4.1

Document for:	Discussion

Introduction

In RAN#95e meeting, the Rel-18 RAN4-led WI on enhanced NR support for high speed train scenario in FR2 has been approved as [1]. This Rel-18 can be regarded as the continuous enhancement over the Rel-17 feature of NR support of FR2 HST, in which Rel-17 WI RAN4 has focused on train roof-mounted high-power devices for NR SA single carrier scenario in FR2, by studying the FR2 HST deployment scenario and specifying the channel modelling, RF, RRM and demodulation requirements for FR2 HST. Accordingly, in RAN4 #106-bis, the WF for simultaneous multi-panel operation for train roof-mounted FR2 high power was agreed [2]. In this contribution, we would like to share our further analysis of simultaneous multi-panel operation for train roof-mounted FR2 high power devices for FR2 HST enhancement. Meanwhile, the outcomes of R18 NR_FR2_multiRX_DL WI discussion are listed out and summarized in the appendix, which can help us synchronize the progress we made for the two WIs so far.

Discussion

Overview of RAN4 progress and Samsung’s view on multi-panel operation for train roof-mounted FR2

The RRM requirements are categorized in Table 1 based on progress on RRM req. impact for Rel-18 NR FR2 HST multiRX_DL till RAN4#106bis-e, and our views on each under discussion item are provided as well.

Proposal 1: Our views on RRM requirement impact for Rel-18 NR FR2 HST multiRX_DL are summarized in Table 1.

Table 1 Overview of RAN4 progress and Samsung’s view on multi-panel operation for train roof-mounted FR2

| RRM Req. Main Category (TS 38.133) | Sub-category | Progress on RRM req. impact for Rel-18 NR FR2 multiRX_DL HST till RAN4#106bis-e | Samsung’s View |
| --- | --- | --- | --- |
| 6 RRC_CONNECTED state mobility | 6.1 Handover 6.2 RRC Connection Mobility Control | Under discussion | Precluded in Rel-18 FR2 HST enhancement WI. (The detailed analysis is in sub-section 2.2.1) |
| 7 Timing | 7.6 MRTD | Introduce requirements. Define RRM requirements for MRTD > CP | New requirements is required |
| 8 Signalling characteristics | 8.1 Radio Link Monitoring: 8.1.2 Requirements for SSB based radio link monitoring/8.1.3 Requirements for CSI-RS based radio link monitoring | Under discussion | New requirements is required Follow the principle from FR2 multi-RX. (The detailed analysis is in sub-section 2.2.2) |
|  | 8.1.7 Scheduling availability of UE during radio link monitoring | Under discussion | Reuse the conclusion from Rel-18 FR2 multi-Rx FFS the necessity of explicit RRM requirement impact for FR2 HST |
|  | 8.5 Link Recovery Procedures:8.5.2-8.5.6 | Under discussion | - SSB based BFD New requirements is required - SSB based CBD Reuse the conclusion from Rel-18 FR2 multi-Rx FFS the necessity of explicit RRM requirement impact for FR2 HST |
|  | 8.5.7/8.5.8 Scheduling availability of UE during BFD/CBD | Under discussion | Reuse the conclusion from Rel-18 FR2 multi-Rx FFS the necessity of explicit RRM requirement impact for FR2 HST |
|  | 8.10 Active TCI state switching delay | Not discussed | Precluded |
|  | 8.12 Uplink spatial relation switch delay | Under discussion | Precluded |
| 9 Measurement Procedure | 9.2 NR intra-frequency measurements | Under discussion with some agreements. - Simultaneous L3 on one panel and L1 measurements on the same panel: Not support | - Simultaneous L3 measurements on different panels: Precluded in Rel-18 FR2 HST enhancement WI - Simultaneous L3+L1 on different panels: Precluded in Rel-18 FR2 HST - Scheduling restriction: New requirements is required Follow the principle from FR2 multi-RX. |
|  | 9.3 NR inter-frequency measurements |  |  |
|  | 9.10 CSI-RS based L3 measurements : 9.10.2.6 Scheduling availability of UE during CSI-RS based intra-frequency measurements |  |  |
|  | 9.5 L1-RSRP measurements for Reporting: 9.5.4 L1-RSRP measurement requirements | Under discussion | New requirements is required Follow the principle from FR2 multi-RX. - Regarding GBBR: Rel-17 GBBR should not be used as a prerequisite to define requirement for simultaneous reception in FR2 HST (The detailed analysis is in sub-section 2.2.4) |
|  | 9.5.5 Measurement restriction for CSI-RS and SSB for L1-RSRP measurement | Under discussion | - Wait and reuse the conclusion from Rel-18 FR2 multi-Rx FFS the necessity of explicit RRM requirement impact for FR2 HST - SSB+SSB simultaneous reception Precluded in Rel-18 FR2 HST |
|  | 9.5.6 Scheduling availability of UE during L1-RSRP measurement | Under discussion | New requirements is required Follow the principle from FR2 multi-RX. |
|  | 9.8 L1-SINR measurements for Reporting | Not discussed | Precluded |

Analysis on specific issues

In the last meeting, the following issues were agreed to further study.

2.2.1 RRC_CONNECTED state mobility-L3 measurements

From our perspective, the issue is Rel-18 FR2 HST specific. Since that based on the agreed transmission scheme for FR2 HST multi-panel simultaneous reception that RAN4 to focus on NC JT as the main transmission scheme, both intra-cell and inter-cell scenarios are not precluded in FR2 HST discussion, which does not align with the operation scenarios agreed in Multi_Rx DL WI. Besides, mobility related requirements are HST specific, which are not in the scope of NR_FR2_multiRX_DL WI.

- Handover requirement

A recap from TS 38.133, the FR2 handover delay Dhandover is defined as applicable RRC procedure delay plus the interruption delay Tinterrupt, that is

Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms

In HST, the common case is that the target cell is known. And if the target cell is a known cell, then Tsearch = 0 ms and RX beam number has no impact on requirements. In this sense, there is no need to consider handover enhancement in Rel-18 FR2 HST scenario.

Observation 1: In HST scenario, if the target cell is a known cell, then Tsearch = 0 ms and RX beam number has no impact on requirements

- RRC connection re-establishment requirement

No doubt that RRC re-establishment is an essential functionality for UE mobility, based on the defined RRC re-establishment requirement to unknown cell for HST FR2, copied as below, we analysis this issue for each scenario (Set).

Considering faster beam sweeping (sweeping scaling factor reduction) incurred by multi-Rx simultaneous reception, we replace N2 with N2/2 no matter scenario A or scenario B. Then, the corresponding Tidentify_intra_NR would keep 1000 ms for set 1, and reduce from 2400 ms to 1200 ms for set 2 with TSMTC=40 ms. Although the existing RRC connection re-establishment delay is relative long versus enhanced Tidentify_intra_NR especially for set 2, we do not observe the need of enhancement of the delay requirement for Rel-18 NR FR2 multiRX_DL HST, since the cell coverage is large in SFN deployment, and PC6 UE takes approximate 28.7 s with 350km/h speed to across a cell.

Proposal 2: There is no need to further enhance the RRC connection re-establishment delay requirement in Rel-18 FR2 HST enhancement WI

- Intra-frequency/Inter-frequency measurements requirement

Take intra-frequency measurements requirement as example, and consider Tidentify_intra_without_index, which defined as

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + TSSB_measurement_period_intra) ms,

where TPSS/SSS_sync_intra is time period for PSS/SSS detection and TSSB_measurement_period_intra measurement period for intra-frequency measurements.

Take SMTC period equals to 40 ms, DRX cycle equals to 80 ms and 3 samples into account, and for simplicity, Kp, Klayer1_measurement, CSSFintra are assumed to be 1.

For Rel-17 FR2 HST, the scaling factors M1= 6 for Set 1 and M1=18 for Set 2. Accordingly, the total delays are for 1080 ms and 2880 ms, respectively. And then for the speed up to 350 km/h, the PC6 UE can moves about 105 m for set 1, and moves about 280 m for set 2. Considering faster beam sweeping and using the shorten scaling factors (M1= 3 for Set 1 and M1=9 for Set 2), the total delay can be reduced to 1000 ms and 1440 ms in Rel-18 FR2 HST, and then cell identification are approximately completed during 97.2m distance and 140 m for the two sets.  From our perspective, although the enhanced the intra-frequency time period requirement can reduce beam overlapping requirement for mobility measurement, the current requirements are robust enough to handle the ISI and ensure overall mobility management performance. That is to say, the current intra-frequency measurements requirement can still work for Rel-18 NR FR2 multiRX_DL HST and no need to consider L3 measurement enhancement. From above analysis, we have the following proposal:

Proposal 3: Preclude L3+L3 measurement on different panels in Rel-18 FR2 HST enhancement WI

2.2.2 L1 measurements

Based on the conclusion from Rel-18 FR2 multi-Rx represented in Appendix, from our perspective, the RRM requirements enhancement of L1+L1 measurements is also need to be introduced in FR2 HST scenario based on faster beam sweeping with multi-Rx chains. However, unlike the cases in Rel-18 NR_FR2_multiRX_DL WI, which indicate the requirements enhancement discussion focus on all L1 measurements, and available RSs assumption for L1-measurement requirements at least include CSI-RS+CSI-RS and SSB+SSB, in Rel-18 FR2 HST, we believe it is necessary to distinguish the cases:

- The legacy RRM requirements which are applicable to HST scenarios but are not enhanced.

The case contains CSI-RS based RLM/BFD/CBD/L1-RSRP/L1-SINR and SSB based CBD requirements. For these requirements, we prefer that they do not need to be additionally enhanced in Rel-18 FR2 HST but can wait the conclusion from Rel-18 NR_FR2_multiRX_DL WI and reuse the same requirements as for enhanced L1 measurement agreed in such WI. But considering the stated enhanced RF requirements for multi-RX WI is PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized,

## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO - Enhanced RF requirements: - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs - The legacy spherical coverage requirement for reception from a single direction will be kept - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized

we believe it is necessary to discuss the necessity of explicit RRM requirement impact for FR2 HST even though we agree to reuse the conclusion from multi-RX WI.

- The existing RRM requirements which need to be further enhanced in Rel-18 FR2 HST due to multi-Rx simultaneous reception.

The case corresponding to the existing Rel-17 FR2 HST L1 measurement requirements including SSB based RLM/BFD /L1-RSRP/L1-SINR. For these requirements, it is reasonable to define new enhanced FR2 HST specific requirements by following the handling principle agreed in Rel-18 NR_FR2_multiRX_DL WI. The agreed principle are:
1) Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains; 2) Do not consider further enhancement to sharing factor for L3 and L1.

Proposal 4: In Rel-18 FR2 HST, the L1+L1 measurement RRM requirements are divided to the following cases:

- For the legacy RRM requirements which are applicable to HST scenarios but are not enhanced case.

-Wait the conclusion from Rel-18 NR_FR2_multiRX_DL WI and reuse the same requirements as for enhanced L1 measurement agreed in such WI.

FFS the necessity of explicit RRM requirement impact for FR2 HST

- For the existing RRM requirements which need to be further enhanced in Rel-18 FR2 HST due to multi-Rx simultaneous reception

- Define new enhanced FR2 HST specific requirements by following the handling principle agreed in Rel-18 NR_FR2_multiRX_DL WI.

Proposal 5: Regarding the RSs type assumption for L1-measurement in Rel-18 FR2 HST, study further the possible impacts of SSB configurations on RRM requirements

RX beam sweeping factor reduction for FR2 HST multi-panel simultaneous reception

Based on the train roof-mounted high-power CPE parameters specified in TS 38.854, copied as below, since UE can perform two parallel Rx beam sweeping with two separate panel simultaneously in the bi-directional deployment scenario in Rel-18, the RX beam sweeping factor N1 can be further reduced. And then, for the L1 measurement requirement including L1-RSRP, for set 1 configuration the value can be further reduced to 1, for set 2 configuration the value can be further reduced to 3.

5.3.1 Number of panels per CPE The number of panels per CPE was open and for discussion. Concerning the number of panels per CPE following will be considered: - 2 panels per CPE each for both for TX and RX. Each panel points in opposite directions. For the bi-directional Operation for Two Panels in the CPE following is considered: - Follow the Rel-15/16 principle of “only one active Rx/Tx panel at a time”. 5.3.2 Placement of CPE panel(s) UE antenna panel(s) for forward and backward directions: - RAN4 to consider CPE to be equipped with two panels pointed forward and backward along the track.

Proposal 6: In Rel-18 FR2 HST, the sweeping factor N1 can be reduced

- For set1 configuration the value can be further reduced to 1, for set2 configuration the value can be further reduced to 3

Measurement restriction for CSI-RS and SSB for L1-RSRP measurement

Regarding the measurement restriction discussion, we do not observe FR2 HST specific issues on L1+L1 measurements and L1 measurement + data reception with multi-RX. Therefore, the conclusion from Rel-18 NR_FR2_multiRX_DL WI can be directly reused for Rel-18 FR2 HST. Considering in R18 FR2 multi-Rx, the measurement restriction issues are under on-going discussion, we suggest to wait and reuse the conclusion from Rel-18 NR_FR2_multiRX_DL WI . From this, considering the limitation of Multi-RX WI scope, we have the following proposal ,

Proposal 7:

Regarding simultaneous L1 measurements and data+ L1 measurement

- Simultaneous L1 (L1 + L1) measurements shall be supported in Rel-18 FR2 HST enhancement WI

- Simultaneous data reception + L1 measurement shall be supported in Rel-18 FR2 HST enhancement WI

Regarding measurement restriction requirement enhancement

- Wait and reuse the conclusion from Rel-18 NR_FR2_multiRX_DL WI, no specific enhancements are needed for HST FR2 scenario

- FFS the necessity of explicit RRM requirement impact for FR2 HST

2.2.3 Scheduling availability of UE during L1 measurement

- Simultaneous data reception and RLM on different panels

The requirement of scheduling availability of UE during RLM is not enhanced for HST. It implicitly means that the current requirements have covered both non-HST and HST cases. From this, in our understanding, the conclusion of scheduling restriction relaxation for RLM achieved in Rel-18 NR_FR2_multiRX_DL WI can be reused in Rel-18 FR2 HST, considering the limitation of Multi-RX WI scope .

Proposal 8: The requirement of scheduling restriction of simultaneous RLM and data reception on different panels in Rel-18 FR2 HST can reuse the conclusion from Rel-18 FR2 multi-Rx. FFS the necessity of explicit RRM requirement impact for FR2 HST

- Simultaneous data reception and L1-RSRP on different panels

Different from non-HST, whether scheduling restrictions shall apply for one symbol before and one symbol after resources (SSB, CSI-RS, etc) used for L1-RSRP measurements is need additional considered in HST.

Proposal 9: Define new FR2 HST specific requirement of scheduling restriction of simultaneous L1-RSRP and data reception on different panels in Rel-18 FR2 HST enhancement WI.

2.2.4 RRM impact of GBBR for simultaneous reception in Rel-18 HST

In this sub-section, we provide our concern on RRM impact of GBBR for simultaneous reception in Rel-18 HST. We take bi-directional RRH deployment scheme 2 as example for further analysis, shown in Figure 1.

Figure 1 An illustrative example of Rx simultaneous multi-panel reception for Rel-18 FR2 HST in bi-directional RRH deployment scheme 2 scenario

There could be three cases when UE travels from one RRH to the next along the track

- Case A: UE within the coverage of any of the RRH or in the coverage hole. In such case, the Rel-17 GBBR is not applicable, and it would fall back to Rel-17 FR2 HST operation case. Since considering the goal of beam reporting enhancement is to facilitate inter-RRH beam pairing, it is not feasible for one RRH to transmit multiple beams simultaneously in FR2.

- Case B: UE arrived at the middle point between two RRHs or UE within the overlapping area covered by two adjacent Tx beams from different RRHs. In this case, Rel-17 GBBR may not be feasible to define requirement for simultaneous reception in FR2 HST. Although with the Rel-17 GBBR, association of beams and the RRHs can be established, and it can guarantee the simultaneous reception on the UE side without any problem of incorrect reporting or missing useful beam pair, the GBBR is in principle for simultaneous PDSCH reception with MRTD<CP assumption based on RAN1 discussion, which runs counter to MRTD>CP assumption agreed in Rel-18 FR2 HST

- Case C: UE is farther away from a RRH (e.g. RRH1) and is close to the adjacent RRH. In such case, the Rel-17 GBBR may not be applicable. When the highest quality of the Tx beams from RRH1 measured by UE is not sufficient and does not meet the differential L1-RSRP measurement criterion specified in TS 38.214, UE would not group and report the two beams (one from RRH 1 and the other from RRH2) with different channel properties, and then the Rel-17 GBBR loses its effectiveness

<TS 38.214> For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size, if the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', or if the higher layer parameter groupBasedBeamReporting-r17 is configured, the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. The mapping between the reported L1-RSRP value and the measured quantity is described in [11, TS 38.133].

Observation 2: There is no need to consider Rel-17 GBBR in FR2 HST scenario

Proposal 10: In Rel-18 FR2 HST, Rel-17 GBBR should not be used as a prerequisite to define requirement for simultaneous reception in FR2 HST

- There is no need to consider RRM impact of GBBR for simultaneous reception in Rel-18 HST

2.2.5 Simultaneous reception of SBB+SSB in Rel-18 HST

As agreed in previous meetings, RRM requirements specified in RAN4 will not consider SSB overlapping cases across different RRHs in the same and different cells, and not use adjacent SSBs in FR2 HST. In this sense, considering SSB+SSB simultaneous reception seems to be redundant in Rel-18 HST. Then, we suggest that

Proposal 11: Preclude SSB+SSB simultaneous reception in Rel-18 FR2 HST enhancement WI

< WFs in RAN4 #100-e [R4-2115335]> Non-overlapping SSB location Agreement: - RRM requirements specified in RAN4 will NOT consider SSB overlapping case. < WFs in RAN4 #103-e [R4-2210608]> Issue 2-1-1: Scheduling restriction on SSB Agreement: FR2 HST requirement is applicable only when the two SSBs from adjacent RRHs are not on the adjacent symbols.

Conclusion

Observation 1: In HST scenario, if the target cell is a known cell, then Tsearch = 0 ms and RX beam number has no impact on requirements

Observation 2: There is no need to consider Rel-17 GBBR in FR2 HST scenario

Proposal 1: Our views on RRM requirement impact for Rel-18 NR FR2 HST multiRX_DL are summarized in Table 1.

Proposal 2: There is no need to further enhance the RRC connection re-establishment delay requirement in Rel-18 FR2 HST enhancement WI

Proposal 3: Preclude L3+L3 measurement on different panels in Rel-18 FR2 HST enhancement WI

Proposal 4: In Rel-18 FR2 HST, the L1+L1 measurement RRM requirements are divided to the following cases:

- For the legacy RRM requirements which are applicable to HST scenarios but are not enhanced case.

-Wait the conclusion from Rel-18 NR_FR2_multiRX_DL WI and reuse the same requirements as for enhanced L1 measurement agreed in such WI.

FFS the necessity of explicit RRM requirement impact for FR2 HST

- For the existing RRM requirements which need to be further enhanced in Rel-18 FR2 HST due to multi-Rx simultaneous reception

- Define new enhanced FR2 HST specific requirements by following the handling principle agreed in Rel-18 NR_FR2_multiRX_DL WI.

Proposal 5: Regarding the RSs type assumption for L1-measurement in Rel-18 FR2 HST, study further the possible impacts of SSB configurations on RRM requirements

Proposal 6: In Rel-18 FR2 HST, the sweeping factor N1 can be reduced

- For set1 configuration the value can be further reduced to 1, for set2 configuration the value can be further reduced to 3

Proposal 7:

Regarding simultaneous L1 measurements and data+ L1 measurement

- Simultaneous L1 (L1 + L1) measurements shall be supported in Rel-18 FR2 HST enhancement WI

- Simultaneous data reception + L1 measurement shall be supported in Rel-18 FR2 HST enhancement WI

Regarding measurement restriction requirement enhancement

- Wait and reuse the conclusion from Rel-18 NR_FR2_multiRX_DL WI, no specific enhancements are needed for HST FR2 scenario

- FFS the necessity of explicit RRM requirement impact for FR2 HST

Proposal 8: The requirement of scheduling restriction of simultaneous RLM and data reception on different panels in Rel-18 FR2 HST can reuse the conclusion from Rel-18 FR2 multi-Rx. FFS the necessity of explicit RRM requirement impact for FR2 HST

Proposal 9: Define new FR2 HST specific requirement of scheduling restriction of simultaneous L1-RSRP and data reception on different panels in Rel-18 FR2 HST enhancement WI.

Proposal 10: In Rel-18 FR2 HST, Rel-17 GBBR should not be used as a prerequisite to define requirement for simultaneous reception in FR2 HST

- There is no need to consider RRM impact of GBBR for simultaneous reception in Rel-18 HST

Proposal 11: Preclude SSB+SSB simultaneous reception in Rel-18 FR2 HST enhancement WI

Appendix

RAN4 progress on RRM requirement impact for Rel-18 NR_FR2_multiRX_DL WI[3]-[5].

| RRM Req. Main Category (TS 38.133) | Sub-category | Progress on RRM req. impact for Rel-18 NR_FR2_multiRX_DL WI (for reference) |
| --- | --- | --- |
| 4 SA: RRC_IDLE state mobility | 4.2 Cell Re-selection | Deprioritize. It is agreed that the L3 related requirements enhancements are deprioritized except for L3 measurements in RRC Connected mode. |
| 6 RRC_CONNECTED state mobility | 6.1 Handover 6.2 RRC Connection Mobility Control | Preclude. It is agreed that RAN4 focus on intra-cell multi-TRP operation scenario; No requirements will be introduced for inter-cell multi-TRP operation in R18 |
| 7 Timing | 7.6 MRTD | Under discussion. Define RRM requirements for MRTD < CP. |
|  |  | The issues to be discussed in RAN4#107 - Whether to consider RTD larger than CP in multi-RX WI - Study impact of RTD from mTRPs on beam pair selection for group-based beam reporting - MRTD for UE not supporting RTD>CP |
| 8 Signalling characteristics | 8.1 Radio Link Monitoring: 8.1.2 Requirements for SSB based radio link monitoring | Enhance. It is agreed that : - Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains based on UE capabilities - Option 1: Introduce beam sweeping factor reduction - Other options not precluded - FFS on specific conditions when enhancement apply - FFS if any signalling is needed - FFS on details of UE capabilities - Do not consider enhancement to sharing factor for L3 and L1 |
|  | 8.1.3 Requirements for CSI-RS based radio link monitoring |  |
|  | 8.1.7 Scheduling availability of UE during radio link monitoring | Under discussion |
|  | 8.3 SCell Activation and Deactivation Delay | Deprioritized due to the WI is focused on single CC |
|  | 8.5 Link Recovery Procedures 8.5.2 Requirements for SSB based beam failure detection | Enhance. It is agreed that : - Introduce enhanced RRM requirements based on faster beam sweeping with multi-Rx chains based on UE capabilities - Option 1: Introduce beam sweeping factor reduction - Other options not precluded - FFS on specific conditions when enhancement apply - FFS if any signalling is needed - FFS on details of UE capabilities - Do not consider enhancement to sharing factor for L3 and L1 |
|  | 8.5.3 Requirements for CSI-RS based beam failure detection |  |
|  | 8.5.5 Requirements for SSB based candidate beam detection |  |
|  | 8.5.6 Requirements for CSI-RS based candidate beam detection |  |
|  | 8.5.7/8.5.8 Scheduling availability of UE during BFD/CBD | Under discussion |
|  | 8.10 Active TCI state switching delay 8.10.2 Known conditions for TCI state | Under discussion with some agreements. It is agreed that RAN4 to define TCI state switch delay requirements only for the case where to-be-activated/switched TCI state upon the TCI switch command(s) reception includes two different QCL-TypeD reference resources and the reference resources of the two TCI states are included in the group-based L1-RSRP measurement report - Note: This is one of the condition and other conditions are not precluded; Only for TCI state switching scenarios: Single TCI to dual TCI and dual TCI to dual TCI |
|  | 8.10.3 MAC-CE based TCI state switch delay | Under discussion with some agreements - Consider MAC CE based dual TCI states switch delay4 requirements for known case - FFS if it is to be limited to known case only - TCI state switching scheme for PDCCH: - Single DCI non-SFN+multi-DCI non-SFN |
|  | 8.10.4 DCI based TCI state switch delay | Under discussion with some agreements - Consider DCI based dual TCI states switch4 delay requirements for known case only - Both single DCI and based multi DCI based are considered |
|  | 8.10.5 RRC based TCI state switch delay | Under discussion |
|  | 8.10.6 Active TCI state list update delay | Under discussion with some agreements - For active TCI state list update for addition of a new dual TCI states, legacy requirements can be used as baseline - FFS if T/F tracking for the dual TCI states are based on different SSBs |
|  | 8.18 TRP specific Link Recovery Procedures | Under discussion. Focus on: - PTRP discussion - Whether to reuse R17 TRP specific BFD/CBD requirement for multi-RX operation |
| 9 Measurement Procedure | 9.2 NR intra-frequency measurements | Preclude except for L3 measurement scheduling/measurement restriction requirements discussion1. |
|  | 9.3 NR inter-frequency measurements |  |
|  | 9.10 CSI-RS based L3 measurements : 9.10.2.6 Scheduling availability of UE during CSI-RS based intra-frequency measurements |  |
|  | 9.5 L1-RSRP measurements for Reporting: 9.5.4 L1-RSRP measurement requirements | Enhance. The same agreements as in 8.1.2/8.1.3 |
|  |  | - RRM impact of GBBR for simu. Reception. It is agreed that - Rel-17 GBBR is used as a prerequisite to define requirement for simultaneous reception - If GBBR can be introduced, the impacted requirements: Under discussion |
|  | 9.5.5 Measurement restriction for CSI-RS and SSB for L1-RSRP measurement | Under discussion. Note: Simultaneous reception term agreed in the WI includes Data/Data, RS/RS and Data/RS simultaneous reception cases. |
|  | 9.5.6 Scheduling availability of UE during L1-RSRP measurement | Under discussion |
|  | 9.6 NE-DC: Measurements | Deprioritized due to the WI is focused on single CC |
|  | 9.8 L1-SINR measurements for Reporting | Under discussion. |
| Note: * Requirements’ classification categories: - Enhance: The requirement to be enhanced. - 1: It is indicated in the revised RRM objectives contained in the WID[RP-223527] that - NOTEs: The work on L3 measurement related aspects for scheduling/measurement restriction requirements is not precluded; Further check in RAN #100 whether to include other L3 measurement related aspects and objectives subject to RAN4 progress. It is agreed in RAN4#106 Rel-18 NR_FR2_multiRX_DL WI that simultaneous L3 and L1 measurements is not supported in this release for multi-Rx - NOTE: The agreement has no impact on scheduling restriction discussion for L3 measurements. Feasibility and benefit of scheduling restriction relaxation for L3 measurements is FFS. It is agreed in RAN4#106-bis Rel-18 NR_FR2_multiRX_DL WI that not to consider enhancement to sharing factor for L3 and L1 |  |  |

Reference

[1] RP-220985, “New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[2] R4-2306340, “WF on simultaneous multi-panel operation for train roof-mounted FR2 high power

devices”, Samsung.

[3] R4- 2306319, “WF on FR2_multiRx_part2”, Ericsson.

[4] R4-2306318, “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)”, vivo.