3GPP TSG-RAN WG4 Meeting # 107	R4-2308469

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.23.3.6

Source: 	OPPO

Title: 	Discussion on carrier phase positioning

Document for:	Approval

1	Introduction

Carrier phase measurement is introduced in Rel-18 to achieve centimetre level positioning accuracy. As a new type of measurement, the corresponding latency and accuracy requirements should be defined in RAN4. Based on the conclusion reached in previous meetings [1]. This contribution will provide our initial considerations on the potential RRM impacts.

2	Discussion

#### Issue 2-2-1: Measurement period requirements for DL carrier phase measurement: < Way forward > - Option 1: - If RSCPD/RSCP being reported with other legacy measurements together, RAN4 can define the measurement reporting delay requirements for them with the existing requirements for the associated legacy measurement. - Option 2: - Current core requirements for PRS measurement are used as baseline for carrier phase measurement. Adaptations can be FFS e.g. sample number and applicable PRS configuration. - Option 3: - Other options are not precluded.

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed.

As shown above, RAN1 agreed to report DL RSCP/RSCPD together with the legacy Rx-Tx time difference/RSTD measurements. In this case, RSCP/RSCPD measurement results are considered as a supplement to improve the estimated time of arrival. This is similar as the situation when PRS RSRP is configured with RSTD or Rx-Tx time difference in Rel-16. And RAN4 agreed to perform the two measurements simultaneously and follow the delay requirements of primary measurements, i.e. PRS RSRP measurement period shall meet RSTD or Rx-Tx time different measurement period requirements. Such principle should be reused for carrier phase measurement in Rel-18. Some companies expressed concerns during the last meeting since RSCP/RSCPD measurement is defined for single PFL and perhaps with single measurement instance, which are different from that of legacy PRS measurements. For example, two PFLs are configured for RSTD while only the PFL #1 is configured with RSCPD. We think UE could perform RSTD and RSCPD measurements simultaneously for the PFL #1 and perform RSTD measurement only for PFL #2. In this way, the existing measurement period could be applicable.

Proposal 1: When RSCP/RSCPD is reported together with legacy positioning measurement, the existing requirements for the associated legacy measurement should apply.

#### Issue 2-2-2: Applicable RF frequencies for DL RSCP measurement period requirements: < Way forward > - Option 1: - The basic measurement requirement for DL RSCP is defined for each RF frequency. And RAN4 to discuss whether to define measurement requirements for multiple RF frequencies. #### Issue 2-2-3: Applicable RF frequencies for DL RSCPD measurement period requirements: < Way forward > - Option 1: - The RF frequencies for target TRP and reference TRP in RSCPD measurement can be different.

Agreement The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default. - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them.

Carrier phase measurement is associated with a specific RF frequency. During the last meeting, RAN1 reached a further conclusion that the specific RF frequency is defined as the center frequency of DL PFL by default. And whether a frequency other than the center frequency will be introduced is still open in RAN1. In our understanding, RAN1 is to define a single RF frequency, either the default center frequency or the other frequency, for one PFL. If so, multiple RF frequencies can be excluded since carrier phase consider single PFL case only.

Proposal 2: Not consider multiple RF frequencies for RSCP/RSCPD measurement if a single RF frequency is defined for a PFL.

Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_INACTIVE state

NR DL carrier phase measurements in both RRC_CONNECTED and RRC_INACTIVE states are supported, which is the common understanding between RAN1 and RAN4 groups. For UE in RRC_CONNECTED state, at least MG-based measurement is supported in RAN1 and FFS MG-less measurement.

Proposal 3: Support DL carrier phase measurement within MG.

Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP

To design report mapping tables, the following issues should be clarified firstly. (1) What kind of unit should be used for carrier phase, degree or radian? We understand they are equivalent but the consensus needs to be reached in RAN4. (2) And what is the reporting range? Generally, [0, 2π] (or [0, 360°]) could be considered as the value range for absolute carrier phase. But situation for differential carrier phase may be a little complicated and further discussion is required. (3) Last but not the least, what is the resolution for carrier phase report? For RSTD and Rx-Tx time difference measurements, 6 reporting tables with different granularities are introduced to satisfy different accuracy demands. Similarly, more than one granularity should also be supported for carrier phase measurement.

Proposal 4: For absolute and differential carrier phase measurement, discuss the following issues to define report mapping tables:

- The unit for carrier phase report

- The range for carrier phase report

- The granularity for carrier phase report

#### Issue 2-3-7: Reporting granularity: < Way forward > - Option 1: - For Rel18 CPP measurements, the reporting granularity for DL RSCP, DL RSCPD and UL RSCP should be configurable up to UE capability. - Option 2: - For carrier phase positioning, RAN4 should consider the higher positioning resolution compared to the one for the time-based measurement metric.

Option 1 includes two features, one is that the reporting granularity is considered as UE capability and the other is that the reporting granularity can be configurable by LMF. This is approach is used for time-based PRS measurement but single reporting granularity is used for RSRP/RSRPP and AoA/ZoA measurement. Which approach should be defined for carrier phase measurement is not clear now and further input from RAN1 is needed as well. For option 2, we understand a higher positioning accuracy can be achieved for carrier phased compared with time-based measurement. For example, the min reporting granularity for RSTD in FR1 is 22Tc=4Tc, representing about 0.6m. When the corresponding RF frequency is 3GHz, the wavelength is 0.1m, which is less than 0.6m.

Proposal 5: Whether to introduce configuration or UE capability for the reporting granularity for DL RSCP/RSCPD can be FFS based on further conclusions in RAN1.

3	Conclusion

Based on latest progress in RAN1 and RAN4, this contribution gave our considerations on defining RRM requirements for carrier phase positioning and the following proposals:

Proposal 1: When RSCP/RSCPD is reported together with legacy positioning measurement, the existing requirements for the associated legacy measurement should apply.

Proposal 2: Not consider multiple RF frequencies for RSCP/RSCPD measurement if a single RF frequency is defined for a PFL.

Proposal 3: Support DL carrier phase measurement within MG.

Proposal 4: For absolute and differential carrier phase measurement, discuss the following issues to define report mapping tables:

- The unit for carrier phase report

- The range for carrier phase report

- The resolution for carrier phase report

Proposal 5: Whether to introduce configuration or UE capability for the reporting granularity for DL RSCP/RSCPD can be FFS based on further conclusions in RAN1.

4	Reference

- R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT