3GPP TSG-RAN WG4 Meeting # 107		R4-2308480

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.9.2.1

Source: 	OPPO

Title: 	Discussion on L3 enhancement for FR2 Scell activation delay reduction

Document for:	Approval

1	Introduction

In RAN4#106bis meeting, a WF on FR2 SCell activation delay reduction [1] was approved.

In this contribution, we further discuss L3 part enhancement for FR2 SCell activation delay reduction.

2	Discussion

Sub-topic 1-1: L3 reporting during SCell activation

Issue 1-1-1: when to report L3 measurement results for unknown FR2 SCell activation enhancement - Agreements (GTW, Monday Apr 17, 2023) - UE needs to report the L3 measurement result after SCell activation command - FFS if additional solutions should be considered. Decision on additional solutions need to be made no later that in RAN4 #107. Issue 1-1-2: FFS on how to trigger L3 report for unknown FR2 SCell activation enhancement - Agreements (GTW, Monday Apr 17, 2023) - The L3 measurement report for unknown FR2 Scell activation after Scell activation command is triggered by Scell activation command. - FFS whether to have additional events for report triggering - Send LS (R4-2306321) to let RAN2 to decide. Issue 1-1-3: FFS on how to configure L3 report for unknown FR2 SCell activation enhancement - Agreement: - The one-time report after SCell activation command is assumed in this enhancement - Send LS (R4-2306321) to let RAN2 to decide. Issue 1-1-4: FFS on how to report L3 measurement result for unknown FR2 SCell activation enhancement - Agreement: - Send LS (R4-2306321) to let RAN2 to decide. - Technical enhancement goal: RAN4 had following agreements in RAN4 #106-bis-e meeting, - Agreements (GTW, Monday Apr 17, 2023) - UE needs to report the L3 measurement result after SCell activation command - FFS if additional solutions should be considered. Decision on additional solutions need to be made no later that in RAN4 #107.. - Action Request to RAN2: RAN4 requests RAN2 to design corresponding signaling for “report of L3 measurement result” after SCell activation command for unknown FR2 SCell activation enhancement, and it’s up to RAN2’s decision on which layer’s signaling shall be used.. Issue 1-1-5: FFS on content in valid L3 measurement report for unknown FR2 SCell activation enhancement Agreement: - L3 measurement result with SSB index

Issue 1-1-2: FFS on how to trigger L3 report for unknown FR2 SCell activation enhancement - Agreements (GTW, Monday Apr 17, 2023) - The L3 measurement report for unknown FR2 Scell activation after Scell activation command is triggered by Scell activation command. - FFS whether to have additional events for report triggering - Send LS (R4-2306321) to let RAN2 to decide.

As agreed the measurement report is triggered by the SCell activation command. NW can provide one or more measurement report configurations for aperiodic reporting in RRC, or trigger RSRP report via MAC-CE along with scell activation command. RAN4 has sent LS to RAN1/2 asking for the solution to transmit the valid L3 measurement report upon receiving the SCell activation command. So, RAN4 can wait for RAN2’s decision.

Proposal 1: RAN4 wait for RAN2’s reply on signalling design, including potential additional events for report triggering.

Issue 1-1-6: If measurement results are available, the UE will report them to the NW. How to determine the measurement result is available? FFS: - Option 1 (Apple, OPPO, Huawei, Xiaomi, vivo): - No need to define criteria to determine the L3 measurement result is available or not for FR2 unknown SCell activation enhancement. - Option 2 (Nokia, Ericsson): - The L3 measurement report is considered as valid only if it fulfils the measurement requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2). - Option 3 (Intel): - If UE already has available L3 measurement result when Scell activation command is triggered, the measurement result can be reported only if it satisfied the following conditions: - Measurement should satisfy the requirement for a deactivated Scell as specified in TS38.133 Table 9.2.5.2-3 (for FR1) and Table 9.2.5.2-4 (for FR2) - the time duration between measurement and Scell activation command is less than [X]s. - Option 4 (MTK): - RAN4 to define a new window [W]s to determine the freshness of the available L3-RSRP measurements at the UE prior to the reception time of Scell activation command. - Proposal 6: If L3-RSRP measurements are within [W], then the UE can send L3-RSRP report. - Option 6 (vivo): - The criteria to determine whether a measurement result is valid (i.e. available), including the signal quality thresholds and the maximum time-delta of the measurement before the reporting, are configured by network - Option 7 (CMCC, ZTE): - for L3 measurement report after Scell activation command, the reported SS-RSRP, SS-RSRQ, and SS-SINR measurements need to meet the accuracy requirements spedified in Clause 10. - Option 8 (ZTE): - To address the concern of the quick report is valid or not, the verification lies in two aspects: - 1)Whether the measurement report is fresh enough. - 2)Whether the quality of measurement result in the report is good or available enough. - The 1st aspect can be ignored since same issue exists in legacy L3 report. The 2nd aspect should be considered to guarantee the NW can obtain useful RSRP and SSB index info from the report. - Option 9 (Apple, CMCC, LGE, CTC, QC, OPPO): - Option 1 and the valid reporting must meet the existing measurement delay/accuracy requirement.

We think UE can follow the existing reporting criteria. and there is no need to define criteria to determine the L3 measurement result is available or not for FR2 unknown SCell activation enhancement.

Proposal 2: No need to define criteria to determine the L3 measurement result is available or not for FR2 unknown SCell activation enhancement.

Issue 1-1-7: FFS on necessity of L3 measurement reporting if UE has no valid measurement results? FFS: - Option 1 (Apple, ZTE, CTC, OPPO, Ericsson, Xiaomi, HW, vivo, QC): No need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results. - Option 2 (Nokia, Intel, vivo, Ericsson): The UE may continue the measurement and report a valid L3 measurement result when it is ready, if there is no valid L3 measurement result at the time of Scell activation. - Option 3 (LGE, CTC, MTK): If a UE has no valid measurement results, UE needs to report even the lowest RSRP reported value to notice whether a UE has valid measurement results to the network. - Option 4 (vivo, MTK): If the reporting of the results is carried by uplink L1L2 signaling, and UE has no valid measurement results at the L1L2 reporting occasion, UE may report -Inf value, e.g. ‘RSRP<-156dBm’. - Option 5 (MTK): when report triggering conditions are met, but UE has no fresh L3-RSRP measurements within [W] period, then instead of sending the report UE indicates unknown status for the target Scell.

From UE perspective, the measurement results should be valid once they are reported. There is no necessity of L3 measurement reporting if UE has no valid measurement results.

Proposal 3: No need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results

Issue 1-1-8: FFS: If valid L3 measurement results are reported, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported. FFS: - Option 1 (Apple, Nokia, Intel, QC, MTK, LGE, Xiaomi, vivo, CMCC, ZTE, CTC, OPPO): - if UE reports valid L3 measurement result, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported; - Proposal 1a (Apple, CTC, QC): after receiving SCell activation command, - if UE reports valid L3 measurement result before n+X slot (n is the slot when UE received SCell activation command), L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported; - otherwise, it shall be treated as legacy unknown case. - Proposal 1b(QC, CTC): - RAN4 shall define the reporting delay requirements from MAC-CE to trigger report and actual measurement report. If UE do not report measurement results during the reporting delay requirements, the new measurement configuration shall be expired. - Proposal 1c(vivo): - Send LS to RAN2, asking RAN2 to design a timer, so as to supervise the longest period between gNB triggering the reporting and the actual result being sent by the UE in PUSCH. The RRM requirements are not enhanced in R18 for the case when a UE does no report a valid measurement result before this timer expires. - Proposal 1d(OPPO): - FFS the side conditions. - Proposal 1e(Nokia): - A-TRS for fast SCell activation can be triggered based on the valid L3 measurement report after SCell activation command. - Proposal 1f(Xiaomi): - If no valid result to be reported, the UE needs to perform L3 measurement and L1 measurement with/without enhanced requirement according to UE capability. - Option 2 (HW, vivo, Apple (bullet #2)): (moderator: this option is also up to the conclusion in issue 1-1-1) - When the new triggered measurement reporting is reported before SCell activation command, it can be considered as known cases. - When the new triggered measurement reporting is reported after SCell activation, it can be considered as unknown case where the uncertainty for TCI can be defined as the time period between TCI configuration/activation relative to the first valid L1-RSRP reporting and the new triggered L3 measurement report which occurs earlier. - Option 3 (Ericsson): - RAN4 to agree that L3 part can be skipped and whether L1 part can be skipped or not pending on whether NW sends TCI command or not. - Option 4 (moderator): - use option 1 as starting point, - FFS on the detailed requirement design, e.g. option 2.

As discussed in last meeting, companies pointed out that it is up to NW decision whether reported measurement results with SSB ID is used for TCI activation. If valid L3 measurement results are reported after Scell activation command and before TCI activation, at least L3 parts can be skipped. For L1 parts, if the reported SSB with indexes becomes not detectable and UE receives TCI activation command with reported SSB ID, L1 measurement procedure can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported.

Proposal 4: If UE reports valid L3 measurement result, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported;

Sub-topic 1-2: Beam related enhancement for L3 part

Issue 1-2-1: Beam sweeping factor enhancement in L3 and L1 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception) Agreement: - For unknown FR2 SCell activation enhancement, introduce the UE capability to support Rx beam sweeping factor less than 8 for cell detection part and SSB based L1-RSRP measurement. - if UE has full set (N=8) of beam sweeping during AGC settling part during FR2 unknown SCell activation procedure. - Introduce beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement - Beam sweeping factor capability X1/X2 are two independent capabilities - Note: above enhancement only applies for FR2 unknown SCell activation enhancement - Note: How to capture in spec can be discussed in CR stage - The candidate values for X1/X2 are FFS Issue 1-2-2: can X1(Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation) be zero? (not related with WI of FR2 multi-Rx chain DL reception) The discussion is related with the previous agreement: Agreement in last meeting R4-2303228 Issue 1-3-3: enhancement of “8*Trs” part of current FR2 unknown Scell activation delay Agreement: - To keep “X1*Trs” part of current FR2 unknown Scell activation delay in the delay requirement for FR2 Scell activation enhancment. - X1 can be less than 8 in the beam sweeping factor reduction discussion for cell detection stage in L3 part based on UE capability.. FFS: - Option 1 (Apple, MTK, Xiaomi, HW (option 4b in issue 1-2-1 comments), Intel, QC, OPPO): no - Option 2 (Ericsson, vivo, Nokia, HW (option 4a in issue 1-2-1 comments)): Yes

As agreed in last meeting, beam sweep factors X1 and X2 are independent with each other, and network can assist UE to decide whether the reported UE capability can apply by configured explicit indication. RAN4 also agreed to introduce the UE capability to support Rx beam sweeping factor X1 and X2 less than 8 for cell detection part of L3 and for SSB based L1-RSRP measurement, respectively. So we think X1 cannot be zero, which means cell detection part of L3 cannot be skipped.

Proposal 5: X1 cannot be zero as cell detection part of L3 cannot be skipped.

3	Conclusion

Proposal 1: RAN4 wait for RAN2’s reply on signalling design, including potential additional events for report triggering.

Proposal 2: No need to define criteria to determine the L3 measurement result is available or not for FR2 unknown SCell activation enhancement.

Proposal 3: No need to report L3 measurement reporting after receiving SCell activation command if UE has no valid measurement results

Proposal 4: If UE reports valid L3 measurement result, L3 and L1 parts can be skipped, i.e., network can perform TCI activation after valid L3 measurement results are reported;

Proposal 5: X1 cannot be zero as cell detection part of L3 cannot be skipped.

4	Reference

[1] R4-2306315, WF on R18 RRM enhancement- FR2 SCell activation delay reduction, Apple