3GPP TSG-RAN WG4 Meeting # 107	R4-2308507

Incheon, KR, May 22 – May 26, 2023

Agenda Item: 8.8.3.5

Source: MediaTek Inc.

Title: Discussion on dual TCI state switch for multi-Rx UEs

Document for: Discussion

# Introduction

According to WF [1] and discussion summary [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WI is divided into six agenda items to be discussed: (1) general issues, (2) L1 measurement, (3) RLM/BFD/CBD, (4) scheduling/measurement restriction, (5) dual TCI state switching and (6) receive timing difference. The discussion in this paper focus on the “dual TCI state switching”.

# Discussion

In the following sections, below issues will be discussed sequentially.

- Requirement applicability of known and unknown TCI state

- Detectable condition

- Active TCI state list update

- RRC based TCI state switch

## 2.1 Requirement applicability of known and unknown TCI state

In last meeting, some companies discussed known and unknown TCI state in [2].

Agreements: - Requirements for DCI based dual TCI states switch delay for PDSCH reception are defined for known case only. - Requirements for MAC CE based dual TCI states switch delay for PDCCH reception are defined for known case. FFS if it is to be limited to known case only.

For dual TCI state switching, there is one open issue regarding whether to consider unknown TCI state switching in the last meeting. The discussed cases are (1) known TCI state + known TCI state (2) known TCI state + unknown TCI state (3) unknown TCI state + unknown TCI state. To our understanding, dual TCI state switching is for simultaneous reception of two PDSCHs, i.e., increase data throughput. If network indicates UE switches to unknown TCI states, the performance cannot be guaranteed, and it will waste UE power if beam failure happens. This is very similar to CQI/RI reporting, i.e., typically network only schedule rank-4 PDSCH after UE report RI=4 and the corresponding CQI. So, we do not see the needs and reason why network indicates UE switches to an unknown TCI state. Therefore, we think DCI based dual TCI states and MAC CE based dual TCI states can be limited to known case only.

Proposal 1: For dual TCI state switching, only define the requirement for the case when two indicated TCI states are known.

In addition, some companies discussed if dual TCI state was included in a group report in [2].

Issue 2-5-2: Definition of known condition: Way forward: GBBR is agreed as prerequisite to enable simultaneous reception. Does that mean dual TCI states are configured based on beams reported in GBBR or can other RS which are QCLed to beams reported in GBBR can be configured. Please consider following to provide your further analysis. - Option 1: dual TCI states are only configured based on beams reported in GBBR. - Option2: dual TCI states which are QCLed to beam pair reported in GBBR can be configured

For issue 2-5-2, we think both option 1 and 2 are OK to us. Basically, multi-RX operation is based on Rel-17 L1 RSRP group-based beam report, this is to select best beam pair to support simultaneous DL reception with better performance. But option 1 is more limited than option 2. So, we can agree both options for flexible condition for dual TCI states.

Proposal 2: Known TCI state definition for dual TCI states:

- The dual TCI states of reference signals were included in Rel-17 group-based beam reporting, or

- The dual TCI states which are QCL type D with the reference signals were included in Rel-17 group-based beam reporting.

## 2.2 Detectable condition

As below, there is one open issue regarding detectable condition in the last meeting.

#### Issue 1-3-3: Detectable condition of RS signals - Proposals - Option 1: - For detectable condition, all RSs in the same TCI chain for the target TCI state should remain detectable during the entire measurement/switch period. - Option 2: - Both RSs and their associated signals in the QCL type D infos are detectable during the entire measurement/evaluation/TCI state switching period.

UE may take the other RS (E.g., SSB) which is in the same TCI chain as the tracking RS to perform UE Rx beam selection. Therefore, it would be more reliable if all RSs in the same TCI chain remain detectable during the entire measurement and evaluation period. Furthermore, as same QCL Type-D means that the 2 RS are transmitted from the same Tx spatial filter, we tend to believe that the detectability should be the same for all RS in the same TCI chain. So, the following proposal is suggested.

Proposal 3: For detectable condition, all RSs in the same TCI chain for the target TCI state should remain detectable during the entire measurement/evaluation/TCI state switch period.

## Active TCI state list update

As below, there is one open issue regarding active TCI state list update in the last meeting.

Issue 2-6-1: Active TCI state list update Agreements: For active TCI state list update for addition of a new dual TCI states, legacy requirements can be used as baseline. FFS if T/F tracking for the dual TCI states are based on different SSBs. Proposals - Option 1(Intel): RAN4 to further discuss whether to define two types of TCI state list update delay requirement for sDCI and mDCI respectively, or just define a common one - Option 2(vivo): For active TCI state list update, requirements for addition of dual TCI states should be specified. - Option 3(vivo): For active TCI state list update for addition of a new dual TCI states, legacy requirements can be used as baseline. FFS if T/F tracking for the dual TCI states are based on different SSBs. - Option 4(Ericsson): Reuse the legacy requirements for the scenario when the RS occasions fully overlap. FFS for the case when the RS occasions are partially overlap. - Option 5(Ericsson): For switching/replacing of one of the two active TCI states, the existing requirements can be reused, but some clarification in the specification is necessary. - Option 6(Nokia): If a pair of TCI states is activated by MAC-CE command, consider the first SSB of each TCI state for activation time while arriving at the active TCI state list update delay requirements for dual TCI states.

From above agreement, active TCI state list update for addition of a new dual TCI states, legacy requirement can be reused as baseline. For the FFS, we think dual TCI states basically are associated with different SSBs. In this WI, RAN4 only consider intra-cell mTRP and the dual TCI states with different QCL type D must base on different SSBs. So, the following proposal is suggested.

Proposal 4: T/F tracking for the dual TCI states are based on different SSBs in this WI.

## 2.4 RRC based TCI state switch

As below, there is one open issue regarding RRC based TCI state switch in the last meeting.

Issue 2-4-1: Whether to define requirements for RRC based TCI state switch Way forward: Please bring further analysis to next meeting. Please confirm if dual to single TCI state switching through RRC reconfiguration can be considered for defining RRC based TCI state switching.

From the scenario of RRC based TCI state switch defined in TS 38.133, this is used for only one configured TCI state in RRC TCI state list. In multi-RX WI, at least, we support dual TCI states for simultaneous DL reception. We don’t see a reason to consider RRC based TCI state switch in this WI, but we are also fine to discuss possible scenarios. So, the following proposal is suggested.

Proposal 5: Not to consider RRC based TCI state switch in this WI.

Ref. TS 38.133 8.10.5 RRC based TCI state switch delay

# Summary

In this paper, the discussion of R18 multi-Rx chains is provided. We have the following proposal:

Proposal 1: For dual TCI state switching, only define the requirement for the case when two indicated TCI states are known.

Proposal 2: Known TCI state definition for dual TCI states:

1. The dual TCI states of reference signals were included in Rel-17 group-based beam reporting, or

2. The dual TCI states which are QCL type D with the reference signals were included in Rel-17 group-based beam reporting.

Proposal 3: For detectable condition, all RSs in the same TCI chain for the target TCI state should remain detectable during the entire measurement/evaluation/TCI state switch period.

Proposal 4: T/F tracking for the dual TCI states are based on different SSBs in this WI.

Proposal 5: Not to consider RRC based TCI state switch in this WI.

# Reference

- R4-2306319, WF on FR2_multiRx_part2, Ericsson, RAN4 #106-bis-e

- R4-2302768, Topic summary for [106bis-e][203] FR2_multiRx_part2, Ericsson, RAN#106-bis-e