3GPP TSG-RAN WG4 Meeting #107	R4-2308521

Seoul, Korea, May 22nd – 26th 2023

Agenda Item:	8.29.2.2

Source:	Ericsson

Title:	NCR-MT remaining RF requirements

Document for:	Discussion

# 1	Introduction

During RAN4#106bis, most open issues for NCR-MT RF requirements were resolved. In this contribution, we analyze the remaining open issues of emissions requirements and ACS.

# 2	Discussion

Summation of emissions from NCR-FWD and NCR-MT

For the downlink, support for simultaneous operation of NCR-FWD and NCR-MT is mandatory. Since only the NCR-FWD transmits in DL, emissions requirements need only be defined for NCR-FWD.

Downlink (i.e., UE side) TX emissions requirements are only defined for the NCR-FWD.

For the uplink, support of simultaneous NCR-FWD and NCR-MT is optional. If simultaneous operation is supported, then the emissions limit should be on the combined emissions from NCR-FWD and NCR-MT. If it is declared not supported, then the emissions limits for UL could be met by NCR-FWD and NCR-MT separately.

If the NCR supports simultaneous MT and FWD transmission, then the UL (i.e. BS side) TX emissions requirements should be defined on the total emissions from MT and FWD.

If the NCR does not support simultaneous MT and FWD transmission, then the UL (i.e. BS side) TX emissions requirements can be defined separately for the FWD and MT

It is important to note that defining emissions on the combination of NCR-FWD and NCR-MT does not imply that the emissions must be measured simultaneously during the conformance test. If, for example the NCR-FWD and NCR-MT have separate connectors, it is feasible to measure the emissions for each part on each connector, calculate the sum and then compare the sum to the limit.

If the emissions requirements are defined on the total emissions from MT and FWD, this does not imply that the emissions of MT and FWD need to be measured together as part of conformance testing.

Where the emissions are met by the combination of FWD and MT, it is necessary to consider how to define the OBUE requirements and the ACLR. The OBUE requirements are based on offset from the RF bandwidth. Where emissions are defined on the combination of MT and FWD, the RF bandwidth should be based on the maximum RF bandwidth considering FWD passband and MT carrier positions. The core requirement can be defined in the same manner as for the Rel-17 repeater, based on the bandwidth edge.

If the TX emissions for the BS side are defined on the sum of MT and FWD, the RF bandwidth should be the total bandwidth of passband and the MT carrier.

The ACLR should take into account emissions from both FWD and MT. The MT will have a carrier bandwidth, and the FWD part has a nominal carrier bandwidth defined. When the emissions are defined on the combination of FWD and MT then, in order that the MT part can be measured, the ACLR carrier bandwidth for the combined emissions should defined as the MT carrier bandwidth.

If the TX emissions for the BS side are defined on the sum of MT and FWD, the bandwidth for the ACLR requirement shall be the MT channel bandwidth.

Emissions scaling and regulatory limits

In the previous meeting, it was proposed to scale spurious emissions according to the number of TAB connectors, or 9dB in the case of OTA. In some regions, regulatory limits do not allow for emissions scaling. For these requirements, the scaling should not be applied (in the same manner as the BS specification).

Emissions scaling shall not be defined where it is not allowed by regulations.

ACLR requirement for LA repeater

For IAB, the ACLR for the LA IAB-MT class is based on the BS specification. During RAN4#106bis-e, it was proposed that the NCR-MT ACLR should be based on the UE requirement for the LA class. Since the NCR LA class is based on UE power levels and transmits in the uplink only, it is reasonable to adopt the UE ACLR for the NCR-MT. However, in this case then, in order to enable the emissions to be defined on both NCR-MT and NCR-FWD together if they can transmit simultaneously, it is necessary to mandate that the NCR-FWD and NCR-MT belong to the same class.

It is OK to allow the ACLR and emissions requirements for the LA IAB-MT class to be the same as for the UE. However, then the same class should be applied to both NCR-FWD and NCR-MT in uplink.

ACS requirement for LA repeater

Similar to the ACLR, for an IAB the ACS is defined based on the BS specification. For an LA NCR-MT, since the NCR-MT can only receive during DL slots it is reasonable to apply the UE ACS, as proposed at RAN4#106bis-e.

It is OK to use UE ACS for the LA NCR-MT

ON/OFF power and transient time

Since the TDD DL/UL pattern applies to both the NCR FWD and NCR MT, logically the FWD and MT, if switching, must switch at the same time.

The transient time could be defined and measured on FWD and MT together. However, it does no harm to define and measure the transient time separately either.

The transient time can be defined for FWD and MT separately

Due to simultaneous operation of FWD and MT, the OFF power level should be met by the combined output of MT and FWD for DL, and also for UL if simultaneous operation is supported, in the same manner as for emissions.

The OFF power should be defined as the total OFF power from the MT and FWD

# Conclusion

In the previous sections we made the following observations:

Observation 1	If the emissions requirements are defined on the total emissions from MT and FWD, this does not imply that the emissions of MT and FWD need to be measured together as part of conformance testing.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Downlink (i.e., UE side) TX emissions requirements are only defined for the NCR-FWD.

Proposal 2	If the NCR supports simultaneous MT and FWD transmission, then the UL (i.e. BS side) TX emissions requirements should be defined on the total emissions from MT and FWD.

Proposal 3	If the NCR does not support simultaneous MT and FWD transmission, then the UL (i.e. BS side) TX emissions requirements can be defined separately for the FWD and MT

Proposal 4	If the TX emissions for the BS side are defined on the sum of MT and FWD, the RF bandwidth should be the total bandwidth of passband and the MT carrier.

Proposal 5	If the TX emissions for the BS side are defined on the sum of MT and FWD, the bandwidth for the ACLR requirement shall be the MT channel bandwidth.

Proposal 6	Emissions scaling shall not be defined where it is not allowed by regulations.

Proposal 7	It is OK to allow the ACLR and emissions requirements for the LA IAB-MT class to be the same as for the UE. However, then the same class should be applied to both NCR-FWD and NCR-MT in uplink.

Proposal 8	It is OK to use UE ACS for the LA NCR-MT

Proposal 9	The transient time can be defined for FWD and MT separately

Proposal 10	The OFF power should be defined as the total OFF power from the MT and FWD