3GPP TSG-RAN WG4 Meeting # 107	R4-2308568

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on simulation assumption to derive the isolation distance for ATG scenario

Agenda Item:	8.14.1.2

Document for:	Approval

Introduction

In last meeting, we have the way forward [1] on ATG co-existence evaluation. For the non-synchronized scenario, we have the following Way forward.

Issue 1-3-2: location relationship between ATG UE and TN network for non-synchronized scenario to conclude isolation distance FFS and conclude in May meeting to simulate the worst case. - ATG UE and TN cluster are on the same side of ATG BS, i.e. ATG BS actually point at TN cluster to consider the worst case. - Minimum distance between ATG BS and ATG UE is 20/50km - There is no additional restriction between ATG BS and ATG CPE. Companies are welcome to provide their results for worst case with detailed deployment illustration. - Following show one example for simulation - if isolation distance between TN cluster and ATG BS is larger than 20/50km, ATG UE is always on top of TN cluster - if isolation distance between TN cluster and ATG BS is less than 20/50km, ATG UE is dropped 20/50km distant from ATG BS - The isolation distance is the distance from the ATG BS to the nearest TN BS. The ATG BS sector and nearest TN BS sector should be pointing towards one another in azimuth. The diagram below is illustrative and does not capture this.

In this paper, we’d like to share our views on the simulation assumption on the non-synchronized scenario.

Discussion

Referring to TR 38.876 [2], we have the following non-synchronized scenarios.

Table 1 Non-synchronized scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz | ATG BS => TN BS | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz | TN BS => ATG BS | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | FFS |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 TN BS => ATG BS | FFS |

For scenarios 5, 7 and 14, these non-synchronized scenarios are related to the mutual interference between ATG BS and TN BS. For the other scenarios 6, 8, 13, these non-synchronized scenarios are related to the mutual interference between ATG UE and TN UE. For interference scenarios between ATG UE and TN UE, the airplane should be always over the Center of TN cluster. The mechanism are different, so I think we can decouple these two kinds of scenarios.

Proposal 1: To decouple the interference scenarios 5, 7 and 14 between ATG BS and TN BS from the interference scenarios 6, 8 and 13 between ATG UE and TN UE. And isolation distance is derived from interference scenarios 5, 7 and 14.

For the interference scenarios 5, 7 and 14 between ATG BS and TN BS, we can consider the following network layout in figure 1.

Firstly, we must define the isolation distance between ATG BS and TN cluster accurately. We provide a definition:

Proposal 2: Isolation distance is defined between ATG BS site and the central point of TN cluster.

Secondly, we should derive the range of isolation distance and step size. We can refer to the ISD of TN. The assumptions of TN ISD are 7.5km ISD for 2GHz and 3.5km ISD for 4GHz. So we have the following proposal.

Proposal 3: some assumptions for isolation distance.

| Minimum value of isolation distance | Step size of isolation distance | Maximum value of isolation distance |
| --- | --- | --- |
| 3*ISD | 3*ISD (coarse precision) FFS (Fine precision) | less than 100km |

Finally, we have provided some coexistence results in contribution [3]. It is shown that no serious interferences are observed.

Observation 1: no serious interferences are observed for scenarios 6, 8 and 13 between ATG UE and TN UE.

Proposal 4: scenarios 6, 8 and 13 between ATG UE and TN UE can be considered to verify whether the existing 30dB ACLR and 33dB ACS requirements can be reused for ATG UE instead of deriving isolation distance.

Summary

Proposal 1: To decouple the interference scenarios 5, 7 and 14 between ATG BS and TN BS from the interference scenarios 6, 8 and 13 between ATG UE and TN UE. And isolation distance is derived from interference scenarios 5, 7 and 14.

Proposal 2: Isolation distance is defined between ATG BS site and the central point of TN cluster shown below.

Proposal 3: some assumptions for isolation distance.

| Minimum value of isolation distance | Step size of isolation distance | Maximum value of isolation distance |
| --- | --- | --- |
| 3*ISD | 3*ISD (coarse precision) FFS (Fine precision) | less than 100km |

Observation 1: no serious interferences are observed for scenarios 6, 8 and 13 between ATG UE and TN UE.

Proposal 4: scenarios 6, 8 and 13 between ATG UE and TN UE can be considered to verify whether the existing 30dB ACLR and 33dB ACS requirements can be reused for ATG UE instead of deriving isolation distance.

- References

[1] R4-2306607, WF on ATG co-existence evaluation, CMCC

[2] TR 38.876, Study on Air-to-ground network for NR, v0.3.0

[3] R4-2305372, Discussion on 2GHz and 4GHz simulation result for ATG scenario, Huawei, HiSilicon