3GPP TSG-RAN WG4 Meeting # 107	R4-2308578

Incheon, KR, May 22 – May 26, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on Ka band NTN UE

Agenda Item:	8.27.4

Document for:	Approval

Introduction

In RAN4#106bis meeting, a WF [1] on NTN UE RF requirements in Ka-band was approved with the following agreements.

Issue 1-1: UE types

- Encourage companies to provide the regulation differences leading to different RF/RRM requirements between the mobile and fixed NTN UEs, and further discuss whether RAN4 needs to define the different UE types.

- Discuss possible differentiation of NTN UE types in above 10 GHz, based on RF parameters (noise figure, antenna types, antenna gain and transmission power, or alternatively minimum EIRP in the peak direction, Effective Isotropic Sensitivity EIS, etc.).

- The impact on RF and RRM requirements will be investigated, and analysis on RF requirement impact is prioritized.

Issue 2-1:  Beam tracking capability

Tentative agreement:

the (Rx and Tx) beam pointing error are compliant with at least the relevant ETSI harmonized standard. (e.g.

      - EN 303 978 “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands”;

      - EN 303 979 “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands”).

In last meeting, RAN4 has no consensus on whether to specify two types for NTN UE, i.e. mobile and fixed NTN UEs. Companies are encouraged to investigate the regulation differences leading to different RF/RRM requirements between the mobile and fixed NTN UEs. In this paper, we’d like to provide our observations on this issue. In addition, the noise figure of NTN UE was discussed in last meeting, we’d like to provide our technical investigation and observations for this issue in this paper.

Discussion on the regulation differences for NTN UEs

Frequency range differences

Based on ITU radio regulations [2], there are two basic kinds of satellite services, i.e. fixed-satellite service (FSS) and mobile-satellite service (MSS). The definitions for two kinds of satellite services are shown below.

1.21 fixed-satellite service: A radiocommunication service between earth stations at given positions, when one or more satellites are used; the given position may be a specified fixed point or any fixed point within specified areas; in some cases this service includes satellite-to-satellite links, which may also be operated in the inter-satellite service; the fixed-satellite service may also include feeder links for other space radiocommunication services.

1.25 mobile-satellite service: A radiocommunication service: – between mobile earth stations and one or more space stations, or between space stations used by this service; or – between mobile earth stations by means of one or more space stations. This service may also include feeder links necessary for its operation.

For fixed-satellite service, the earth stations are at given positions which may be a specified fixed point or any fixed point within specified areas.

For mobile-satellite service, mobile earth stations are movable.

Observation 1: mobile and fixed NTN UEs belong to two different services referring to ITU radio regulations.

We just copy one segment of frequency range allocation for different services in Ka band UL part. Mobile-satellite service are allowed to be used in frequency range 29.5~30GHz, but Fixed-satellite service are allowed to be used in frequency range 27.5~30GHz.

Observation 2: The allocated frequency range for mobile and fixed satellite service are different from regulations’ perspective.

Referring to the WF [3] in RAN4#106 meeting, band n511 and n510 were agreed to be used for exemplary bands. However, ESIM operation are not included for band n510. That’s why two different bands (band n511 and n510) are arranged based on FCC 47 CFR part 25.

In total, frequency range which are basic RF requirements are different for mobile and fixed NTN UEs referring to ITU and other regional regulations. Thus, it’s necessary to design some mechanisms to distinguish mobile and fixed NTN UEs. At least, RAN4 need to specify two sets of frequency range requirements for fixed and mobile NTN UEs respectively.

Proposal 1: it’s necessary to design some mechanisms to distinguish mobile and fixed NTN UEs. At least, RAN4 need to specify two sets of frequency range requirements for fixed and mobile NTN UEs respectively.

Satellite orbit differences

In ITU and other regional regulations, geostationary-satellite orbit (GSO) VS non-geostationary satellite orbits (NGSO) is another key difference. Referring to ITU Satellite issues [4]: WRC-19 has the following agreement for ESIM communicating with geostationary (GSO) space stations.

WRC-19 agreed on the regulatory and technical conditions under which the frequency bands 17.7‑19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) can be used by the three types of ESIM communicating with geostationary (GSO) space stations in the fixed-satellite service (FSS).

Observation 3: referring to WRC-19 agreements, frequency bands 17.7‑19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) can be used by the three types of ESIM communicating with geostationary (GSO) space stations in the fixed-satellite service (FSS).

Referring to the WF [3] in RAN4#106 meeting, band n512 were agreed to be used for exemplary band considering ECC Decision(05)01 [5] and ECC Decision (13)01. In ECC Decision (13)01 [6], the purpose of this ECC Decision is to:

Observation 4: In ECC Decision (13)01, the purpose of this ECC Decision is to harmonise the use and allow the free circulation and exemption from individual licensing of ESOMPs of geostationary satellite networks operating within the frequency bands 17.3~20.2 GHz (receive band) and 27.5~30 GHz (transmit band).

Thus, for mobile NTN UEs which operate in FSS frequency range, only geostationary satellite networks are allowed by some regional regulations.

Proposal 2: it’s necessary to design some mechanisms to distinguish mobile and fixed NTN UEs, since some regulations only allow mobile NTN UEs to operate in FSS frequency range with geostationary satellite networks.

Discussion on the noise figure for NTN UE

Actually, RAN4 has studied and discussed the noise figure over the 7~24GHz range in one Rel-16 SI [7]. All the outcomes were captured into the TR 38.820 [8]. In the clause 5.5.1 of TR 38.820, RAN4 has investigated the key parameter noise figure over the 7 ~ 24 GHz range. Some high level general aspects for noise figure are copied below.

Receiver Noise Figure (NF) is one of the essential metrics for determining receiver requirements. For determining typical NF value some fundamental principles should be considered: - The NF is not given by the LNA alone, but also by bandwidth, linearity and dynamic range dependencies as there is a delicate balance which should be considered when future requirements are specified. - A full RF receiver chain all the way up to radiating elements should be addressed as all parts in the chain would contribute to the overall receiver performance including switch (for TDD), routing and filter losses, etc. - For some compact and highly integrated building practices with many transceivers and antennas, the power efficiency and heat dissipation in small area/volume is necessary needs to be considered. - As an example, in the context of NF, it might be possible to reduce the noise contribution from ADC by using more bits, but this would have significant implication in terms of power consumption and heat dissipation aspect as a single added bit to ADC would result in four times higher power consumption. In general the ADC power consumption is proportional to BW2 · DRACD where DRACD is the ADC dynamic range. For UE, the typical NF values considering the usage of similar technologies as for BS, should be similar but slightly higher to allow more flexibility in the implementation and taking to account the size limitations.

Observation 5: The NF is not given by the LNA alone, but also by bandwidth, linearity and dynamic range dependencies as there is a delicate balance which should be considered when future requirements are specified.

Observation 6: A full RF receiver chain all the way up to radiating elements should be addressed as all parts in the chain would contribute to the overall receiver performance including switch (for TDD), routing and filter losses, etc.

Observation 7: For some compact and highly integrated building practices with many transceivers and antennas, the power efficiency and heat dissipation in small area/volume is necessary needs to be considered.

In addition, ETSI has published an update of TR 101 854 [9] on fixed radio systems in April 2019, which contains a table on typical NF values and associated industrial margins (IMF) of the radio receivers. This table is extracted below in table 3-1. The IMF considers various performance variations of system elements over e.g. temperature extremes, voltage variations, or aging by capturing production spread of RF circuits.

Table 3-1: Typical Noise Figures (NF) and associated Industrial Margins (IMF)

| Frequency band (GHz) | Typical Noise Figure (NF) (dB) | Industrial margin (IMF) (dB) |
| --- | --- | --- |
| 1,3 - 3 | ~4 | +3 |
| 3 - 5 | ~5 | +3 |
| 6 - 15 | ~5 | +3 |
| 18 - 23 | ~6 | +3 |
| 26 - 28 | ~7 | +3 |
| 32 | ~7 | +3 |
| 38 - 42 | ~8 | +3 |
| 48 - 50 | ~9 | +3 |
| 52 - 55 | ~10 | +3 |
| 71 - 76 | ~13 | +4 |
| 81 - 86 | ~13 | +4 |

NOTE: 	The above ETSI originated Industrial Margins (IMF) shall not be confused with the Implementation Margins (IM) used in process of RAN4 conformance requirements derivation.

Observation 8: Referring to ETSI TR 101 854, the typical noise figure in frequency range 18 – 23GHz is 6 dB.

Finally, we investigated the some IEEE published papers. In paper [10], the performances of state-of-the-art LNA-only were summarized below.

In paper [11], the Performance Summary and Comparison for LNA-only were summarized below.

In paper [12], the Performance comparison of recently published LNAs were summarized below.

In paper [13], the Performance comparison with other LNAs were summarized below.

Referring to published papers from IEEE, the value range of noise figure given by LNA alone at 17GHz is about 2.2dB ~ 6 dB. If we consider the whole Rx chain, the noise figure for the receiver is worse than the value given by LNA alone. When NTN UE Rx requirements are discussed, at least RAN4 need to assume noise figure as 6 dB considering the outcomes in 7~24GHz SI, ETSI TR 101 854 and IEEE published papers.

Proposal 3: When NTN UE Rx requirements are discussed, at least RAN4 need to assume noise figure as 6 dB considering the outcomes in 7~24GHz SI, ETSI TR 101 854 and IEEE published papers.

Summary

Observation 1: mobile and fixed NTN UEs belong to two different services referring to ITU radio regulations.

Observation 2: The allocated frequency range for mobile and fixed satellite service are different from regulations’ perspective.

Proposal 1: it’s necessary to design some mechanisms to distinguish mobile and fixed NTN UEs. At least, RAN4 need to specify two sets of frequency range requirements for fixed and mobile NTN UEs respectively.

Observation 3: referring to WRC-19 agreements, frequency bands 17.7‑19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) can be used by the three types of ESIM communicating with geostationary (GSO) space stations in the fixed-satellite service (FSS).

Observation 4: In ECC Decision (13)01, the purpose of this ECC Decision is to harmonise the use and allow the free circulation and exemption from individual licensing of ESOMPs of geostationary satellite networks operating within the frequency bands 17.3~20.2 GHz (receive band) and 27.5~30 GHz (transmit band).

Proposal 2: it’s necessary to design some mechanisms to distinguish mobile and fixed NTN UEs, since some regulations only allow mobile NTN UEs to operate in FSS frequency range with geostationary satellite networks.

Observation 5: The NF is not given by the LNA alone, but also by bandwidth, linearity and dynamic range dependencies as there is a delicate balance which should be considered when future requirements are specified.

Observation 6: A full RF receiver chain all the way up to radiating elements should be addressed as all parts in the chain would contribute to the overall receiver performance including switch (for TDD), routing and filter losses, etc.

Observation 7: For some compact and highly integrated building practices with many transceivers and antennas, the power efficiency and heat dissipation in small area/volume is necessary needs to be considered.

Observation 8: Referring to ETSI TR 101 854, the typical noise figure in frequency range 18 – 23GHz is 6 dB.

Proposal 3: When NTN UE Rx requirements are discussed, at least RAN4 need to assume noise figure as 6 dB considering the outcomes in 7~24GHz SI, ETSI TR 101 854 and IEEE published papers.

- References

[1] R4-2306624, WF on NTN UE RF requirements in Ka-band, ZTE

[2] Radio Regulations, ITU

[3] R4-2302998, WF for system parameters of above 10 GHz NTN band, Moderator (THALES)

[4] https://www.itu.int/en/mediacentre/backgrounders/Pages/Earth-stations-in-motion-satellite-issues.aspx Satellite issues: Earth stations in motion (ESIM)

[5] ECC Decision (05)01, The use of the band 27.5-29.5 GHz by the Fixed Service and uncoordinated Earth stations of the Fixed-Satellite Service (Earth-to-space). Approved 18 March 2005. Amended 8 March 2013

[6] ECC Decision 13(01), The harmonised use, free circulation and exemption from individual licensing of Earth Stations On Mobile Platforms (ESOMPs) within the frequency bands 17.3-20.2 GHz and 27.5-30.0 GHz, Approved 8 March 2013 and latest amended 2 July 2021

[7] RP-182884		Study on 7 - 24 GHz Frequency Range, SID

[8] TR 38.820, 7 - 24 GHz frequency range

[9] ETSI TR 101 854: “Fixed Radio Systems; Point-to-point equipment; Derivation of receiver interference parameters useful for planning fixed service point-to-point systems operating different equipment classes and/or capacities”

[10] Na Peng, Dixian Zhao, “A Ku-Band Low-Noise Amplifier in 40-nm CMOS”, 2019 IEEE International Conference on Integrated Circuits, Technologies and Applications

[11] Syed Sharfuddin Ahmed, Hermann Schumacher, “Low Power Ku- and Ka-Band SiGe HBT Low-Noise Amplifiers”, 2020 Austrochip Workshop on Microelectronics

[12] Chenxi Zhao, Kai Kang, “A Ku-band Miniaturized LNA in 0.18-μm CMOS Process for Low-cost Phased Array Application”, 2019 IEEE

[13] Bing Liu, Kaixue Ma, Shouxian Mou, Fanyi Meng, “A Ku-Band LNA in 0.35-μm SiGe BiCMOS Technology”, 2017 IEEE