3GPP TSG-RAN WG4 Meeting #107 	R4-2308621

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On BS RF requirements for Network energy savings for NR

Agenda item:	8.35.3

Document for:	Discussion

Introduction

In RAN4#106bis, the following agreements were made regarding the scope of this work item work in RAN4 [1]:

Issue 2-1-1: Priorities for feasibility study - Agreement： - RAN4 prioritize the discussion on SSB-less Scell operation for inter-band CA for FR1 and co-located cell. Issue 2-2-6: How to define the RF requirements for inter-band CA for SSB-less Scell operation - Agreement: - No impact to the legacy requirements of inter-band CA TAE defined in TS38.104. Whether or not to define feature-specific TAE requirements for SSB-less operation (NW enabling energy savings) for FR1 co-located inter-band CA is FFS. Issue 2-3-1: RF requirements for Cell DTX - Agreement: - FFS on the necessity to define ON-OFF power requirement waiting for more inputs from RAN1/2. Issue 2-4-1: RF requirements for spatial domain techniques - Agreement: - FFS on the necessity to the updates in manufacturer declarations, switching period and DL EVM requirement waiting for more inputs from RAN1/2.

In this contribution, we further discuss about open issues presented in the WF.

Discussion

In this TDoc, we discuss about the issues related to feasibility study on SSB-less SCell operation, RF requirements for cell DTX, spatial domain techniques, etc.

Issue 2-1-1: Priorities for feasibility study

In the last meeting RAN4-RRM has agreed the following scenarios to be considered for SSB-Less SCell operation for FR1 inter-band collocated CA.

• Agreements - The following scenarios are considered for inter-band SSB-less SCell requirements definition - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell. - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis

Scenario 1 is originated from the intra-band SSB-less scenario hence is well received during the discussion. As Scenario 1 is originated from intra-band SSB-less SCell operation, we suggest RAN4 prioritize the study of FR1 inter-band SSB-less operation in this scenario. Our paper on RAN4 RRM [5], provides more analysis about the reason for prioritizing Scenario 1.

RAN4 to prioritize the SSB-less SCell operation for FR1 inter-band collocated CA in Scenario 1 identified in RRM.

Furthermore, the conditions under which the SSB-less SCell operation could be feasible for the above mentioned scenarios has been discussed as well in RRM. These conditions are

- Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell

- The difference of the reception power with the FR1 inter-band active serving cell

- FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell)

- FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations

Some of the conditions listed above are relevant or impacting to the existing RAN4 RF core specifications.

At the starting point, RAN4-RF could identify the list of parameters relevant to RF core specifications and provide to RAN4-RRM to proceed efficiently in the RRM feasibility study.

RAN4 to consider providing, if necessary, a list of RF-core specification parameters to RAN4-RRM to efficiently proceed with the RRM feasibility study.

Issue 2-2-6: How to define the RF requirements for inter-band CA for SSB-less Scell operation

As agreed in the previous meeting, RAN4 should not discuss about changing the legacy requirements of inter-band CA TAE, defined in TS38.104. As proposed in our RAN4 RRM [5] paper, RRM could use 3us for the TAE while deriving the RTD in the feasibility studies.

Regarding whether or not to define feature-specific TAE requirements for SSB-less operation (NW enabling energy savings) for FR1 co-located inter-band CA, we believe RAN4 could still wait for the conclusions of the feasibility study to make concrete agreements. In the meantime, RAN4 could also try to identify the implications of reducing the TAE below 3 us from UE RF architecture point of view. In summary, we believe that by introducing new lower TAE value other than specified one, there may also be compatibility issues from both BS and UE sides.

RAN4 should not change the currently specified TAE value 3 us even as a support feature for network energy savings, until conclusions are made from the feasibility study in RRM.

Issue 2-3-1: RF requirements for Cell DTX

RAN1 has made the following agreements related to cell DTX operation in NES WI [2].

R1-2304014 Discussion summary #2 of issues for enhancements on cell DTX/DRX mechanism Agreement From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting) - FFS: - PDCCH in USS - UE behavior for retransmission - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation - PDCCH in Type-3 CSS - UE behavior for retransmission - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation - PRS - CSI-RS configured by measObjectNR (for RRM) - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD) - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking) - Periodic/Semi-persistent CSI-RS (for BM) - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: Whether the list of impacted signals/channels can be configurable - FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period R1-2304015 Discussion summary #3 for enhancements on cell DTX/DRX mechanism Agreement Study L1 signalling for enhancing cell DTX/DRX including activation/deactivation for a single configuration which will have the following characteristics: - PDCCH based signaling - FFS: Whether enhancing legacy DCI or introducing new DCI - FFS: DCI content - FFS: Whether L1 signaling is UE specific DCI or group common DCI - FFS: Timer or validity duration based activation/deactivation of cell DTX/DRX - FFS: whether to specify a reference time for activation/deactivation of cell DTX/DRX - FFS: If multiple Cell DTX/DRX patterns are to be supported - FFS on detailed UE behavior upon reception of L1 signaling at least including application delay - FFS how to guarantee reliability of the L1 signaling - FFS whether the L1 signal can be monitored in non-active periods. Agreement From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - FFS: SRS for positioning - FFS: - HARQ feedback for SPS PDSCH - FFS whether there will be exception case(s) for UE transmitting listed signals/channels during non-active periods of DRX - FFS Whether the listed signals/channels can be configurable by gNB - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period R1-2304119 Discussion summary #4 for enhancements on cell DTX/DRX mechanism Further study the following in RAN1: - Handling of HARQ-ACK codebook generation when configured with cell DTX/DRX - Handling of PUCCH deferral operation during non-active periods of cell DRX - Handling of overlapping channels where a least a channel overlaps with non-active periods of cell DTX/DRX - Handling of signals/channels that can be received/transmitted repeatedly during non-active periods of cell DTX/DRX - Handling of PUCCH switching during non-active period to an active cell - Other enhancements are not precluded. Agreement For PDDCH monitoring, further work on Rel-18 NES in RAN1 is to follow the RAN2 agreement below: 10. The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH). Working Assumption Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disenabling the feature) from RAN1 perspective. - This does not imply that L1 activation/deactivation is supported in Rel-18\ - Note: Reliability, overhead, and benefits are FFS

Based on current RAN1 agreements, the discussion is mainly focused on identifying the signals and channels transmitted by the gNB to the UE during non-active periods of cell DTX, required L1 signalling for enhancing cell DTX/DRX operation (activation and deactivation), identifying the signals and channels transmitted by the UE during to the gNB during non-active periods of cell DTX, etc.

RAN1 currently discusses identifying signals and channels transmitted by BS/UE during cell DTX/DRX, and other signalling aspects to enable cell DTX/DRX operation.

One concern raised during the email discussion was the applicability of TDD ON-OFF power requirements in cell DTX/DRX operation for TDD and FDD operations.  We believe applying the TDD ON-OFF requirements in cell DTX/DRX operation is not straightforward, and that needs further investigations. Therefore, RAN4 should not take any decisions related to specifying ON-OFF power requirements for cell DTX/DRX operation without further investigations.

RAN4 should not take decision to specify ON-OFF power requirements for TDD or FDD operation without further investigations.

Issue 2-4-1: RF requirements for spatial domain techniques

Regarding the necessity to the updates in manufacturer declarations, the conformance testing specification may already contain the required manufacture declarations to support energy saving spatial domain techniques, e.g., to support different muting patterns of antenna elements. However, energy saving feature may introduce many different muting patterns, and relevant beam patterns.

The BS vendors have to comply with the OTA conformance test specification (38.141-2), and that would generate a lot of testing effort for possible huge number of muting patterns of antenna elements. RAN4 may need to investigate how to minimize this testing effort (i.e., the time and complexity of testing) without reducing the test coverage in conformance testing part.

The BS vendors have to comply with the OTA conformance test specification (38.141-2), and that would generate a lot of testing effort for possible huge number of muting patterns of antenna elements.

RAN4 may need to investigate how to minimize the testing effort (i.e., the time and complexity of testing) without reducing the test coverage in conformance testing part.

Regarding the element pattern switching period the following agreements has been made in RAN1 #112 [3].  This aspect has been discussed a bit in RAN1#112bis-e, but no clear consensus has been reached.

Agreement Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that - Whether there is a need for transition time per adaptation (for UE) - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

RAN1 has not had any consensus about element pattern switching period in the last two concluded meetings.

RAN4 to wait until RAN1 agrees about element pattern switching periods, which are relevant to RAN4.

In previous meeting, [4] has pointed out that due to frequent element switching the downlink EVM performance could be impacted. We are not clear about why the switching may happen frequently (as mentioned in [4]), and also whether the EVM degradation is observed during the switching period or after switching is completed. We would appreciate if the proponent company could provide further clarification about this observation.

Conclusion

In this paper, we have made the following observations and conclusions:

- RAN4 to prioritize the SSB-less SCell operation for FR1 inter-band collocated CA in Scenario 1 identified in RRM.

RAN4 to consider providing, if necessary, a list of RF-core specification parameters to RAN4-RRM to efficiently proceed with the RRM feasibility study.

RAN4 should not change the currently specified TAE value 3 us even as a support feature for network energy savings, until conclusions are made from the feasibility study in RRM.

RAN4 should not take decision to specify ON-OFF power requirements for TDD or FDD operation without further investigations.

RAN4 may need to investigate how to minimize the testing effort (i.e., the time and complexity of testing) without reducing the test coverage in conformance testing part.

RAN4 to wait until RAN1 agrees about element pattern switching periods, which are relevant to RAN4.

- References

- R4-2306639, WF on RF feasibility study of Network Energy Savings for NR, Nokia, Nokia Shanghai Bell, RAN4 #106-bis-e, April 17 – April 26, 2023.

- RAN1 Chair’s Notes, RAN1 #112-bis-e, April 17 – April 26, 2023.

- RAN1 Chair’s Notes, RAN1 #112, February 27 – March 3, 2023.

- R4-2305529, Discussion on RF requirement impacts from NES perspective, ZTE Corporation, RAN4 #106-bis-e, April 17 – April 26, 2023.

- R4-2307362, SSB-less operation for FR1 inter-band co-located CA, Nokia, Nokia Shanghai Bell