3GPP TSG-RAN WG4 Meeting #107 	R4-2308633

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.858: Feasibility of FR2 UE aspects

Agenda item:	8.20.2.2.4

Document for:	Discussion

Introduction

Some introductory

The new RAN1/RAN4 study item on evolution of duplex operation for NR TDD systems in unpaired spectrum was adopted [1]. The assumptions are listed as follows:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

While the work item objectives are the following:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this document, we discuss the text proposals on the feasibility of FR1 UE aspect based on the agreed text proposal in [2].

Discussion

Similar to our contribution about the feasibility of FR1 UE aspects [3], we note that by assuming a fixed NF model, the AGC is not modeled. Therefore, we propose an update to the text proposal.

AGC is not modelled if a fixed NF is used as a function of the input power.

Consider the change to the text proposal to TR 38.858.

Text Proposal

Begin changed section ******************************************************

10.7 FR2-1 Feasibility of UE aspects

10.7.1    Interference analysis

10.7.1.1  UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

10.7.1.1.1 Receiver aspects

Existing co-channel UE RX performance requirements

For legacy UEs, the current UE RF architecture can be assumed without any RF architecture modification.

Currently there are no RF requirements for UE co-channel Rx performance.

Sub-band filtering and legacy UEs

For legacy UEs, no sub-band filtering is implemented, and therefore RAN4 has not assumed any subband filtering.

Sub-band filtering for a SBFD-aware UEs – UEs with a new feature

SBFD-aware UEs support half duplex operation while receiving the sub-band configuration from the network. For this kind of UE, the RF architecture and applicable RF requirements need further study for future releases. In the present study, legacy UEs are the main focus.

For new SBFD aware UEs, we need further study on whether sub-filtering can be considered or not. The subband/in-channel selectivity needs further study.

Thermal self-noise aspects (both adjacent channel and co-channel)

RAN4 decided on a simple fixed-value noise figure model for the UE receiver. Generally, the receiver noise figure will vary with the input power level, however the single value noise figure model was considered to be sufficient for the purpose of system studies for SBFD, therefore AGC is not modelled. RAN4 decided on a NF of [7.5 to 10dB].

Subband in-channel selectivity

It is worth noting that the RF degradations can cause inter-subband interference . We analyzed the design of the FR2-1 receiver. There are multiple considerations in the receiver design … for example residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions. The effect of all these distortions is lumped into a single parameter we call selectivity. Based on the discussion and analysis from the meeting, contributions suggested possible the sub-band selectivity values from 20 dB to 34 dB. The receiver performance is simply modelled as being [20 to 34 dB] below the total input power level Pin. Pin the total power into the receiver, whether it is signal or jammer.

The definition of Sub-band/In-channel selectivity is introduced for clarity in the SBFD feasibility study:

- Sub-band/In channel selectivity is the ratio of the interference power in the desired DL to the total input power at the receiver input. The total input power includes both desired signal and the undesired jammers. The selectivity includes the entire receiver all the way through the output of the FFT.

FFT leakage and selectivity

As in FR1, the FFT leakage aspect is not a dominant factor in the UE receiver, and we do not include any factor for this effect.

Jammer frequency and time offset were studied are not significant factors influencing UE-UE interference.

10.7.1.1.2 Transmitter aspects

Inband emissions (co-channel)

RAN4 has decided to use the IBE requirements from 38.101-2 clause 6.4.2.3.4 (power class 3 UE). It is understood these requirements are minimum performance requirements as opposed to typical requirements. RAN4 has agreed to use typical requirements for the UE parameters, however, did not conclude on the typical values so we are using the formulation from the MPS.

It should also be assumed the LO location is in the center of the channel for the purposes of system studies in RAN4. The LO location is important as it allows placement of the image.

Analysis indicates that the IBE interference is higher and dominates the sub-band co-channel selectivity,

Apart from the selectivity, the degradation can be caused by transmitter leakage from the UL sub-band into the DL sub-band. For co-channel case, the leakage was agreed to be modelled using IBE based model. Additionally, the IQ image contribution for the IBE model for co-channel CLI can be ignored for the DUD configuration.

For UE co-channel Tx model, UE IBE in TS 38.101-2 can be used in the feasibility study as shown in Table 10.7.1.1-1. This model consists of three parts, General, IQ image, Carrier leakage. In the system level simulation, the general and IQ image parts shall be considered, while the carrier leakage part can be ignored in the feasibility study. For DUD configuration, the IQ image from the uplink is fully contained in the UL sub-band and does not land in the DL subband, thus the IQ image can also be ignored in the simulation.

Table 10.7.1.1-1: Requirements for in-band emissions in TS 38.101-2 (For Power class 3)

| Parameter description | Unit | Limit (NOTE 1) |  |  | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- |
| General | dB |  |  |  | Any non-allocated (NOTE 2) |
|  |  |  | Output power for FR2-1 | Output Power for FR2-2 |  |
| IQ Image | dB | -25 | > 10 dBm | > 8.1 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -20 | ≤ 10 dBm | ≤ 8.1 dBm |  |
| Carrier leakage | dBc | -25 | > 0 dBm | > -1.9dBm | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -13 dBm ≤ Output power ≤ 0 dBm | -14.9 dBm ≤ Output power ≤ -1.9 dBm |  |

10.7.1.2  UE-UE adjacent channel CLI modeling

Editor's note: This section captures the CLI modeling.

10.7.1.2.1 Receiver aspects

Effect of adjacent channel aggressor UE jammer

We analyzed the design of the FR2-1 receiver. There are multiple considerations in the receiver design … for example residual sideband, reciprocal mixing, integrated phase noise, IM3 distortion, and ADC distortions. The receiver performance is simply modelled as being 34 dB below the total input power level Pin. Pin the total power into the receiver, whether it is signal or jammer.

10.7.1.2.2 Transmitter aspects

ACLR (adjacent channel)

ACLR is one aspect modelled as an interference aspect from a nearby aggressor UE transmitting in an adjacent UL subband. UE ACLR is modeled as [24 dB] at max power, improving 1 dB/dB with backoff up to a maximum of 10 dB of improvement. Therefore, when the backoff is 10 dB, the ACLR is [34] dB. This model is an approximation of the performance of a typical UE.

10.7.2	Summary

Editor's note: This section captures the conclusion of feasibility.

End changed section***********************************************************

Conclusion

In this document, we provided changes to the text proposal on feasibility of FR2 UE aspects for SBFD. The following observations and proposals were made:

- AGC is not modelled if a fixed NF is used as a function of the input power.

- Consider the change to the text proposal to TR 38.858.

- References

- RP-223041, Revised SID: Study on evolution of NR duplex operation, CMCC, RAN #98-e, Electronic Meeting, December 12-16, 2022.

- R4-2305970, TP to TR 38.858 on Feasibility of FR1 UE aspects, MediaTek Inc., Online, RAN4 #106-bis-e, April 2023.

- R4-2308632, Feasibility of FR1 UE aspects with TP to TR 38.858, Nokia, Nokia Shanghai Bell