3GPP TSG-RAN WG4 Meeting #107	R4-2308655

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on RRM test cases for SDT

Source: 	Huawei, HiSilicon

Agenda item:	5.2.7.2

Document for:	Discussion

Introduction

RRM test cases for SDT are discussed in RAN4#106, and two CRs [1-2] are agreed. Based on the two CRs, the power levels for different time periods (T1 to T5) in the TC are finalized, but the time interval between different time points (TA to TK) and resulted durations of time periods are still TBD.

In this paper we will provide our views on the remaining issues in SDT RRM TC.

Discussion

Figure 1 is an illustration of the test setup copied from the latest 38.133.

Figure 1: illustration of the test setup from 38.133

The parameters that impacts the time interval between different time points (TA to TK) include

- DRX cycle: it is 640ms in FR1 TC and 1.28s in FR2 TC, and we suggest to align it to 640ms

- CG-SDT periodicity: it is 640ms in FR1 TC and 40ms in FR2 TC, and we suggest to align it to 320ms

- SMTC periodicity: it is 20ms in both FR1 and FR2 TC, but since the W1 and W2 are 480ms for FR2, it is better to make sure the measurement time based on CONNECTED requirements are smaller than this window as discussed next, we suggest to 10ms SMTC period for FR2.

Proposal 1: Adopt the following setup for the test cases.

- DRX cycle: 640ms for FR1 and FR2

- CG-SDT periodicity: 320ms for FR1 and FR2

- SMTC periodicity: 20ms for FR1 and 10ms for FR2

The resulted serving cell measurement time is

- 640ms for FR1 and 3200ms for FR2 in INACTIVE based on requirements in cl. 4.2.2.2

- 200ms for FR1 and 400ms for FR2 in CONNECTED based on requirements in cl. 9.2.5.2

The resulted RSRP measurement window (W1 and W2) is

- 640ms for FR1 and 480ms for FR2 based on requirements in cl. 5.5.3.

TA to TB

We do not see strong restriction for this time interval since UE is already in CONNECTED mode before TA. We suggest it to be 100ms.

TB to TC

Interval between TB and TC must be larger than W1 because UE would consider any measurement completed in this interval as valid RSRP1.

In last meeting, there were some discussion on the case where UE may complete the measurement within TB-TC in Figure 1 but some of the filtered samples are obtained before TB. In this case, UE still considers RSRP result as valid RSRP1 but it may be different from P1 which is the intended power level for RSRP1. As a result, in FR2 TC parameter Y is added and the value is FFS.

We think it is a valid concern, but since UE anyway has the option to measure RSRP after receiving the RRC release message, i.e. after TC, we do not see strong need to extend the interval between TB and TC. Besides, if we follow the same logic, the interval between TE and TF, TG and TH, TI and TG all need to be extended based on measurement time in INACTIVE, which will make the test time long and test design complex. Therefore, we suggest to keep the interval between TB and TC as W1.

TC to TD

Interval between TC and TD must be larger than W1 because UE would consider any measurement completed in this interval as valid RSRP1. The assumption is that UE would follow CONNECTED requirements for measuring RSRP1, and since the measurement time is less than W1, there is no need to extend the interval beyond W1.

TD to TE

There is no explicit restriction on the interval between TD and TE, so we suggest to set it to 40ms.

An implicit restriction is that start of timer T_delay_modeB must be before TC and the duration of the timer can only be set in seconds. With the interval between TD and TE set to 40ms, the test equipment should start timer T_delay_modeB

- at 60ms after TA with duration of 2s for FR1

- at TC with duration of 1s for FR2

TE to TF

Interval between TE and TF must be larger than W2 because UE would consider any measurement completed in this interval as valid RSRP2.

TF to TG

As discussed in earlier meetings, UE may need to perform a new round of measurements for TA validation after UL data arrives at lower layer. This means W3 should include two parts:

- Z which is the measurement and processing time between UL data arrival and TA validation, and

- 640ms which is the allowed time between TA validation and CG-SDT transmission

For Z we suggest to re-use the measurement time in CONNECTED plus [20]ms processing time. This leads to the interval between TF and TG (W3) as 860ms and 1060ms for FR1 and FR2.

TG to TH

The power level is constant as P2 since TE. Interval between TE and TG is 1500ms for FR1 and 1580ms for FR2 which is larger than W1, so we do not see strong restriction for this time interval. A correct UE will consider P2 for new RSRP1 and will not fail the test. We suggest the interval to be 100ms.

TH to TI

Interval between TH and TI must be larger than W1 because UE would consider any measurement completed in this interval as valid new RSRP1. The assumption is that UE would follow CONNECTED requirements for measuring new RSRP1, and since the measurement time is less than W1, there is no need to extend the interval beyond W1.

TI to TJ

Interval between TI and TJ must be larger than W2 because UE would consider any measurement completed within W2 before TJ as valid new RSRP2. Assuming the second UL data is triggered at TH, i.e. the timer of T_delay_modeB is restarted at TH (this is pending on RAN5 conclusion), interval between TH and TJ needs to be integer second because timer T_delay_modeB can only be set in seconds. Since TH to TI is 640ms for FR1 and 480ms for FR2, we suggest to use 2s timer for FR1 and 1s timer for FR2. This leads to interval between TI and TJ as 1360ms for FR1 and 520ms for FR2.

TJ to TK

Technically the interval between TJ and TK should be infinite, but we suggest to re-use the interval between TF and TG assuming UE still transmits CG-SDT within 640ms after TA validation when it incorrectly determines TA is valid.

Summary

Based on discussions in section 2.1 to 2.10, we have the following proposal.

Proposal 2: Adopt the following time intervals between different time points (TA to TK):

- TA to TB: 100ms

- TB to TC: 640ms for FR1, 480ms for FR2

- TC to TD: 640ms for FR1, 480ms for FR2

- TD to TE: 40ms

- TE to TF: 640ms for FR1, 480ms for FR2

- TF to TG: 860ms for FR1, 1060ms for FR2

- TG to TH: 100ms

- TH to TI: 640ms for FR1, 480ms for FR2

- TI to TJ: 1360ms for FR1, 520ms for FR2

- TJ to TK: 860ms for FR1, 1060ms for FR2

Conclusions

In this paper we provided our views on remaining issues in SDT RRM TC.

Proposal 1: Adopt the following setup for the test cases.

- DRX cycle: 640ms for FR1 and FR2

- CG-SDT periodicity: 320ms for FR1 and FR2

- SMTC periodicity: 20ms for FR1 and 10ms for FR2

Proposal 2: Adopt the following time intervals between different time points (TA to TK):

- TA to TB: 100ms

- TB to TC: 640ms for FR1, 480ms for FR2

- TC to TD: 640ms for FR1, 480ms for FR2

- TD to TE: 40ms

- TE to TF: 640ms for FR1, 480ms for FR2

- TF to TG: 860ms for FR1, 1060ms for FR2

- TG to TH: 100ms

- TH to TI: 640ms for FR1, 480ms for FR2

- TI to TJ: 1360ms for FR1, 520ms for FR2

- TJ to TK: 860ms for FR1, 1060ms for FR2

Reference

- R4-2303160, CR correction of SDT test cases, Nokia, Nokia Shanghai Bell

- R4-2303298, CR on SDT RRM test case for FR2, Huawei, HiSilicon