3GPP TSG-RAN WG4 Meeting #107	R4-2308661

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on inter-RAT MG-less measurement

Source: 	Huawei, HiSilicon

Agenda item:	8.10.3.2

Document for:	Discussion

Introduction

RRM requirements for inter-RAT MG-less measurement are discussed in RAN4#106-bis-e and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Using scenario

- UE capability

- General principle

- Measurement period

- Scheduling restriction

In this paper we will provide our views on RRM requirements for inter-RAT MG-less measurement.

Discussion

Using scenario

Issue 2-1-1: Differentiate scenarios for Case b-2 < Way forward/Agreement >: - FFS on whether to differentiate the following scenarios in case b-2. - Scenario 1: Inter-RAT LTE measurement for LTE CRS rate-matching feature - Scenario 2: Inter-RAT LTE measurement for CRS-IM receiver

In our view, having LTE channel contained in active BWP of NR serving cell is already a sufficient condition for measurement without MG, and the other condition on DSS is not needed. Also, “UE is in DSS” is a bit vague, and what it means e.g. in terms of configuration, should be further clarified. Option 2b is not supported because scaling due to multiple layer measurement is not considered. It should be noted that in Rel-16 inter-frequency without MG, the inter-frequency MO within active BWP of serving cell are still accounted in CSSF outside MG.

Proposal 1: RAN4 not to differentiate sub-scenarios in Case b-2.

UE capability

Issue 2-2-2: UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2) < Way forward >: - A new per-UE capability to support Case b-2 should be defined. - FFS on indication of “no gap with interruption” because of AGC,T/F offset issue Issue 2-6-1: Interruption requirements for inter-RAT measurement without gap < Way forward/Agreement >: - Option 1: - interruption requirements can be defined - for case a-1 and case b-1, - Option 1a: - interruption requirements can be defined - for case a-1 and case b-1, - FFS for case b-2 - Option 1b: - Interruption requirements can be defined - for inter-RAT NR measurements without gap(case a-1)

For Case b-2, we assume it is similar to Rel-16 inter-frequency measurement without MG. As no RF re-tuning is needed to perform the measurement, no interruption is needed.

Proposal 2: Measurement with interruption is not considered for Case b-2.

General principle

Issue 2-3-2: Searcher limitation < Way forward/Agreement >: - Option 1: - Inter-RAT LTE measurement without gap(case b-2) can be performed in parallel with NR measurement without searcher limitation - Option 2: - Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported

In option 1, some companies suggest that UE can do parallel measurement for LTE and NR MOs and as such the CSSF for inter-RAT LTE measurement would equal to the number of LTE MOs. We do not support this assumption. In our view, LTE and NR measurements could share the same memory and computation resources, so even LTE measurement can be taken outside SMTC window for NR measurement, it does not mean LTE measurement will have no impact on NR measurement or they can be done in parallel.

When RAN4 defined requirements for inter-frequency without gap in Rel-16, CSSF outside MG is defined as the total number of intra- and inter-frequency MOs to be measured outside MG. Following the same principle, when inter-RAT LTE measurement is considered, the CSSF should be further updated by considering all MOs to be measured outside MG.

Proposal 3: Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported.

Measurement period

Issue 2-4-2: Measurement period requirements for case a-1 with “no gap but interruption allowed” < Way forward/Agreement >: - FFS on measurement period requirements for case a-1 with “no gap but interruption allowed” Issue 2-4-3: Measurement period requirements for case a-1 with “no gap no interruption” < Way forward/Agreement >: - FFS on measurement period requirements for case a-1 with “no gap no interruption”

For inter-RAT NR measurement without gap, it is straightforward to follow requirements for inter-frequency NR measurement based on NFG. Looking at the existing requirements for inter-RAT NR measurement in cl. 8.1.2.4 of 36.133, one needs to adapt the measurement cycle from Max(MGRP, SMTC period) to SMTC period when no interruption is expected, or to TmeasCycle when interruption is expected. Definition of TmeasCycle is discussed in our companion paper for NFG.

In addition, Nfreq should be defined to account for total number of LTE and NR MOs measured outside MG, and Kp should be defined for account for SMTC occasions punctured by MG.

Proposal 4: For Case a-1, adopt the following updates based on the existing requirements for inter-RAT NR measurement

- Max(MGRP, SMTC period) is changed to TmeasCycle for “no gap but interruption allowed”, and changed to SMTC period for “no gap no interruption”

- Nfreq is defined as the total number of LTE and NR MOs measured outside MG

- Kp is defined to account for SMTC occasions punctured by MG.

Issue 2-4-5: Measurement period requirements for case b-1 no gap no interruption allowed < Way forward/Agreement >: - The measurement requirement for case b-1 and case b-2 can be defined by the framework below. TIdentify, interRAT_EUTRAN_gapless=TBasicIdentifya??480TInter1a??CSSFinterRAT msTIdentify, interRAT_EUTRAN_gapless=TBasicIdentify*480TInter1*CSSFinterRAT ms - FFS on update definition of TBasicIdentify,Tinter1 and CSSFinterRAT Issue 2-4-7: Measurement period requirements for case b-2 no gap no interruption allowed < Way forward/Agreement >: Leverage the same agreements from issue 2-4-5. - The measurement requirement for case b-1 and case b-2 can be defined by the framework below. TIdentify, interRAT_EUTRAN_gapless=TBasicIdentify*480TInter1*CSSFinterRAT ms, - FFS on update definition of TBasicIdentify,Tinter1 and CSSFinterRAT

For inter-RAT LTE measurement without gap, we suggest to follow the existing LTE inter-frequency requirements without gaps as baseline.

A UE that is capable of identifying and measuring inter-frequency and/or inter-RAT cells without gaps shall follow requirements as if Gap Pattern Id #0 had been used and the minimum available time Tinter1 of 60 ms shall be assumed for the corresponding requirements.

In LTE inter-frequency requirements without gaps, TBasicIdentify = 480ms, and Tinter1 = 60ms as derived based on 40ms effective MGRP. These two definitions can be re-used for inter-RAT measurement. Nfreq is the total number of LTE MOs, and for inter-RAT measurement, CSSFinterRAT is used and it should account for the total number of LTE and NR MOs measured outside MG.

Proposal 5: For case b-1 and b-2,

- TBasicIdentify = 480ms

- Tinter1 = 60ms, derived based on 40ms effective MGRP

- CSSFinterRAT = total number of LTE and NR MOs measured outside MG

Scheduling restriction

Issue 2-5-2: Scheduling restriction for inter-RAT LTE measurements < Way forward/Agreement >: - Scheduling restriction due to inter-RAT LTE measurement are applicable when - UE does not support simultaneous Tx and Rx on the serving cell and target band - FFS on Serving cell and target MO have mixed SCS and they are in the same band

For NE inter-frequency measurement without MG, either when inter-frequency SSB is within UE active BWP, or when UE has vacant RF chain for the inter-frequency layer, scheduling restriction is allowed for the case when serving cell and target MO have mixed SCS and are in the same band, when UE does not support mixed SCS, i.e. simultaneousRxDataSSB-DiffNumerology.

For inter-RAT LTE measurement, the SCS of target MO is always 15kHz, but the serving cell SCS can be same or different. We suggest to follow the same principle as NR inter-frequency measurement, i.e. to allow scheduling restriction based on UE capability.

Proposal 6: Define scheduling restriction for Case b-1 and b-2 when

- serving cell and target MO have mixed SCS and they are in the same band, and

- UE does not support mixed SCS between serving cell and target MO

Issue 2-4-9: Effiective measurement window < Way forward >: - Option 1: vivo, MTK, Huawei, Ericsson - RAN4 to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap. - Option 1a: Ericsson - The effective measurement window can be defined based on measurement duration, measurement periodicity and offset. - Option 1b: MTK - FFS on the overlap between NR SMTC and LTE SMTC-lte (i.e. the effective measurement window).

RAN4 also needs to discuss where the scheduling restriction is located. Unlike NR measurement which is based on SMTC, LTE measurement can be taken any time, and in order to regulate the scheduling restriction location, we think it is meaningful to define effective measurement window. The periodicity and duration of the window can be based on measurement requirements e.g. 40ms period and 6ms duration if inter-RAT LTE requirements are to be defined based on gap pattern 0. The open part is the offset of the window, and we think it can be either fixed in the spec or configured by the NW.

Proposal 7: Define effective measurement window to restrict the location of scheduling restriction due to inter-RAT LTE measurement.

Conclusions

In this paper we provided our views on RRM requirements for inter-RAT MG-less measurement.

Proposal 1: RAN4 not to differentiate sub-scenarios in Case b-2.

Proposal 2: Measurement with interruption is not considered for Case b-2.

Proposal 3: Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported.

Proposal 4: For Case a-1, adopt the following updates based on the existing requirements for inter-RAT NR measurement

- Max(MGRP, SMTC period) is changed to TmeasCycle for “no gap but interruption allowed”, and changed to SMTC period for “no gap no interruption”

- Nfreq is defined as the total number of LTE and NR MOs measured outside MG

- Kp is defined to account for SMTC occasions punctured by MG.

Proposal 5: For case b-1 and b-2,

- TBasicIdentify = 480ms

- Tinter1 = 60ms, derived based on 40ms effective MGRP

- CSSFinterRAT = total number of LTE and NR MOs measured outside MG

Proposal 6: Define scheduling restriction for Case b-1 and b-2 when

- serving cell and target MO have mixed SCS and they are in the same band, and

- UE does not support mixed SCS between serving cell and target MO

Proposal 7: Define effective measurement window to restrict the location of scheduling restriction due to inter-RAT LTE measurement.

Reference

- R4-2306331, WF on NR_MG_enh2 Part 2, Intel Corporation