3GPP TSG-RAN WG4 Meeting #107	R4-2308674

Incheon, KR, 22 – 26 May, 2023

Title: 	Discussion on option A for BWP without restriction

Source: 	Huawei, HiSilicon

Agenda item:	8.11.2.1

Document for:	Discussion

Introduction

RRM requirements for option A for BWP without restriction are discussed in RAN4#106-bis-e and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Clarification on CSI-RS based RLM/BFD/BM requirements

- Clarification on existing timing requirements

In this paper we will provide our views on RRM requirements for option A for BWP without restriction.

Discussion

Clarification on CSI-RS based RLM/BFD/BM requirements

Issue 2-1: Any clarification on CSI-RS based RLM/BFD/BM requirements for supporting Option A <Agreement >: - In general, no specification impact on RLM/BFD/BM requirements for UE supporting option A for BWP operation without restriction. - FFS if there is impact on FR2 CSI-RS base RLM requirements specifically.

The FFS bullet was raised by some companies during the first round discussion last meeting.

One issue is related to the following requirement on P factor.

- P=1, when the RLM-RS resource is not overlapped with measurement gap and also not overlapped with SMTC occasion.

The requirement is introduced in Rel-15, and the principle is that

- when RLM-RS collides with MG, the RLM-RS occasion is dropped

- when RLM-RS outside MG partially collides with SMTC, the RLM-RS occasion is dropped

- when RLM-RS outside MG fully collides with SMTC, RLM and L3 will 1:2 share the occasions

We do not see any issue in applying the requirements for BWP without restriction with option A. RLM-RS is the CSI-RS within active BWP, and L3 measurement is performed with MG on SSB outside active BWP.

Another issue is the UE capability on CSI-RS based RLM/BFD/BM. The issue was separately discussed in issue 6.2 in [1], and it was concluded that UE capability for Option A will be discussed in RAN1 and/or RAN2. Therefore, we do not see further clarification needed in RAN4 spec.

Proposal 1: RAN4 not to make further clarification for FR2 CSI-RS base RLM requirements for option A.

Clarification on existing timing requirements

Issue 2-2: Any clarification on existing timing requirements when CD-SSB is outside active BWP <Way forward >: - Proposals - Option 1: (Ericsson, Apple, Nokia, Intel, Huawei, CATT, MediaTek, OPPO) - No clarifications on existing timing requirements are needed. - Option 2: (vivo) - It is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not. - A note is added for timing requirements that when SSB is outside active BWP, availability of SSB is at least relevant to configuration of measurement gap, number of measurement objects and gap sharing factor. - Option 3: (Ericsson, Intel) - The condition to configure gaps to meet the existing UE transmission timing error requirements in clause 7.1 of TS 38.133, when the UE is performing BM/RLM/BFD based on option A, is NOT needed. - A possible compromise is to clarify in clause 7.1.2 of TS 38.133, that the availability of the SSB at the UE is for the purpose of acquiring the timing of the reference cell

The current UE timing requirements are defined based on the assumption that SSB is used for time tracking. The question is then whether and how UE could meet the timing requirements when no SSB is contained in the active BWP. Two options have been discussed in earlier RAN4 meeting:

- Option 1: UE uses TRS for time tracking

- Option 2: UE uses MG to access SSB outside active BWP for time tracking

- Option 3: UE access SSB outside active BWP without MG for time tracking

In our view, all the options can work, and which option to use can depend on NW and UE implementation.

It is true that the current UE timing requirements are defined based on SSB, but it does not mean other signals cannot be used for time tracking and enable UE to meet the current requirements. Based on cl. 5.1.6.1.1 of 38.214, the BW of TRS is larger than SSB and the periodicity of TRS is no larger than 80ms. As such we do not see any issue for UE to meet the current timing requirements based on TRS.

Option 2 and 3 are also feasible. Option 2 requires NW to configure a MG for UE to meet the timing requirements. For RedCap this may not be an issue as in most cases UE would need MG to perform intra-frequency measurement, but for normal UE this may be unnecessary. For example, some UEs may support intra-frequency measurement on SSB outside BWP with NCSG/interruption or without any interruption, and this leads to option 3.

For option 1, based on cl. 5.1.6.1.1 of 38.214, NW would always configure TRS to UE.

A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.

For option 2 and 3, NW would ensure UE can perform intra-frequency measurement for the serving cell, with or without configuring MG, depending on UE capability, and this is up to NW implementation.

As such, we think the spec support for option A is complete, and no additional RAN4 work is needed.

Proposal 2: RAN4 to confirm that spec support for option A is complete, and no additional RAN4 work is needed.

Conclusions

In this paper we provided our views on RRM requirements for option A for BWP without restriction.

Proposal 1: RAN4 not to make further clarification for FR2 CSI-RS base RLM requirements for option A.

Proposal 2: RAN4 to confirm that spec support for option A is complete, and no additional RAN4 work is needed.

Reference

- R4-2306397, WF on completion of specification support for BWP operation without restriction, vivo