3GPP TSG-RAN WG4 Meeting #107	R4-2308705

Incheon, KR, 22th – 26th May, 2023

Title: 	Discussion on UL timing adjustment for R18 FR2 HST

Source: 	Huawei, HiSilicon

Agenda item:	8.13.4.3

Document for:	Discussion

Introduction

In this contribution, we will provide the discussion on UL timing adjustment solution for R18 FR2 HST.

Discussion

MAC-CE based solution for cross-RRH TCI state switch

In RAN4#106bis-e meeting, RAN4 achieved the following agreements on UL transmit timing adjustment for NR FR2 HST:

Issue 1-1-1: MAC-CE signalling GtW Agreement: - Introduce MAC-CE based solution with 1bit indication to inform UE on the TCI state switch across RRHs Issue 1-1-2: Information indicated in MAC-CE Agreement: - Introduce 1-bit TCI State Indication in UE-specific PDCCH MAC CE for whether or not UE shall follow the Rel-17 UL timing solution for the indicated TCI state ID. - FFS in RAN4, UE behaviour after receiving the 1-bit indication. Issue 1-1-3: Impact of signalling on RACH-based timing adjustment procedure Way forward: Open issue needs further discussion - Option 1: Agreed signalling can be used to trigger RACH procedure also if RACH procedure is used for timing adjustment at cross-RRH TCI state switch. - Option 2: Do not take signalling into account for triggering RACH procedure

RAN4 agreed to introduce 1-bit indication in MAC-CE to inform UE on the TCI state switch across RRHs. The UE behaviour after receiving the 1-bit indication need to be discussed, i.e. whether or not UE shall follow the Rel-17 UL timing solution for the indicated TCI state switching.

In Rel-17, the UL timing adjustment requirements at TCI state switch for FR2 HST in TS38.133 are defined as follows:

- 7.1.2.3 One shot large UL timing adjustment for FR2 Power Class 6 UE When highSpeedMeasFlagFR2-r17 is configured and highSpeedLargeOneStepUL-TimingFR2-r17 is enabled for UE supporting FR2 power class 6 and [largeOneStepUL-timingFR2-r17] capability, the following requirements apply to the UE: - If the absolute value Told-Tnew≤CP/4, the requirement in clause 7.1.2.1 apply to the first UL transmission after a TCI state switch. - Otherwise, the UE transmit timing immediately after TCI state switch shall be Tnew-(NTA+NTA offset)+2 (Told-Tnew) and clause 7.1.2.1 requirements don’t apply. - The UE UL transmission timing error after the TCI state switching procedure shall be less than or equal to ±Te as specified in clause 7.1.2 if the new target TCI state is within active TCI state list, otherwise ±7*64*Tc, and the reference point is Tnew-(NTA+NTA offset)+2 (Told-Tnew). Above, - Tnew (in Tc units) is the DL timing defined as the time when UE receives downlink frame with new target TCI state. - Told (in Tc units) is the DL timing defined as the time when UE receives downlink frame with old source TCI state.

UE determines whether to perform one-shot large UL timing adjustment on the first UL transmission after TCI state switching according to the DL timing change caused by TCI state switching. If the DL timing change is above the threshold, UE performs one-shot large UL timing adjustment on the first UL transmission after TCI state switching, otherwise, UE performs gradual UL timing adjustment on the first UL transmission after TCI state switching.

In R18, based on the agreed MAC-CE based solution, if the UE does not receive the 1-bit indication for cross-RRHs TCI state switching, i.e. intra-RRH TCI state switching, then UE assume the DL timing change caused by TCI state switching will not be large, and UE performs gradual UL timing adjustment after TCI state switching. When UE receives the 1-bit indication for cross-RRHs TCI state switching, it does not mean there will always be a large DL timing change. For example, when UE move to the middle of two RRHs, the propagation delay between two RRHs can be assumed to be very close, and the TCI state switching between two RRHs will not cause a large DL timing change as UE side. So, UE still needs to whether to perform one-shot large UL timing adjustment on the first UL transmission after TCI state switching according to the DL timing change caused by TCI state switching.

Proposal 1: For Rel-18 FR2 HST scenario, if UE receives the 1-bit indication for the TCI state switch across RRHs, the existing R17 UL timing adjustment requirements in clause 7.1.2.3 apply to the first UL transmission after TCI state switch, otherwise the gradual timing adjustment requirements in clause 7.1.2.1 apply to the first UL transmission after TCI state switch.

In RAN4, whether the 1-bit indication for cross-RRHs TCI state switching can be used to trigger RACH procedure was discussed. Usually, RACH procedure is triggered when UE needs to access a cell or when beam failure is detected, and the network needs to estimate the RTT for indicating a TA. However, TCI state switching will not occur when no link problem is detected. Besides, no matter gradual timing adjustment or one-shot timing adjustment is applied for TCI state switching, both belong to UE autonomous timing adjustment which would not lead to a link problem. Hence, RACH procedure is not needed when UE performs timing adjustment at TCI state switching.

Proposal 2: For Rel-18 FR2 HST scenario, there is no need to take the 1-bit indication for TCI state switch into account for triggering RACH procedure.

UL transmit timing adjustment at UL spatial relation switch

For FR2 HST scenario, UE can perform UL spatial relation switch between RRHs. RAN4 discussed whether to define UL transmit timing adjustment at UL spatial relation, and the following options are captured in [1].

- FFS the scenario in which UL timing exceed allowed timing adjustment range at UL spatial relation: - Option 1: Apply existing one-shot larger UL timing adjustment mechanism (Clause 7.1.2.3) at UL spatial relation switch - Option 2: UL spatial relation shall always be executed strictly when corresponding DL TCI state switches - Option 3: The existing gradual timing adjustment requirements can be applied, and there is no need to define additional UL transmit timing adjustment.

For Rel-18 FR2 HST, multi-panel reception is considered to be supported at UE side. However, supporting of multi-panel reception does not mean to support multi-panel transmission. Based on RAN4’s previous agreements, UE is still assumed to perform UL transmissions with single beam direction. The UE needs to perform TDM-ed UL transmissions to two RRHs with different beam directions.

Figure 1: Example for simultaneous multi-panel receptions from two RRHs

As shown in Figure 1, UE capable of simultaneous multi-panel reception can receive signals from both RRH with different beam directions, but UE only can perform UL transmissions to one RRH at the same time. UE can perform UL spatial relation switch between RRH1 and RRH2. However, UL spatial relation switch only indicates the beam direction change of uplink transmission. There is no TRS indicated in UL spatial relation information. The uplink transmit timing is derived from the detected downlink timing minus a TA value. However, it has not been specified from which RRH the downlink timing is to be used as the reference timing for the uplink transmission timing. It is up to UE implementation to select which RRH downlink timing as reference timing. If UE performs UL spatial relation switch from RRH1 to RRH2, there could be no timing change when the downlink timing for two RRHs are the same. Even there has timing change at UL spatial relation switch, the existing gradual timing adjustment requirements can be applied. There is no need to define additional UL transmit timing adjustment requirements for UL spatial relation switch.

Proposal 3: For UL timing change at UL spatial relation switch, the existing gradual timing adjustment requirements can be applied, and there is no need to define additional UL transmit timing adjustment.

Other aspects

In RAN4, it was mentioned that the potential impacts of large jump in propagation delay on UE MAC timeAlignmentTimer in the case of inter-RRH TCI state switch need to be further studied.

One shot large UL timing adjustment belongs to UE autonomous timing adjustment. UE performs autonomous timing adjustment without receiving TA command. After one shot large UL timing adjustment, UE still needs to perform gradual timing adjustment. Then, the network can detect UL receive timing change slowly without large jump. The IE timeAlignmentTimer controls how long the MAC entity considers the serving cells belonging to the associated TAG to be uplink time aligned. The magnitude of UE autonomous timing adjustments, including both gradual timing adjustment and one-shot large UL timing adjustment, is still very limited and within CP length for most cases. The network shall be able to following the uplink timing change. We do not observe the obvious impact on TA adjustment and timeAlignmentTimer.

Proposal 4: There is no need to introduce timeAlignemntTimer enhancements at UL timing adjustment, since no obvious impact on TA adjustment and timeAlignmentTimer due to one-shot large UL timing adjustment is observed.

Conclusions

In this paper we provided our analysis on UL timing adjustment solution for R18 FR2 HST. The followings are provided.

Proposal 1: For Rel-18 FR2 HST scenario, if UE receives the 1-bit indication for the TCI state switch across RRHs, the existing R17 UL timing adjustment requirements in clause 7.1.2.3 apply to the first UL transmission after TCI state switch, otherwise the gradual timing adjustment requirements in clause 7.1.2.1 apply to the first UL transmission after TCI state switch.

Proposal 2: For Rel-18 FR2 HST scenario, there is no need to take the 1-bit indication for TCI state switch into account for triggering RACH procedure.

Proposal 3: For UL timing change at UL spatial relation switch, the existing gradual timing adjustment requirements can be applied, and there is no need to define additional UL transmit timing adjustment.

Proposal 4: There is no need to introduce timeAlignemntTimer enhancements at UL timing adjustment, since no obvious impact on TA adjustment and timeAlignmentTimer due to one-shot large UL timing adjustment is observed.

Reference

- R4-2306398, “WF on NR FR2 HST UL Timing Adjustment Solutions and Tunnel Deployment”, Nokia, Nokia Shanghai Bell.