3GPP TSG-RAN WG4 Meeting #107	R4-2308730

Inchon, Korea, 22 – 26 May, 2023

Source:	ZTE Corporation

Title:	Discussion on RRM aspects of Network energy saving for NR

Agenda Item:	8.35.4

Document for:	Approval

- Introduction

In RAN plenary #98 meeting, a new WID[1] for Rel-18 was approved to specify network energy saving techniques for NR. The related SI phase has been finished[2] and during SI phase no RAN4 aspects was studied. While during the WI phase, some RAN4 aspects are considered, the objectives of the core part of the WID are listed as follows. The objectives related to RAN4 were highlighted.

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3] - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

During the SI phase, it has been concluded that the power consumption of radio access network can be split into two parts: the dynamic part which is only consumed when data transmission/reception is ongoing, and the static part which is consumed all the time to maintain the necessary operation of the radio access devices, even when the data transmission/reception is not on-going. As a result, the network energy saving has been investigated through time, frequency, spatial and power domains. One important technique from the time domain perspective is SSB-less operation. In term of SSB-less operation, intra-band SSB-less SCell has been supported by the current specification via scellWithoutSSB. Furthermore, SSB-less SCell for inter-band CA is referred as a possible frequency domain technique in [2]. Therefore, during this WI phase, from RAN4 perspective, the feasibility of SSB-less SCell for inter-band CA to derive the SCell’s time/frequency synchronization should firstly be verified. Then after the verification achieved, some other potential enhancement can be further discussed.

During last meeting, warm discussion was held and the following agreements were achieved in [3].

Issue 2-1-1: Which scenario to be considered for SSB-Less SCell operation for FR1 inter-band collocated CA - Agreements - The following scenarios are considered for inter-band SSB-less SCell requirements definition - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell. - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis Issue 2-1-2: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-1? Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-1 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a Issue 2-1-3: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-2? Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-2 - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a Issue 2-1-4: From RRM perspective from UE side, under which conditions SSB-less SCell operation can work in scenario-2a? Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-2a - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a Issue 2-3-1: Requirements for SSB-less SCell activation Agreements - SSB-less based SCell activation delay is to be specified for inter-band CA for FR1 and co-located cells based on feasibility study. Issue 3-1: RRM impacts due to Cell DTX/DRX (obj#2) Agreements - RRM impacts due to cell DTX can wait for more inputs from RAN1/2. Issue 3-2: RRM impacts due to Spatial and power domain techniques (obj#3) Agreements - Discuss the RRM requirement for spatial and power adaptation in NES after RAN1 has more concrete conclusions. Issue 3-3: RRM impacts due to Cell selection/reselection (obj#4) Agreements - wait further RAN2 progress Issue 3-4: RRM impacts due to CHO (Obj#5) Agreements - RRM impact due to CHO enhancement in NES mode can wait for more inputs from RAN2.

In this document, we provide some analysis on the SSB-less operation for inter-band CA in FR1 co-located deployment.

For SSB-less operation, we provide our views from the following aspects:

- The discussion on the scenario

- The discussion on the feasibility

- The discussion on the side condition

- The impact on the RRM requirements

- Regular L1/L3 measurements

- Discussion

2.1 The scenarios of SSB-less

Around the SSB-less operation for inter-band CA in FR1 co-located deployment, after the discussion in last meeting, three scenarios were identified to be considered:

- Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell.

Here the Scenario 2a is proposed by us, the motivation of Scenario 2a lies in the following aspects:

- There is really use case demand. Multiple UL heavier use cases were identified in NR, including remote driving, machine vision and factory vedio surveillance. Under these use cases, UL traffic is much more heavier with quite limited downlink transmission. So the SSB-less SCell can be mainly used for the UL traffic supplementation.

- From the perspective of NW power saving, for TDD, the NW power consumption for DL and UL are separately modeled, according to the relative power values statistics of Table 5.1-3 in [4], it can be shown that the power consumption of Active UL is obviously much less than that of Active DL. For FDD, it is summarized as “For simultaneous DL and UL transmission for FDD, the power for UL reception is neglected in this study” in Chapter 5.1 of [4].

Table 5.1-3: Relative power values P for reference configuration Set 1, Set 2 and Set 3

| Power state | BS Category 1 |  |  | BS Category 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Set 1 | Set 2 | Set 3 | Set 1 | Set 2 | Set 3 |
| Deep sleep | 1 |  |  | 1 |  |  |
| Light sleep | 25 |  |  | 2.1 |  |  |
| Micro sleep | 55 | 50 | 38 | 5.5 | 5 | 3 |
| Active DL | 280 | 200 | 152 | 32 | 26 | 17.6 |
| Active UL | 110 | 90 | 80 | 6.5 | 5.8 | 4.2 |

Therefore, shutting down the NW DL transmitter at the SSB-less SCell can achieve significant power saving. Based on the above analysis, Scenario 2a is beneficial to achieve the significant NW power saving based on the SSB-less SCell operation, which should be supported in this WI. When we analysis the feasibility and detailed side conditions, Scenario 2a should be considered.

Observation 1: Multiple UL heavier use cases were identified in NR, including remote driving, machine vision and factory vedio surveillance. Under these use cases, the SSB-less SCell can be mainly used for the UL traffic supplementation.

Observation 2: According to the study in SI status, no matter for TDD or FDD, the NW power consumption in active DL is much severer than active UL.

Proposal 1: By shutting down the NW DL transmitter at the SSB-less SCell, Scenario 2a is beneficial to achieve the significant NW power saving effect.

Regarding other two scenarios, we believe something needs to be further clarified.

For Scenario 1, no SSB but with TRS, here the TRS should be at least P-TRS, since according to Chapter 5.1.6.1.1 in [5], for the configuration of TRS, two cases are supported: 1) Only configuring P-TRS; 2) Configuring both P-TRS and A-TRS. For the latter case, the configured P-TRS and A-TRS should locate in the same PRB and guarantee QCL-Type A relation for FR1. So either only P-TRS is invloved in Scenario 1 or both P-TRS and A-TRS are involved in Scenario 1. It should be noted that the frequency bandwith of TRS is not smaller than 28 PRBs or not smaller than 32 PRBs or equals to min{52, NBWP,isize} PRSs, so it is much wider than SSB in frequency domain. While regarding the time domain configuration, either two symbols in a slot or four symbols in two contiguous slots for each occasion, the period of occasions is slots where 10, 20, 40 or 80. Therefore, comparing with the Scenario 1 and Scenario 2, since without the P-TRS, Scenario 2 can achieve better NW power saving effect than Scenario 1.

Observation 3: For the sake of NW power saving, the always-on signals should be avoided. While based on RAN1 specification, at least P-TRS is involved in Scenario 1.

So based on the above analysis, from the perspective of NW power saving effect, Scenario 2a is better than Scenario 2, and Scenario 2 is better than Scenario 1.

Proposal 2: From the perspective of NW power saving effect, Scenario 2a is better than Scenario 2, and Scenario 2 is better than Scenario 1.

2.2 The feasibility of SSB-less

For co-located FR1 inter-band CA deployment, Scenario 1 and 2 are similar with the SCellwithoutSSB case in legacy intra-band contiguous CA. The exact frequency range of band combinations can be further identified in RF session, it can be seen that for inter-band CA combinations, the frequency range between two bands can be narrower or comparable than some intra-band contiguous CA. So the SSB-less SCell operation in Scenario 1 and Scenario 2 are feasible.

For Scenario 2a, the logic is more direct compared with Scenario 1/2. Since no DL transmission happens in the SSB-less SCell, so during the SCell activation procedure, the components of AGC, fine T/F tracking and TCI state indication are all skipped. Only the coarse DL time tracking is necessary for the sake of further UL synchronization. Which only demands a subset of the conditions in Scenario 1/2. So the SSB-less SCell operation in Scenario 2 are feasible.

Observation 4: For co-located FR1 inter-band CA deployment, Scenario 1 and 2 are similar with the SCellwithoutSSB case in legacy intra-band contiguous CA. For Scenario 2a, since the components of AGC, fine T/F tracking and TCI state indication can be skipped, so which only demands a subset of conditions of Scenario 1/2.

Proposal 3: Considering the frequency range between two bands can be narrower or comparable than some intra-band contiguous CA, so the SSB-less SCell operation in Scenario 1, 2 and 2a are feasible.

Furthermore, some company proposed to perform the PDCCH/PDSCH performance evaluaion for SSB-less SCell operation. To our understand, for Scenario 1 and 2, since applying similar logic as legacy intra-band CA, so the key points is whether the side conditions such as RTD and power difference can met rather than the PDCCH/PDSCH performance. The validition of side conditions can not be proved by PDCCH/PDSCH performance evaluation, so we can not see the necessity of performing performance evaluation. For Scenario 2a, since no PDSCH/PDCCH transmission, it is far from the performance evaluation.

Proposal 4: For all Scenario 1, 2 and 2a, no need to perform PDCCH/PDSCH performance evaluation.

2.3 The side conditions of SSB-less

For Scenario 1, we believe the side conditions include the following aspects:

- Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell

- The difference of the reception power with the FR1 inter-band active serving cell

- frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations.

Regarding the RTD condition, we prefer to reuse the same demand as in legacy intra-band contiguous CA, i.e. 260ns.

Regarding the difference of the reception power, we still prefer to reuse the same demand as in legacy intra-band contiguous CA, i.e. 6dB.

With respect to the frequency domain separation, which can be further identified in RF session.

Besides all above three side conditions, no need to specify additional condition for the sake of TCI state indication, since the above three side conditions are sufficient to guarantee the reuse of AGC, coarse timing and fine T/F info from an reference cell.

Proposal 5: The side conditions for Scenario 1 include:

- Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell -- 260ns;

- The difference of the reception power with the FR1 inter-band active serving cell -- 6dB;

- Frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC -- further determine in RF session.

For Scenario 2, since no TRS transmission, so whether the fine T/F synchronization can be achieved, which should be confirmed. Comparing the CP length and the candidate RTD, it can be seen that 260ns is much smaller that the CP length for FR1. So in our view, once the side condition of RTD not larger than 260ns can be met, no need to concern on the T/F synchronization, both the coarse timing and fine timing infos can borrow from the reference cell. Based on such analysis, we believe the side conditions above in Scenario 1 can be reused for Scenario 2.

Proposal 6: The side conditions for Scenario 1 can be reused for Scenario 2.

However we noticed some UE vendor refers to the different RX chain assumption between intra-band CA and inter-band CA, so we wonder the exact quantity of the time margin between two Rx chains under inter-band CA case. Under the same side condition of RTD not larger than 260ns, fine T/F synchronization can be achieved given the same Rx chain assumed, while fine T/F synchronization can not be achieved given the independent Rx chains assumed, the key point lies in the time margin between the two Rx chains. So it is better to make sure the magnitude of the time margin, so that we can comprehend the timing issue between intra-band CA and inter-band CA.

Proposal 7: To address UE vendors’ concern on different Rx chain assumptions between intra-band CA and inter-band CA, the time margin between multiple Rx chains can be clarified.

For Scenario 2a, no need to concern any DL transmission, so the side condition is some sub-set of Scenario 1/2. Only considering the received time difference (RTD) between the SSB-less SCell and the reference cell is enough. For details, the RTD should be not larger than 260ns.

Proposal 8: The side conditions for Scenario 2a includes:

- Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell -- 260ns;

2.4 The impact of the RRM requirements of SSB-less

First we want to discuss the RRM impact of Scenario 2a. Under this scenario, no DL transmission from NW, NW only need to receive UL. So the AGC, fine T/F tracking and TCI state indication can all be skipped during the SCell activation procedure. UE only need to obtain the coarse time synchronization of the SSB-less SCell. If the side condition of RTD not larger than 260ns can meet, than reuse the coarse time synchronization of the reference cell is fine. So from the RRM impact perspective, the SCell activation latency is 3 ms for this scenario.

Regarding the RAN1 impact, to our understand, Scenario 2a has no RAN1 impact. Some companies show concerns on the DL synchronization and fine T/F maintaining, UL power determination, we given our analysis during comments reply in last meeting, here we repeat again.

With respect to the DL synchronization and fine T/F maintaining, since no PDSCH/PDCCH in the SSB-less SCell, UE does not need to perform AGC, fine T/F tracking and receiving TCI state indication during the SCell activation. UE only needs to prepare for the UL transmission, i.e. determine the UL transmission timing and UL transmission power for the SSB-less SCell. Regarding the fine T/F tracking, which is used for guaranteeing the PDCCH/PDSCH decoding performance. While if no PDSCH/PDCCH, this component can be ignored. Referring to the UL timing, configuring this SSB-less SCell and the reference cell at a same TAG is typical for the co-located FR1 deployment, so UE maintains the same TA for the SSB-less SCell and the reference cell. Based on the coarse DL syn and the maintained TA, the UL timing in the SSB-less SCell can be identified.

Regarding how to determine the UL power for the SSB-less SCell, PL-RS is allowed to be cross cell configured between the SCell and PCell through the IE pathlossReferenceLinking. So by configuring the PL-RS in the reference cell(PCell), UE can derive the pathloss of this SSB-less SCell given that the two co-located cells experience similar pathloss. Additionally, if the cross cell PL-RS monitoring is not enough, through the configuration of P0, NW can further adjust the UE transmission power at the SSB-less SCells. Since similar propagation experienced between the co-located cells, so the UL power offset between cells mainly lies in the frequency. NW completely know about the frequency of each cell, by configuring suitable P0, the NW can help UE to determine the UL power of the SSB-less SCell.

Regarding RRC signalling, it seems that the NW needs to indicate UE which active serving cell is the reference cell to borrow the coarse time synchronization for the SSB-less SCell.

Proposal 9: Scenario 2a has the following impact:

- For RRM requirements, the SCell activation latency is 3 ms for this scenario;

- No RAN1 impact;

- For RRC signalling, the NW indicates UE which active serving cell is the reference cell to borrow the coarse time synchronization for the SSB-less SCell when the side condition met.

For Scenario 2, it seems the above analysis for Scenario 2a also fit Scenario 2.

For Scenario 1, the SCell activation latency is also 3 ms.

2.5 The regular L1/L3 measurements of SSB-less

- Due to lack of SSB, so there are two solutions for the regular L1/L3 measurements of SSB-less SCell:

- Solution 1: Performing regular L1/L3 measurements on CSI-RS resource

- Solution 2: Sharing the L1/L3 measurement results between the SSB-less SCell and reference cell

- For Scenario 1 and 2, both Solution 1 and 2 are applicable. Comparing between the Solution 1 and 2, we prefer Solution 2 given that the side conditions are met.

Proposal 10: For Scenario 1 and 2, both Solution 1 and 2 are applicable. Comparing between the Solution 1 and 2, we prefer Solution 2 given that the side conditions are met.

- Solution 1: Performing regular L1/L3 measurements on CSI-RS resource

- Solution 2: Sharing the L1/L3 measurement results between the SSB-less SCell and reference cell

- For Scenario 1, either applying Solution 2 or ignore this issue since no demands. For L1 measurements, which aims to help NW to derive the RSRP/SINR/CSI/beam info so as to determine the CSI/TCI state, while for the SSB-less SCell under Scenario 2a, no need to determine the CSI/TCI state since no PDSCH/PDCCH transmission, NW only receive the UL transmitted from UE, so NW only need to determine the UL CSI through SRS reception. For L3 measurements, which are used for mobility management. While the decision of mobility management on such SSB-less SCell can be same as that for co-located reference cell, it is natural and typical. As a result, For Scenario 2a, both legacy L1/L3 measurement can be ignored.

Proposal 11: For Scenario 2a, no need to determine the CSI/TCI state since no PDSCH/PDCCH transmission, so legacy L1 measurement can be ignored. For L3 measurement, it is nature and typical to share the L3 measurement results between the co-located SSB-less SCell and reference cell.

- Conclusion

In this contribution, we have the following proposals for the Network power saving:

Observation 1: Multiple UL heavier use cases were identified in NR, including remote driving, machine vision and factory vedio surveillance. Under these use cases, the SSB-less SCell can be mainly used for the UL traffic supplementation.

Observation 2: According to the study in SI status, no matter for TDD or FDD, the NW power consumption in active DL is much severer than active UL.

Proposal 1: By shutting down the NW DL transmitter at the SSB-less SCell, Scenario 2a is beneficial to achieve the significant NW power saving effect.

Observation 3: For the sake of NW power saving, the always-on signals should be avoided. While based on RAN1 specification, at least P-TRS is involved in Scenario 1.

Proposal 2: From the perspective of NW power saving effect, Scenario 2a is better than Scenario 2, and Scenario 2 is better than Scenario 1.

Observation 4: For co-located FR1 inter-band CA deployment, Scenario 1 and 2 are similar with the SCellwithoutSSB case in legacy intra-band contiguous CA. For Scenario 2a, since the components of AGC, fine T/F tracking and TCI state indication can be skipped, so which only demands a subset of conditions of Scenario 1/2.

Proposal 3: Considering the frequency range between two bands can be narrower or comparable than some intra-band contiguous CA, so the SSB-less SCell operation in Scenario 1, 2 and 2a are feasible.

Proposal 4: For all Scenario 1, 2 and 2a, no need to perform PDCCH/PDSCH performance evaluation.

Proposal 5: The side conditions for Scenario 1 include:

- Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell -- 260ns;

- The difference of the reception power with the FR1 inter-band active serving cell -- 6dB;

- Frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC -- further determine in RF session.

Proposal 6: The side conditions for Scenario 1 can be reused for Scenario 2.

Proposal 7: To address UE vendors’ concern on different Rx chain assumptions between intra-band CA and inter-band CA, the time margin between multiple Rx chains can be clarified.

Proposal 8: The side conditions for Scenario 2a includes:

- Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell -- 260ns;

Proposal 9: Scenario 2a has the following impact:

- For RRM requirements, the SCell activation latency is 3 ms for this scenario;

- No RAN1 impact;

- For RRC signalling, the NW indicates UE which active serving cell is the reference cell to borrow the coarse time synchronization for the SSB-less SCell when the side condition met.

Proposal 10: For Scenario 1 and 2, both Solution 1 and 2 are applicable. Comparing between the Solution 1 and 2, we prefer Solution 2 given that the side conditions are met.

- Solution 1: Performing regular L1/L3 measurements on CSI-RS resource

- Solution 2: Sharing the L1/L3 measurement results between the SSB-less SCell and reference cell

Proposal 11: For Scenario 2a, no need to determine the CSI/TCI state since no PDSCH/PDCCH transmission, so legacy L1 measurement can be ignored. For L3 measurement, it is nature and typical to share the L3 measurement results between the co-located SSB-less SCell and reference cell.

- Reference

- RP-223540 “New WID: Network energy savings for NR”, Huawei

- RP-222821, 3GPP TR 38.864 V1.0.0, Study on network energy savings for NR.

- R4-2306369, “WF on RRM requirements for NR network energy saving”, Huawei

- TR 38.864-i10

- TS38.214-h50