3GPP TSG-RAN WG4 Meeting #107 	R4-2308768

Incheon, Korea, May 22nd – 26th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Bandwidth part operation without restriction in NR – Option A

Agenda item:	8.11.2.1

Document for:	Discussion

Introduction

RAN#99 approved a new WID to address the issues on BWP without restriction in [1]. Objectives can be found below:

In RAN4#106bis meeting RAN4 discussed option A and reached agreements as captured in [2]. In this paper we give our input to the remaining aspects related Option A and the possible UE, network and system impacts.

Discussion

The solution related to option A is based on following assumption:

- Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP

Hence, it is assumed that CD-SSB is not within the active BWP and the network has configured the UE dedicated CSI-RS for performing the necessary RLM, BFD and Beam Management measurements.

In RAN4#106bis meeting it was agreed:

Issue 2-1: In general, no specification impact on RLM/BFD/BM requirements for UE supporting option A for BWP operation without restriction.

Issue 2-3: Existing requirements for intra-frequency measurement with gap shall apply for UE supporting option A and no update is needed.

Hence, L3 intra-frequency measurement requirements need no further discussion.

And following was still open:

Issue 2-1: if there is impact on FR2 CSI-RS base RLM requirements specifically?

Issue 2-2: Any clarification on existing timing requirements when CD-SSB is outside active BWP?

The objective is:

- For Option A

  - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4)

Following we shortly address these aspects.

Network signalling overhead and power impact

Option A is based on the idea that the network transmits CSI-RS within the active BWP to the UE. Hence, each UE in the cell is configured with CSI-RS for RLM/BFD and BM.

It should be noticed that the network will be providing the UE dedicated CSI-RS continuously to allow the UE to use the CSI-RS for RLM/BFD and BM.

CSI-RS based RLM in FR2 has already been specified. We do not see a reason why the existing requirements would not be directly applicable to Option A. CSI-RS for RLM is assumed being available within the active BWP of the UE and hence the conditions for the existing requirements being applicable are fulfilled.

There is no impact on existing FR2 CSI-RS base RLM requirements due to Option A.

Timing Requirements

UE operating in a BWP without CD-SSB has been defined since Rel-15. The existing timing requirements has been applicable since Rel-15 and account this scenario. Hence, there is no reason to clarify the existing timing requirements as this may cause confusion whether legacy/existing requirements do apply for the Option A scenario.

No clarification on existing timing requirements when CD-SSB is outside active BWP is needed.

Conclusion

In this paper we take a further look at bandwidth part operation without restriction in NR – Option A. We look at the two remaining issues from last meeting. We observe following related to Issue 2-1:

There is no impact on existing FR2 CSI-RS base RLM requirements due to Option A.

And related to Issue 2-2 we propose:

- No clarification on existing timing requirements when CD-SSB is outside active BWP is needed.

- References

- RP-230805, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Moderator (Vodafone), vivo, Mar. 2023.

- R4-2306397, WF on completion of specification support for BWP operation without restriction, vivo