3GPP TSG-RAN WG4 Meeting #107 	R4-2308774

Incheon, Korea, May 22nd – 26th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Network A requirements

Agenda item:	8.26.2.3

Document for:	Discussion

Introduction

In the last RAN4 meeting, RAN4 discussed the aspects concerning UE requirements related to the MUSIM gaps introduced in Rel-17.

One agreement was made in RAN4#106bis meeting on Issue 3-1-3 regarding network A requirements. Several issues remain open in RAN4, and in this paper we continue the discussion related to these issues in this paper.

Discussion

On network A requirements

2 issues are currently open for discussion regarding network A operations:

- Principle on layer 1 and layer 3 measurement requirements after gap collision handling (3-1-1)

- On parameters for L1/L3 measurement requirements (3-1-2)

We still see that issue 3-1-2 depends on outcome of the first issue. And in general the discussion depends on the progress and agreements related to the priority discussion.

Principle on layer 1 and layer 3 measurement requirements after gap collision handling (3-1-1)

Different proposals were discussed in last meeting. However, one proposal gained majority interest. The proposal was:

- Reuse the principle used in Rel-17 concurrent gaps WI as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured.

Hence, the proposal is to re-use the principle introduced with concurrent gaps. However, as also pointed out in last meeting, the Issue depends on clear resolution and agreements related to gap collisions.

Re-use the principle introduced with concurrent gaps. Settle other dependent issues first.

However, as several aspects are still open for discussion in the collision and priority discussion, we do not see a need for further solid agreement on the details of defining layer 1 and layer 3 measurement requirements after gap collision.

Define the detailed NW-A’s requirement once RAN4 reach consensus on gap priorities.

On parameters for L1/L3 measurement requirements (3-1-2)

As concluded in last meeting this issue depends on Issue 3-1-1 which again depends on RAN4 settling the discussion on how to handle gap collisions in general, including MUSIM gaps, Type-1 and Type-2 measurement gaps.

This issue depends on Issue 3-1-1 (and other collision related agreements).

Conclusion

In the last RAN4 meeting in Athens, RAN4 discussed the aspects concerning UE requirements related to the MUSIM gaps introduced in Rel-17. The discussions left RAN4 with 3 Issues regarding network A operations. These we have addressed in this paper and propose:

- Re-use the principle introduced with concurrent gaps. Settle other dependent issues first (3-1-1).

- Define the detailed NW-A’s requirement once RAN4 reach consensus on gap priorities (3-1-1).

- This issue depends on Issue 3-1-1 (and other collision related agreements) (3-1-2).

- References

- R4-2306357, WF on NR Dual TxRx Multi-SIM, vivo.

- R4-2303248, WF on NR Dual TxRx Multi-SIM, vivo.