3GPP TSG-RAN WG4 # 107	R4-2308802

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.22.1

Source:	MediaTek Inc.

Title:	Discussion on RAN4 works in Rel-18 AIML SI

Document for:	Discussion

- Introduction

In RAN4#106bis-e meeting, a WF on AI/ML RAN4 studies was approved [1]. In this contribution, we present our views on RAN4 impact and considerations for AI/ML in the air interface.

- Discussion

2.1	LCM requirements

In last meeting, RAN4 agreed to study how to define LCM related requirements and test cases. In this part, we will share our views on this.

RAN4#106bis-e Agreements: - Following LCM related requirements are to be studied: - Model/Functionality select/switch/activate/deactivate/fallback - Model/Functionality monitoring - FFS if requirements for data collection (in particular for training) could/need be defined - FFS if requirements for transfer/delivery/update - NOTE: RAN4 study should be aligned with the agreements in other working groups. - Further study under LCM related tests, if they are defined. - how the framework can address the possibility of updates/activation/deactivation/switching to the functionalities/models after the deployment of the devices in the filed - RAN4 to investigate how to define performance requirements/tests for the following candidate procedures: - model/functionality monitoring - model/functionality selection - model/functionality activation/deactivation/switching/fallback - FFS whether data collection should be considered - FFS whether model update/transfer/delivery should be considered

Model/Functionality select/switch/activate/deactivate/fallback

According RAN2#121bis-e agreement, for all the use cases, model/function selection/(de)activation/switching/fallback can be UE-initiated or NW-initiated. RAN4 needs to discuss how to define the requirements and test cases for both UE-initiated and NW-initiated model/function selection/(de)activation/switching/fallback.

For the CSI compression and beam management use cases, model/function selection/(de)activation/switching/fallback can be UE-initiated or gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model). For the positioning use case, model/function selection/(de)activation/switching/fallback can be UE-initiated or LMF-/ gNB-initiated. FFS how the different cases are different (e.g. applicability to UE-sided vs network sided model).

Proposal 1: RAN4 needs to discuss how to define the requirements and test cases for both UE-initiated and NW-initiated model/function selection/(de)activation/switching/fallback.

Regarding NW-initiated model/function selection/(de)activation/switching/fallback, a command from NW is expected. And the triggering command can be used as the starting point of the delay requirements. For the ending point, it depends on whether a complete indication will be defined in other WGs or not. If a complete indication will be defined in other WGs, then the complete indication can be used as the ending point.

In the delay requirements, at least two components are essential. One is the time to decode the triggering command. The exact time depends on the command is RRC/MAC/DCI. Another is the time needed for model/function change or model/function status change. UE may need some time to load the model or parameters, and model/function complexity may have impact on the delay requirements, especially for model/function selection/(de)activation/switching. In other words, model update may be needed during model/function selection/(de)activation/switching. As for the other components, we think more progress from other WGs is necessary for further discussion.

Proposal 2: For NW-initiated model/function selection/(de)activation/switching/fallback, the starting point of the delay requirements can be the triggering command to be defined in other WGs.

Proposal 3: For NW-initiated model/function selection/(de)activation/switching/fallback, the ending point of the delay requirements can be the complete indication to be defined in other WGs, if any.

Proposal 4: At least two components are essential in the delay requirements of NW-initiated model/function selection/(de)activation/switching/fallback. One is the time to decode the triggering command. Another is the time needed for model/function change or model/function status change.

Proposal 5: Further discuss whether and how model/function complexity impact the delay requirements of model/function selection/activation/switching.

Regarding UE-initiated model/function selection/(de)activation/switching/fallback, it is very similar to conditional handover/PSCell change in our understanding. Some conditions should be predefined or configured by NW to trigger model/function selection/(de)activation/switching/fallback. In both the requirements and test cases, monitoring would be one of the procedures to check whether the trigger condition is satisfied or not. When the trigger condition is satisfied, the procedure will be basically the same as NW initiated model/function selection/(de)activation/switching/fallback.

Proposal 6: For UE-initiated model/function selection/(de)activation/switching/fallback, Model/Functionality monitoring would be one of the procedures in both the delay requirements and test cases.

Proposal 7: For UE-initiated model/function selection/(de)activation/switching/fallback, when the trigger condition is satisfied, the procedure will be basically the same as NW initiated model/function selection/(de)activation/switching/fallback.

Model/Functionality monitoring

Monitoring is to continuously identify a right time to activate/deactivate AI operation or switch to a different AI model. This is very similar to the background measurements on both serving and neighbouring cells in order to trigger the handover or PSCell addition. Which parameter to monitor would be different per use cases. It is better to discuss Model/Functionality monitoring per use cases.

Proposal 8: Discuss Model/Functionality monitoring per use cases.

Model update/transfer/delivery

For Model update/transfer/delivery, there are mainly two steps. The first step is transferring or delivering the model parameters and/or model structure from one side to another, e.g., from gNB to UE. The second step is that UE updates the model parameters and/or model structure. In our understanding, RAN4 does not need to define the requirements for the first step. For the second step which is model updating at UE, RAN4 can define the delay requirements. In our understanding, the information to deliver for open format and proprietary format will be different. The action on receiving the model information will be also different for open format and proprietary format. So will be the delay requirements. For open format, the parts to update would have impact on the delay requirements, e.g., only to update part of parameters or also update the structure. And further discussion is needed.

Proposal 9: Discuss how to define the delay requirement of model updating (not include model parameters and/or model structure transfer procedure).

Proposal 10: Discuss the delay requirements of model updating for open format and proprietary format separately.

2.2	Use cases discussion

2.2.1	CSI feedback enhancement

RAN1 use intermediate KPI, SGCS and NMSE, to evaluate inference accuracy in AI/ML. Also, raw channel matrix, eigenvector(s) or feedback CSI information are used as the input/output to AI/ML model in RAN1 discussion.

CSI compression - AI/ML model input (for CSI generation part)/output (for CSI reconstruction part) types for evaluations: - Raw channel matrix (in frequency or delay domain), e.g., channel matrix with dimensions of Tx, Rx, and frequency unit - Precoding matrix (as a group of eigenvectors or an eTypeII-like reporting) CSI prediction - The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc. - Including assumptions on the observation window, i.e., number/time distance of historic CSI/channel measurements - The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

In RAN4 legacy CSI related requirements, different test metrics are applied for CQI, PMI and RI requirements, which are:

- CQI requirements: medium CQI and BLER

- PMI requirements: relative throughput ratio between follow PMI and random PMI

- RI requirements: relative throughput ratio between follow RI and fixed RI

All these test metrics are based on the CQI/PMI/RI reporting from UE side and ACK/NACK in UE uplink transmission. Considering the testability from TE perspective, RAN4 can use throughput/BLER or CSI report (CQI/PMI/RI) as test metrics for CSI feedback enhancement as a priority. From RAN4 perspective, if a certain BLER/throughput or CSI report (CQI/PMI/RI) accuracy can be achieved, the intermediate KPI, SGCS and NMSE, are not relevant.

Proposal 11: RAN4 can prioritize to consider throughput/BLER or CSI report (CQI/PMI/RI) as test metrics for CSI feedback enhancement.

Reference encoder/decoder for Two-sided model

The AI/ML CSI compression sub-case involves two-sided AI/ML operations performed at both UE and gNB. UE is configured with the AI encoder and gNB is configured with the corresponding decoder, where the AI encoder is for CSI compression, and the decoder is to recover CSI. The network side AI/ML model design/implementation will impact the UE performance verification. Similarly, the UE side AI/ML model design/implementation will also impact the network performance verification. Therefore, to minimize the impact from either UE side/network side. The reference encoder/decoder should be considered and there are lots of options after RAN#106bis-e. We think option3 maybe the simplest way to perform the test. Therefore, RAN4 can choose option3 as a starting point to discuss reference encoder/decoder.

- Reference Decoder for test implementation for 2-sided models in the UE performance tests - Following options should be studied for the reference decoder for test implementation in the UE performance tests - Option 1: reference decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained - Option 2: reference decoder is provided by the vendor of the decoder so that the encoder and decoder are jointly designed and trained - Option 3: The reference decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. - Option 4: The reference decoder(s) are partially specified and captured in RAN4 spec. - Option 5: Option 1, 2, 3 or 4 depending on the test - Option 6: Test decoder is specified and captured in RAN4 and is provided by test environment vendor. The encoder and decoder can be jointly trained. - Other options can be discussed depending on companies’ inputs - Reference decoder defined for the tester in the UE performance tests should not limit the implementation of different models at the network side - Reference Encoder for test implementation for 2-sided models in the gNB performance tests - Following options should be studied for the reference encoder for test implementation in the gNB performance tests - Option 1: reference encoder is provided by the vendor of the decoder under test so that the encoder and decoder are jointly designed and trained - Option 2: reference encoder is provided by the vendor of the encoder so that the encoder and decoder are jointly designed and trained - Option 3: The reference encoders are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. - Option 4: The reference encoders are partially specified and captured in RAN4 spec. - Option 5: Option 1, 2, 3 or 4 depending on the test - Option 6: Test encoder is specified and captured in RAN4 and is provided by test environment vendor. The encoder and decoder can be jointly trained. - Other options can be discussed depending on companies’ inputs - Reference decoder defined for the tester in the gNB performance tests should not limit the implementation of different models at the UE side.Further discuss the difference between reference encoder/decoder and test encoder/decoder.

Proposal 12: It is the simplest way to perform the test if reference encoder/decoder are fully specified in specification. For the reference encoder/decoder in two-sided model, RAN4 can first study how to fully specify reference encoder/decoder in specification.

2.2.2	Beam management

The intention of both spatial beam prediction and temporal beam prediction is to reduce overhead. RAN1 is evaluating overhead reduction ratio using AI/ML in beam management. Overhead reduction ratio is an important metric, but we don’t think RAN4 needs to test this metric directly. When Set A and Set B are determined as well as the input of the model and the quantity to report to NW. The overhead reduction ratio will be determined correspondingly. In our understanding, RAN1 will further discuss the procedures, the input of the model and the reported quantity. When defining the performance requirements and/or test cases, we think RAN4 needs to discuss how to choose Set A & Set B and also the simulation or test parameters. When these are fixed, the overhead reduction ratio is also determined. We prefer not to discuss how to choose Set A & Set B and also the simulation/test parameters in this SI, as there are still some open issues in RAN1. Take the progress in spatial beam prediction as an example. As shown below, RAN1 has not reached the consensus on how to choose Set A & Set B and the input of the model yet. It is too early to discuss how to choose Set A & Set B and also the simulation/test parameters for defining requirements and test case in RAN4.

- BM-Case1: Spatial-domain Downlink beam prediction for Set A of beams based on measurement results of Set B of beams - Consider: Alt. 1): AI/ML model training and inference at NW side. Alt. 2): AI/ML model training and inference at UE side. - Consider: Alt. i): Set A and Set B are different (Set B is NOT a subset of Set A). Alt. ii): Set B is a subset of Set A. Note: Set A is for DL beam prediction and Set B is for DL beam measurement. The beam patterns of Set A and Set B can be clarified by companies. - AI/ML model input: Alt 1): Only L1-RSRP measurement based on Set B; Alt.2): L1-RSRP measurement based on Set B and assistance information; Alt. 3): CIR based on Set B; Alt. 4): L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID.

Proposal 13: For AI/ML of beam management, RAN4 does not need to define requirements or test cases for the metric “overhead reduction”.

Proposal 14: Not to discuss how to choose Set A & Set B and also the simulation/test parameters for defining requirements and test case in RAN4 in this SI.

Another metric that RAN1 uses to evaluate the performance of AI/ML model for beam management is “throughput”. In RRM test cases, usually we don’t use throughput as a metric. In our understanding, the intention to use the metric throughput in RAN1 is to verify that using AI/ML model for beam prediction can reduce the overhead with little performance loss. If this is already verified in RAN1, RAN4 does not need to test it.

Proposal 15: Not to use throughput as a test metric for beam prediction AI/ML model.

Regarding the metrics to define requirements and test cases for, we think the output of AI/ML model should be considered.

As shown in draft TR 38.843 [2], RAN1 has not concluded on AI/ML model output yet. But from the following alternatives, there are mainly two metrics. One is the predicted L1-RSRP, the other is the predicted top-N DL Tx beams. In the next, we will discuss on the two metrics one by one.

The following alternatives for AI/ML model output are studied: - Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - e.g., N predicted beams can be the top-N predicted beams - Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information - e.g., N predicted beams can be the top-N predicted beams - Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams - e.g., N predicted beams can be the top-N predicted beams

Proposal 16: To evaluate the performance of BM AI/ML models, use AI/ML model output as a basis for further discussion on the metrics.

Predicted L1-RSRP: In our understanding, the input of the model should at least contain L1-RSRP measurement based on Set B. As the input L1-RSRP is measured by UE, the input L1-RSRP should satisfy the legacy L1-RSRP measurement accuracy requirements. The accuracy of the output would not be better than the input. Therefore, the accuracy requirements of the output L1-RSRP of BM AI/ML model should not be tightened than legacy L1-RSRP accuracy requirements.

Proposal 17: The accuracy requirements of the output L1-RSRP of BM AI/ML model should not be tightened than legacy L1-RSRP accuracy requirements.

Predicted top-N DL Tx beams: RAN1 has also discussed the beam prediction accuracy as shown in draft TR 38.843 [2]. In our understanding, the outcome of RAN1 discussion on beam prediction accuracy can be used as a metric in defining the test cases. In RRM test cases, we usually define the past rate during repeated tests. The beam prediction accuracy discussed in RAN1 can be used as a reference when define the pass rate during repeated tests.

- Beam prediction accuracy (%): - Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam” - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams” - Top-1/K (%) (Optional): the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams” - Where K >1 and values can be reported

Proposal 18: The beam prediction accuracy discussed in RAN1 can be used as a reference when define the pass rate during repeated tests.

2.3	Others

Complexity of AI/ML

In RAN1 discussion, the complexity is also evaluated as a KPI, including model complexity and computational complexity. The computational complexity can be reported via the metric of floating point operations (FLOPs), and the model complexity could be measured by memory storage in terms of AI/ML model size and number of AI/ML parameters. The different complexity of AI/ML will result in different performance. Therefore, when defining requirements for AI/ML, the complexity should be taken into consideration. RAN4 should define requirements with reasonable complexity and how to measure the complexity and what is the reasonable complexity can be further discussed.

Proposal 19: The different complexity of AI/ML model will result in different performance. When introducing the requirements for AI/ML, the complexity should be considered. RAN4 should define requirements with reasonable complexity and how to measure the complexity and what is the reasonable complexity can be further discussed.

- Conclusion

In the contribution, we provided our views on RAN4 AI/ML SI. We have the following proposals:

Proposal 1: RAN4 needs to discuss how to define the requirements and test cases for both UE-initiated and NW-initiated model/function selection/(de)activation/switching/fallback.

Proposal 2: For NW-initiated model/function selection/(de)activation/switching/fallback, the starting point of the delay requirements can be the triggering command to be defined in other WGs.

Proposal 3: For NW-initiated model/function selection/(de)activation/switching/fallback, the ending point of the delay requirements can be the complete indication to be defined in other WGs, if any.

Proposal 4: At least two components are essential in the delay requirements of NW-initiated model/function selection/(de)activation/switching/fallback. One is the time to decode the triggering command. Another is the time needed for model/function change or model/function status change.

Proposal 5: Further discuss whether and how model/function complexity impact the delay requirements of model/function selection/activation/switching.

Proposal 6: For UE-initiated model/function selection/(de)activation/switching/fallback, Model/Functionality monitoring would be one of the procedures in both the delay requirements and test cases.

Proposal 7: For UE-initiated model/function selection/(de)activation/switching/fallback, when the trigger condition is satisfied, the procedure will be basically the same as NW initiated model/function selection/(de)activation/switching/fallback.

Proposal 8: Discuss Model/Functionality monitoring per use cases.

Proposal 9: Discuss how to define the delay requirement of model updating (not include model parameters and/or model structure transfer procedure).

Proposal 10: Discuss the delay requirements of model updating for open format and proprietary format separately.

Proposal 11: RAN4 can prioritize to consider throughput/BLER or CSI report (CQI/PMI/RI) as test metrics for CSI feedback enhancement.

Proposal 12: It is the simplest way to perform the test if reference encoder/decoder are fully specified in specification. For the reference encoder/decoder in two-sided model, RAN4 can first study how to fully specify reference encoder/decoder in specification.

Proposal 13: For AI/ML of beam management, RAN4 does not need to define requirements or test cases for the metric “overhead reduction”.

Proposal 14: Not to discuss how to choose Set A & Set B and also the simulation/test parameters for defining requirements and test case in RAN4 in this SI.

Proposal 15: Not to use throughput as a test metric for beam prediction AI/ML model.

Proposal 16: To evaluate the performance of BM AI/ML models, use AI/ML model output as a basis for further discussion on the metrics.

Proposal 17: The accuracy requirements of the output L1-RSRP of BM AI/ML model should not be tightened than legacy L1-RSRP accuracy requirements.

Proposal 18: The beam prediction accuracy discussed in RAN1 can be used as a reference when define the pass rate during repeated tests.

Proposal 19: The different complexity of AI/ML model will result in different performance. When introducing the requirements for AI/ML, the complexity should be considered. RAN4 should define requirements with reasonable complexity and how to measure the complexity and what is the reasonable complexity can be further discussed.

- Reference

- R4-2306299, “WF on AI/ML RAN4 studies”, Qualcomm Incorporated

- R1-2304148, “TP for TR for Study on AI-ML for NR Air Interface”, Qualcomm Incorporated