3GPP TSG-RAN WG4 Meeting # 107	R4-2308857

Incheon, KR, May 22 – May 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0372 | rev |  | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 38.101-4: Updates to FR2-2 UE performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Perf |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The brackets is still existing for PDSCH requirements and some configurations - FRC for FR1+FR2-2 CA requirements is still TBD - RMC for PDCCH is captured in wrong clause - PDSCH allocation for FR1+FR2-2 CA are wrong |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the brackets - Add FRC for FR1+FR2-2 CA - Remove the PDCCH RMC from clause A.3.2.2 to A.3.3.2 - Correct the PDSCH allocation for FR1+FR2-2 CA test |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The brackets will still be existing - The FRC for FR1+FR2-2 CA will still be missing - The PDCCH RMC will still be in wrong clause - PDSCH allocation for FR1+FR2-2 CA is still wrong |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2, A.3.2, A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.2	PDSCH demodulation requirements

The parameters specified in Table 7.2-1 are valid for all PDSCH demodulation tests unless otherwise stated.

Table 7.2-1: Common Test Parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| PTRS epre-Ratio |  |  |  | 0 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 60 or 120 or 480 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-2 [7] for tested channel bandwidth and subcarrier spacing |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot for 120 KHz SCS (Xs, Ys) = (4, 1) for 480 KHz SCS |
|  | Symbols with PDCCH |  |  | 0 |
|  | Number of PRBs in CORESET |  |  | Table 7.2-2 for tested channel bandwidth and subcarrier spacing |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI state |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| Cross carrier scheduling |  |  |  | Not configured |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 6 for CSI-RS resource 1 and 3 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 for CSI-RS resource 1,2,3,4 120 kHz SCS: 160 for CSI-RS resource 1,2,3,4 480 kHz SCS: 640 for CSI-RS resource 1, 2, 3, 4 |
|  | CSI-RS offset |  | Slots | 60 kHz SCS: 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 120 kHz SCS: 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 480 kHz SCS: 320 for CSI-RS resource 1 and 2 321 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  | 3 for 2 CSI-RS ports and 5 for 4 CSI-RS ports |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 12 |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 120 kHz SCS: 160 480 kHz SCS: 640 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Row index (Note 3) |  |  | 5 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 120 kHz SCS: 160 480 kHz SCS: 640 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |
| CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2 |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 |
|  | CSI-RS periodicity |  | Slots | 60 kHz SCS: 80 for CSI-RS resource 1,2 120 kHz SCS: 160 for CSI-RS resource 1,2 480 kHz SCS: 640 for CSI-RS resource 1,2 |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | Repetition |  |  | ON |
|  | QCL info |  |  | TCI state #1 |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| PTRS configuration | Frequency density (KPT-RS) |  |  | 2 |
|  | Time density (LPT-RS) |  |  | 1 |
|  | Resource Element Offset |  |  | 2 |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 120 KHz SCS: 4 480 KHz SCS: 16 |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection with Wideband size and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 for FR2-1 tests OP.1 TDD as defined in Annex A.5.2.1 for FR2-1 tests No OCNG symbols on unused REs for FR2-2 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-2 [7] for tested channel bandwidth and subcarrier spacing. Note 3: Refer to Table 7.4.1.5.3-1 in [9] |  |  |  |  |

Table 7.2-2: Number of PRBs in CORESET

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 30 | 66 | 132 | 264 |
| 480 | N.A | N.A | N.A | 66 |

Table 7.2.2.2.1-2: Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | For Test 1-1 and 1-2: 3 for CSI-RS resource 1 and 3 7 for CSI-RS resource 2 and 4 |
|  | CSI-RS offset | Slots | For Test 1-2: 82 for CSI-RS resource 1 and 2 83 for CSI-RS resource 3 and 4 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL4 for Test 1-4, 2-3, and 4-6 1/AL8 for other tests |
|  | Symbols with PDCCH |  | 0,1 for Test 4-6 0 for other tests |
|  | Number of PRBs in CORESET |  | 18 for Test 4-6 Table 7.2-2 for other tests |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 for Test case 4-6 1 for other Tests |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband for Test 1-1, 2 for other tests |
|  | Resource allocation type |  | Test 2-1: Type 1 with start RB = 30, LRBs = 6 Test 4-6: Type 1 with start RB = 24, LRBs = 20 Other tests: Type 0 |
|  | RBG size |  | Test 2-1: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 for Test 1-1, 1-3, 1-4, 2-2, 2-4, 4-1, 4-2, 4-4, 10 for Test 2-1, 2-3, 2-5, 2-6, 3-1 16 for Test 1-2, 4-3, 4-5, 4-6 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

7.2.2	2RX requirements

7.2.2.1	FDD

(Void)

7.2.2.2	TDD

7.2.2.2.1	Minimum requirements for PDSCH Mapping Type-A

For PDSCH Type-A scheduling, the requirements are specified in Table 7.2.2.2.1-3, 7.2.2.2.1-4, 7.2.2.2.1-5, and 7.2.2.2.1-6 with the addition of the parameters in Table 7.2.2.2.1-2 and the downlink physical channel setup according to Annex C.5.1. The purpose is to verify the performance of PDSCH Type-A scheduling.

The test purposes are specified in Table 7.2.2.1.1-1.

Table 7.2.2.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance in FR2-1 under 2 receive antenna conditions and with different channel models, MCSs andnumber of MIMO layers in FR2-1. | 1-1, 1-3, 1-4, 2-1, 2-2, 2-3, 2-4, 2-5, 2-6 |
| Verify the PDSCH mapping Type A HARQ soft combining performance in FR2-1 under 2 receive antenna conditions. | 1-2 |
| Verify the PDSCH mapping Type A performance requirements for Enhanced Receiver Type 1 in FR2-1 under 2 receive antenna conditions. | 3-1 |
| Verify the PDSCH mapping Type A normal performance in FR2-2 under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 4-1, 4-2, 4-3, 4-4, 4-5, 4-6 |

Table 7.2.2.2.1-2: Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | For Test 1-1 and 1-2: 3 for CSI-RS resource 1 and 3 7 for CSI-RS resource 2 and 4 |
|  | CSI-RS offset | Slots | For Test 1-2: 82 for CSI-RS resource 1 and 2 83 for CSI-RS resource 3 and 4 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL4 for Test 1-4, 2-3, and 4-6 1/AL8 for other tests |
|  | Symbols with PDCCH |  | 0,1 for Test 4-6 0 for other tests |
|  | Number of PRBs in CORESET |  | 18 for Test 4-6 Table 7.2-2 for other tests |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 for Test case 4-6 1 for other Tests |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband for Test 1-1, 2 for other tests |
|  | Resource allocation type |  | Test 2-1: Type 1 with start RB = 30, LRBs = 6 Test 4-6: Type 1 with start RB = 24, LRBs = 20 Other tests: Type 0 |
|  | RBG size |  | Test 2-1: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 for Test 1-1, 1-3, 1-4, 2-2, 2-4, 4-1, 4-2, 4-4, 10 for Test 2-1, 2-3, 2-5, 2-6, 3-1 16 for Test 1-2, 4-3, 4-5, 4-6 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

Table 7.2.2.2.1-3: Minimum performance for Rank 1 (FRC) for FR2-1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 1-1 | R.PDSCH.5-1.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-1A | TDLC60-300 | 2x2 ULA Low | 70 | -0.4 |
| 1-2 | R.PDSCH.5-2.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 30 | 1.7 |
| 1-3 | R.PDSCH.5-3.1 TDD | 100 / 120 | 64QAM, 0.46 | FR2.120-1 | TDLA30-300 | 2x2 XPL Medium | 70 | 12.4 |
| 1-4 | R.PDSCH.5-10.1 TDD | 50 / 120 | 256QAM 0.67 | FR2.120-1 | TDLD30-75 | 2x2 ULA Low | 70 | 20.2 |

Table 7.2.2.2.1-4: Minimum performance for Rank 2 (FRC) for FR2-1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 2-1 | R.PDSCH.5-4.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 4.1 |
| 2-2 | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.4 |
| 2-3 | R.PDSCH.5-5.2 TDD | 50 / 120 | 16QAM,0.48 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 14.0 |
| 2-4 | R.PDSCH.5-2.3 TDD | 200 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 70 | 14.2 |
| 2-5 | R.PDSCH.4-1.1 TDD | 50 / 60 | 16QAM, 0.48 | FR2.60-1 | TDLA30-75 | 2x2 ULA Low | 70 | 14.3 |
| 2-6 | R.PDSCH.5-6.1 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 70 | 18.6 |

Table 7.2.2.2.1-5: Minimum performance for Rank 2 (FRC) for Enhanced Receiver Type 1 for FR2-1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 3-1 | R.PDSCH.5-5.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-2 | TDLA30-75 | 2x2 ULA Medium | 70 | 19.0 |

Table 7.2.2.2.1-6: Minimum performance for Rank 1 (FRC) for FR2-2

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of max through-put (%) | SNRBB (dB) |
| 4-1 | R.PDSCH.5-1.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 70 | 0.9 |
| 4-2 | R.PDSCH.5-2.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-200 | 2x2 ULA Low | 70 | 9.0 |
| 4-3 | R.PDSCH.5-2.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 30 | 2.7 |
| 4-4 | R.PDSCH.5-3.2 TDD | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | 70 | 11.6 |
| 4-5 | R.PDSCH.8-1.1 TDD | 400 / 480 | QPSK, 0.30 | FR2.480-1 | TDLA10-200 | 2x2 ULA Low | 70 | 1.0 |
| 4-6 | R.PDSCH.8-2.1 TDD | 400 / 480 | 16QAM 0.48 | FR2.480-1 | TDLD10-200 | 2x2 ULA Low | 70 | 8.3 |

9.2A.1.1	NR CA between FR1 and FR2-2

The performance requirements for SCell on FR2-2 band are specified in Table 9.2A.1-5. The test parameters for SCell are specified in Table 7.2.2.2.1-2 with additional change of PDSCH resource allocation type specified in Table 9.2A.1-1:

Table 9.2A.1-1: PDSCH resource allocation type

| Test numer | Allocation Type | Start RB | LRBs |
| --- | --- | --- | --- |
| 1-1 | Type 1 | 98 | 66 |
| 1-2 |  |  |  |
| 1-3 |  |  |  |
| 1-4 |  |  |  |
| 1-6 | ` | 24 | 20 |
| 1-5Note1 | Type 0 | N/A | N/A |
| Note 1: Full BWP is allocated for PDSCH |  |  |  |

The test parameters for PCell in Table 9.2A.1-3 and the downlink physical channel setup according to Annex C.3.1. In this test, Pcell is in FR1 and Scell is in FR2-2 and only requirements for Scell should be verified.

The test purposes are specified in Table 9.2A.1-2.

Table 9.2A.1-2: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance in FR2-2 Scell CC in CA between FR1 and FR2-2 under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1,1-2,1-3,1-4,1-5,1-6 |

Table 9.2A.1-3: Test parameters for PCell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| TDD pattern |  |  | FR1.30-1 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 3 |

Table 9.2A.1-4: K1 values for each CC

| Cells | K1 values |
| --- | --- |
| Pcell CC | {8,7,6,5,4,3,2} |
| SCell CC with 120kHz | {8,8,8,8,7,7,7,7} |
| SCell CC with 480kHz (Note 1) | {8,8,8,7} |
| Note 1: One k1 value applies for 4 TBs scheuled by one DCI |  |

Table 9.2A.1-5: Minimum performance for Rank 1 (FRC) in Scell CC

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of max through-put (%) | SNRBB (dB) |
| 1-1 | R.PDSCH.9-1.1 TDD | 400 / 120 | QPSK, 0.30 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 70 | 0.9 |
| 1-2 | R.PDSCH.10-1.1 TDD | 400 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-200 | 2x2 ULA Low | 70 | 9.0 |
| 1-3 | R.PDSCH.10-1.1 TDD | 400 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 30 | 2.7 |
| 1-4 | R.PDSCH.11-1.1 TDD | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | 70 | 11.6 |
| 1-5 | R.PDSCH.12-1.1 TDD | 400 / 480 | QPSK, 0.30 | FR2.480-1 | TDLA10-200 | 2x2 ULA Low | 70 | 1.0 |
| 1-6 | R.PDSCH.13-1.1 TDD | 400 / 480 | 16QAM 0.48 | FR2.480-1 | TDLD10-200 | 2x2 ULA Low | 70 | 8.3 |

A.3.2	Reference measurement channels for PDSCH performance requirements

For PDSCH reference channels if more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit).

A.3.2.2.8	Reference measurement channels for SCS 480 kHz FR2-2

Table A.3.2.2.8-1: PDSCH Reference Channel for TDD UL-DL pattern FR2.480-1 (QPSK)

| Parameter |  | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | R.PDSCH.8-1.1 TDD |  |  |  |  |
| Channel bandwidth |  | MHz | 400 |  |  |  |  |
| Subcarrier spacing |  | kHz | 480 |  |  |  |  |
| Allocated resource blocks |  | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  |  | N/A |  |  |  |  |
| For Slots i = 320, 321 |  |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  |  | 11 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  |  | 477 |  |  |  |  |
| MCS table |  |  | 64QAM |  |  |  |  |
| MCS index |  |  | 4 |  |  |  |  |
| Modulation |  |  | QPSK |  |  |  |  |
| Target Coding Rate |  |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  |  | N/A |  |  |  |  |
| For Slots i = 320, 321 |  |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  |  | 12 |  |  |  |  |
| Overhead for TBS determination |  |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | Bits | 4480 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | Bits | 5504 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | CBs | N/A |  |  |  |  |
| For Slots i = 320, 321 | CBs |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | Bits | 15048 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | Bits | 18216 |  |  |  |  |
| Max. Throughput averaged over 2 frames |  | Mbps | 129.632 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |  |

Table A.3.2.2.8-2: PDSCH Reference Channel for TDD UL-DL pattern FR2.480-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.8-2.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 400 |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |
| Allocated resource blocks | PRBs | 20 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | N/A |  |  |  |  |
| For Slots i = 320, 321 |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | 11 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 477 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | N/A |  |  |  |  |
| For Slots i = 320, 321 |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 3904 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 4736 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | CBs | N/A |  |  |  |  |
| For Slots i = 320, 321 | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 8160 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 10080 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 111.6224 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

A.3.2.2.9	Reference measurement channels for FR1+FR2-2 CA with 120 kHz SCS (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.9-1.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 9 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 13 |  |  |  |  |
| Allocated slots per 40 slots |  | 8 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 12 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 3624 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 | Bits | 5504 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 16 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | CBs | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | CBs | 1 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=80,81 | Bits | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 12210 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 18282 |  |  |  |  |
| Max. Throughput averaged over 160 slots | Mbps | 7.504 |  |  |  |  |
| Note 1: Slot i is slot index per 160 slots |  |  |  |  |  |  |

A.3.2.2.10	Reference measurement channels for FR1+FR2-2 CA with 120 kHz SCS (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.10-1.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 9 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 13 |  |  |  |  |
| Allocated slots per 40 slots |  | 8 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 12 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 11272 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 | Bits | 17424 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 24 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | CBs | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | CBs | 2 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 | CBs | 3 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=80,81 | Bits | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 24420 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 36564 |  |  |  |  |
| Max. Throughput averaged over 160 slots | Mbps | 23.675 |  |  |  |  |
| Note 1: Slot i is slot index per 160 slots |  |  |  |  |  |  |

A.3.2.2.11	Reference measurement channels for FR1+FR2-2 CA with 120 kHz SCS (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.11-1.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 9 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 13 |  |  |  |  |
| Allocated slots per 40 slots |  | 8 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 17 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.43 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 12 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 15112 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 | Bits | 23568 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 24 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | CBs | N/A |  |  |  |  |
| For Slot i=80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | CBs | 2 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 | CBs | 3 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=80,81 | Bits | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 36630 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 54846 |  |  |  |  |
| Max. Throughput averaged over 160 slots | Mbps | 31.970 |  |  |  |  |
| Note 1: Slot i is slot index per 160 slots |  |  |  |  |  |  |

A.3.2.2.12	Reference measurement channels for FR1+FR2-2 CA with 480 kHz SCS (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.12-1.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 400 |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 160= {14,15,16,17,18,19,21,…,159} |  | N/A |  |  |  |  |
| For Slot i=320,321 |  | N/A |  |  |  |  |
| For Slots i= 13 |  | 11 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,12,20} and i≠ 320,321 |  | 13 |  |  |  |  |
| Allocated slots per 640 slots |  | 58 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 160= {14,15,16,17,18,19,21,…,159} |  | N/A |  |  |  |  |
| For Slot i=320,321 |  | N/A |  |  |  |  |
| For Slots i= 13 |  | 12 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,12,20} and i≠ 320,321 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 160= {14,15,16,17,18,19,21,…,159} | Bits | N/A |  |  |  |  |
| For Slot i=320,321 | Bits | N/A |  |  |  |  |
| For Slots i= 13 | Bits | 4480 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,12,20} and i≠ 320,321 |  | 5504 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 160= {14,15,16,17,18,19,21,…,159} | Bits | N/A |  |  |  |  |
| For Slot i=320,321 | Bits | N/A |  |  |  |  |
| For Slots i= 13 | Bits | 24 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,12,20} and i≠ 320,321 |  | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 160= {14,15,16,17,18,19,21,…,159} | CBs | N/A |  |  |  |  |
| For Slot i=320,321 | CBs | N/A |  |  |  |  |
| For Slots i= 13 | CBs | 1 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,12,20} and i≠ 320,321 | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 160= {14,15,16,17,18,19,21,…,159} | Bits | N/A |  |  |  |  |
| For Slot i=320,321 | Bits | N/A |  |  |  |  |
| For Slots i= 13 | Bits | 15048 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,12,20} and i≠ 320,321 |  | 18216 |  |  |  |  |
| Max. Throughput averaged over 40 slots | Mbps | 4.077 |  |  |  |  |
| Note 1: Slot i is slot index per 640 slots |  |  |  |  |  |  |

A.3.2.2.13	Reference measurement channels for FR1+FR2-2 CA with 480 kHz SCS (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.13-1.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 400 |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |
| Allocated resource blocks | PRBs | 20 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 160= {,15,16,17,18,19,21,…,159} |  | N/A |  |  |  |  |
| For Slot i=320,321 |  | N/A |  |  |  |  |
| For Slots i= 14 |  | 11 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,13,20} and i≠ 320,321 |  | 12 |  |  |  |  |
| Allocated slots per 640 slots |  | 58 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 160= {,15,16,17,18,19,21,…,159} |  | N/A |  |  |  |  |
| For Slot i=320,321 |  | N/A |  |  |  |  |
| For Slots i= 14 |  | 12 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,13,20} and i≠ 320,321 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 160= {,15,16,17,18,19,21,…,159} | Bits | N/A |  |  |  |  |
| For Slot i=320,321 | Bits | N/A |  |  |  |  |
| For Slots i= 14 | Bits | 3904 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,13,20} and i≠ 320,321 |  | 4736 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 160= {,15,16,17,18,19,21,…,159} | Bits | N/A |  |  |  |  |
| For Slot i=320,321 | Bits | N/A |  |  |  |  |
| For Slots i= 14 | Bits | 24 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,13,20} and i≠ 320,321 |  | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 160= {15,16,17,18,19,21,…,159} | CBs | N/A |  |  |  |  |
| For Slot i=320,321 | CBs | N/A |  |  |  |  |
| For Slots i= 14 | CBs | 1 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,13,20} and i≠ 320,321 | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 160= {,15,16,17,18,19,21,…,159} | Bits | N/A |  |  |  |  |
| For Slot i=320,321 | Bits | N/A |  |  |  |  |
| For Slots i= 14 | Bits | 8160 |  |  |  |  |
| For Slot i mod 160= {0,1,2,…,13,20} and i≠ 320,321 |  | 10080 |  |  |  |  |
| Max. Throughput averaged over 40 slots | Mbps | 3.510 |  |  |  |  |
| Note 1: Slot i is slot index per 640 slots |  |  |  |  |  |  |

A.3.3	Reference measurement channels for PDCCH performance requirements

A.3.3.2	TDD

<Unchangged skipped>

A.3.3.2.6	Reference measurement channels for SCS 480 kHz FR2-2

Table A.3.3.2.6-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.6-1.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 60 |  |  |  |  |  |
| CORESET time domain allocation |  | 1 |  |  |  |  |  |
| Aggregation level |  | 8 |  |  |  |  |  |
| DCI Format |  | 1_1 |  |  |  |  |  |
| Payload (without CRC) | Bits | 56 |  |  |  |  |  |

Table A.3.3.2.6-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.6-2.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 60 |  |  |  |  |  |
| CORESET time domain allocation |  | 2 |  |  |  |  |  |
| Aggregation level |  | 16 |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |
| Payload (without CRC) | Bits | 40 |  |  |  |  |  |

Table A.3.2.2.8-2: PDSCH Reference Channel for TDD UL-DL pattern FR2.480-1 (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.8-2.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 400 |  |  |  |  |
| Subcarrier spacing | kHz | 480 |  |  |  |  |
| Allocated resource blocks | PRBs | 20 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | N/A |  |  |  |  |
| For Slots i = 320, 321 |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | 11 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 477 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} |  | N/A |  |  |  |  |
| For Slots i = 320, 321 |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} |  | 12 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 3904 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 4736 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 24 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | CBs | N/A |  |  |  |  |
| For Slots i = 320, 321 | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | CBs | 1 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 20) = {15,16,17,18,19} for i from {0,…,639} | Bits | N/A |  |  |  |  |
| For Slots i = 320, 321 | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 20) = 14 for i from {0,…, 639} | Bits | 8160 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1…13} for i from {1,…,639} | Bits | 10080 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 111.6224 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |