3GPP TSG-RAN WG4 Meeting # 107	R4-2308865

# Incheon, KR, May 22 – May 26, 2023

Title: 	TP on 38.878: Introduction of parameters for link level evaluation

Source: 	Huawei, HiSilicon

Agenda item:	9.19.1

Document for:	Approval

- Background

As per [1], many simulation assumptions for link level evaluation have been agreed. It’s time to submit the section on parameters for link level evaluation.

Advanced receiver to cancel inter-user interference for MU-MIMO

Scenario and interference modelling

Receiver structure

Analysis on the required information

Link performance characterization

4.4.1	 Parameters for link level evaluation

Parameters captured in Table 4.4-1 are used to study performance of different receiver types assuming every parameter of co-scheduled UE is known to target UE.

Table 4.4-1: Parameters for receiver type performance study purpose

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | Rank 1+1:13 Rank 2+2: 13,17 | Rank 1+1: QPSK Rank 2+2: QPSK and 64QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | Scrambling ID |  | Same scrambling ID for both UEs |  |
| MIMO configuration |  |  | For Rank 1+1: 2T2R ULA Medium For Rank 2+2: 4T4R ULA Low |  |
| Number of CDM groups without data |  |  | 1 for paired UE allocated in same CDM groups and 2 for paired UE allocated in different CDM groups |  |
| HARQ process number |  |  | 4 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | For Rank 2+2: Select the precoding matrix to ensure orthogonality with target UE For Rank 1+1: Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
|  | Frequency domain allocation |  | Full bandwidth allocation |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | For Rank 1+1: TDLC300-100 For Rank 2+2:TDLA30-10 |  |
| Receiver type |  |  | MMSE-IRC,E-MMSE-IRC, R-ML | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

Parameters captured in Table 4.4-2 are used to study the performance of blind detection on modulation order of co-scheduled UE assuming every parameter of co-scheduled UE is known to target UE except for modulation order. To reduce the blind detection complexity, target UE assumes all interference layers have same modulation in each PRB.

Table 4.4-2: Parameters for modulation order blind detection study purpose

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | For Rank 1+1: 13 For Rank 2+2 and Rank 1+2: 17 | Rank 1+1: QPSK and 16QAM For Rank 2+2 and Rank 1+2: 16QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 1 | 2 |
|  |  |  | 2 | 2 |
|  | Scrambling ID |  | Same scrambling ID for both UEs |  |
| MIMO configuration |  |  | For Rank 1+1: 2T2R Medium For Rank 1+2: 4T4R MediumA For Rank 2+2: 4T4R Low |  |
| Number of CDM groups without data |  |  | 1 for paired UE allocated in same CDM groups and 2 for paired UE allocated in different CDM groups |  |
| HARQ process number |  |  | 4 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | For Rank 2+2 and Rank 1+2: Select the precoding matrix to ensure orthogonality with target UE For Rank 1+1: Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
|  | Frequency domain allocation |  | Full bandwidth allocation |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | For Rank 1+1: TDLC300-100 For Rank 2+2 and Rank 1+2:TDLA30-10 |  |
| Receiver type |  |  | R-ML | N/A |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |

Parameters captured in Table 4.4-3 are used to study the performance of blind detection on DMRS port and frequency domain resource allocation of co-scheduled UE assuming every parameter of co-scheduled UE is known to target UE except for DMRS port and frequency domain resource allocation

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 |  |  |
| Duplex mode |  |  | FDD |  |  |
| MCS |  |  | 13 | Rank 1+1: QPSK Rank 2+2: 64QAM |  |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |  |
|  |  |  | 2 | 2 |  |
|  | Scrambling ID |  | Same scrambling ID for both UEs |  |  |
| MIMO configuration |  |  | For Rank 1+1: 2T2R Medium For Rank 2+2: 4T4R Low |  |  |
| Number of CDM groups without data |  |  | 1 for paired UE allocated in same CDM groups and 2 for paired UE allocated in different CDM groups |  |  |
| HARQ process number |  |  | 4 |  |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | For Rank 2+2: Select the precoding matrix to ensure orthogonality with target UE For Rank 1+1: Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |  |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | Frequency domain allocation |  | Full bandwidth allocation |  | Full bandwidth allocation, partial bandwidth allocation PDSCH is scheduled from RB0 to RB24 |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Propagation conditions |  |  | For Rank 1+1: TDLC300-100 For Rank 2+2:TDLA30-10 |  |  |
| Receiver type |  |  | E-MMSE-IRC, R-ML | N/A |  |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |  |

4.4.2	 Link level simulation results

4.4.3	 Summary of link level evaluation

Conclusions

Reference

- R4-2305914 WF on advanced receiver for MU-MIMO scenario. China Telecom.