3GPP TSG-RAN WG4 Meeting # 107	R4-2308873

# Incheon, KR, May 22 – May 26, 2023

Title: 	Simulation results on 8Rx CQI requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.5.3.1.4

Document for:	Discussion

Background

As per [1], in this paper we provide our simulation results on 8Rx CQI requirements.

Simulation results

The simulation assumptions are captured in Table 2-1:

Table 2-1: Proposed simulation assumptions for 8RX CQI test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-i1CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | The codebook is restricted to i1,1=0, i1,2=0,i1.3=0 and i2=0 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | TBD |  |  |  |

The median CQI distribution is shown in Figure 2-1:

Figure 2-1: Median CQI distribution simulation results

BLER statistics are captured in Table 2-2:

Table 2-2: BLER statistics

| SNR | Median CQI | BLER of median CQI | BLER of median CQI-1 | BLER of median CQI+1 |
| --- | --- | --- | --- | --- |
| 0 | 3 | 1 | 0.05 | 1 |
| 1 | 3 | 1 | 0 | 1 |
| 2 | 4 | 0.98 | 0 | 1 |
| 3 | 4 | 1 | 0 | 1 |
| 4 | 5 | 1 | 0 | 1 |
| 5 | 5 | 0.92 | 0 | 1 |
| 6 | 6 | 0.95 | 0 | 1 |
| 7 | 6 | 1 | 0 | 1 |
| 8 | 7 | 1 | 0 | 1 |
| 9 | 7 | 1 | 0 | 1 |
| 10 | 8 | 0.99 | 0 | 1 |
| 11 | 8 | 1 | 0.05 | 1 |
| 12 | 9 | 0.98 | 0 | 1 |
| 13 | 9 | 1 | 0 | 1 |
| 14 | 10 | 1 | 0 | 1 |
| 15 | 10 | 1 | 0.02 | 1 |
| 16 | 11 | 1 | 0 | 1 |
| 17 | 11 | 1 | 0 | 1 |
| 18 | 12 | 1 | 0.03 | 1 |
| 19 | 13 | 1 | 0 | 1 |

Conclusion

In this contribution we provide our simulation results for 8Rx CQI requirements

- 4   Reference

- R4-2302942 WF for 8Rx UE performance requirements, Huawei, HiSilicon