3GPP TSG-RAN WG4 Meeting #107	R4-2308896

# Incheon, 22nd - 26th May, 2023

Title: 	Simulation results on UE demodulation requirements for LTE NTN IOT

Source: 	Huawei, HiSilicon

Agenda item:	9.7.7.1

Document for:	Information

Background

During RAN4#106-bis-e meeting, WF [1] on IoT-NTN UE demodulation and CQI reporting requirements was approved. In this contribution, we provide our updated simulation results about UE demodulation requirements for LTE NTN IOT.

Simulation

Test cases for Cat-M1

The ideal simulation results for the test cases for Cat-M1 are shown as Table 2.1-1 based on the simulation assumption in [1].

Table 2.1-1 Ideal simulation results for the test cases for Cat-M1

| Test number | CBW | MCS | Transmission mode | PDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR@70% maximum throughput [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 1.4MHz | 16QAM 1/2 | TM1 | 1 | NTN-TDLC5-30 | 1x1 | 8ms | 9.45 |
| 2 | FDD 1.4MHz | QPSK 1/3 | TM1 | 8 | NTN-TDLA100-200 | 1x1 | 8ms | -6.49 |
| 3 | FDD 1.4MHz | QPSK 1/10 | TM1 | 64 | NTN-TDLA100-10 | 1x1 | 8ms | -14.49 |

Test cases for NB-IoT

The ideal simulation results for the test cases for NB-IoT are shown as Table 2.2-1 based on the simulation assumption in [1].

Table 2.2-1 Ideal simulation results for the test cases for NB-IoT

| Test number | CBW | Carrier type | MCS | NPDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR@70% maximum throughput [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 200kHz | Anchor | QPSK 1/2 (Reuse R.NB6.FDD) | 32 | NTN-TDLC5-200 | 1x1 | 8ms | -6.68 |
| 2 | FDD 200kHz | Non-anchor | QPSK 1/3 (Reuse R.NB6-1.FDD) | 128 | NTN-TDLA100-10 | 1x1 | 8ms | -14.32 |

Summary

In this contribution, we provide our updated simulation results about UE demodulation requirements for LTE NTN IOT.

Reference

R4-2305960, WF on IoT-NTN UE demodulation and CQI reporting requirements, RAN4#106-bis-e, MediaTek